3GPP TSG RAN WG1 #90bis	 R1-1718698

# Prague, Czech Republic, 9-13 Oct 2017

Source:	Motorola Mobility, Lenovo

Title:	Discussion on higher rank Type II codebook and feedback overhead reduction

Agenda Item:	7.2.2.6

Document for:	Discussion and Decision

1 Introduction

In the RAN1 #89 meeting in Hangzhou, Type II channel state information (CSI) codebook was agreed for supporting full-dimension (FD) multiple-input multiple-output (MIMO) as follows [1,2].

Agreements: - Slides 4 to 24 in R1-1709232 are agreed - For slide 20, FFS whether or not support frequency-dependent parameterization and if so, the details - FFS whether or not to further enhance analog beamforming related operations especially for >1 layers

In this contribution, we discuss Type II CSI feedback in a system with 4 Rx UEs.  Simulation results of such a system are presented both the current rank 1-2 codebook and a rank 1-4 extended version of this codebook. To reduce total feedback overhead, a framework for a high resolution CSI codebook which is suitable for higher rank transmission but which uses beam space compression is also presented.

2 Type II Codebook for Higher Rank CSI – Review

In the Type II CSI agreement in [1,2], a channel matrix consisting of MIMO channel matrices  for  total subcarriers

is compressed in WB such that

by exploiting a set of  selected discrete Fourier transform (DFT) beams, i.e., B∶=b1,⋯,bL∈CNtx×L. Note that Ntx is the number of transmit antenna ports and Nrx is the number of receive antenna ports for each polarization. Realistic MIMO channels are spatially correlated and the channel matrix consists of only a limited number of dominant (meaningful) propagation paths. The channel compression procedure based on  is effective for quantizing high resolution CSI because the set of selected DFT beams captures the characteristics of the MIMO channels. For the rest of this contribution, we refer to the compressed channel matrix as the beamspace channel matrix.

After constructing the beamspace channel matrix in WB, the 2L-dimensional right dominant singular vector of beamspace channel matrix

is quantized in each SB, s∈{1,⋯,S}, to generate a precoding vector for each transmission layer. Note that  denotes the number of SBs. Quantizing the 2L-dimensional vector in a layer and SB specific process increases the total feedback overhead required for SB CSI reporting in  proportion to the number of transmission layers and the number of SBs.

Fig. 1: Performance comparison of different CSI feedback schemes

Table 1: Extended Type II feedback overhead calculations

In order to fully exploit the benefits of multiuser (MU) FD-MIMO systems, it is necessary to support rank > 2 MU transmission based on high resolution CSI. One straightforward solution is to extend the Type II codebook based on the codebook design principle agreed in [1,2]. To compute rank 3 and rank 4 precoding vectors in the extended Type II codebook, we could use the same quantization approach as that for rank 1 and rank 2 precoding vectors in the current Type II codebook. To be more specific, 2L-dimensional vectors should be quantized in each SB for computing rank 3 and rank 4 precoding vectors.

It is expected that for extended rank > 2, Type II CSI will increase system performance gain considerably [3,4]. We thus present simulation results to verify data rate performance (bits/s/Hz/user) of the rank 1-4 Type II CSI in the 3D UMi scenario and in a 32 port (4,4,2) configuration. For system level simulations, we consider a FTP scenario with 50% target resource utilization (RU). The rest of the simulation parameters are listed in Table 2. In Fig. 1, average user packet throughputs (UPTs) are plotted for rank 1-2 Type II CSI and rank 1-4 extended Type II CSI. Note that 4 Rx antennas were used in both cases. These results indicate that significant performance gain, 14.8% in average UPT, and 9.5% in cell edge UPT, can be obtained from rank 3-4 multiplexing above that provided by improved receive spatial diversity and array gain provided by 2 additional UE Rx. Therefore if 4 Rx UE operation is to be supported, it is desirable that ranks 3 and 4 Type II codebooks be included in the specification.

We next summarize total feedback overhead required by the extended Type II CSI reporting. While Fig. 1 show gains for extending the Type II CSI codebook to include ranks 3 and 4, the associated feedback overhead of the extended Type II scheme is large as will be described below. As shown in Table 1, if we simply extend the Type II codebook, the total feedback overhead for rank 1-4 Type II SB CSI reporting would be twice that for rank 1-2 Type II SB CSI reporting. A simple extension of the current Type II codebook would cause an excessive burden on feedback links since feedback overhead would increase in proportion to the maximum transmission rank . It is desirable to investigate how feedback overhead can be reduced while retaining the increased throughput from ranks 3 and 4 multiplexing.

Fig. 2. Empirical cdf of normalized power in beam direction of selected DFT beams in WB

Observation 1: Extending Type II CSI to ranks 3 and 4 will increase system performance gains considerably at the cost of excessive total feedback overhead.

3 Empirical Study – Channel Gains Contained in Selected DFT beams

Before discussing a CSI design framework suitable for higher rank transmission, we first discuss distributions of channel gains contained in selected DFT beams. For verifying the channel gain distributions, we compute the normalized channel gains contained in each DFT beam such that

The cumulative distribution function (cdf) of the normalized channel gains are depicted in Fig. 2. It is shown that more than 83% of channel gains are contained in the beam directions of the first and second dominant DFT beams (with probability of 0.7) and more than 58% of channel gains are contained in a beam direction of first dominant DFT beam (with probability of 0.7). Most of channel gains are therefore contained in the first and second dominant beams and only small amount of channel gains are contained in the third and fourth dominant beams.

Observation 2: Most of channel gains are contained in beam directions of first and second dominant DFT beams and only small amount of channel gains are contained in beam directions of third and fourth dominant DFT beams.

4 Beam Space Channel Matrix Compressions

The Type II CSI codebook supporting higher rank transmission scenario should be designed carefully by considering limited feedback resources. The maximum number of transmission layers and the number of SBs are fixed in the system and therefore these system parameters cannot be controlled for the purpose of reducing CSI feedback overhead. To reduce feedback overhead, one possible solution is to modify the number of basis vectors. Reducing the number of basis vectors is in line with additionally compressing the beam space channel matrix in WB such that

where  denotes the channel compression matrix and  denotes the channel compression level.

Proposal 1: Study channel compression matrix  and channel compression levels   for developing the ranks 3 and 4 Type II CSI codebook.

Extending the block diagonal structure implicit in the Type II codebook to the channel compression matrix results in a block diagonal channel compression matrix  as follows:

Proposal 2: Study the following channel compression patterns for each transmission layer .

ㆍAlt 0: Compression sub-matrix is polarization specific such that

where Ara denotes the sub-matrix for each polarization a∈{h,v} and ala is the (unit norm) compression vector.

ㆍAlt 1: Compression sub-matrix is polarization common such that

In Alt 0, the beamspace matrix in each polarization is compressed separately to construct a better compression matrix, while this approach requires more feedback overhead compared to Alt 1, which compresses the beamspace matrix for both polarizations identically.

5 Conclusions

CSI design framework suitable for higher rank transmission is discussed for new radio (NR). The proposals and observations discussed in this contribution are summarized as follows.

Observation 1: Extending Type II CSI to ranks 3 and 4 will increase system performance gains considerably at the cost of excessive total feedback overhead.

Observation 2: Most of channel gains are contained in beam directions of first and second dominant DFT beams and only small amount of channel gains are contained in beam directions of third and fourth dominant DFT beams.

Proposal 1: Study channel compression matrix  and channel compression levels   for developing the ranks 3 and 4 Type II CSI codebook.

Proposal 2: Study the following channel compression patterns for each transmission layer .

ㆍAlt 0: Compression sub-matrix is polarization specific such that

where Ara denotes the sub-matrix for each polarization a∈{h,v} and ala is the (unit norm) compression vector.

ㆍAlt 1: Compression sub-matrix is polarization common such that

References

- R1-1709232, WF on Type I and II CSI codebooks, Samsung

- 3GPP, RAN1#89, Chairman’s Notes

- R1-1710674, On higher rank (rank 3 and 4) Type II CSI, Samsung

- R1-1710454, The remaining issues for Type II codebook, Huawei, HiSilicon

Appendix A: Simulation Assumptions

Table 2: Simulation Parameters

| Channel model | 3D UMi 200m ISD |
| --- | --- |
| eNB height | 10 m |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Oversampling (O1,O2) | (4,4) |
| eNB antenna configurations H,Vx(N1,N2,P) | 32 ports: 4,8x(4,4,2) with 1x2 virtualization, 108° tilt |
| UE antenna configurations | 4 UE antennas: (2,1,2), dv=0.8 λ 2 UE antennas: (1,1,2) |
| Antenna polarizations | eNB (+45°,-45°) UE (0°, 90°) |
| eNB antenna spacings (dh,dv) | (0.5,0.8)λ |
| eNB power | 41 dBm (UMi) |
| Cell layout | 57 cells |
| Traffic model | FTP Traffic Model 1 Non full buffer (50% Target RU, 500kB packet size) |
| Scheduling | MU, Proportional fair, Max 8 MU layers |
| HARQ | Max 4 retransmission |
| CSI periodicity and CSI delay | 5 ms |
| UE speed | 3 km/h |
| Target BLER | 0.1 |
| Receiver | MMSE – IRC |
| Transmission rank | 1 – 4 |