3GPP TSG RAN WG1 Meeting 90bis 		R1-1719025

Prague, CZ, 9th – 13th, October 2017

Source:	Qualcomm

Title:	Summary of Remaining Details on RACH Procedure

Agenda Item:	7.1.4.2

Document for:	Discussion and Decision

# Background

This document summarizes the contributions and offline discussions regarding remaining details on RACH procedure.  Following issues have been discussed:

- SS Block to PRACH transmission time/frequency and preamble index mapping

- Size of RAPID

- RA-RNTI calculation

- Threshold for SS Block Selection

- Spatial QCL relationship during RACH procedure

- Msg2/Msg3 formats and timeline

- Timing advance (TA) granularity and TA size

- Subcarrier spacing of PDCCH triggered contention free random access procedure

- CORESET monitoring during RACH procedure

- Msg3 transmission parameters

- Signaling of RACH resource configuration

- Other items regarding RAN2’s LS

# Offline Agreements on October 11

offline agreement:

- Let X be the number of PRACH transmission occasions in the RACH resource configuration period which are associated (by the SSB to RACH resource association rule) with the actually transmitted SS blocks in the SS burst set period.

  - X can be greater than, equal to or smaller than the number of actually transmitted SS blocks in the SS burst set

Suggested offline agreement:

- UE selects only one PRACH transmission occasion for Msg1 transmission before the expiration of RAR window

Offline agreement:

- NR supports both slot based PDCCH, PDSCH and PUSCH, and non-slot based PDSCH/PUSCH transmissions for Msg2/Msg3/Msg4 transmission

  - For the non-slot based transmission, 2, 4 and 7 OFDM-symbol durations for the PDSCH/PUSCH is supported

  - FFS the handling of PDCCH for non-slot based transmissions

  - FFS Time gap during RACH procedure applied to non-slot based transmissions

  - Note: Whether to support simultaneous uplink transmission of slot and non-slot based transmission from UE’s perspective will be finalized in the control channel session

Offline working assumption:

- Bit field length of RAPID is fixed in the spec.

- Bit field length of RAPID is 6 bits.

  - RAN1 is discussing if 8 bits should be considered for bit field length of RAPID

  - FFS: How RACH occasion is conveyed to UE

Offline conclusion

Maximum size of TA command is 11 bits.

- FFS: If size of TA command needs to increase to cover higher cell sizes or decrease for some PRACH formats.

- Objected by: Nokia. Nokia prefers 12 bits as the maximum size of TA command to support higher cell size.

# SS Block to RACH transmission occasion and preamble mapping

## SSB to RACH transmission occasion mapping

Mapping from SS block to RACH transmission time/frequency

- Alt 1: One to one mapping

- Alt 2: Many to one mapping

- Alt 3: One to many mapping

In alternative 1 and 2, RACH transmission occasion is confined to one time/frequency region based on the selected SS block. In alternative 3, UE has the flexibility to select one time/frequency region from multiple ones based on the selected SS block. Fig. 1-3 show these mapping scenarios. The figures have been drawn based on similar figures of [1].

Suggested offline working assumption from October 10, 2017:

- Let X be the smallest number of PRACH transmission occasions after which the associations between actually transmitted SS blocks in the SS burst set and PRACH transmission occasions are repeated.

  - In one to one mapping, X is equal to the number of actually transmitted SS blocks

  - In many to one mapping, X is less than the number of actually transmitted SS blocks

  - In one to many mapping, X is greater than the number of actually transmitted SS blocks

- Note: following definition of ‘RACH transmission occasion’ was agreed in RAN NR AH1

- “ For 4-step RACH procedure, a RACH transmission occasion is defined as the time-frequency resource on which a PRACH message 1 is transmitted using the configured PRACH preamble format with a single particular tx beam”

Figure 1: 1st example of one to one mapping between SS block to RACH transmission occasion

Figure 2: 2nd example of one to one mapping between SS block to RACH transmission occasion

Figure 3: 3rd example of one to one mapping between SS blocks to RACH transmission occasions

Figure 4: 4th example of one to one mapping between SS block to RACH transmission occasion

Figure 5: Many to one mapping between SS blocks to RACH transmission occasion

Figure 6: 1st example of one to many mapping between SS block to RACH transmission occasions

Figure 7: 2nd example of one to many mapping between SS block to RACH transmission occasions

Suggested agreement:

- At least, support one-to-one and many-to-one mapping between SS blocks and RACH transmission occasion within X PRACH transmission occasions.

  - Support one-to-one and many-to-one

    - Yes: Everyone present in offline session

  - FFS: SS Block to RACH preamble index mapping in scenarios where many-to-one mapping between SS blocks and RACH transmission occasion within X PRACH transmission occasions supported

  - FFS: One-to-many mapping between SS block and RACH transmission occasions within X PRACH transmission occasions

  - Support one-to-many

    - Yes: ETRI, NOKIA, Ericsson, Samsung, Motorola, Lenovo, Docomo, ZTE, Sanechip, Intel

    - No: Huawei, Hisilicon, Qualcomm, LGE, CATT

Offline agreements

- NR supports

  - Mapping between one SS block to only one PRACH transmission occasion in the RACH configuration

  - Mapping between many SS blocks to only one PRACH transmission occasion in the RACH configuration

  - Mapping between one SS block to many PRACH transmission occasions in the RACH configuration

  - Mapping between many SS blocks to many PRACH transmission occasions in the RACH configuration

Offline agreement:

- Original proposal from LGE:

- Let X be the number of PRACH transmission occasions in the RACH resource configuration which are associated (by the SSB to RACH resource association rule) with any one of the actually transmitted SS blocks in the SS burst set period.

  - In one to one mapping, X is equal to the number of actually transmitted SS blocks

  - In many to one mapping, X is less than the number of actually transmitted SS blocks

  - In one to many mapping, X is greater than the number of actually transmitted SS blocks

## SSB to RACH preamble mapping

Is SSB to RACH preamble index mapping unique in one-to-one mapping from SSB to RACH transmission occasion?

- Alt 1: Yes

- Alt 2: No

- Alt 3: Up to Gnb configuration.

Is SSB to RACH preamble index mapping unique in many-to-one mapping from SSB to RACH transmission occasion?

- Alt 1: Yes

- Alt 2: No

- Alt 3: Up to Gnb configuration

In alternative 1, RACH preamble index itself conveys the SS block index to gNB.

In one-to-one mapping, through alternative 2, RACH transmission time/frequency conveys the SSB to Gnb [3].  In many-to-one mapping, alternative 2 may be useful if gNB relies on digital beamforming and beam correspondence to obtain the DL Tx beam from the detected preamble [1].

Suggested offline agreement: Down-select from above options for both scenarios.

## Ordering of SSB to RACH transmission time/frequency/preamble mapping

SSB to RACH transmission time/frequency/preamble are mapped in the order of:

Alt 1 [4]:

- Increasing cyclic shifts of a root index

- Increasing root index

- Increasing time allocation

Alt 2 [3]:

- Increasing cyclic shifts of a root index

- Increasing root index

- Increasing subcarrier region

Alt 3 [5]:

Allow two different mapping. For one-to-one mapping between SSB and RACH transmission time/frequency, support:

- Increasing cyclic shifts of a root Zadoff-Chu sequence

- increasing root index

- increasing NR-RACH preamble time instances within a slot

- increasing slot index

Note: the last two steps assume that SSB to RACH preamble index mapping is unique in one-to-one scenario.

For many-to-one mapping between SSB and RACH transmission time/frequency, support:

- the available cyclic shifts of a root Zadoff-Chu sequence

- increasing NR-RACH preamble time instances within a slot

- increasing root index

- increasing frequency allocation

- increasing slot index

Alt 4 [7]:

- Increasing cyclic shifts within one root sequence

- Root sequence within one PRACH preamble format

- PRACH preamble formats within a RACH slot and the same PRB allocation

- PRB RACH allocations within a RACH slot

- RACH slots within the RACH slot pattern time interval

Several other alternatives have also been proposed in [1-20]. Suggestion is to discuss these alternatives after agreeing on 2.1 and 2.2.

# Size of RAPID

Support of variability regarding the size of RAPID

- Alt 1: Fixed in the spec

- Alt 2: Variable

Signaled by the network

Suggested offline agreement: Down-select from above options.

Offline working assumption:

- Bit field length of RAPID is fixed in the spec.

- Bit field length of RAPID is 6 bits.

  - RAN1 is discussing if 8 bits should be considered for bit field length of RAPID

  - FFS: How RACH occasion is conveyed to UE

If size of RAPID is fixed, what is the value of RAPID? Most companies prefer 6 bits. But this could also be related to RACH capacity enhancement discussion.

Suggested offline agreement: If RAPID size is fixed in the spec, support [6] bits for RAPID size.

# RA-RNTI calculation

RA-RNTI is calculated based on:

- Alt 1: SS block index associated with the RACH preamble in addition to slot index and frequency allocation in which the preamble was transmitted.

- Alt 2: Similar as in LTE.

  - RAPID contains starting slot index and symbol index of RACH transmission

- Alt 3: Similar to LTE. But the time index is defined based on starting slot number and subframe number of RACH transmission

  - RAPID contains starting symbol index of RACH transmission

- Alt 4: Similar to LTE. But the time index is defined based on the starting symbol, slot and subframe number of RACH transmission.

- Alt 5: Similar to LTE. But the time index is defined based on RACH occasion index in the radio frame [6]

- Alt 6:

  - SS block index where SSB index in expressed in an ascending order of actually transmitted SS block associated with the specified RACH occasion.

  - Time index is expressed in an ascending order of the specified RACH occasion in time domain.

  - Frequency index is expressed in an ascending order of the specified RACH occasion in frequency domain.

Comment: RAN2 has agreed in the last meeting that the calculation of RA-RNTI will be decided in RAN2. RAN1 can discuss if down-selection among above alternatives should be decided in RAN1.

## Dependency of RA-RNTI on UL carrier index

When more than one UL carriers are available, whether NR RA-RNTI should be a function of UL carrier index? [9] supports this relationship between NR RA-RNTI and UL carrier index.

Note that, the following agreement happened in the previous meeting [9]:

- For NR UE initial access based on RACH configuration for an SUL carrier

  - RACH configuration for the SUL carrier is broadcasted in RMSI

  - The configuration information for the SUL carrier is sufficient for UEs to complete RACH procedure via only that SUL carrier

    - In particular the configuration information includes all necessary power control parameters

  - The configuration information for the SUL carrier includes a threshold. The UE selects that SUL carrier for initial access if and only if the RSRP measured by the UE on the DL carrier where the UE receives RMSI is lower than the threshold

- If the UE starts its RACH procedure on the SUL carrier, then the RACH procedure is completed with all uplink transmission taking place on that carrier

Comment: Discuss if the dependency of UL carrier index on RA-RNTI needs to be defined in RAN1 or not? If yes, discuss if this dependency is supported.

# Threshold for SS Block selection

## Nature of RSRP threshold

Nature of RSRP threshold (whether it is absolute RSRP or relative RSRP with respect to the previously selected SS block) for SS Block selection for RACH (re)transmission during initial access

- Alt 1: Absolute

    - Supported by: Qualcomm, Docomo [8], Huawei [9],

- Alt 2: Relative

    - Supported by: LGE

Note that, in the last meeting, we made the following agreement,

“For initial access, threshold for SS block selection for RACH resource association is configurable by network, where the threshold is based on RSRP

  - FFS details, including ping-pong effect handling”

Suggested offline agreement: “UE can select an SS block with a filtered RSRP above the threshold for Msg1 (re)transmission.”

## Need for additional restrictions on SS Block Selection

[7] has mentioned the following for SS block selection, “Restrict RACH resource selection to ones that are associated to SS blocks that fulfil the cell (re-) selection criteria or threshold.”

Comment: Discuss these restrictions further.

## Threshold for SS block/CSI-RS Selection during Handover

The agreement on the use of RSRP for SS block selection during the last meeting focused on initial access. [13] proposes that this should be extended to handover scenarios as well.

Suggested offline agreement: “For handover scenarios, threshold for SS block/CSI-RS selection for both CBRA/CFRA resource association is configurable by network, where the threshold is based on RSRP”

## Threshold for SSB/CSI-RS Selection on SUL or common UL

Since supplemental uplink is agreed, there are proposals to configure RSRP threshold and indicated via handover command for CFRA for selecting SSB/CSI-RS RACH on either SUL or common UL [13].

Comment: Discuss above proposal.

# Spatial QCL Relationship during RACH Procedure

RAN1 made the following conclusion in the last meeting:

- FFS Message 2 PDCCH/PDSCH is received by the UE assuming that the PDCCH/PDSCH DMRS conveying message 2 is QCL'ed with the SS block which the preamble/RACH occasion the UE sent is associated to

- FFS Message 3 is transmitted by the UE assuming that the same Rx beam as was used for PRACH preamble reception by gNB to which the received RAR is associated to

- FFS If there is no beam reporting in RACH message 3, Message 4 PDCCH/PDSCH is received by the UE assuming that the PDCCH/PDSCH DMRS conveying message 4 is QCL'ed with that of Msg 2

  - FFS: If there is beam reporting in RACH message 3

  - FFS: If and how beam reporting in RACH message 3 impacts message 4 Tx QCL assumption

## Spatial QCL Relationship during Msg2

- Alt 1: UE may assume that the PDCCH DMRS and PDSCH DMRS conveying Msg2 are QCL'ed with the SS block which is associated with the preamble/RACH occasion which the UE sent

    - Supported by: Nokia, Intel, Qualcomm, Samsung

- Alt 2: NR should support QCL association of RAR signals with suitable subsets of SSB transmissions during an SSB burst

    - Supported by: Ericsson

- Suggested offline agreement: UE may assume that the PDCCH DMRS and PDSCH DMRS conveying Msg2 are QCL'ed with the SS block which is associated with the preamble/RACH occasion that the UE sent

## Spatial QCL relationship during Msg3

Two versions of a similar proposal were proposed by companies.

- DMRS of PUSCH transmitting Msg3 is QCLed with PRACH preamble for which the UE received RAR

    - Mentioned by Nokia

- Msg-3 is transmitted by the UE assuming that the same Rx beam as was used for PRACH preamble reception by gNB to which the received RAR is associated to

    - Same as the conclusion during the last meeting and mentioned by Intel [10]

Suggested offline agreement: The DMRS of PUSCH transmitting Msg3 is QCLed with the PRACH preamble for which the UE received RAR

## Spatial QCL relationship during Msg4

Following proposal appeared in [8]

- UE may assume that the PDCCH DMRS and PDSCH DMRS conveying Msg2 are QCL'ed with the SS block which the preamble/RACH occasion the UE sent is associated to

However, several contributions supported beam reporting in Msg3 [3], [11], [12]; whereas several others did not support beam reporting in Msg3 [13]-[16]. Due to this, we suggest the following proposal:

Suggested offline agreement: If there is no beam reporting in RACH message 3, UE may assume that the PDCCH DMRS and PDSCH DMRS conveying Msg4 are QCL’ed with the SS block which is associated with the preamble/RACH occasion that the UE sent

    - FFS: If there is beam reporting in RACH message 3

    - FFS: If and how beam reporting in RACH message 3 impacts message 4 Tx QCL assumption

# Msg2/Msg3 Formats and Timeline

## Support of non-slots during RACH procedure

- Alt 1: Non-slot should not be supported in Rel-15

- Alt 2: Non-slot based PDCCH and PDSCH/PUSCH should be supported in NR

    - Mentioned by: Ericsson, AT&T

- Alt 3: NR supports non-slot based PDSCH/PUSCH but only slot based PDCCH transmission

    - Supported by: Qualcomm

- Alt 4: Non-slot based transmission of PDSCH/PUSCH with a minimum CORESET monitoring periodicity [125us] should be specified.

    - Supported by: Huawei

- Alt 5: NR supports slot based and non-slot based PDSCH/PUSCH transmission but only slot based PDCCH transmission during RACH procedure in Rel-15.

    - FFS: Timing gap between Msg2 reception and Msg3 transmission with non-slot transmission should not be shorter than that with slot based transmission.

    - HARQ timing applied to non-slot based transmissions should not be shorter than that for slot based transmissions.

    - Non-simultaneous transmission of slot and non-slot based transmission from UE’s perspective

    - Supported by: Mediatek

Suggested offline agreement:

- NR supports both slot based PDCCH, PDSCH and PUSCH, and non-slot based PDSCH/PUSCH transmissions for Msg2/Msg3/Msg4 transmission

  - For the non-slot based transmission, 2, 4 and 7 OFDM-symbol durations for the PDSCH/PUSCH is supported

  - FFS the handling of PDCCH for non-slot based transmissions

  - FFS HARQ timing applied to non-slot based transmissions

  - should not be shorter than that for slot based transmissions.

  - Note: Whether to support simultaneous uplink transmission of slot and non-slot based transmission from UE’s perspective will be finalized in the control channel session

NR supports slot based and non-slot based PDSCH/PUSCH transmission but only slot based PDCCH transmission during RACH procedure in Rel-15.

    - FFS: Timing gap between Msg2 reception and Msg3 transmission with non-slot transmission should not be shorter than that with slot based transmission.

    - HARQ timing applied to non-slot based transmissions should not be shorter than that for slot based transmissions.

    - Note: Whether to support simultaneous uplink transmission of slot and non-slot based transmission from UE’s perspective will be finalized in the control channel session

## Timeline between Msg2 and Msg3

Following aspects were mentioned regarding time gap between Msg2 and Msg3 in different contributions

- NR supports multiple minimum time gap between Msg2 and Msg3 for different service types [9]

- NR shall discuss whether the maximum TA value among all preamble formats is taken into account when specifying the minimum timing gap between Msg2 reception and Msg3 transmission [6]

- NR supports the gap (expressed in number of slots) between Mg2 reception and Msg3 transmission to depend on numerology [3]

Suggestion is to discuss the minimum time gap between Msg2 and Msg3 after agreeing on other key features. If time does not permit, this should be decided in the next meeting.

# Timing Advance

## Timing Advance Granularity

Dependency on the granularity of timing advance on other parameters.

- Alt 1: Depends on tone spacing of Msg3

  - Supported by: CATT, Ericsson, LGE,

- Alt 2: Depends on RACH preamble format

  - Supported by: Samsung

- Alt 3: Depends on tone spacings of Msg1

  - Supported by: Nokia, Huawei

- Alt 4: Does not depend on tone spacing and/or RACH preamble format

Suggested proposal: Down-select among above options

Note that timing accuracy can be worse than timing granularity.

## Size of TA Command

- Alt 1: 12 bits

    - Objective is to increase supportable cell size (proposed by Nokia)

- Alt 2: 11 bits

    - Supported by: Samsung, [CATT]

    - CATT mentions that RAN1 should discuss if there is a need to support higher cell sizes and comments higher cell size can be supported by changing TA granularity, too

- Alt 3: 5 bits for PRACH format A0, A1 and B1, and 11 bits for remaining NR-PRACH formats

    - Supported by: Ericsson

Suggested offline working assumption:

Size of TA command is 11 bits.

- FFS: If size of TA command needs to increase to cover higher cell sizes or decrease for some PRACH formats.

# Subcarrier spacing of PDCCH triggered CFRA procedure

Subcarrier spacing of Msg1 of PDCCH triggered CFRA.

- Configured in the RACH configuration.

  - Supported by: Intel, CATT

    - Suggested offline agreement: SCS of Msg1 of PDCCH triggered CFRA is configured in the RACH configuration.

Subcarrier spacing of Msg2 of PDCCH triggered CFRA

- Alt 1: Configured in the RACH configuration

    - Supported by: Intel

- Alt 2: The same as the SCS of the PDCCH order

    - Supported by: CATT

- Alt 3: Configured in the PDCCH order

    - Supported by: CATT

Suggested offline agreement: Down-select from above three options

# CORESET monitoring during RACH procedure

## CORESET for monitoring RAR

- Alt 1: CORESET for monitoring RAR is same as that of RMSI

- Alt 2: CORESET for monitoring RAR is same as that of RMSI apart from CORESET periodicity of RAR

  - CORESET periodicity of RAR may need to be configured in RMSI

- Alt 3: CORESET for monitoring RAR is different from that of RMSI

  - Note: This is especially applicable if summation of SSB and RMSI CORESET’s BW are not confined within minimum UE bandwidth but if some companies prefer this restriction for RAR CORESET

Suggested offline agreement: Down-select from above options.

## CORESET for monitoring the PDCCH for Msg3 retransmission

- Alt 1: Same as the CORESET used for RAR

- Alt 2: CORESET is configured in RAR

Suggested offline agreement: CORESET for monitoring the PDCCH for Msg3 (re)transmission is same as the RAR CORESET.

## CORESET for monitoring the Msg4 reception

- Alt 1: Same as the CORESET used for RAR

- Alt 2: CORESET can be configured differently from RAR and from the PDCCH for Msg3 (re)transmission

- Alt 3: CORESET is same as the PDCCH for Msg3 (re)transmission and but can be configured differently from RAR

Suggested offline agreement: Down-select from above options.

# Msg3 Transmission Parameters

## HARQ Retransmission of Msg3

Following proposal was mentioned in [7], “NR supports asynchronous adaptive HARQ retransmissions for RACH message 3.”

Suggested offline agreement: NR supports asynchronous adaptive HARQ retransmissions for RACH message 3.

Both [4] and [14] mentioned that that the HARQ process ID for Msg3 PUSCH retransmission can be fixed in the spec. [4] further proposed to fix the HARQ process ID as 0 index.

Suggested offline agreement:

HARQ process ID for Msg3 PUSCH retransmission is fixed in the spec.

- HARQ process ID for Msg3 PUSCH retransmission is fixed as 0 index.

# Signaling of RACH Resource Configuration

- Alt 1: Support only semi-static signaling of RACH resource configuration

- Alt 2: Support dynamic indication signaling of RACH time-frequency resource configuration in addition to semi-static signaling [21]

Suggested offline agreement: Down-select from above options.

# Other Items regarding reply to RAN2’s LS [20]

## Reply regarding size of Uplink Grant Field

Q2 in RAN2 LS: What is the size of the Uplink grant field?

[14] proposes the following answer, “RAN1 is still discussing resource allocation in general, so it is FFS for the size of the uplink grant field.”

Suggested offline agreement: confirm above reply.

## Reply regarding Temporary C-RNTI

Q4 in RAN2 LS: What is RAN1's preferred size of the Temporary C-RNTI?

[14] proposes the following answer: It is RAN1 understanding that network identifiers are determined by RAN2 and the temporary C-RNTI is no different. RAN1 considers [16] bits should be sufficient for the temporary C-RNTI.

Suggested offline agreement: confirm above reply.

## Reply regarding Additional Field

Q6 in RAN2 LS: Has RAN1 agreed to add any additional fields (in RAR)?

[14] proposes the following answer: No. RAN1 has not agreed so far on the need for any additional fields.

Suggested offline agreement: RAN1 is currently discussing need for additional fields but has not agreed to it yet.

# References

- R1-1717036, “Remaining details of RACH Procedure”, ZTE, Sanechips.

- R1- 1718716, “Remaining details of RACH Procedure”,  Ericsson.

- R1-1718532, “PRACH procedure considerations”, Qualcomm.

- R1-1717931, “RACH Procedure”,  LGE Electronics.

- R1-1718715, “Remaining details on PRACH Formats”, Ericsson

- R1-1718340, “Remaining details on RACH Procedure”, Mediatek

- R1-1718303, “Remaining details on RACH Procedure”, Nokia

- R1-1718184, “Discussion on Remaining Details on RACH Procedure”, NTT Docomo

- R1-1717051, “Remaining Issues in RACH Procedure”, Huawei, HiSilicon

- R1-1717358, “NR RACH Procedures”, Intel

- R1-1718665, “Considerations on RACH Procedure”, Sony

- R1-1718381, “Remaining details of RACH Procedure and RACH Configuration”, AT&T

- R1-1717582, “Remaining details on RACH Procedure”, Samsung

- R1-1717803, “Further details on NR 4 Step RACH Procedure”, CATT

- R1-1718052, “Discussion on NR 4-Step Random Access Procedure” Guangdong OPPO Mobile Telecom.

- R1-1718004, “Remaining details on RACH procedure”, ETRI

- R1-1717715, “Discussion on RACH Procedure”, Fujitsu

- R1-1718314, “Numerology for Msg2 and Msg4 for RRC_CONNECTED UEs”, Panasonic.

- R1-1718359, “RACH configuration of Multiple Msg1 transmissions before the end of a monitored RAR window”, Interdigital

- R1-1715353, “LS on RACH agreements”, RAN2

- R1-1718696, “Remaining details on random access procedure”, Motorola Mobility, Lenovo.