3GPP TSG RAN WG1 #90b                                                    	R1-1719072

Prague, Czech, 9th – 13th October 2017

Source:	vivo

Title:	Summary of Offline Discussions on DL PRB Bundling

Agenda Item:	7.2.1.4

Document for:	Discussion

Introduction

Agreement in RAN1#AH2

For DL data transmission:

- PRB bundling size values include

  - Case 1: one or more values down-selected from the following set

    - {[1], 2, 4, 8 and 16};

    - FFS the relationship with RBG size;

  - Case 2: values equal to consecutively scheduled bandwidth in frequency;

- For UE-specific PRB bundling size indication, support dynamically indicated PRB bundling size with up to 1 bit overhead;

  - FFS implicit indication to reduce configuration overhead, e.g., based on DMRS configuration etc;

  - FFS the usage of above 1 bit, e.g. whether to switch between Case 1 and Case 2 or between two configured Case 1 values;

  - FFS other aspects related to MU-MIMO pairing and higher-layer signaling

Agreements: in RAN1#90

- PRB bundle is based on absolute PRB-grid of a component carrier

Agreements: in RAN1#90

- For DL unicast data transmission:

  - Case 1 PRB bundling size values are at least 2 and 4

    - FFS whether or not to additionally support PRB bundling size 1 – companies are encouraged to perform analysis and evaluations especially w.r.t. PRB bundling sizes 2 and 4

- FFS: PRG configuration for broadcast PDSCH

Agreement in RAN1#AH3

The PRB bundle defined in the absolute PRB grid of the wideband CC from network perspective is aligned with RBG boundary

Agreement: in RAN1#AH3

- Support the 1 bit DCI indication for PRB bundling size

  - Dynamic PRB bundling is part of UE capability signalling discussion;

    - FFS details;

  - If UE does not support dynamic PRB bundling, then only one PRB bundling size is higher layer configured;

  - FFS the detailed usage of the 1 bit;

Key issue 1: PRB bundling size

About values of PRB bundling size, the major consideration includes bundled processing gains, scheduling flexibility (MU-MIMO, CoMP), possible beamforming gains and precoders cycling performance gains. There are the following issues:

  - Against the value of 1 (Huawei, HiSilicon, vivo, Samsung, Qualcomm)

- Support the value of 1 (Intel, ZTE, Sanechips, CATT, LG, E///)

  - Support of 8 and 16 (Samsung)

  - Maximum should not exceed 8 (IDC)

Conclusion: FFS PRB bundling size value of 1 until RAN1 #91;

Key issue 2: usage of the 1 bit

The following usage of the 1 DCI bit is proposed:

  - Switch between case 1 and case 2; (Huawei, HiSilicon, LG)

  - Flexibly used to switch between configured values; (ZTE, Sanechips, vivo, Samsung, E///, Nokia)

    - Support using 1 bit to switch between Case 1 values only if the UE may assume that the precoding granularity of co-scheduled ports (in MU-MIMO) and their resource allocation is aligned with the indicated PRG (otherwise support switching only between case 1 and case 2) (Qualcomm)

  - Switch between an explicit value and the implicit rule; (IDC)

Offline agreement:

- When 1-bit DCI field is present, the following configuration is supported

  - Candidate values: {[1], 2, 4, scheduled BW}

  - The DCI bit field indicated “1” : select one value from one or two RRC configured candidate values

    - When two candidate values are configured, one value is implicitly determined

      - FFS details of implicit determination (e.g., scheduled BW, RBG-based, subband size, PDCCH REG bundling size, etc.)

  - The DCI bit field indicated “0”: select one RRC configured candidate value

  - When a UE is configured with RBG=2, the UE is not expected to be configured with PRG=4

- Example of further configuration when 1-bit DCI is configured to be present by RRC

  - ({2}, {4})

    - The DCI bit field indicated “1” :  PRG size {2}

    - The DCI bit field indicated “0”: PRG size {4}

  - ({2, 4}, {scheduled BW})

    - The DCI bit field indicated “1” :  PRG size {2} or {4}, determined implicitly

    - The DCI bit field indicated “0”: scheduled BW

Key issue 3: Presence of the 1 bit DCI

The following ways of configuring the presence of the 1 bit DCI is proposed:

  - The presence of 1 bit in DCI for DL scheduling, i.e. the dynamic bundle switch, is configured by RRC signalling to the UE together with the two PRB bundle states 1 and 2. (E///, vivo, ZTE, Sanechips)

  - The 1 bit is always present in DCI and State 1 = State 2 = [Case 1: 2] by default, unless one or both states 1 and 2 are configured to the UE by RRC signalling

Offline agreement:

- Down select between two alternatives in RAN1 #90bis

  - Alt-1: 1-bit DCI field is always present when a UE has capability

  - Alt-2: 1-bit DCI field can be configured to be present by RRC

Key issue 4: PRB bundling for broadcast/groupcast channels

The following ways of determining broadcast/groupcast PDSCH PRB bundling size is discussed

- Specified as 2 all the time (Huawei, HiSilicon, E///, Nokia)

- Support {2 and consecutively scheduled PRBs } for broadcast/groupcast channels (vivo, Qualcomm)

  - Dependent on how the REG bundling size of corresponding PDCCH for groupcast/broadcast channel is determined. (vivo)

- Fixed to one value (LGE)

Key issue 5: Relationship between RBG and PRB bundling size

- The following views on relationship between RBG and PRB bundling size are expressed:

  - For case 1, if RBG is {1,2,4}, PRG is equal to RBG. If RBG is {8,16} then PRG is 4. (LGE)

  - For case 1, PRG can only be configured to be less or equal than RBG, i.e,, based on current agreements: if RBG=2, PRG is equal to 2, if RBG={4,8,16}, PRG can be equal to {2,4}. (Qualcomm)

Key issue 6: UE assisted PRB bundling size determination and configuration

- Supports UE-assisted PRB bundling size determination and configuration (Huawei, HiSilicon, CATT)

Key issue 7: Joint Configuration of PRB bundling size and BWP

The following possible ways of configuration are discussed:

  - Joint configuration of PRB bundling size and BWP (Huawei)

  - At least for the case that UE does not support dynamic PRB bundling switch, configuration of PRB bundling size depends on bandwidth of BWP (Samsung)

Key issue 8: VRB to PRB mapping granularity

The following possible ways of configuration are discussed:

  - If distributed VRB to PRB mapping is supported for Type 1 resource allocation, then for case 1 values, VRB to PRB mapping granularity is indicated PRB bundling size ; (vivo, Qualcomm)

Others

Intel proposes to reuse the signalling defined for common PRB indexing to align PRGs between UEs.

ZTE proposes layer/beam specific PRB bundling configuration;

E/// proposes to support time domain bundling size indication for multi-slot scheduling;

Qualcomm further proposes two options for case 2 values to handle disjoint cluster resource allocation.

WFs

- References

[1] R1-1717297	PRB bundling size for DL data precoding	Huawei, HiSilicon

[2] R1-1717364	On PRB bundling for DL	Intel Corporation

[3] R1-1717419	On PRB Bundling	ZTE, Sanechips

[4] R1-1717469	Discussion on PRB bundling for DL	vivo

[5] R1-1717597	PRB bundling for NR DMRS	Samsung

[6] R1-1717809	PRB bundling for DL transmission	CATT

[7] R1-1717938	Discussion on PRB bundling	LG Electronics

[8] R1-1718427	PRB bundling for DL	Ericsson

[9] R1-1718479	Discussion on Dynamic PRG Size Indication	InterDigital, Inc.

[10] R1-1718509	On remaining issues on PRB bundling	Nokia, Nokia Shanghai Bell

[11] R1-1718538	Discussion on PRB bundling for DL	Qualcomm Incorporated