3GPP TSG RAN WG1 Meeting #90bis	R1-1719087

# Prague, Czechia, 9th – 13th, October 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.873 | CR | 0034 | rev | 1 | Current version: | 12.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on 3D-InH channel model in TR 36.873 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, CATR, Huawei, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_LTE_3D_channel |  |  |  |  |  |  | Date: |  | 2017-10-9 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TR36.873 has an error in 3D-InH channel model specification. - In Table 7.2-1: Pathloss model in LOS state of 3D-InH, the working distance range can not satisfy the topology distance to calculate interference in the indoor hotspot. Especially for 12TRPs in the network layout, the distance between UE and the farmost TRP is larger than 100m. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Extend the largest distance of d2D from 100m to 150m for the LOS state of 3D-InH in Table 7.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Users of channel model do not know how to model the 3D-InH when the working distance is larger than 100m. UE distribution cannot cover the case when UE is located under a TRP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sections 7.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Text Proposal

< Unchanged parts are omitted >

Table 7.2-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and distance is in meters | Shadow fading std [dB]7) | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| 3D-UMi | LOS | PL = 22.0log10(d3D) + 28.0 + 20log10(fc) PL = 40log10(d3D)+28.0+20log10(fc) –9log10((d'BP)2+(hBS - hUT)2) | σSF=3 σSF =3 | 10m < d2D < d'BP1) d'BP < d2D < 5000m1) hBS = 10m1), 1.5m ≦ hUT≦ 22.5m1) |
|  | NLOS | For hexagonal cell layout: PL = max(PL3D-UMi-NLOS, PL3D-UMi-LOS), PL3D-UMi-NLOS = 36.7log10(d3D) + 22.7 + 26log10(fc) – 0.3(hUT - 1.5) | σSF =4 | 10 m < d2D < 2000m2) hBS = 10m 1.5m ≦ hUT≦ 22.5m |
| 3D-UMa | LOS | PL = 22.0log10(d3D) + 28.0 + 20log10(fc) PL = 40log10(d3D)+28.0+20log10(fc) –9log10((d'BP)2+(hBS - hUT)2) | σSF =4 σSF =4 | 10m < d2D < d'BP4) d'BP < d2D < 5000m4) hBS = 25m4), 1.5m ≦ hUT ≦ 22.5m4) |
|  | NLOS | PL = max(PL3D-UMa-NLOS, PL3D-UMa-LOS), PL3D-UMa-NLOS = 161.04 – 7.1 log10 (W) + 7.5 log10 (h) – (24.37 – 3.7(h/hBS)2) log10 (hBS) + (43.42 – 3.1 log10 (hBS)) (log10 (d3D)-3) + 20 log10(fc) – (3.2 (log10 (17.625)) 2 - 4.97) – 0.6(hUT - 1.5) | σSF =6 | 10 m < d2D < 5 000 m h = avg. building height, W = street width hBS = 25 m, 1.5m ≦ hUT≦ 22.5m, W = 20m, h = 20 m The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1.5 m ≦ hUT ≦ 22.5 m Explanations: see 6) |
| 3D-RMa | LOS | PL1 = 20log10(40dfc /3) + min(0.03h1.72,10)log10(d3D) – min(0.044h1.72,14.77) + 0.002log10(h)d3D PL2 = PL1 (dBP) + 40 log10(d3D /dBP) | SF = 4 SF = 6 | 10 m < d2D < dBP, 8) dBP < d < 10 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m h = avg. building height, W = street width The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1 m < hUT < 10 m |
|  | NLOS | PL = 161.04 – 7.1 log10 (W) + 7.5 log10 (h) – (24.37 – 3.7(h/hBS)2) log10 (hBS) + (43.42 – 3.1 log10 (hBS)) (log10 (d3D)-3) + 20 log10(fc) – (3.2 (log10 (11.75 hUT)) 2 - 4.97) | SF = 8 | 10 m < d2D < 5 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m h = avg. building height, W = street width The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1 m < hUT < 10 m |
| 3D-InH | LOS | PL = 16.9 log10(d3D) + 32.8 + 20 log10(fc) | SF = 3 | 3 m < d2D < 150m hBS = 3-6 m hUT = 1-2.5 m |
|  | NLOS | PL = 43.3 log10(d3D) + 11.5 + 20 log10(fc) | SF = 4 | 10 m < d2D < 150 m hBS = 3-6 m hUT = 1-2.5 m |
| NOTE 1: Break point distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In 3D-UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. NOTE 2: PL3D-UMi-LOS = Pathloss of 3D-UMi LOS outdoor scenario. NOTE 4: Break point distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In 3D-UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. The function C(d2D, hUT) is defined in Table 7.2-2. NOTE 6: PL3D-UMa-LOS = Pathloss of 3D-UMa LOS outdoor scenario. NOTE 7: The shadow fading values are reused from [4] for simplicity. NOTE 8: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. |  |  |  |  |