3GPP TSG RAN WG1 Meeting 91 		R1-1721185

Reno, USA, November 27th – December 1st, 2017

Title:	[Draft] LS on HARQ-ACK feedback for eFeMTC

# Release:	Rel-15

Work Item:	LTE_eMTC4-Core

Source:	Qualcomm [RAN1]

To:	RAN2

# Contact Person:

# Name:	Alberto Rico

E-mail Address:	albertor@qti.qualcomm.com

# 1. Overall Description:

RAN1 discussed HARQ-ACK feedback for PUSCH in eFeMTC. RAN1 reached the following working assumption:

- One unused DCI state in each CE mode is used for indicating:

  - Early termination of MPDCCH monitoring and early termination of any ongoing PUSCH transmission

- Another unused DCI state in each CE mode is used for indicating:

  - Early termination of any ongoing PUSCH transmission (without early termination of MPDCCH monitoring)

- It can be left up to RAN2 whether the new feedback signaling should also be used for termination of MPDCCH monitoring not related to UL HARQ (re)transmissions (e.g. MPDCCH monitoring related to DL transmissions).

For early termination of MPDCCH monitoring, RAN1 assumed that this may be used in the following cases:

1) To acknowledge the receipt of the RRCConnectionRelease message.

2) To acknowledge all pending uplink HARQ processes, so that the UE may transition to DRX earlier.

It is RAN1 understanding that the Case 1 does not need any change to RAN2 specification. For Case 2, RAN1 would like to ask RAN2 if that use case is supported by RAN2 for early termination of MPDCCH monitoring. In case it is not, RAN1 will revisit the working assumption.

# 2. Actions to RAN2:

RAN1 respectfully requests RAN2 to provide feedback on the above working assumption, and whether Case 2 is supported by RAN2 for early termination of MPDCCH monitoring.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #92	Feb. 26 – Mar. 2, 2018	      Athens (Greece)

TSG RAN WG1 Meeting #92b	Apr. 16 – Apr. 20, 2018	      China