3GPP TSG RAN WG1 Meeting #91                                            R1-1721443

Reno, USA, November 27th – December 1st, 2017

Agenda Item:	7.2.3.2

Source:	Huawei, HiSilicon

Title:	Summary of remaining issues on CSI-RS

Document for:	Discussion and decision

# Summary and proposals

This is a summary of remaining issues on CSI-RS. Based on previous inputs from companies in [90b-NR-19], several proposals are prepared by the moderator as follows. The proposals here do not include those already agreed in from [90b-NR-19] (listed below) or those are “not to be supported”. And, these proposals will be further updated based on the pre-meeting offline over NR reflector.

Proposal 1: Support including the OFDM symbol index within a slot in the formula for c_init.

Proposal 2: Support including slot index within a radio frame in the formula for c_init.

Proposal 3: Support mapping CSI-RS sequence to the resource grid at RE-level granularity.

Proposal 4: Support reusing the allowed CSI-RS-to-PDSCH power offset values in LTE for NR.

Proposal 5: At least for CSI acquisition, within a CSI-RS resource, the CSI-RS ports within one CDM group are QCLed w.r.t. all parameters, while the CSI-RS ports across different CDM groups can be QCLed or non-QCLed.

Agreements:

- At least the following set of RRC parameters are to be used for configuring CSI-RS

  - Decide in RAN1#91 on whether or not to support the parameters in brackets and possible revisions

| RRC Parameter Name | Description | Value range / Sub-parameters |
| --- | --- | --- |
| NZP-CSI-RS-ResourceConfig | NZP CSI-RS resource configuration |  |
| NZP-CSI-RS-ResourceConfigId | NZP-CSI-RS resource configuration ID | 0 .. NZP-CSI-RS-ResourceMax – 1 |
| ResourceConfigType | Time domain behavior of resource configuration | aperiodic, semi-persistent, or periodic |
| CSI-RS-timeConfig | Contains periodicity and slot offset for periodic/semi-persistent CSI-RS | Periodicity: {5, 10, 20, 40, 80, 160, 320, 640} slots Slot offset: For CSI-RS periodicity P, the supported offsets are 0…P-1 |
| NrofPorts | Number of ports | 1, 2, 4, 8, 12, 16, 24, 32 |
| CSI-RS-ResourceMapping | Include parameters to capture OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource FFS: how to configure CSI-RS in different slots for fine time/frequency tracking | FFS |
| CDMType | Includes parameters to capture CDM value (1, 2, 4, or 8), CDM pattern (freq only, time and freq, time only) | FFS |
| CSI-RS-Density | Density of CSI-RS resource measured in RE/port/PRB | {1/2, 1, 3} for X = 1 {1/2, 1} for X = {2, 16, 24, 32} {1} for X = {4, 8, 12} Note: Other densities and restriction based on functionality can be added later if agreed |
| CSI-RS-FreqBand | Includes parameters to enable configuration of wideband and partial band CSI-RS | FFS |
| Pc_PDSCH | Power offset of NZP CSI-RS RE to PDSCH RE | FFS |
| [Pc_PDCCH_DMRS] | [Power offset of NZP CSI-RS RE to PDCCH DMRS RE] | FFS |
| Pc_SS | Power offset of NZP CSI-RS RE to SS RE Note: This parameter is optional | FFS |
| [CC/BWP-Info] | [Indication of which CC/BWP the configured CSI-RS is located in FFS: whether or not this parameter belongs within a CSI-RS resource configuration or within a CSI-RS resource set configuration] | FFS |
| [QCL-Info] | [QCL info (e.g., QCL relation(s), QCL parameters, QCL type) between reference RS(s) (e.g., SS, CSI-RS on same or different CC/BWP) and the target CSI-RS, also captures QCL amongst ports within the CSI-RS resource FFS: whether to merge this parameter to TCI] | [For QCL amongst ports within the CSI-RS resource, include the following two cases: Case 1: All ports are QCLed with respect to all parameters Case 2: No QCLed (FFS on QCLed w.r.t. partial parameters; FFS on QCLed within the same port group and number of port groups)] |
| [NonPMI-FeedackPortIndices] | [For non-PMI feedback, configuration of port index selection for RI/CQI calculation per rank] | FFS |
| ScramblingID | Scrambling ID | FFS |
| ZP-CSI-RS-ResourceConfig | ZP CSI-RS resource configuration | FFS |
| ZP-CSI-RS-ResourceConfigId | ZP-CSI-RS resource configuration ID | 0 .. ZP-CSI-RS-ResourceMax - 1 |
| ZP-CSI-RS-ResourceMapping | Include parameters to capture OFDM symbol and subcarrier occupancy of the ZP CSI-RS resource within a slot | FFS |
| ZP-CSI-RS-timeConfig | Contains periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS | FFS |
| ZP-CSI-RS-FreqBand | Includes parameters to enable configuration of wideband and partial band ZP-CSI-RS | FFS |

# Sequence initialization

## Bit-width for scrambling ID

Alternatives:

- 16 bits

- 10 bits (analogous to LTE)

- Matching bit-width of C-RNTI

| NEC | 16 bits |
| --- | --- |
| ZTE, Sanechips | 10 bits |
| Intel | 10 bits |
| Samsung | 10 bits |
| Huawei, HiSilicon | 16 bits to increase the flexibility of CSI-RS planning. In NR, there may be many TRPs/beams may interfere with one another within the same cell or between neighboring cells. An increased size of scrambling ID provides a larger pool of CSI-RS sequences for a given symbol and slot number and, in turn, reduces the chance of both intra-cell and inter-cell collision (or unacceptably large cross correlation) and/or facilitates more flexible CSI-RS planning to avoid such collision (or unacceptably large cross correlation). With a short scrambling ID (such as 10 bits), the CSI-RS collision rate increases and adversely impacts the CSI estimation quality especially in lower CSI-RS densities. Simulations have been performed to quantitatively show the effect of increasing the size of scrambling ID on reducing collision chance (large cross correlation). |
| LGE | 10 bits |
| Ericsson | In our view, it is beneficial to adopt a common sequence initialization formula for CSI-RS and DL DMRS, including the same bit width for the scrambling ID for both CSI-RS and DL DMRS. From a MU-MIMO perspective for DL DMRS, it is beneficial to support multiple scrambling IDs per cell. Since the number of cells is 1008, in our view, the bit width of the scrambling ID should be greater than 10 bits to allow configuration of multiple scrambling IDs per cell. This will ease network planning. In this sense, we agree with Huawei that more than 10 bits can be beneficial for CSI-RS (inheriting the bit width for DMRS). However, 16 may be too many since 16 + 16 > 31 (bit width of c_init). Hence, we propose that the bit width is in the range 10 < x <=15, where x is FFS. |
| OPPO | 10bits |
| CATT | Support 16 bits to increase CSI-RS planning flexibility, where different CSI-RS resources configured for a UE may use different CSI-RS sequences. |
| Qualcomm | 10bits |
| vivo | The number of Scrambling ID should be larger than that of cell ID, i.e. 1008 for different TRPs/beams within cells. |

RRC Impact: The bit width will only affect the range of possible values for the already existing RRC parameter ScramblingID. No new RRC parameter is needed.

## Timing information

### OFDM symbol index within a slot

| NEC | We support this parameter to be included. |
| --- | --- |
| ZTE, Sanechips | Similar with LTE, both OFDM symbol index and slot index are used. |
| Intel | Support OFDM symbol |
| Samsung | Open to include both slot and symbol index analogous to LTE. However, the potential limited payload for sequence initialization should be considered first. |
| Huawei, HiSilicon | Support OFDM symbol index in slot |
| DOCOMO | Support OFDM symbol |
| LGE | Support OFDM symbol index |
| Ericsson | Support OFDM symbol index within a slot in the formula for c_init |
| OPPO | Support OFDM symbol index with a slot. |
| CATT | Same as LTE |
| Nokia | Support OFDM symbol index |
| Qualcomm | Support OFDM symbol index within a slot |
| vivo | Support OFDM symbol index |
| mtk | We support OFDM symbol index within slot |

RRC Impact: None

### Slot index

| NEC | We support different ranges of slot index are defined for different SCS values. In addition, for larger SCS, the slot index may take the value modulo a fixed value to make sure the length of c_init not exceed 31. |
| --- | --- |
| ZTE, Sanechips | Similar with LTE, both OFDM symbol index and slot index are used. |
| Intel | Support slot index |
| Samsung | Open to include both slot and symbol index analogous to LTE. However, the potential limited payload for sequence initialization should be considered first. |
| Huawei, HiSilicon | Support slot index |
| DOCOMO | Support slot index |
| LGE | Support slot index |
| Ericsson | Support slot index within a radio frame in the formula for c_init |
| OPPO | Support slot index |
| CATT | Same as LTE |
| Nokia | Support slot index |
| Qualcomm | Support slot index with a fixed range for all SCS values |
| vivo | Support slot index within a radio frame |
| mtk | As what we have in LTE: slot number within a radio frame |

RRC Impact: None

## Frequency information

### Subcarrier index within an RB

| NEC | We support the sequence is commonly generated with RB index, and generated with 12 values for 12 subcarriers within one RB. And within one RB, for different CSI-RS component, values are abstracted from corresponding subcarriers. For RB level, the sequence values of CSI-RS with low density are abstracted from corresponding RBs. |
| --- | --- |
| Intel | Same PN sequence for all ports in PRB. |
| Huawei, HiSilicon | We do not see any need for this. |
| LGE | Not support |
| Ericsson | Not clear exactly what this means; however, our view is that sequences should be generated on a per-RE basis within the carrier bandwidth and mapped to CSI-RS REs resulting in a resource specific design. In other words, the CSI-RS symbol values depend on the PRB position of the CSI-RS resource within the carrier bandwidth, like in LTE. |
| OPPO | No need |
| CATT | Not support |
| Nokia | Initialization is not related with RE index |
| ZTE, Sanechips | No need for c_init |
| Qualcomm | No need |
| vivo | Not support |
| mtk | It is not needed |

RRC Impact: None

### RB index

| Intel | Modulation symbol should be different from different PRBs |
| --- | --- |
| Samsung | No support (prefer to use frequency information for sequence generation but not for sequence initialization.) |
| Huawei, HiSilicon | We do not see any need for this. |
| LGE | Not support. |
| Ericsson | Not clear what this means |
| OPPO | No need |
| CATT | Not support |
| Nokia | Not support |
| ZTE, Sanechips | No need for c_init |
| Qualcomm | No need |
| vivo | Not support |
| mtk | Not needed |

RRC Impact: None

## Subcarrier spacing

| NEC | We support to include SCS in the c_init generation. And for different subcarrier spacing values, the ranges of some parameters are different, e.g. slot index as mentioned above. |
| --- | --- |
| ZTE, Sanechips | Not support |
| Intel | Open to include this parameter for c_init |
| Huawei | We do not see any need for this. |
| LGE | Not support |
| Ericsson | Do not support inclusion of the sub-carrier spacing in the formula for c_init |
| OPPO | No need |
| Samsung | Open to include related parameter(s) |
| CATT | Not support |
| Nokia | Not support |
| Qualcomm | No need |
| vivo | Not support |
| mtk | Not needed |

RRC Impact: None

## CP type, e.g., NCP, ECP

| NEC | We support the initialization includes parameter CP type for 60kHz only. |
| --- | --- |
| ZTE, Sanechips | Can be omitted |
| Intel | Open to include this parameter for c_init |
| Samsung | Can be omitted. |
| LGE | Including the CP type for c_init is not necessary. |
| Ericsson | Do not support inclusion of the CP type in the formula for c_init |
| OPPO | No need |
| CATT | Same as LTE |
| Qualcomm | No need |
| vivo | Not support |
| mtk | Not needed |

RRC Impact: None

# Mapping of sequence values to CSI-RS REs

Alternatives:

- per RE

- per RB

- per OFDM symbol

- per slot

| ZTE, Sanechips | Per RE. the final sequence is generated by Kronecker product between two Gold sequences, where the length of the first one is the maximal number of PRBs, and the second length is 12 |
| --- | --- |
| Intel | Same mapping approach as in LTE. To avoid power imbalance issue we also prefer to apply OCC code cycling across PRBs. |
| Samsung | Per RE. |
| Huawei | Per RE. A longer CSI-RS sequence is generated to guarantee each CSI-RS RE is mapped with a different QPSK symbol. |
| LGE | Per RE |
| Ericsson | Per RE mapping. Similar to Intel’s proposal, we prefer that for a given port index, the corresponding OCC is cycled across PRBs to avoid a power imbalance. Like our comment in Section 1.3.1, this should be done in a resource specific manner. Contrary to Intel’s proposal, we don’t see a need to apply OCC across CDM groups. |
| OPPO | Per RE similar to LTE |
| CATT | Same as LTE |
| Nokia | Per RE |
| Qualcomm | Per RE. The length of the LTE legacy gold sequence is 2^31, which is large enough to support the needed number of REs. The extraction can be based on the subcarrier index per CSI-RS transmission bandwidth. |
| vivo | Per RE |
| mtk | Per RE, and we also consider the cycling of OCC code |

RRC Impact: None

# Periodicity and offset

## Periodicity

- Note: Periodicities of {5, 10, 20, 40, 80, 160, 320, 640} slots are already agreed for CSI-RS transmission

- Whether/how to restrict the periodicity as a function of subcarrier spacing

| ZTE, Sanechips | no restriction |
| --- | --- |
| Intel | no restriction |
| DOCOMO | no restriction |
| Ericsson | No restriction |
| OPPO | {5, 10} is not needed for 60kHz/120kHz SCS, {320, 640} is not needed for 15kHz/30kHz SCS |
| CATT | No restriction |
| Qualcomm | Restriction based on SCS: {5,10} no needed for 60kHz/120kHz |
| vivo | {5} is not needed for 60kHz/120kHz SCS. The restriction of {320, 640} seems not necessary for 15kHz/30kHz SCS. |
| mtk | We suggest to have some restriction in RAN1. We have similar view as QC to restrict {5, 10} not for 60k/120k SCS |

RRC Impact: The supported periodicities are to be included in the already existing RRC parameter CSI-RS-timeConfig.

## Offset

Alternatives:

- E.g., the same values of slot offsets of LTE, for SCS of 15 kHz

- E.g., N times number of slot offsets of LTE, for SCS of N * 15 kHz

- E.g., the same values of slot offsets of LTE, for all SCS

| ZTE, Sanechips | For CSI-RS periodicity P, the supported offsets are 0…P-1 |
| --- | --- |
| Intel | For CSI-RS periodicity P, the supported offsets are 0…P-1 |
| DOCOMO | For CSI-RS periodicity P, the supported offsets are 0…P-1 |
| Ericsson | For CSI-RS periodicity P, the supported offsets are 0…P-1 |
| OPPO | For CSI-RS periodicity P, the supported offsets are 0…P-1 |
| Samsung | For CSI-RS periodicity P, the supported offsets are 0…P-1 |
| CATT | Same values as LTE for all SCS |
| Qualcomm | For CSI-RS periodicity P, the supported offsets are 0…P-1 |
| vivo | For CSI-RS periodicity P, the supported offsets are 0…P-1 |
| mtk | the supported offsets are 0…P-1 |

RRC Impact: The supported slot offsets are to be included in the already existing RRC parameter CSI-RS-timeConfig.

# Densities for CSI-RS, i.e., D

- Note: D = 1, 3 is already agreed

## For X = 1

| ZTE, Sanechips | 2, 3, 4, 6 |
| --- | --- |
| Intel | 2, 3, 4, 6 |
| Samsung | 3/2, 3, 6 |
| LGE | At least 3 |
| Ericsson | At least D = 3 |
| OPPO | 3/2, 3, 6 |
| CATT | 2, 3, 4, 6 |
| Nokia | 2, 4, 6, 12 |
| Qualcomm | 3 for TRS (agreed already), 1 for CSI acquisition (agreed), 1 and 3 for BM |
| vivo | 3; 2 should not be supported due to the limited RE sharing possibility between 2 and 3; |
| mtk | D=3 for TRS and CSIRS BM, D=1 for CSI acquisition |

RRC Impact: The supported density(ies) are to be included in the existing RRC parameter CSI-RS-Density.

## For X = 2 for BM

- Note: D = 1 (subject to RAN4 feedback)

| LGE | According to RAN4 reply in R4-1711697, D=1 is feasible for X=2 |
| --- | --- |
| Ericsson | D = 1 only (according to previous RAN1 agreement and RAN4 LS reply R4-1711697). |
| OPPO | D=3/2 (can achieve better performance than D = 1) |
| Samsung | Support D=1 for X=2 by R4-1711697 |
| Qualcomm | D = 1 |
| vivo | D = 1 |

RRC Impact: The supported density is to be included in the existing RRC parameter CSI-RS-Density.

## Indication of D

| Ericsson | This is indicated by the already existing RRC parameter CSI-RS-Density |
| --- | --- |
| Qualcomm | RRC configured |

RRC Impact: This is indicated by the already existing RRC parameter CSI-RS-Density

# Non-uniform mapping of component patterns

- For {X=24, 32}-port CSI-RS which span N=4 OFDM symbols, whether or not to support different number of component CSI-RS RE patterns between two pairs where each pair is composed of two adjacent OFDM symbols

| NEC | We also support to consider different number of component CSI-RS RE patterns between two pairs for multiplexing with other RS/channels, while we think it’s better to restrict the pair with less number of components nested within the pair with more number, which will maintain “uniform” as much as possible. And this restriction may also have benefit to reduce the configuration signaling overhead. |
| --- | --- |
| ZTE, Sanechips | Not support |
| Intel | No support |
| Samsung | Support given that the current agreements already support full flexibility for 1-port CSI-RS resource, which means there will be no more RRC impact or RRC overhead. |
| DOCOMO | No support. These exceptions can be avoided by allowing additional symbol location as in Q6. |
| LGE | Support the different number of component CSI-RS RE patterns, but we can agree possible restrictions according to complexity issue as NEC commented. Similar to Samsung’s comment, we don’t see any extra RRC impact or RRC overhead for this non-uniform mapping of component patterns. Furthermore, even if additional symbol location will be agreed, the different number of component RE patterns is difficult to be avoided in consideration of various slot formats and RSs. |
| Ericsson | No need to support. 24-port CSI-RS can already coexist with TRS. For 32 port, there are sufficient configuration possibilities for both CSI-RS and TRS to enable coexistence. |
| OPPO | No need. |
| Huawei, HiSilicon | No need. |
| CATT | Support considering flexibility. |
| Nokia | Not support |
| Qualcomm | Not supported |
| vivo | Not supported |
| mtk | We support LGE proposal |

RRC Impact: None

# Additional symbol location for CSI-RS within a slot

- Note: The {6, 7, 8, 9, 10, 11, 13, 14}-th OFDM symbol (indexing start at 1) are agreed

| ZTE, Sanechips | Support 4, 5, 12 in addition to {1, 2, 3, 6, 7, 8, 9, 10, 11, 13, 14} |
| --- | --- |
| Intel | 4, 5, 12 |
| DOCOMO | 4 and 12 to accommodate four adjacent component patterns in time domain. |
| LGE | At least 4, 5 |
| Ericsson | In our understanding only symbol locations {6,7,8,9,10,11,13,14} have been agreed so far. We propose adding symbol 5 |
| CATT | 4, 5, 12 |
| Nokia | 4, 5 |
| Qualcomm | 4, 5 |
| vivo | Adding symbol index 5 |
| mtk | TRS is agreed to be at 5th symbol. So, CSI-RS should also support it |

RRC Impact: Symbol locations are to be included in the existing RRC parameter CSI-RS-ResourceMapping.

# Configuration parameters for CSI-RS bandwidth

- Values for N and X when CSI-RS spans a BW smaller the BWP

| ZTE, Sanechips | N = 4, X = 0 (i.e. up to gNB implementation) |
| --- | --- |
| Samsung | N is equal to the minimum bandwidth of CSI reporting, X=1 (prefer explicit signaling on the starting RB and ending RB for CSI-RS transmission.) |
| Ericsson | Starting PRB position of CSI-RS resource requires specification as well |
| CATT | N equals to minimum bandwidth of CSI reporting, X=1 |
| Intel | Our understanding that CSI-RS can be discontinuous in the frequency domain. Some bitmap for a given BWP may be needed to indicated which PRBs are occupied. |
| Qualcomm | X = 20 |
| vivo | N is equal to the configured reporting subband size; X=1; |

RRC Impact: Parameters specifying CSI-RS bandwidth are to be included in the already existing RRC parameter CSI-RS-FreqBand.

# Allowed CSI-RS-to-PDSCH power offset values

- E.g., same values as in LTE

| NEC | Same values as in LTE can be taken as a starting point. And we support different power offset values for different CSI-RS components, for example, different components in different symbols. |
| --- | --- |
| ZTE, Sanechips | Same values as LTE |
| Intel | Same values as supported by LTE. |
| Samsung | Same values as LTE. |
| LGE | Same values as LTE |
| Ericsson | RRC Impact: Supported power offset values are to be included in existing RRC parameter Pc. |
| OPPO | Same values as LTE |
| Huawei, HiSilicon | Consider supporting different power offset values for different CSI-RS patterns (e.g., different densities) |
| CATT | Same values as LTE. |
| Nokia | Same values as LTE |
| Qualcomm | Similar to LTE |
| vivo | For CSI acquisition, same values as LTE |
| mtk | Same as LTE |

RRC Impact: Supported power offset values are to be included in existing RRC parameter Pc.

# Power offset between CSI-RS and SSS

| Intel | Given the agreement in BM session to support joint CSI-RS / SS block based RSRP calculation, RRC signalling should also support power offset indication between these two types of the reference signals. Power offset values for CSI-RS relative to SS block are 0 dB, 3 dB, 4.7 dB, 6dB (some refinement may be needed depending on the agreed CSI-RS densities ) |
| --- | --- |
| Ericsson | RRC Impact: Propose adding new RRC parameter Pc_SSB defined within the existing RRC parameter NZP-CSI-RS-ResourceConfig. |
| Nokia | RRC signaling is supported to inform the power offset |
| ZTE, Sanechips | Support this new RRC parameter, but negative values should also be included for power offset values of CSI-RS relative to SSB because SSB is more important |
| Qualcomm | Support this new RRC parameter, but wide range of values (e.g., negative values) should be considered |

RRC Impact: Propose adding new RRC parameter Pc_SSB defined within the existing RRC parameter NZP-CSI-RS-ResourceConfig.

# Indication of position of CSI-RS units within a slot constituting NZP CSI-RS resource

| Ericsson | This already captured in the current draft of 38.211, where the CSI-RS component locations are parameterized by starting OFDM symbol locations and a bitmap specifying the frequency domain positions (see Table 7.4.1.5.2-1: CSI-RS locations). In our view the starting OFDM symbol locations and the bitmap should be configurable, hence there is indeed RRC impact. However, an RRC parameter intended to capture this configurability is already included in the list of RRC parameters, namely CSI-RS-ResourceMapping. |
| --- | --- |
| Huawei, HiSilicon | Bitmap for specifying the time and frequency position of the component patterns |
| Samsung | We suggest to have separated RRC parameters for frequency and time domain RE location indication for simpler design. For example, frequency domain location can be captured by the existing “NZP-CSI-RS-ResourceConfig” and the time domain location can be covered by CSI-RS-ResourceMapping. |
| CATT | Separate RRC parameters for frequency domain and time domain RE location indication |
| Intel | Two bitmaps - one in frequency of length 12/Y and another in time domain of length 14 bits. Kronecker product of two vectors (corresponding to the bitmaps) indicates positions of CSI-RS components in the slot. |
| Qualcomm | Supported, both time and frequency position included in the CSI-RS-ResourceMapping |
| vivo | Consider some signaling overhead reduction method rather than two bitmaps. |
| ZTE, Sanechips | Explicitly configure each component position in the frequency domain and time domain. Bitmaps are not preferred. |

RRC Impact: OFDM symbol location of the CSI-RS components are to be included in the already existing parameter CSI-RS-ResourceMapping.

# QCL configuration/indication for CSI-RS

According to the agreement in the QCL agenda item in RAN1#90b about signaling the QCL relationship between a reference RS (e.g., SSB, CSI-RS) and a target CSI-RS, there is RRC impact on the configuration of a CSI-RS resource.

| Ericsson | RRC Impact: Propose adding a new RRC parameter QCL-Info defined within the existing RRC parameter NZP-CSI-RS-ResourceConfig to capture QCL info (QCL relation(s), parameters, type, etc.) between source RS(s) and the target CSI-RS. This is analogous to 36.331 where for Type B QCL, such an IE is defined within a CSI-RS resource configuration CSI-RS-ConfigNZP. |
| --- | --- |
| Intel | QCL info should also include QCL information for other CSI-RS or SS block including carrier index and other BWPs per agreement in QCL session. |
| ZTE, Sanechips | QCL information between reference RS(s) and target CSI-RS should be decoupled from RS configuration. It is better to be included in TCI configuration via higher signaling. |
| Qualcomm | Support this RRC parameter, but need to indicate the cross carrier/BWP information, e.g., carrier index and BWP index |
| Samsung | Indication/configuration of QCL relation between a reference RS (e.g. SSB, CSI-RS) and a target CSI-RS (P/SP CSI-RS) can be included as part of the RRC configuration of TCI states. Indication of QCL relation between a reference RS (SSB or P/SP CSI-RS) and the target RS (AP CSI-RS) is provided via DCI signaling – No RRC impact is foreseen if the target RS is AP CSI-RS |
| vivo | Should be part of CSI-RS configuration referring to TCI state. |
| LGE | Proposals 4, 5, 6 in Ericsson’s Tdoc (R1-1720730 for RAN1#91) seems reasonable and straightforward to resolve this issue considering all the cases of AP/SP/P-CSI-RS types, which captured below for convenience (where Proposal 5 may need to be double-checked on whether the MAC-CE activation message should also carry such beam association signaling together or beam association signaling can be separated from this activation signaling): Proposal 4. For a configured target aperiodic (ap)-CSI-RS resource in a resource set used for beam management and/or CSI acquisition, the source RS used for spatial QCL purposes (an SSB, semi-persistent (sp)-CSI-RS, or periodic (p)-CSI-RS resource) is indicated by DCI signalling of a TCI state ID in the same DCI message that contains the trigger of the ap-CSI-RS resource set. Proposal 5. For a configured target semi-persistent (sp)-CSI-RS resource in a resource set used for beam management and/or CSI acquisition, the source RS used for spatial QCL purposes (an SSB) is indicated by MAC-CE signalling of a TCI state ID in the same MAC-CE message that contains the activation of the sp-CSI-RS resource(s)/resource set. Proposal 6. For a configured target periodic (p)-CSI-RS resource used for beam management and/or CSI acquisition, the source RS used for spatial QCL purposes (an SSB) is indicated by an IE within the RRC configuration of the target p-CSI-RS resource, e.g., QCL_Info |

RRC Impact: Propose adding a new RRC parameter QCL-Info defined within the existing RRC parameter NZP-CSI-RS-ResourceConfig to capture QCL info (QCL relation(s), parameters, type, etc.) between source RS(s) and the target CSI-RS. This is analogous to 36.331 where for Type B QCL, such an IE is defined within a CSI-RS resource configuration CSI-RS-ConfigNZP.

# QCL type indication among CSI-RS ports within one CSI-RS resource

According to the agreement in the QCL agenda item in RAN1#89 about QCL relationship among CSI-RS ports within one resource, two QCL types are defined. It has RRC impact on the configuration of QCL type among CSI-RS ports within one resource.

| ZTE, Sanechips | Propose adding a new RRC parameter QCL-type-ports to configure the QCL type of CSI-RS ports within one resource. 0: QCL w.r.t average gain, delay spread, Doppler spread, Doppler shift, and average delay parameters, spatial Rx parameters; 1: Not QCL’ed. Agreement: - CSI-RS ports within a CSI-RS resource have at least two types of QCL assumptions - QCL w.r.t average gain, delay spread, Doppler spread, Doppler shift, and average delay parameters, spatial Rx parameters - Not QCL’ed (e.g. for beam selection based on beamformed CSI-RS codebook) - FFS whether some parameters can still be QCL’ed |
| --- | --- |
| Qualcomm | Support 2 QCL groups and CSI-RS ports within one CDM group are QCLed (Similar to DMRS agreements) |
| LGE | We share with ZTE’s view above. In this case, port index reporting should be supported as well to use Not QCLed case for the beam management. |
| NEC | We share similar view with ZTE. Within one CSI-RS resource, both QCLed and non-QCLed across CDM groups should be supported. |
| CATT | We don’t see use case for not QCLed ports within one resource, multi panel, multi TRP has been de-prioritized in September plenary. |

RRC Impact: Propose adding a new RRC parameter QCL-type-ports defined within the existing RRC parameter .

# Indication of whether UE is to measure CSI or L1-RSRP

| Intel | It is better that the UE could know whether it should measure the L1-RSRP or CSI when receiving a CSI-RS. |
| --- | --- |
| Ericsson | Reporting quantity (CSI or L1-RSRP) is already captured in the existing RRC parameter ReportQuantity |
| Samsung | We don’t see a critical reason to limit usage of a configured RS itself. The corresponding feature can be provided by CSI reporting configuration. |
| Qualcomm | No need to limit the usage of a configured RS resource |
| vivo | Support such indication. |
| LGE | Support such indication to simplify UE implementation via removing ambiguity (e.g. if UE is configured to report CRI only) |

RRC Impact: Reporting quantity (CSI or L1-RSRP) is already captured in the existing RRC parameter ReportQuantity.

# Port index indication for non-PMI feedback

| ZTE | This parameter needs to be added in CSI-RS configuration based on the following agreement. Agreement: For non-PMI feedback, support the following port index indication method: - Port index indication is signaled to UE for RI/CQI calculation in non-PMI feedback - Port index indication per CSI-RS resource is configured by RRC to select the CSI-RS port(s) used for RI/CQI calculation per rank - Identity matrix is assumed by UE on the selected CSI-RS ports for RI/CQI calculation - N ports are selected for rank N - The CSI-RS resource can be dynamically selected for CSI reporting in CSI framework In this parameter, a list of N fields is signaled for N-port CSI-RS, where N<=8. The n-th field contains a list of n port indices, and value range of each port index varies from 0 to N-1. |
| --- | --- |
| Qualcomm | No need |
| CATT | No need |

RRC Impact: Add one RRC parameter PortIndexIndication for each CSI-RS resource.

# BWP/CC configuration of CSI-RS resources

| LGE | CC configuration seems not needed since the change of CCs can be operated by RRC reconfiguration. BWP configuration however needs to be configured per CSI-RS resource or per resource setting because active BWP can be switched dynamically by DCI. UE may assume that resource settings for inactive BWPs are invalid. |
| --- | --- |
| ZTE, Sanechips | Considering configuring multiple resources for each CC should be supported and CSI-RS triggering is per resource set, we support to configure CC information in resource set. |
| CATT | This is related to RRC structure, it could be other way round, BWP configuration includes CSI-RS configuration. In our view, it is better to coordinate with BWP discussion before we agree on new RRC parameter. |

RRC Impact: Add one RRC parameter BWP-Info for each CSI-RS resource or each resource setting.

# Number of antenna ports in CSI-RS resource and codebook.

| Intel | For efficient use of RS and support UEs with different CSI processing capability, NR can support antenna ports on CSI-RS to be larger than or equal to the number of antenna ports in the codebook. |
| --- | --- |