3GPP TSG RAN WG1 Meeting #92                                          R1-1801832

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.1.2.3.2

Source:	Spreadtrum Communications

Title:	Remaining issues on CSI-RS

Document for:	Discussion and decision

Introduction

The current equation of physical resource mapping of CSI-RS captured in latest CR [1] to TS 38.211 has not been fully correct. The contribution provides correction to the existing mapping.

Discussion

The problems comes from ρ=0.5, where the equation does not well reflect the RB-level comb structure, where either even or odd PRBs can be configured to transmit CSI-RS.

Checking the current mapping equation, it seems like no further change is required except limiting the value of n to all even or odd integers based on a higher layer configuration comb-offset, i.e.,

So we have the following mapping shown in Table 1. Such a resource mapping supports CSI-RS sharing under the same density ρ, number of ports X, and RB-level comb offset between CSI-RS of different bandwidths, while resource sharing between different density is not supported, which is acceptable.

Table 1 CSI-RS sequence mapping from CRB0

| Frequency Resource |  | X=1 |  | X>1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Even PRB | Odd PRB | Even PRB | Odd PRB |
| CRB3 | 11 |  |  |  |  |
|  | ⋮ |  |  |  |  |
|  | 1 |  |  |  | r4 |
|  | 0 |  | r(1) |  | r3 |
| CRB2 | 11 |  |  |  |  |
|  | ⋮ |  |  |  |  |
|  | 1 |  |  | r3 |  |
|  | 0 | r(1) |  | r2 |  |
| CRB1 | 11 |  |  |  |  |
|  | ⋮ |  |  |  |  |
|  | 1 |  |  |  | r2 |
|  | 0 |  | r(0) |  | r1 |
| CRB0 | 11 |  |  |  |  |
|  | ⋮ |  |  |  |  |
|  | 1 |  |  | r1 |  |
|  | 0 | r(0) |  | r0 |  |

Proposal: Capture the RB-level comb offset for ρ=0.5 in clause 7.4.1.5.3 of TS 38.211.

Text Proposal for 38.211

/************************ Start of Text Proposal **************************/

##### 7.4.1.5.3	Mapping to physical resources

For each CSI-RS configured, the UE shall assume the sequence  being mapped to resources elements  according to

when the following conditions are fulfilled:

-	the resource element  is within the resource blocks occupied by the CSI-RS resource for which the UE is configured

The reference point for  is subcarrier 0 in common resource block 0.

The value of  is given by the higher-layer parameter CSI-RS-Density, and the comb offset is configured by higher layer parameter CSI-RS-Comb-Offset when .

/************************ Unchanged parts omitted**************************/

/************************ End of Text Proposal **************************/

Conclusion

In this contribution, we provided a modification to the CSI-RS resource mapping equation to resolve the remaining issue of ρ=0.5. Based on the discussion, we have the following proposal and the text proposal is also present.

Proposal: Capture the RB-level comb offset for ρ=0.5 in clause 7.4.1.5.3 of TS 38.211.

Reference

- 3GPP R1-1801291, “draftCR to 38.211 capturing the Jan18 ad-hoc meeting agreements”, Ericsson,  RAN1 AH 1801, Vancouver, Canada, Jan. 2018.