3GPP TSG RAN WG1 Meeting #93	R1-1805930

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.8.2

Source:	Huawei, HiSilicon

Title:	Considerations and evaluation results for IMT-2020 for high mobility

Document for:	Discussion and decision

Introduction

In this contribution, the considerations on self evaluation and evaluation results for mobility is presented.

Evaluation method

The general evaluation method and procedure for mobility evaluation is defined in Report ITU-R  M.2412 [1]. This procedure includes system level simulation (SLS) part and link level simulation (LLS) part. In RAN1#92bis meeting, the detailed evaluation method and parameters for these two parts are discussed [2].

For SLS part, the pre-processing SINR is used. The pre-processing SINR is defined on a Rx antenna port with respect to a Tx antenna port. The details of calculating pre-processing SINR is refer to Section 2.1.1 in [2].

For LLS part, the antenna pattern options are proposed, and inter-port distance is specified. There are three options for the use of antenna port pattern [2]. 0dBi omni-directional gain is assumed for Tx and Rx antenna ports is used in this contribution.

The evaluation method in [2] is used to derive the evaluation results in this contribution.

Mean ZoD spread of system level channel model

In [2], there is a remaining issue on mean value of ZoD spread. According to ITU-R guideline, the ZoD spread in link level channel model should be scaled to the mean value in system level channel model. It is agreed in [2] that,

Use the 50%-tile point of CDF of the mean value of ZoD spread of all UEs based on UE dropping in system level simulation. The mean value of ZoD spread of a UE is calculated according to the formula given in Table 1 and Table 2 for Dense Urban and Rural test environment, respectively, where d2D and hUT of a UE is determined by the UE dropping.

Table 1 Mean value calculation for UMa ZoD spread

|  | Frequency | Parameters | UMa_x |  |
| --- | --- | --- | --- | --- |
|  |  |  | LOS | NLOS |
| UMa_A | 0.5 GHz≤ fc ≤6 GHz | lgZSD | max[-0.5, -2.1(d2D/1000) -0.01 (hUT - 1.5)+0.75] | max[-0.5, -2.1(d2D/1000) -0.01(hUT - 1.5)+0.9] |
|  | 6 GHz< fc ≤ 100 GHz | lgZSD | max[-0.5, -2.1(d2D/1000) -0.01 (hUT - 1.5)+0.75] | max[-0.5, -2.1(d2D/1000)-0.01(hUT - 1.5)+0.9] |
| UMa_B | 0.5 GHz≤ fc ≤ 100 GHz | lgZSD | max[-0.5, -2.1(d2D/1000)- 0.01 (hUT -1.5)+0.75] | max[-0.5, -2.1(d2D/1000)-0.01(hUT - 1.5)+0.9] |

Table 2 Mean value calculation for RMa ZoD spread

|  | Frequency | Parameters | RMa_x |  |
| --- | --- | --- | --- | --- |
|  |  |  | LOS | NLOS |
| RMa_A RMa_B | RMa_A：0.5 GHz≤ fc ≤ 6 GHz, RMa_B：0.5 GHz≤ fc ≤ 7 GHz | lgZSD | max(-1, -0.17*( d2D /1000)-0.01*( hUT -1.5)+0.22) | max(-1, -0.19*( d2D /1000) - 0.01*( hUT -1.5)+0.28) |

Detailed method of deriving mean value of ZoD spread for LOS and NLOS

In link level simulation, LOS and NLOS channel will be evaluated separately. Therefore, the mean value of ZoD spread should be derived for LOS and NLOS, separately. Based on the above agreement, the detailed derivation is as follows:

- In the UE drop in system level simulation, determine “LOS” UE and “NLOS” UE according to LOS probability from system level channel model (“LOS UE” means the channel state is LOS for UE to its serving TRxP; “NLOS UE” means the channel state is NLOS for UE to its serving TRxP)

- Assume there are N “LOS” UEs, and M “NLOS” UEs; N+M=570 for dense urban and rural test environment. Calculate the value of lgZSD  for “LOS” UE and “NLOS” UE according to “LOS” and “NLOS” column in Table 1 or Table 2, respectively.

- Plot CDF of lgZSD for “LOS” UEs, based on the N samples, and find the 50%-tile point as the mean value of ZoD spread in case of LOS. Similarly, plot CDF of lgZSD  for “NLOS” UEs, based on the M samples, and find the 50%-tile point as the mean value of ZoD spread in case of NLOS.

Proposal 1: Use the detailed method shown in Section 3.1 to derive the mean value of ZoD spread for LOS and NLOS.

The results of mean value of  ZoD spread

According to the above-mentioned method, the mean value of ZoD in degree is shown is Table 3 (see gray grids), and the CDF figures are provided in Appendix 1.

Proposal 2: Use the mean value of ZoD in Table 3 in link level simulation for mobility evaluation for 4 GHz and 700 MHz.

Table 3 Mean value of ZoD spread for Dense Urban and Rural – eMBB test environment

| Parameters | Indoor Hotspot-eMBB |  |  |  |  |  | Dense Urban-eMBB |  |  |  | Rural-eMBB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Config A (4 GHz) |  | Config B (30 GHz) |  | Config C (70 GHz) |  | Config A (4 GHz) |  | Config B (30 GHz) |  | Config A/B (700 MHz/4 GHz) |  |
| Link-level Channel model | LOS: CDL/TDL-iv | NLOS: CDL/TDL-i | LOS: CDL/TDL-iv | NLOS: CDL/TDL-i | LOS: CDL/TDL-iv | NLOS: CDL/TDL-i | LOS: CDL/TDL-v | NLOS: CDL/TDL-iii | LOS: CDL/TDL-v | NLOS: CDL/TDL-iii | LOS: CDL/TDL-v | NLOS: CDL/TDL-iii |
| Delay spread scaling parameter (ns) | 20 | 39 | 19.5 | 26 | 19.5 | 20 | 93 | 363 | 79 | 263 | 32 | 37 |
| AoA angular spreads scaling parameter (degree) | 42 | 59 | 32 | 50 | 27 | 46 | 65 | 74 | 65 | 48 | 33 | 33 |
| AoD angular spreads scaling parameter (degree) | 40 | 42 | 40 | 42 | 40 | 42 | 14 | 26 | 17 | 22 | 8 | 9 |
| ZoA angular spreads scaling parameter (degree) | 17 | 18 | 11 | 14 | 9 | 13 | 9 | 18 | 9 | 11 | 3 | 4 |
| ZoD angular spreads scaling parameter (degree) | 10 | 12 | 1 | 12 | 0.4 | 12 | 3.3 | 4.6 | TBD from 50%-tile point of CDF of ZoD spread | TBD from 50%-tile point of CDF of ZoD spread | 1.25 | 1.44 |
| K-factor (dB) | 13.3 | - | 13.3 | - | 13.3 | - | 22 | - | 22 | - | 22 | - |

Initial evaluation results

In this section, the initial evaluation results for mobility is provided. In Figure 1, the pre-processing SINR CDFs for eMBB test environment are provided. The assumptions are provided in Appendix 2.

- Rural – eMBB (700 MHz)                                        (b) Rural – eMBB (4 GHz)

(c) Dense Urban – eMBB (4 GHz)                               (d) Indoor Hotspot – eMBB (4 GHz)

Figure 1 UL SINR distribution for eMBB test environments

Based on the above figures, the 50%-tile point of the CDF for different test environments are listed in Table 4.

Table 4 The 50%-tile point of SINR CDF for different test environments

| Test environment | Evaluation configuration | UE mobility | 50%-tile point of SINR CDF (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Channel model A | Channel model B |
| Rural – eMBB | Config. A (700 MHz) | 120 km/h | 10.21 | 10.14 |
|  |  | 500 km/h | 9.67 | 9.65 |
| Rural - eMBB | Config. B (4 GHz) | 120 km/h | 4.66 | 4.50 |
|  |  | 500 km/h | 2.90 | 2.72 |
| Dense Urban – eMBB | Config. A (4 GHz) | 30 km/h | 5.52 | 5.32 |
| Indoor Hotspot – eMBB (12 TRxP) | Config. A (4 GHz) | 10 km/h | 3.90 | 3.95 |
| Indoor Hotspot – eMBB (36 TRxP) | Config. A (4 GHz) | 10 km/h | -0.21 | -0.07 |

Summary

In this document, the consideration and initial evaluation results on mobility is provided. The following proposals are made.

Proposal 1: Use the detailed method shown in Section 3.1 to derive the mean value of ZoD spread for LOS and NLOS.

Proposal 2: Use the mean value of ZoD in Table 3 in link level simulation for mobility evaluation for 4 GHz and 700 MHz.

Furthermore, the median SINR value is presented in Table 4.

- Reference

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- R1-1805643, “Summary of discussion on mobility evaluation method and parameters”, Huawei, 3GPP TSG RAN WG1#92bis, April 2018.

- Appendix 1 	CDF for ZoD spread for LOS and NLOS

The CDF of mean value of ZoD spread for LOS and NLOS for Rural and Dense Urban test environment are plotted from Figure A1-1 to Figure A1-3, respectively.

Figure A1-1 Mean value of ZoD (degree) for Rural (4 GHz)

Figure A1-2 Mean value of ZoD (degree) for Rural (700 MHz)

Figure A1-3 Mean value of ZoD (degree) for Dense Urban (4 GHz)

- Appendix 2 	Simulation assumption of SLS part for mobility

The simulation assumption of system level part for mobility evaluation is listed in Table A2-1.

Table A2-1. Simulation assumptions

|  | Indoor Hotspot – eMBB | Dense urban - eMBB | Rural - eMBB |
| --- | --- | --- | --- |
| Evaluation configuration | Configuration A, Configuration B | Configuration A | Configuration A, Configuration B |
| Carrier frequency for evaluation | 4 GHz | 4 GHz | Configuration A :700 MHz Configuration B : 4 GHz |
| Multiple access | OFDMA | OFDMA | OFDMA |
| Duplexing | FDD | FDD | FDD |
| Transmission scheme | UL SIMO | UL SIMO | UL SIMO |
| BS antenna height | 3m | 25 m | 35 m |
| Total transmit power per TRxP | 21 dBm for 10 MHz bandwidth | 41 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | - | 20% high loss, 80% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance | 20 m | 200 m | 1732 m |
| Number of antenna elements per TRxP | 32 Tx/Rx, (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 32 Tx/Rx, (M,N,P,Mg,Ng) = (8,2,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) |
| Number of UE antenna elements | 1Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) | 1Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) | 1Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) |
| Device deployment | 100% indoor Randomly and uniformly distributed over the area | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area |
| UE speeds of interest | 10 km/h | 30 km/h; | 120 km/h;500km/h; |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 10 MHz | 10 MHz | 10 MHz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) | 1.5 m |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B |
| TRxP number per site | 1 or 3 | 3 | 3 |
| Mechanic tilt | For 1 TRxP per site:180° in GCS (pointing to the ground) For 3 TRxPs per site: 110° | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | 90° in LCS | 105° in LCS | 100° in LCS |
| Handover margin (dB) | 1 | 1 | 1 |
| TRxP boresight | For 1 TRxP per site::- For 3 TRxP per site: 30 / 150 / 270 degrees | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | No wrapping around | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=0m | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 | Model-2 in TR36.873 |
| Power control parameters | = 0.6, P0 = -60 dBm | = 0.9, P0 = -86 dBm | For 700 MHz := 0.8, P0 = -76 dBm For 4 GHz := 0.6, P0 = -60 dBm |
| Numerology | One slot with 15 kHz SCS | One slot with 15 kHz SCS | For 700 MHz: - 120 km/h: one slot with 15 kHz SCS - 500 km/h: one slot with 30 kHz SCS For 4 GHz: - 120 km/h: one slot with 30 kHz SCS - 500 km/h: one slot with 60 kHz SCS |
| Scheduling | PF | PF | PF |
| Receiver | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| Pre-processing SINR calculation | Aligned with Section 2.1.1 in R1-1805643 | Aligned with Section 2.1.1 in R1-1805643 | Aligned with Section 2.1.1 in R1-1805643 |