3GPP TSG RAN WG1 Meeting #93							R1-1806089

Busan, Korea, May 21st –25th, 2018

Source:	vivo

Title:	Enhancements to support NR backhaul link

Agenda Item:	7.7.1

Document for:	Discussion and Decision

- Introduction & Background

A couple of issues related to bahchaul resource partitioning are discussed in last RAN1 meeting, relative outcomes of the conclusions are as follows,

Agreements:

- Mechanisms for scheduling coordination, resource allocation, and route selection across IAB nodes/donors and multiple backhaul hops should be studied.

- Mechanisms for efficient TDM/FDM/SDM multiplexing of access/backhaul traffic across multiple hops considering an IAB-node half-duplex constraint should be studied.

This contribution try to focus on semi-static and dynamic backhaul resource partitionin schemes and offers corresponding proposals.

- Resource partitioning between backhaul and access link

TDM / FDM / SDM multiplexing between BH and AC link

In LTE Relay, the backhual and access link are TDM multiplexed from the relay node perspective, as shown in figure 1 below. In order to support more efficient resource utilization, FDM and SDM multiplexing between backhaul and access link has been proposed in NR IAB. One example of FDM multiplexing is shown in figure 2, where the IAB node simultaneously transmit to the donor and UE, using differnt PRB resources.

Figure 1 TDM between backhaul and access link             Figure 2 FDM between backhaul and access link

Although the FDM/SDM mutliplexing approaches has benefits, it also bring some new challenges as following.

- Symbol level timing alignment between backhaul and access link

As shown in figure 2, the receiving timing of backhaul DL and access UL in slot 1 has to be perfectly aligned, with timing offset less than CP. If the propogation distance between backhual and access link are larger enough, some timing control scheme should be considered to maining the timing alignment.

- Power control

As shown in figure 2, the receiving power of backhaul DL and access UL in slot 1 has to be comparable and reach the IAB node receiver target power so that the IAB node can correctly decode both links. Due to the Tx power difference and propagation difference between the two links, some power control mechanism should be considered.

Proposal 1 : Timing control and power control mechanisms should be studied to enable the FDM/SDM multiplexing approache between backhual and access link .

Scheduler coordination between IAB-node and donor gNB

The scheduler coordination bwteeen IAB-node and donor gNB depends on the ways of partitioning BH and AC link,

- For semi-static partitioning

  - For this case, the BH and AC link resource is pre-configured by higher-layer signalling, including time and/or frequency resources.

  - The relay and donor gNB can implement the scheduler independently within its configured BH and AC link.

- For dynamic partitioning

  - For this case, the BH and AC link resource is NOT pre-configured by higher-layer signalling. Instead of that, L1 signalling is used to determine the BH and AC link resource.

  - For example, the DCI from gNB can indicate to the relay whether the upcoming time-slot is for BH or not. Based on that, the relay scheduler determines whether and how to allocate resource for the relay UEs.

As the NR is designed in a dynamic nature and considering the time variant channel condition and variant requirement of backhual and access link resource due to dynamic traffic demand, the dynamic partitioning between backhaul and access link should be studied. Furthermore, considering LTE is semi-staic BH/AC paritioning by using MBSFN configuration, NR can have improvement than LTE if NR adopts dynamic partitioning.

It is worthwhile to know how to determine the Uu link DL/UL resource usage for IAB-node

- Option 1: donor gNB decides Un link DL/UL resource usage, IAB-node further decides the Uu link D/UL resource usage under its Uu cell

  - IAB-node may need to report the required amount of DL/UL resource within its Uu cell based on the buffered UL/DL traffic demand.

  - gNB indicates the backhaul DL/UL time resource for IAB-node, using L1/L2 signaling

- Option 2: donor gNB and IAB-node decides the Un link and Uu link DL/UL resource usage independently and inform each other, gNB and IAB-node further decides the DL/UL resource usage based on coordination.

  - IAB-node and donor gNB may need to exchange the DL/UL resource configuration

A summary of the above cases are as follows

|  |  | Scheduler |  |
| --- | --- | --- | --- |
|  |  | independently schduler for gNB and IAB-node | IAB-node scheduler determines Uu resource according to the gNB L1/L2 signalling on backhaul DL/UL resource allocation |
| Resource usage between BH and AC link | gNB decides the slots used for Un DL/UL | - IAB-node may need to report the required amount of DL/UL resource within its Uu cell based on the buffered UL/DL traffic demand - Semi-static Uu/Un resource partitioning resource | - IAB-node may need to report the required amount of DL/UL resource within its Uu cell based on the buffered UL/DL traffic demand - dynamic Uu/Un resource partitioning |
|  | Independent determined | - IAB-node and donor gNB may need to exchange the DL/UL resource configuration - Semi-static Uu/Un resource partitioning resource | - IAB-node and donor gNB may need to exchange the DL/UL resource configuration - dynamic Uu/Un resource partitioning |

Proposal 2 : Dynamic partitioning between backhaul/access link should be studied.

For both option 1 and 2, a resource indication for the backhaul/non-backhaul resource is needed from donor gNB to IAB node. Considering whether the backhaul resource can be used also for donor gNB access link and  IAB node access link, serveral ways can be considered as follows,

|  |  | Alt 1 : fullly flexible at both donor gNB and IAB node | Alt 2 : flexible for donor gNB | Alt 3 : flexible for IAB node | Alt 4 :only for Un for IAB node |
| --- | --- | --- | --- | --- | --- |
| Backhaul resource indicated by donor gNB | Un DL by donor gNB | Yes | Yes | Yes | Yes |
|  | Un UL by donor gNB | Yes | Yes | Yes | Yes |
|  | Uu DL by donor gNB | Yes | Yes |  |  |
|  | Uu UL by donor gNB | Yes | Yes |  |  |
|  | Uu DL by IAB node (in case of FDM/SDM between BH&AC) | Yes |  | Yes |  |
|  | Uu UL by IAB node (in case of FDM/SDM between BH&AC) | Yes |  | Yes |  |
| Non-backhaul resource indicated by donor gNB | Uu DL by IAB node, | Yes |  | Yes |  |
|  | Uu UL by IAB node, | Yes |  | Yes |  |
|  | Uu DL by donor gNB | Yes | Yes |  |  |
|  | Uu UL by donor gNB | Yes | Yes |  |  |

Note : ‘Yes’ indicates that backhaul resource can be used for corresponding usage.

Alt 4 is fully fixed resource reserved for Un and is not flexible. Alt 2 is similar to LTE relay. Considering further improvement for NR IAB, we consider Alt 1 as a baseline for NR.

- Conclusion

This contribution focus on semi-static and dynamic backhaul resource partitionin schemes and propose that,

Proposal 1 : Timing control and power control mechanisms should be studied to enable the FDM/SDM multiplexing approache between backhual and access link .

Proposal 2 : Dynamic partitioning between backhaul/access link should be studied.