3GPP TSG RAN WG1 Meeting #93															R1- 1806108

Busan, Korea, May 21st – 25th, 2018

Agenda item:		7.6.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Channel access and co-existence for NR-U operation

Document for:		Discussion and Decision

# 1	Introduction

In RAN plenary #75, a new Study Item “Study on NR-based Access to Unlicensed Spectrum” was approved [1]. One of the objectives is that the NR-based unlicensed access design should allow fair coexistence across RATs and within NR-based systems operating in unlicensed spectrum.

- Study NR-based operation in unlicensed spectrum (RAN1, RAN2, RAN4) including

  - Coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements in e.g., 5GHz , 37GHz, 60GHz bands

    - Coexistence methods already defined for 5GHz band in LTE-based LAA context should be assumed as the baseline for 5GHz operation. Enhancements in 5GHz over these methods should not be precluded. NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier;

In RAN1 #92bis [2], the following details are agreed:

- At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

  - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

- Study whether or not the following techniques enhance performance beyond the baseline LBT mechanisms

  - Techniques to cope with directional antennas/transmissions

  - Receiver assisted LBT: RTS/CTS type mechanism

    - On-demand receiver assisted LBT: For example, receiver assisted LBT enabled only when needed

  - Techniques to enhance spatial reuse

  - Preamble detection

  - Enhancements to baseline LBT mechanisms above 7 GHz

- Note: LTE-LAA LBT mechanism are assumed as baseline for evaluations for 5GHz.

In this contribution, we provide our views of the baseline LBT mechanisms in different unlicensed frequency bands. Furthermore, we discuss potential enhancements on UL channel access and LBT mechanism for wider bandwidth operation.

# 2	Baseline LBT mechanism

As unlicensed spectrum is a shared and open resource, every terminal with the same or different RATs should have the equal rights to access the channel. Therefore, coexistence methods are key design features for NR-based operation in unlicensed spectrum. In this section, we discuss baseline LBT mechanisms for NR unlicensed operation in various candidate unlicensed bands.

Below 7 GHz

In order to ensure coexistence fairness with incumbent systems in 5 GHz unlicensed spectrum, LTE Rel-13 and Rel-14 licensed-assisted access (LAA and eLAA) have studied energy detection based LBT procedure for both downlink and uplink operations. NR-Unlicensed targets to operate as a “good neighbour” towards all legacy systems in 5 GHz unlicensed spectrum, including Wi-Fi, LTE LAA. LBT procedures adopted by LTE-LAA offer a good baseline solution for NR unlicensed. However, potential further enhancements based on channel access framework defined in regulation (ETSI regulation) may be considered during the study item.

As mentioned in our pervious contribution [3], opening significant bands on 6 GHz for unlicensed access is currently considered in US and EU regulatory bodies. This means a “greenfield” channel access design could be considered on 6 GHz unlicensed spectrum. However, coexistence fairness should also be considered for a new channel access mechanism, as Wi-Fi systems are also expected to start to use this spectrum and other incumbent systems (such as fixed satellite services (earth-to-space) and fixed services (links)) have been deployed widely. If NR unlicensed access is allowed on 6 GHz, 3GPP could also use LTE-LAA LBT as the baseline coexistence method and may study new coexistence methods and provide the result to corresponding regulatory bodies.

Proposal 1: In below 7 GHz unlicensed spectrum, NR unlicensed can use LTE-LAA LBT procedures as a baseline solution. Further enhancements and new coexistence methods can be considered during the study item.

It should be noted that the total RF output power is restricted by EIRP limitation in both ETSI and FCC regulation. And, both antenna gain and additional beamforming gain should be taken into account into this limitation. That means directional transmission with high beamforming gain cannot lead to additional coverage, if the device is able to reach maximum output power with omni-direction. And, it is more cost efficiency to increase the maximum out power than to introduce additional beamforming gain Hence, the transmission is more favorable to single beam operation in low frequency unlicensed spectrum (e.g., <7 GHz), which leads to much higher efficiency compared to beamforming with beam sweeping operation. If beamformed transmission will not be considered in 5 GHz unlicensed bands, directional LBT is not required.

Proposal 2: Do not support directional LBT for NR-U in below 7 GHz unlicensed bands.

Above 7 GHz (60 GHz)

While the 60 GHz frequencies show high propagation path loss, directional transmissions become highly attractive by providing additional beamforming gain. Furthermore, beamformed transmission and reception can allow for additional spatial multiplexing. As show in Figure 1, Devices with (partially) overlapped coverage can perform beamformed transmission simultaneously.

Figure 1. spatial multiplexing by means of directional transmission.

Since legacy channel access mechanisms in LAA and eLAA are mainly designed for omni-directional transmission, co-existence methods need to be enhanced for beamformed transmission with high antenna gain in 60 GHz spectrum. In the 60 GHz band, the main incumbent systems are 802.11ad and its further evolution 802.11ay, which apply 2.16 GHz channel bandwidth. An LBT mechanism, which is defined by ETSI [2], is mandatory at least in Europe for 60 GHz. Coexistence design for NR unlicensed in 60 GHz should be compliant with this regulation for coexistence fairness. However, potential further enhancements (e.g., to enhance the spatial reuse and to protect beam sweeping operation) may be considered during the study item.

Proposal 3: Coexistence design for NR unlicensed in 60 GHz shall be compliant to regulations in different regions. However, potential further enhancements (e.g., to enhance the spatial reuse and to protect beam sweeping operation) may be considered during the study item.

# 3	UL Channel access enhancement

With the experience of LAA/eLAA channel access design, we find that energy detection based LBT procedure may suffer from the hidden node issue. As shown in Figure 2, the gNB A and the gNB B cannot sense each other due to outside the sensing range. However, the UE A is able to sense both gNBs. Before the gNB A transmits UL grant to the UE A, the gNB senses the channel to be idle with LBT category 4 procedure, although gNB B is transmitting data to UE B. After receiving the UL grant, UE A may sense the channel to be busy, when it performs 25 us one-shot LBT before its UL transmission. Hence, the scheduled UL resources are wasted due to failed UL LBT, which may degrade the UL performance dramatically and result in inefficient spectrum utilization.

Observation 1: Hidden nodes may reduce UL channel access probability in unlicensed spectrum.

Figure 2. Hidden node issue for UL transmission in unlicensed spectrum.

To minimize the impact of hidden nodes on UL transmission, over-booked UL scheduling and channel access can be considered. More specifically, the gNB could schedule UL transmissions for different UEs on the same time-frequency resource. By managing different transmission starting positions, collisions caused by overbooking can be reduced, and UL access probability is increase. Alternatively, with fast energy detection based on necessary signals, the gNB can identify the colliding UEs.

Proposal 4: Overbooked UL transmissions with optional sensing result feedback can be considered to increase UL access probability.

# 4	Channel access for wider bandwidth operation

Different from LTE system, NR supports wider bandwidth operation. The maximum channel bandwidth per NR carrier is 100MHz for <6GHz and 400 MHz for 6~52.6GHz in Rel-15. As indicated in our previous contribution [3], there are hundreds of MHz of spectrum available in 5 GHz and 6 GHz unlicensed frequency bands and multiple GHz of spectrum available in the 60 GHz unlicensed band. Hence, NR-U shall also support wider bandwidth operation due to the benefit of higher spectrum utilization and lower baseband complexity. In the following, our discussion focuses on channel access for wider bandwidth operation in 5 GHz unlicensed spectrum.

As agreed in RAN1 92bis [2]:

- At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

  - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

The LBT can be performed in units of 20 MHz in 5 GHz unlicensed bands.

To perform LBT mechanism on a wideband carrier, one potential way is to split a wideband carrier into multiple 20 MHz sub-bands. This is illustrated in Figure 3, considering subcarrier spacings of 15 kHz, 30 kHz, and 60 kHz. In ETSI regulation, two types of LBT mechanisms are defined for multi-channel operation.

- Type 1: Cat 4 LBT is performed on each 20 MHz operating channel individually.

- Type 2: Cat 4 LBT is performed on a selected primary channel. Other operating channels only need to perform a clear channel assessment (CCA) check of at least 25 us immediately before the transmission. And, the primary channel is 1. chosen uniformly randomly or 2. arbitrarily determined and not changed more than once per second.

Figure 3. Possible NR BWs for 4k FFT and different subcarrier spacings. “20” denotes a 20 MHz sub-band

Proposal 5: Existing multi-channel LBT operation defined in ETSI or LTE-LAA can be used as baseline channel access mechanism for NR-U with wider bandwidth for below 7 GHz unlicensed bands.

Regardless of which type of multi-channel LBT operation is used, it may happen that only a subset of all 20 MHz sub-bands are ready for transmission. This brings a new challenge for wider bandwidth operation in unlicensed spectrum. One straightforward solution is that the device (gNB or UE) can wait for all 20 MHz bandwidth sub-bands to be ready for transmission (e.g., by means of deferred access). However, this approach results in very low channel utilization and puts wideband NR-U into unfavourable position in a coexistence scenario. Another potential solution is to support dynamic bandwidth adaptation according to the outcome of multi-channel LBT. Bandwidth adaptation can increase the efficiency of wideband operation dramatically. More details can be found in our companion contribution [4]. We see that this aspect needs to be carefully considered in the NR-U Study Item.

Proposal 6: LBT for wider bandwidth operation should be studied considering dynamic bandwidth adaptation, including operation with both Carrier Aggregation as well as Bandwidth Parts.

# 5. 	Conclusions

In this contribution, we have discussed channel access related issues for NR unlicensed. Based on the discussion, we make the following observations and proposals:

Proposal 1: In below 7 GHz unlicensed spectrum, NR unlicensed can use LTE-LAA LBT procedures as a baseline solution. Further enhancements and new coexistence methods can be considered during the study item.

Proposal 2: Do not support directional LBT for NR-U in below 7 GHz unlicensed bands.

Proposal 3: Coexistence design for NR unlicensed in 60 GHz shall be compliant to regulations in different regions. However, potential further enhancements (e.g., to enhance the spatial reuse and to protect beam sweeping operation) may be considered during the study item.

Observation 1: Hidden nodes may reduce UL channel access probability in unlicensed spectrum.

Proposal 4: Overbooked UL transmissions with optional sensing result feedback can be considered to increase UL access probability.

Proposal 5: Existing multi-channel LBT operation defined in ETSI or LTE-LAA can be used as baseline channel access mechanism for NR-U with wider bandwidth for below 7 GHz unlicensed bands.

Proposal 6: LBT for wider bandwidth operation should be studied considering dynamic bandwidth adaptation, including operation with both Carrier Aggregation as well as Bandwidth Parts.

# References

- RP-172021, “New SID on NR-based Access to Unlicensed Spectrum”, Qualcomm

- Chairman’s Notes, RAN1 WG #92bis

- R1-1802283, “Spectrum band candidates for NR unlicensed”, Nokia, Nokia Shanghai Bell

- R1-1806112, “On Wideband Operation for NR-U”, Nokia, Nokia Shanghai Bell