3GPP TSG RAN WG1 Meeting #93		R1-1806352

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	6.2.5.4

Source:	Lenovo, Motorola Mobility

Title:	Discussion on Latency Reduction for V2X Phase 2

Document for:	Discussion and Decision

Introduction

This is an updated contribution of R1-1804206.

During the RAN1#91 and #92bis meeting, regarding latency reduction, the related agreements are made as follows:

# Agreement

- The minimum value of T2 can be reduced to support Layer 1 latency reduction.

- (Pre)configuration based selection of minimum value of T2 is supported.

- The minimum value of T2 is selected from a set of values.

  - The set of values includes at least 20ms, and a value lower than 20ms (FFS how many additional values).

  - FFS: whether the (pre)configuration is per PPPP, CBR range, per carrier, or if it intends to have a similar behaviour as a rel-14 UE, etc.

# Agreement

The minimum (pre-)configurable T2min is [10]ms.

The maximum (pre-)configurable T2min is 20ms.

The determination of T2min

- For each PPPP, the T2min is (pre-)configured by RRC.

Note: The actual value of T2 (>=T2min) is left to UE implementation.

In this contribution, we discuss the potential issues and enhancements due to the minimum T2min is reduced to [10]ms for V2X mode 4 autonomous sensing and resource selection .

Discussion

After T2min is reduced to [10]ms, i.e., the resource selection windows size is shortened to 10ms (or less), the number of the candidate resource may not meet the requirement of 20% of all the resource  in selection window. Even physical layer still reports at least 20% candidate resource of  by increasing the parameter by 3dB. Some of the reported candidate resources may have lower SINR. From system’s perspective, it will reduce the range of sidelink transmission and increase error/collision probability. So the reporting percentage of  and the  increment needs to be enhanced in stringent latency requirement case.

Proposal 1: The reporting percentage of and the  increment need to be enhanced in stringent latency requirement case.

As mentioned above, the reporting percentage of and the  increment can be adjusted in stringent latency requirement case. But blindly reducing the reporting percentage of and restricting  increment may decrease the number of reported candidate resource, which will also lead the potential collision in resource selection step at the higher layer. Especially, when the sidelink payload is relatively low, this kind of adjustment is not needed. Because eNB can control the number of active UEs and sidelink payload in configured resource pool, the parameters can be (pre)configured or reconfigured by eNB to adapt , Moreover, eNB can also configure the out of coverage UE to determine the parameters corresponding to  and UE physical layer measured sidelink information, e.g., channel busy ratio (CBR).

Proposal 2: The reporting percentage of and  increment can be (pre)configured by eNB corresponding to.

Proposal 3: UE physical layer can determine the reporting percentage of and increment corresponding to  and measured channel busy ratio.

Conclusion

In this contribution, we focus latency reduction for V2X mode 4 communication. We have the following proposals:

Proposal 1: The reporting percentage of and the  increment need to be enhanced in stringent latency requirement case.

Proposal 2: The reporting percentage of and  increment can be (pre)configured by eNB corresponding to.

Proposal 3: UE physical layer can determine the reporting percentage of and increment corresponding to  and measured channel busy ratio.

- References

- [1] R1- 1804206, Discussion on Latency Reduction for V2X Phase 2, Lenovo and Motorola Mobility, RAN1#92bis

- [2] 3GPP TS 36.213 V14.5.0 (2017-12), “Physical layer procedures (Release 14)”

- [3] 3GPP TR 22.886 V15.1.0 (2017-03), “Study on enhancement of 3GPP support for 5G V2X services”