3GPP TSG RAN WG1 Meeting #93		R1-1806376

Busan, Korea, May 21st – 25th, 2018

Source:	CMCC

Title:	Initial self evaluation results for eMBB spectrum efficiency

Agenda Item:	7.8.2

Document for: 	Discussion

- Introduction

At RAN#77 meeting, the preliminary work plan of self evaluation was agreed in [1], where a three-step plan is made. At RAN#78 meeting, it was agreed in [2] that 3GPP will start Step 2 activity for self evaluation study, which includes the performance evaluation against the eMBB, mMTC and URLLC technical performance requirements as defined in Report ITU-R M.2410 (see [3]).

According to Report ITU-R M.2412 (see [4]), simulation is needed to evaluate spectral efficiency (average, 5th percentile user), mobility, user experience data rate, connection density and reliability.

In this contribution, based on the summary of offline discussion in RAN1#92 (see [5]), some evaluation results on DL spectral efficiency, in the Indoor-eMBB, Dense Urban-eMBB and Rural-eMBB test environment, are shown.

- Test environments for eMBB DL spectral efficiency

According to Report ITU-R M.2410 (see [3]), the key minimum technical performance requirements for spectral efficiency are provided in Table 1.

Table 1. Minimum requirements for average spectral efficiency and 5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) |  |
| --- | --- | --- |
|  | Average spectral efficiency | 5th percentile user spectral efficiency |
| Indoor Hotspot – eMBB | 9 | 0.3 |
| Dense Urban – eMBB | 7.8 | 0.225 |
| Rural – eMBB | 3.3 | 0.12 |

NOTE 1 – This requirement will be evaluated under Macro TRxP layer of Dense Urban.

The eMBB test environments and their related evaluation configurations are employed in Report ITU-R M.2412 (see [4]) to test the fulfillment of the spectral efficiency requirements. In this contribution, a subset of the eMBB test environments and the related evaluation configurations as given in Table 2 is considered for initial spectral efficiency evaluation.

TABLE 2. Evaluation configuration for initial spectral efficiency evaluation

| Test env. | Evaluation configuration | Carrier frequency | ISD | Duplex scheme | Bandwidth | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor Hotspot – eMBB | Config. A | 4 GHz | 20 m | TDD | 20MHz |  |
| Dense Urban – eMBB | Config. A | 4 GHz | 200 m | TDD | 20MHz | Macro layer only |
| Rural - eMBB | Config. A | 700 MHz | 1732 m | TDD | 20MHz |  |

- Initial evaluation results

Based on the above discussion, initial evaluation results for average spectral efficiency and 5th percentile spectral efficiency is evaluated. Evaluation assumptions can be found in Appendix.

The initial DL spectral efficiency evaluation results for NR are given in Table 3.

TABLE 3. Initial DL spectral efficiency evaluation for TDD

| Test env. | Evaluation config. | Average spectral efficiency (bit/s/Hz/TRxP) |  | 5th percentile spectral efficiency (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BW=20MHz | Req. | BW=20MHz | Req. |
| Indoor Hotspot | Config. A (4 GHz); 32T4R | 10.88 | 9 | 0.30 | 0.3 |
| Dense Urban | Config. A (4 GHz); 32T2R | 14.46 | 7.8 | 0.35 | 0.225 |
| Rural | Config. A (700 MHz);8T2R | 7.22 | 3.3 | 0.23 | 0.12 |

Observation 1: NR with sub 6 GHz has the potential to fulfill DL spectral efficiency requirements in all test environments.

- Conclusions

In this contribution, we provide our preliminary evaluation results for spectral efficiency in eMBB scenario. We have the following observation.

Observation 1: NR with sub 6 GHz has the potential to fulfill DL spectral efficiency requirements in all test environments.

References

- RP-172101, “WF on Work plan of Self Evaluation SI”, Huawei, Ericsson, Telecom Italia, September 2017.

- RP-172532, “Work plan for self evaluation towards 3GPP Update Submission to ITU-R”, Huawei, Ericsson, Telecom Italia, December 2017.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation”, RAN1 #92, Huawei, Athens

Appendix: Evaluation assumptions

Table A-1. Evaluation assumptions for DL TDD

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Test environment | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural – eMBB |
| Evaluation configuration | Configuration A | Configuration A | Configuration A |
| Channel model | InH_A | Uma_A | Rma_A |
| ISD | 20 m (12TRxP) | 200 m | 1732 m |
| TDD frame structure | DDDSU | DDDSU | DDDSU |
| Carrier Frequency | 4 GHz | 4 GHz | 700 MHz |
| System bandwidth | 20MHz | 20MHz | 20MHz |
| Subcarrier spacing | 30 kHz | 30 kHz | 30 kHz |
| Max PRB number | 51 | 51 | 51 |
| Symbols number per slot | 14 | 14 | 14 |
| Number of antenna elements per TRxP | 32Tx cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1) | 128Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) | 64Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1) |
| Number of TXRU per TRxP | 32TXRU | 32TXRU: Vertical 2-to-8 | 8TXRU: Vertical 1-to-8 |
| Number of antenna elements per UE | 4Rx with 0°,90° polarization | 2Rx with 0°,90° polarization | 2Rx with 0°,90° polarization |
| Transmit power per TRxP | 24 dBm | 44 dBm | 49 dBm |
| TRxP number per site | 1 | 3 | 3 |
| Mechanic tilt | 180deg in GCS (pointing to the ground) | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt | 90deg in LCS | 105deg in LCS | 100deg in LCS |
| UT attachment | Based on RSRP (formula) from CRS port 0 | Based on RSRP (formula) from CRS port 0 | Based on RSRP (formula) from CRS port 0 |
| Scheduling | MU-PF | MU-PF | MU-PF |
| MIMO mode | MU-MIMO with rank 2 adaptation per user; Maximum MU layer = 12 | MU-MIMO with rank 2 adaptation per user Maximum MU layer = 12 | MU-MIMO with rank 2 adaptation per user Maximum MU layer = 4 |
| Guard band ratio | 8.2% (for 20 MHz) | 8.2% (for 20 MHz) | 8.2% (for 20 MHz) |
| BS receiver type | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| CSI feedback | 5 slots period based on CSI-RS (ideal) with delay | 5 slots period based on CSI-RS (ideal) with delay | 5 slots period based on CSI-RS (ideal) with delay |
| Precoder derivation | SRS (ideal) based | SRS (ideal) based | SRS (ideal) based |
| Waveform | OFDM | OFDM | OFDM |