3GPP TSG-RAN WG1 Meeting #93	R1-1807005

# Busan, Korea, May 21th – 25th, 2018

Agenda Item:	7.1.3.6

Source:	Apple Inc.

Title:	Enhancements for NR UE Power Saving

Document for:	Discussion, Decision

# Introduction

Power consumption of mobile device has been one of the important issues not only from energy efficiency perspective but also from usability/marketability perspective. Since mobile devices has limited form factor and limited space for battery, being power efficient to certain extent is not enough for its usability/marketability. It must be power efficient enough to keep device on for long enough time while supporting required data transmissions. This is especially true for IoT devices which require long lasting battery life. The enhancements in MTC/NB-IoT in R15 for power efficient signaling made MTC and NB-IoT devices very attractive choices in IoT market in that sense.

Although LTE made a great success with higher throughput and low latency, later it turned out that LTE is not efficient enough in terms of power consumption especially at UE side. One of frequently noted reason for the low energy efficiency is that there are a lot of PDCCH blind decodings with no PDSCH transmissions in RRC CONNECTED mode. Although the continuous blind decoding operation is beneficial to network for scheduling flexibility and low latency in data transmission, it entails a lot of power consumption at UE side. This has been noted many times through thorough evaluations. For example, in [1] it has been observed that PDCCH blind decoding without grant consumes 60~75% of energy in RRC CONNECTED mode. In [2], it has been noted that 65% of power is consumed by PDCCH blind decoding without grant and low data rate services. [3] has reported that about 55% ~ 65% of power used for blind decoding without grant under the assumption of high speed data transmission.

Another factor making LTE less power efficient is un-optimized DRX configuration. The DRX mechanism was originally designed to make UE more power efficient. It is designed as quite flexible so that it can support various different traffic arrival scenarios. However, from field logs, we observe that DRX parameters are fixed most of time. This means system does not adapt to different traffic arrival rates and accordingly it is less effective for UE power saving than intended.

Carrier aggregation is another reason increasing UE power consumption for high rate data transmission. Although it can be activated and deactivated depending on traffic demand, the energy cost is high and less usable than originally intended. It became one of motivations for introducing wider bandwidth in NR and dynamic bandwidth adaptation using BWP.

Although there have been some efforts for NR UE power saving during the NR standardization until Dec. 2017 we think it needs further focused considerations on UE power saving. The current techniques such as configurable PDCCH monitoring period, bandwidth part (BWP), cross-slot scheduling can indeed reduce UE power consumption. However, it is not very clear at this point how much power saving we could expect, and there are many more aspects we didn’t consider due to limited time. We believe that optimizing power consumption of NR UE could greatly increase the usability/marketability of NR especially for high/higher data rate applications.

In this paper, we discuss NR UE power consumption and potential enhancements.

# NR UE Power Consumption and Potential Enhancements

In NR, we have new features to support higher data rates applications (eMBB) entailing higher power consumption. To overcome increased power consumption, techniques such as BWP and periodic PDCCH monitoring were introduced.

## Wider bandwidth and BWP/CA

NR supports higher data rate with wider bandwidth (up to 400MHz for single carrier). This requires much higher rate of data processing in both RF side and baseband side; supporting higher sampling rate, smaller slot lengths, higher encoder/decoder throughput, faster clocks, higher voltage (with higher leakage), larger memory, and LTE-NR dual connectivity may require significantly higher power consumption. Especially due to the support of mmWave, NR needs additional power consumption for beam forming operation and beam management procedure.

The BWP and cross-slot scheduling were introduced to overcome the increased power consumption. Network can use BWPs to adapt bandwidth to the demand for data transmission/reception. Network can configure narrow BWP for monitoring PDCCH only and wider BWP for receiving large-sized data. Channel bandwidth adaptation using BWP is a quite effective power saving technique.

However, whether UE can expect power saving or not is totally up to network configuration / scheduling decision. If network does not configure multiple BWPs with different sizes or if network does not change BWPs considering traffic demand and UE power consumption, then, even with multiple different BWPs configured, there will be no power saving at UE side (e.g., the case when BWP is used just for load balancing purpose). Similar example in LTE is DRX configuration. If only very simple configuration is used for network complexity, then it is hard to expect intended power saving gain at UE side. UE having better knowledge on channel status and traffic demand/pattern/usage may help network to make better decisions for both BWP/CA configuration and scheduling in both CONNECTED mode and IDLE mode as shown in Figure 1.

Observation 1:

- UE having better knowledge on channel status and traffic demand/pattern/usage could help network to make better decisions for both BWP/CA configuration and change.

Figure 1 Current BWP framework (left) and improved BWP framework 
with feedforward/feedback information (right)

### UE-initiated Approach

If UE can indicate its preferred BWP/CA configuration, then UE can potentially achieve higher power saving. It is clear that UE has more information about device owner’s preference on applications, types/amount/duration of traffics to be received/transmitted, etc. This kind of information could be used in conjunction with internal power consumption statistics to determine the right size of BWP and number of CCs to be used. Another reason is that different UE could have different RF capabilities and different implementations. This means optimal BWP/CA for a certain traffic could be different for different UEs. For example, the set of supported sampling rates could be different across UEs. This means that it can happen that, e.g., a UE A is configured with a 6MHz BWP with sampling rate of 8MHz, while another UE B is configured of a 6MHz BWP with sampling rate of 10MHz. This implies that the power consumption of using the 6MHz BWP could be different between UE A and UE B and it might be better for UE B to be configured with a BWP larger than 6MHz (e.g., 10MHz) since the same sampling rate is used anyway. For similar reason, it is also beneficial for UE power saving for UE to indicate network preferred CCs to be configured.

If UE can indicate its preferred BWP/CA change, UE can potentially achieve higher power saving gain. We see that there are strong motivation for network that it configures larger BWP and larger number of component carriers (CC). It is because that, with larger BWPs and larger number of CCs, network can keep its scheduling flexibility high to improve network capacity. Thus, in many cases, we see that component carriers are active without much data transmission. In such case, UE should be able to request to use a smaller BWP or turn off unused/idling component carriers.

Proposal 1:

- NR support that UE send UE-preferred BWP/CA configuration to network for UE power saving.

- NR support that UE send UE-preferred BWP/CA change to network for UE power saving.

## Periodic PDCCH Monitoring

Another important new feature in NR is the support of periodic PDCCH monitoring before UE enters CDRX state. With this feature, UE can monitor PDCCH less frequently and accordingly the chance that UE receives PDCCH (with grant) increases. Consequently, it can reduce the amount of power wasted for PDCCH only monitoring. This kind of approaches of controlling the so-called duty cycle of UE’s ON state could be promising approach for UE power saving.

### Periodicity

With the introduction of mmWave, the importance of periodic PDCCH monitoring gets even higher since it is power consuming operation to monitor slots with short length continuously in mmWave. The current R15 NR supports up to 20 slots of PDCCH monitoring periodicity which corresponds to 20ms with SCS=15kHz. However, since the periodicity was defined in the unit of slot, it corresponds to only 2.5ms with SCS of 120kHz. Due to the design/decision, the maximum time that UE can skip PDCCH monitoring in absolute time depends on SCS and it is not desirable/enough for NR in FR2 to achieve similar power saving gain as NR in FR1.

In Figure 2, we show the PDCCH monitoring periodicity of 10 slots as an example for two different SCS=15kHz and 120kHz. For 15kHz case, UE monitors the second slot of one periodicity and takes a deep sleep for the rest of the period with no PDSCH scheduled. While in 120kHz case, UE has to wake up every 1.25ms (10 slots in 120kHz). Due to associated overhead (RFIC ramp up & RFIC ramp down) and short sleep time (which allows UE to enter shallow sleep state only), the expected power saving gain is much lower than the case of 15kHz.

Observation 2:

- For NR in FR2, it is hard to achieve similar power saving gain as NR in FR1 with currently defined PDCCH monitoring (in general duty cycle) periodicities.

Proposal 2:

- It is required to improve PDCCH monitoring (in general duty cycle) mechanism especially for FR2.

Figure 2 Expected power consumption with PDCCH monitoring periodicity of 10 slots for SCS=15kHz (up) and 120kHz. For 15kHz case, UE can enter deeper sleep state. While for 120kHz case, UE cannot enter deep sleep state due to too short PDCCH monitoring periodicity.

### Irregular Wake up/Sleep Pattern

Another potential problem of the periodic PDCCH monitoring with dynamic PDSCH scheduling is that it introduces irregular wake up pattern in UE side which may not be desirable for UE power saving. Modern wireless modem has different levels of sleep states and modem can enter deeper sleep state when it has longer time to sleep. It is because different component blocks require different amount of time to enter and come out from low power state. Thus, it is desirable to optimize PDCCH monitoring period (or in general duty cycle) and data scheduling jointly such that tx/rx activities occurs as close as possible to each other. This will reduce unnecessary OFF and ON operation of component blocks (e.g., RFIC).

Observation 3:

- Joint optimization of PDCCH monitoring mechanism and scheduling could reduce unnecessary OFF/ON of UE modem component blocks, which could provide longer sleep duration for entering to deeper sleep states.

### Throughput Limitation

If UE monitors PDCCH periodically with periodicity P, the scheduling opportunity is limited by factor of P. This limits UE’s achievable throughput by the factor of P. This may be fine when the data arrival is predictable since then network can choose optimal monitoring periodicity based on the predicted throughput and delay. However, if traffic arrival gets dynamic and bursty, then network may want to adapt monitoring periodicity according to increased or decreased traffic demand to maximize power saving gain.

Observation 4:

- If UE performs periodic PDCCH monitoring with periodicity P, then the UE’s throughput is limited by a factor of P.

Dynamic change of PDCCH monitoring periodicity is supported in BWP framework but it seems that it is not main use case. With current specification, network can configure multiple BWPs with same bandwidth and same center-frequency and dynamically change BWPs among those with same bandwidth and same center-frequency. This operation does not require RF reconfiguration. Thus, the required “re-tunning” time could be minimal. However, this is not the main use case considered during BWP framework design. Furthermore small number of maximum number of BWP (4) is another practical difficulty which makes it hard to use BWP framework for such purpose. One approach to overcome such throughput limitation is to support scheduling of multiple TBs with single DCI. However, this approach would require UE to either monitor additional DCI size or the change of DCI size UE monitors, which is not desirable. Others more promising approaches are to provide additional PDCCH monitoring occasions based on rx/tx activity or dynamically change monitoring periodicity explicitly.

Observation 5:

- In order to support dynamically changing traffic arrival, PDCCH monitoring mechanism needs to be improved to have no or relaxed throughput limitation.

## Explicit Signal based Wakeup & Sleep Approach

In R15 MTC and NB-IoT, it was agreed to introduce power efficient wake up type signal for UE power saving. The focus was on IDLE mode power saving enhancement since IoT devices stay mostly in IDLE state for extending battery life. In this sense, the context is slightly different from NR UE power saving, where we think eMBB is most popular use case. Accordingly, the approach for power saving signal in NR could be very different from that in MTC/NB-IoT.

To achieve meaningful power saving gain, NR needs power saving techniques for CONNECTED mode. Considering that total power contribution from Idle mode is relatively small compared to that in CONNECTED mode [1][2][3], it becomes clear that power saving techniques for CONNECTED mode could be much more effective than that for IDLE mode. So, it should be the focus of NR UE power saving study.

Observation 6:

- It was noted that power saving for CONNECTED mode is more effective than that for IDLE mode.

Proposal 3:

- NR UE power saving SI should focus on UE power saving technique in CONNECTED mode.

As an example, we could think of explicit signal-based UE power saving mechanism, which is also known as wake up and sleep mechanism as shown in Figure 3. Due to the dynamic change of traffic arrivals, configuring static PDCCH monitoring period or CDRX parameters may not be enough to effectively handle dynamic data transmission / reception in power efficient manner. With explicit signaling approach, network can dynamically handle such situation while maximizing power saving gain.

Observation 7:

- The explicit signal-based wake up & sleep mechanism is more effective in supporting dynamic traffic arrivals and UE power saving.

Figure 3 Expected UE power consumption with network assisted go-to-sleep type signal

## Scheduling Considering UE Power Saving

NR has very flexible HARQ scheduling timing. Unlike LTE with fixed HARQ timing, the transmission timing for NR’s PDSCH and corresponding ACK can be dynamically determined and indicated by DCI. The increased flexibility makes network scheduler much easier to serve multiple UEs with different processing capability, different traffic demand, different latency requirement, etc.

We see that a certain way of scheduling could help UE power saving. For example, note that a UE which receives PDCCH in slot N, PDSCH in slot N+4, and sends ACK in slot N+8 can enter micro sleep state between slot N and N+4 and between N+4 and N+8. If network can schedule this UE with PDCCH, PDSCH, and ACK all in the same slot N (self-contained slot scheduling), then the UE can quickly finish data reception and enter deeper sleep mode for longer time, which save more energy as shown in Figure 4. Since there is power cost for UE whenever it wakes up and enters sleep state, it is better to reduce those power cost and make modem sleeps longer. To maximize UE power saving gain while fully taking advantage of flexibility of NR scheduling mechanism, we need to study and identify set of system parameter configurations which help UE saving power and how those configurations could be more easily enabled.

Proposal 4:

- In NR UE power saving SI, identify the set of system parameter configurations helpful for UE power saving and study how those configurations could be more easily enabled.

Figure 4 Power consumption for NR HARQ scheduling with both K0, K1>0 (up) and NR self-contained slot scheduling with K0=K1=0 (down)

## Measurement and CSI Reporting for Power Saving

The absence of cell reference signal (CRS) in NR has affected many aspects of NR design. One example is channel state information framework. In NR, UE can be configured with resources setting, reporting resetting, and measurement link. The combined configuration of them makes UE to perform measurement of certain set of CSI-RS resources and report CSI accordingly. One important observation is that the measurement and reporting could be quite inefficiently configured in terms of power consumption especially when UE is in CDRX state, as shown in Figure 5. To reduce power consumption, it is better to have those measurements and reporting be aligned with UE’s CDRX ON duration. This kind of misalignment is one of the cases we can further optimize in power saving context. As noted in [3], relatively too frequent measurement than required is also one area we can further improve in power saving context.

Observation 8:

- For Power saving, it is required to optimize measurement/reporting timing in both CONNECTED state and IDLE state.

Figure 5 Misalignment between measurement/report configuration and ON duration

# Conclusions

In this contribution, we have identified potential enhancements for NR UE power saving. In our understanding, there is no single magic feature which can significantly reduce UE power consumption. It is rather collection of well-designed/optimized feature sets that can reduce UE power consumption additively. Thus, we think the RAN1 should investigate UE power saving techniques with a scope covering many different aspects of NR described above – BWP/CA, PDCCH monitoring, Scheduling, Measurement. Our observations/proposals are as follows.

Observation 1:

- UE having better knowledge on channel status and traffic demand/pattern/usage could help network to make better decisions for both BWP/CA configuration and change.

Proposal 1:

- NR support that UE send UE preferred BWP/CA configuration to network for UE power saving.

- NR support that UE send UE preferred BWP/CA change to network for UE power saving.

Observation 2:

- For NR in FR2, it is hard to achieve similar power saving gain as NR in FR1 with currently defined PDCCH monitoring (in general duty cycle) periodicities.

Proposal 2:

- It is required to improve PDCCH monitoring (in general duty cycle) mechanism especially for FR2.

Observation 3:

- Joint optimization of PDCCH monitoring mechanism and scheduling could reduce unnecessary OFF/ON of UE modem component blocks, which could provide longer sleep duration for entering to deeper sleep states.

Observation 4:

- If UE performs periodic PDCCH monitoring with periodicity P, then the UE’s throughput is limited by a factor of P.

Observation 5:

- In order to support dynamically changing traffic arrival, PDCCH monitoring mechanism needs to be improved to have no or relaxed throughput limitation.

Observation 6:

- It was noted that power saving for CONNECTED mode is more effective than that for IDLE mode.

Proposal 3:

- NR UE power saving SI should focus on UE power saving technique in CONNECTED mode.

Observation 7:

- The explicit signal-based wake up & sleep mechanism is more effective in supporting dynamic traffic arrivals and UE power saving.

Proposal 4:

- In NR UE power saving SI, identify the set of system parameter configurations helpful for UE power saving and study how those configurations could be more easily enabled.

Observation 8:

- For Power saving, it is required to optimize measurement/reporting timing in both CONNECTED state and IDLE state.

# References

[1] R1-166368 UE Power Consideration based on Days-of-Use, Qualcomm, Aug, 2016

[2] R1-1609557 On Techniques for Enhanced UE Power Efficiency, MediaTek, Oct, 2016

[3] RP-172318 Motivation for Study on UE Power Saving in NR, VIVO, Dec, 2017

This contribution is updated version of RP-1804773.