3GPP TSG-RAN1 Meeting #93			R1-1807889

Busan, Korea, May 21st – 25th, 2018

Title:	Draft Reply LS on the design of PRACH table

Response to:	R1-1805803 (R2-1806227)

Work Item:	NR_newRAT-Core

Source:	Qualcomm

To:	RAN WG2

Cc:

Contact Person:

#### Name: Nazmul Islam

E-mail Address:	mislam@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall Description:

RAN1 thanks RAN2 for the liaison statement “LS on the design of PRACH table” R1-1805803(R2-1806227) and informs the following RAN1 agreement.

- PDCCH order for RACH procedure includes the following fields:

  - Random Access Preamble index – 6 bits. Indicating which Random access preamble to use in case of contention-free random access procedure, or the value 000000 in case of contention-based random access procedure

  - For CFRA only:

    - no BWP index is necessary in Rel-15

    - SUL indicator – 1 bit. Indicating whether to transmit the Random access preamble on SUL or normal uplink carrier

    - SSB index - 6 bits are used to indicate an SSB index, which, in turn, identifies a group of RACH occasions

    - PRACH Mask index – 4 bits are used to indicate PRACH Mask index that corresponds to the RO(s) in the set of RACH occasions identified by the indicated SSB index

      - RAN1 supports RAN2’s agreements regarding the PRACH table as in R1-1805803.

      - The set of RACH occasions identified by the indicated SSB index within one association period are ordered in the following way:

        - In the order of increasing frequency multiplexed RACH occasions and then

        - In the order of increasing time multiplexed RACH occasions within a RACH slot and then

        - In the order of increasing RACH slot indices

2. Actions:

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into consideration.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN1 Meeting #94			20th – 24th August 2018			Gothenburg, Sweden

TSG-RAN1 Meeting #94bis		8th – 12th October 2018			Chengdu, China