3GPP TSG RAN WG1 Meeting #94																R1-1808697

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	Intel Corporation

Title:	Remaining Aspects of eV2X Evaluation Methodology and Assumptions

Agenda item:	7.2.4.1.5

Document for:	Discussion and Decision

Introduction

At the RAN#80 meeting, the study item on NR V2X was approved [1]. This new study is supposed to conduct link level and system level evaluations following eV2X evaluation methodology defined in [2] based on the outcome of the recently completed study [2].

In this contribution, we discuss remaining details that need to be agreed or revised for eV2X link and system level evaluations. Our views on NR V2X design aspects are summarized in companion contributions [5]-[13].

eV2X Link Level Evaluation Assumptions

In eV2X sidelink design, the synchronization and demodulation performance of major sidelink physical channels and signals such as SLSS/PSBCH/PSCCH/PSSCH is expected to be evaluated to come up with the optimized design options for physical structure of sidelink channels and signals, e.g. PSSS/SSSS/PSBCH/PSCCH/PSSCH. In the latest version of TR, the link level evaluation assumptions need to be further refined to cover sidelink synchronization and demodulation frameworks. This step is necessary to align basic parameters and assumptions across companies as well as to simplify comparative analysis of different design proposals.

The following set of link level parameters was proposed in [2] for link level evaluations. In our view, majority of these parameters are solution dependent although it is clearly desirable to have alignment at least on subset of parameters. At RAN1#92, it was agreed each company is expected to clarify many of link level parameters. In our view each parameter needs to be reviewed one-by-one and further discussion is needed whether parameter can be left up to company decision or should be defined by evaluation methodology. Below, we provide our view on how to modify existing section “6.3 Link Level Evaluation Assumptions” in TR 37.885.

  - Update section 6.3 in TR 37.885 as follows

    - Replace current text in section 6.3 with the Text Proposal #1 in this document

--------------------------------------------------------------- Text Proposal #1----------------------------------------------------------------

This section provides link level evaluation assumptions for eV2X study.

6.3.1 Common Link Level Assumptions for Sidelink Synchronization and Demodulation

Carrier frequency

  - Sidelink (PC5)

    - FR1: 6 GHz,

    - FR2: 30 or 63 GHz

  - Downlink/Uplink (Uu)

    - FR1: 4 GHz,

    - FR2: 30 GHz

Channel model (e.g. fast fading)

  - Both TDL and CDL channel models can be used (indicated in evaluation assumptions)

  - Channel model parameters defined in this technical report are used for analysis

Relative vehicle speed

  - 60 km/h (low), 240 km/h (mid), 500 km/h (high)

Number of antennas (at UE and gNB)

  - UE

    - FR1: Up to 8 Tx /Rx antenna elements. Baseline [2 TX, 2 RX] or [2 TX, 4 RX]

    - FR2: Up to 32 Tx /Rx antenna elements. Baseline antenna array configuration

- (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1)

  - gNB

    - FR1: Baseline antenna array configuration (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1)

    - FR2: Baseline antenna array configuration (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2)

AGC settling time

  - Requires input from RAN4 WG. Can be revised based on RAN4 feedback

EVM (at TX and RX)

  - Refer to values defined in TS 38.101-1 and 38.101-2

  - FR1 TS 38.101-1

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

  - FR2 TS 38.101-2

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| Pi/2 BPSK | % | 30 | 30 |
| QPSK | % | 17.5 | 17.5 |
| 16 QAM | % | 12.5 | 12.5 |
| 64 QAM | % | 8 | 8 |

- Note: for demodulation performance smaller typical values can be considered (e.g. 6% for ≤ 64QAM)

TX PA non-linearity model

  - Up to proponent. Assumption for non-linearity model are provided if simulated

TX/RX phase noise model (targeting high frequency band)

  - If considered, Example Model 1 and Model 2 from TR.38.803 can be used

6.3.2 Link Level Evaluation Assumptions for Sidelink Synchronization

This section provides link level evaluation assumptions and parameters for evaluation of sidelink synchronization performance:

The following target requirements should be taken into account in NR SLSS design

  - Robustness against initial frequency offset up to +/-10 ppm

Reasonable complexity for NR SLSS (PSSS/SSSS) detection

  - Good one-shot detection probability at -6 dB received baseband SNR condition with less than 1% false alarm rate

  - Companies report detection probability, the residual timing error and frequency error

The following aspects can be considered (not an exhaustive list)

  - Low system overhead due to NR SLSS (PSSS/SSSS) transmission

  - Low PAPR of waveform for possible power boosting transmission

The following target requirements should be taken into account in the NR PSBCH design

  - Detectable at low received baseband SNR condition

  - Note: it does not mean NR PSBCH should be detectable by one-shot

Performance metrics

For evaluation of sidelink synchronization, the synchronization accuracy for remaining carrier frequency (CFO) and timing offset (TO) is analyzed (e.g. CDF of CFO and TO estimation errors vs SNR).

Note:

  - If multiple UEs transmit the same signal in SFN manner, each specific transmission should be characterized by independent timing and frequency error as well as Doppler. In this case, useful RX power for SNR definition is a power combined from all sources transmitting the same signal.

6.3.3 Link Level Evaluation Assumptions for Demodulation of Sidelink Physical Channels

This section provides additional link level evaluation assumptions and parameters relevant for evaluation of sidelink channel demodulation performance.

Packet size

  - Control channel: 64bits +24bit CRC

  - Shared channel: 200 bytes, 1200 bytes,  24000 bytes

Frequency synchronization error (post sync error)

  - Case 0: Uniform distribution within +/- 0.1ppm for both TX and RX w.r.t. the absolute carrier frequency

  - Case 1: Extreme case, i.e. +0.1 PPM for TX and -0.1 PPM for RX w.r.t. UE’s sync reference.

  - Case 2: Frequency error in each UE is uniformly distributed [-0.1, 0.1] PPM w.r.t. UE’s sync reference.

    - Case 2a: 0ppm (i.e. same reference)

    - Case 2b: +0.05 PPM for TX’s reference and -0.05 PPM for RX’s reference w.r.t. the absolute frequency

Time synchronization error

  - Should be a part of eV2X evaluation methodology  +/- 0.4us w.r.t. reference

Modulation and code rates (for control and data channels)

  - Control channel: Left up to proponents

  - Shared channel: QPSK CR = ½ , 16QAM CR = ½ , 64QAM CR = ½For eV2X evaluations, it is important to conduct analysis for both noise limited and interference limited scenarios.

Noise limited scenario

In case of noise limited scenario, the PER and spectral efficiency are evaluated vs SNR in AWGN conditions.

Interference limited scenario

In case of interference limited scenario, SNR is set to = 25dB. SINR varies in the range -5..20 dB. Single and two dominant interferer scenarios are considered. In case of two dominant interferers, the power difference between two interferer sources is set to 5dB.

Performance metrics

For evaluation of sidelink demodulation performance, the PER and spectrum efficiency statistics are used for evaluation of demodulation performance vs SNR and SINR in noise and interference limited scenarios respectively.

- 6.3.4 Design specific aspects and assumptions

Channel codes (for control and data channels)

  - Effective code rates may be fixed instead

Signal waveform (for control and data channels)

  - Waveforms supported by NR system are baseline

Subcarrier spacing

  - SCSs values defined by NR system are baseline

CP length

  - CP lengths defined by NR system are baseline

Reference signals

  - Details of reference signals used for demodulation are provided by proponents

Number of retransmissions and combining (if applied)

  - Design details are provided by proponents

Transmission diversity scheme (if applied)

  - Design details are provided by proponents

Channel estimation

  - Practical channel estimation. Details up to proponents

UE receiver algorithm

  - Baseline receiver is MMSE-IRC. Implementation details are provided by proponents

---------------------------------------------------End of Text Proposal #1 -----------------------------------------------------

eV2X System Level Evaluation Assumptions

Wrap Around Considerations

During the work on eV2X evaluation methodology, the LTE V2X channel modelling framework was revised based on additional studies and measurement data [14], [15]. In particular, new propagation type NLOSv was introduced for the links between two vehicles if line-of-sight between TX and RX is blocked by intermediate vehicle. In addition, the LOS model was revised to free-space propagation model. The new propagation characteristics have lower attenuation and thus radiated signals can propagate to a longer distances. In order to properly address sidelink interference issues, the proper layout characteristics need to be selected so that wrap-around principle can be applied for system level simulations.

In [2], it is stated that in case of Highway evaluation scenario the “Highway length >= 2000 m. Wrap around should be applied to the simulation area”. To analyze the size of the simulated deployment which is sufficient for evaluations with new pathloss models, we estimate the maximum distances where received signal with 10 PRB bandwidth has Target SNR = 0 dB, -10 dB for typical system configurations. To calculate signal attenuation pathloss and mean blockage values were taken into account. Obtained distance values are provided in Table 1.

Table 1: Estimation of the maximum distances with Target SNR based on TR 37.885 methodology

| Frequency Range | Vehicle UE Parameters | SCS | Target SNR | Max. Distance, NLOSv Case 1 | Max.Distance, NLOSv Case 2 | Max.Distance, NLOSv Case 3 |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 (fc:6 GHz) | Tx Power: 23 dBm Peak antenna gain: 3 dBi Noise Figure: 9 dB | 15 KHz | 0 dB | 14936 m | 3542 m | 8399 m |
|  |  |  | -10 dB | 47230 m | 11200 m | 26560 m |
|  |  | 30 KHz | 0 dB | 10561 m | 2505 m | 5939 m |
|  |  |  | -10 dB | 33397 m | 7920 m | 18781 m |
| FR2 (fc:30GHz) | Tx Power: 23 dBm Peak antenna gain: 5 dBi Noise Figure: 13 dB | 120 KHz | 0 dB | 1057 m | 251 m | 594 m |
|  |  |  | -10 dB | 3340 m | 792 m | 1879 m |

As it can be seen from the table, the large propagation distances are expected for any NLOSv blockage case, and, hence, large road length with the large number of vehicles should be used for proper system-level simulations, especially in FR1 frequency range.

In our view, the channel modeling framework agreed in TR 37.885 [2] has the following disadvantages:

Free-space path loss used for LOS and NLOSv propagation types may significantly underestimates signal propagation attenuation at large distances, caused by earth curvature, terrain irregularity, etc.

Agreed blockage modeling methodology is based on measurements at short and medium distances with limited number of intermediate vehicles and does not reflect the blockage loss increase with distance caused by signal attenuation by multiple vehicles.

In order to improve blockage modeling, we propose to revise agreed blockage modeling at large distances by adding additional blockage loss to the agreed mean blockage value.

To analyze blockage statistics at large distances, the blockage loss in Freeway scenario with passenger vehicles traveling with 70 km/h speed and 2.5s mean time ahead distance has been evaluated according to the blockage calculation methodology from TR 38.901 [16]. Figure 1 shows evaluation results for the above rooftop antenna placement scenario, which can be considered as the least challenging from the signal propagation point of view.

As it can be seen in Figure 1-a, for the medium distances of about 200..300m, evaluated blockage loss statistics is similar to the agreed in TR 37.885 Blockage Case 3 model where for the case of the same Tx/Rx antennas and blockers height value blockage loss is log-normally distributed with 5dB mean and 4dB standard deviation. For the long distance range, the large difference between agreed blockage loss mean value and 50% of the simulated blockage loss value is observed.

To minimize the difference in blockage loss, we propose to introduce additional blockage loss value, dependent on communication distance. For short and medium distances this value is equal to zero, while at the large distances  it may be approximated with the following equation: BLextra = max(0, 15*log10(d)-37).

In Figure 1-b, we provide 50%-tile blockage loss statistics for the above rooftop antenna placement (green curve), extra blockage loss value calculated as the difference between 50%-tile simulated blockage loss and TR 37.885 Blockage Case 3 model 5dB mean value (red curve). The approximated extra blockage loss is also shown in Figure 1-b with blue curve.

| - a) | - b) |
| --- | --- |

Figure 1: Blockage loss distribution for different communication ranges

In Table 2 we provide estimated maximum distances, where Target SNR can be achieved. It can be seen that introduced blockage model correction provides shorter ranges and, hence, reasonable deployment sizes may be used for accurate system-level study. Considering the modified blockage loss model, the actual deployment size may depends on system configuration, but typically should not be less than 4000m.

Table 2: Estimation of the maximum distances with Target SNR based on modified TR 37.885 blockage methodology

| Frequency Range | Vehicle UE Parameters | SCS | Target SNR | Max. Distance, NLOSv Case 1 | Max.Distance, NLOSv Case 2 | Max.Distance, NLOSv Case 3 |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 (fc:6GHz) | Tx Power: 23 dBm Peak antenna gain: 3 dBi Noise Figure: 9 dB | 15 KHz | 0 dB | 14936 m | 1217 m | 1994 m |
|  |  |  | -10 dB | 47230 m | 2350 m | 3849 m |
|  |  | 30 KHz | 0 dB | 10561 m | 999 m | 1636 m |
|  |  |  | -10 dB | 33397 m | 1928 m | 3157 m |
| FR2 (fc:30GHz) | Tx Power: 23 dBm Peak antenna gain: 5 dBi Noise Figure: 13 dB | 120 KHz | 0 dB | 1057 m | 251 m | 439 m |
|  |  |  | -10 dB | 3340 m | 518 m | 847 m |

- Based on the analysis above, we propose to modify the text of the TR 37.885 as follows:

- --------------------------------------------------- Text Proposal #2 -------------------------------------------------------

When a V2V link is in NLOSv, additional vehicle blockage loss is added as follows:

-	The blocker height is the vehicle height which is randomly selected out of the three vehicle types according to the portion of the vehicle types in the simulated scenario.

-	The additional blockage loss is max {0 dB, a log-normal random variable}.

-	Case 1: Minimum antenna height value of TX and RX > Blocker height

-	No additional blockage loss

-	Case 2: Maximum antenna height value of TX and RX < Blocker height

-	Mean: 12.5 + max(0, 15*log10(d)-37) dB, standard deviation: 4.5 dB

-	Case 3: Otherwise

-	Mean: 5 + max(0, 15*log10(d)-37) dB, standard deviation: 4 dB

- ---------------------------------------------------End of Text Proposal #2 ----------------------------------------------------

  - Adopt changes to TR 37.885 provided in the Text Proposal #2 of this document

It should be noted that similar issues exists in Urban scenario, especially for vehicles located in the same street. In Urban scenario the minimum simulation distance was agreed to be 1299 m * 750 m, which is obviously not sufficient for new channel models. One of the challenges in Urban scenario is that its layout is aligned with Macro-cell deployments.

  - Further analyze how to modify assumptions for Urban scenario to address sidelink interference issues considering Macro-cell deployment layouts

In-band Emission Model (IBE)

In this section, we would like to note that accurate IBE model is very important for eV2X evaluations given that it has significant impact on performance. The existing eV2X evaluation methodology [2] does not provide any recommendation on IBE model to be used for analysis. One can think that reuse of the LTE-V2X IBE model may be a straightforward solution. However, we would like to point out that NR has additional set of numerologies which are not covered by LTE-V2X. The alternative approach is to use IBE currently agreed for Uu UL communication. On this, we would like to point out that due to power control the actual accuracy of the NR IBE model is not as critical as it can be for sidelink performance. We would like to recall that during the LTE sidelink discussions the issue on IBE model was raised in [2]. As an outcome, the more strict IBE model was agreed for RAN1 sidelink evaluations, where additional offsets on value of W, X, Y, Z for single cluster transmissions was set to [3, 6, 3, 3] dB (please refer to [4]). Later on this model was also reused for LTE-V2X sidelink evaluations.

In Figure 2, we analyze performance of LTE-V2X sidelink radio-protocol (Mode-4) and using LTE-V2X evaluation methodology for the following system configurations:

Configuration 1: 10MHz BW and IBE on;

Configuration 2: 10MHz BW and IBE off;

Configuration 3: 40MHz BW and IBE on;

Configuration 4: 40MHz BW and IBE off.

Our system level evaluations show that Configuration 2 (i.e. 10MHz BW without IBE impact) gives better performance than Configuration 3 (i.e. 40MHz BW with IBE impact). This means that solutions to reduce the level of IBE are of equal or may be even higher importance as increase of the system bandwidth for eV2X applications and that 3GPP needs to work in both directions.

Figure 2: LTE-V2X protocol performance for 10MHz and 40MHz BW w/ and w/o IBE impact

In addition to system level evaluations, we have analysed the actual level of IBE from major contributing sources including (carrier frequency offset, power amplifier non-linearity, phase noise, IQ imbalance, DAC). Our evaluation analysis with all of the impairments included is shown in Figure 3. Based on this evaluation, we can see that existing RAN4 IBE model is a too conservative and thus may not be suitable for sidelink design and evaluations.

Figure 3: IBE Model Analysis

  - Ask RAN4 WG to provide the model for NR-V2X sidelink evaluations

  - Inform RAN4 WG that IBE can be a limiting factor eV2X performance and ask whether any improvement can be considered for NR V2X sidelink communication

Conclusions

In this contribution, we provided our views on remaining critical aspects for eV2X evaluation methodology. In summary, we have following proposals to finalize eV2X evaluation assumptions:

  - Update section 6.3 in TR 37.885 as follows

    - Replace current text in section 6.3 with the Text Proposal #1 in this document

  - Adopt changes to TR 37.885 provided in the Text Proposal #2 of this document

  - Further analyze how to modify assumptions for Urban scenario to address sidelink interference issues considering Macro-cell deployment layouts

  - Ask RAN4 WG to provide the model for NR-V2X sidelink evaluations

  - Inform RAN4 WG that IBE can be a limiting factor eV2X performance and ask whether any improvement can be considered for NR V2X sidelink communication

References

- RP-181480, “New SID: Study on NR V2X”, Vodafone, La Jolla, USA, June 11th -14th, 2018

- 3GPP TR 37.885, “Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR”, V15.0.0

- R1-133986, “LS on D2D Inband Emission Modeling for D2D System Level Simulations”, Barcelona, Spain, August 2013

- 3GPP TR 36.843, “Study on LTE Device to Device Proximity Services”, V12.0.1

- R1-1808693, “Support of sidelink unicast, groupcast and broadcast modes for NR V2X communication”, Intel Corporation, Gothenburg, Sweden, August, 2018.

- R1-1808694, “Sidelink physical layer structure and procedures for NR V2X communication”, Intel Corporation, Gothenburg, Sweden, August, 2018.

- R1-1808695, “Sidelink synchronization for NR V2X communication”, Intel Corporation, Gothenburg, Sweden, August, 2018.

- R1-1808696, “Sidelink resource allocation mechanisms for NR V2X communication”, Intel Corporation, Gothenburg, Sweden, August, 2018.

- R1-1808698, “Analysis of NR Uu support for advanced V2X use cases”, Intel Corporation, Gothenburg, Sweden, August, 2018.

- R1-1808699, “Enhancements of NR and LTE Uu link to control NR sidelink”, Intel Corporation, Gothenburg, Sweden, August, 2018.

- R1-1808700, “Enhancements of NR Uu link to control LTE sidelink”, Intel Corporation, Gothenburg, Sweden, August, 2018.

- R1-1808701, “QoS management for NR V2X communication”, Intel Corporation, Gothenburg, Sweden, August, 2018.

- R1-1808702, “Coexistence mechanisms for eV2X services”, Intel Corporation, Gothenburg, Sweden, August, 2018.

- R1-1801398, “V2X sidelink measurement results”, Huawei, HiSilicon, Athens, Greece, February, 2018

- R1-1803671, “V2X sidelink channel model”, Huawei, HiSilicon, Sanya, China, April, 2018

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, V 15.0.0

Annex A System and Link Level Evaluation Assumptions

Link Level Evaluation Assumptions

In this section, we provide summary of link level evaluation assumptions used for analysis of IBE mask including system parameter (see Table 3) and considered TX RF impairments (see Table 4):

Table 3: System parameters for IBE mask study

| System parameters | Value |
| --- | --- |
| Carrier frequency | 6 GHz |
| Bandwidth | 40 MHz |
| SCS | 30 kHz |
| Waveform | CP-OFDM |
| Frequency resource allocation | 10 PRB (band edge) |

Table 4: TX RF impairments taken into account for IBE evaluation

| Tx RF Impairments | Value |
| --- | --- |
| Power amplifier | RAN4 UL model (R4-163314), Output power - 23 dBm |
| Phase noise | RAN4 model (R4-1703088) |
| I/Q Imbalance | -25 dB |
| Carrier frequency offset (CFO) | 1200 Hz (2 x 0.1 ppm for 6 GHz) |
| DAC | 12 bits with -14 dB back-off value |

System Level Evaluation Assumptions

In this section, we provide summary of system level evaluation assumptions use for analysis of IBE impact:

Table 5: System Level Evaluation Assumptions

| Parameter | Value |
| --- | --- |
| Deployment |  |
| Type | Freeway, 2 Directions, 3464 m length |
| Number of lanes | 3 Lanes in each direction |
| Lane width | 4 m |
| Vehicle speed | 70 km/h |
| Vehicle deployment | Poisson in each lane; 2.5s V2V Mean Time Ahead |
| Spectrum allocation |  |
| Carrier frequency | 5.9 GHz |
| Bandwidth | Single channel; [10, 40] MHz |
| Subcarrier Spacing | 15 KHz |
| FFT Size | 10 MHz – 1024; 40 MHz - 4096 |
| Channel model |  |
| V2V channel model | According to 3GPP TR 36.885 (LTE V2V R14) |
| Signal Rx Processing |  |
| V2V sensing | According to the LTE R14 procedure |
| Receiver type | MMSE-IRC |
| Physical channels | PSCCH and PSSCH |
| Traffic Model |  |
| Packet generation period | 100 ms |
| Latency | 100 ms |
| Packet size | 190 Bytes |
| Resource Allocation |  |
| SCI/Data resource allocation | Adjacent resource allocation |
| PSSCH Transmission parameters (190 Byte Packet) |  |
| Allocation size | 10 PRB |
| MCS Index | 9 |
| Modulation | QPSK |
| Number of TTI | 2 |