3GPP TSG RAN WG1 Meeting #94			R1-1809076

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.4.5

Source:	AT&T

Title:	Coexistence of Different V2X Services

Document for:	Discussion/Approval

# Introduction

In RAN#80, V2X SI was approved, [1]. In this paper, we give our view on the sidelink design for NR.

# Design principle for NR sidelink

When looking at sidelink and acceslink, a fundamental difference is the structure of communication: accesslink is designed to support hierarchical structure where base-station is the centre of wireless communication while sidelink needs to support more flexible structure such as peer-2-peer structure. Meanwhile, advanced V2X use case also require sidelink to support hierarchical structure, e.g. vehicle platooning is a typical advanced use case where a platoon head is needed to manage a group of vehicles, essentially, it requires sidelink to support hierarchical structure based communications.  Naturally, NR sidelink should be able to support both hierarchical and peer-2-peer communication.

NR access link has been designed to support great flexibility through rich configurability. Many of the design can be reused in NR sidelink design especially for hierarchical structure based communication. Also reuse the access link can minimize the standardization effort as well maximize the possibility of shared hardware between access-link and sidelink. Therefore we have the following two proposals:

Proposal: NR sidelink should target to work on both peer-2-peer and hierarchical network structure.

Proposal: NR sidelink design should be based on access link and target to have common PHY procedures as much as possible.

Another important question for V2X service is on which band it operates on. In multiple countries, Intelligent Transportation System (ITS) band has been allocated to vehicle communication which is a possible spectrum for C-V2X. Another possibility is on the spectrum licensed for cellular service, which includes both FR1 and FR2. Of course, it’s rather expensive to have a dedicated band for C-V2X service, the best way is to share band with cellular service.

Proposal: NR sidelink must be able to operate on wireless licensed spectrum and co-exist with wireless service.

# Three-Party based communication framework

Typical communication is between two nodes, for example, cellular communication is between base-station and UE(s). And it has been further categorized into downlink and uplink based on the direction of data packet flow. No matter which direction it is, base-station serves two roles: as a data packet transmitter/receiver and as a scheduler which decides the radio resource to be used for every data packet, see below figure.

Figure.1 Two-Parties communication diagram

In order to support more flexible communication structure, it’s better to have a separate node dedicated as scheduler node, see figure.2. The function of Node-S is to coordinate the resource usage within a local area. Thus we also call Node-S as local manager. Similar to two parties’ communication, local manager sends DCI to Node-T and Node-R to convey the scheduling decision. Three parties communication is much flexible than traditional two parties counterpart mainly because it doesn’t need formal association between the Node-T/Node-R with Node-S. In C-V2X, a vehicle UE can serve as local manager to surrounding vehicles. That can effectively reduce transmission collision.

With the present of local manager, sidelink communication is in hierarchical structure. In another words, vehicle UEs need to associate with local manager somehow. However, there is no need to have full association just like UE-BS in cellular network since local manager essentially is another UE. It simply helps to coordinate the resource utilization in a certain area.  In our opinion, UE can have “diet” connection to local manager, which means UE synchronizes to local manager and blindly decode control channel following the search space configured for it. Also, when UE switch to a different local manager, no need to have normal handover procedure, UE simply just send an SR to another local manager to ask for new resources.

Figure.2 Three Parties communication diagram

Proposal: NR sidelink should support vehicle UE serves as a local manager to coordinate resource for the communication between other UEs within a local area. Local manager is granted with a resource pool from the cellular network.

# Co-existence of V2X and cellular services

In general, there are two modes of V2X operation, on shared spectrum, such as ITS bands, or on private owned licensed spectrum. Shared spectrum is free but difficult to guarantee QoS. Private owned licensed spectrum can guarantee QoS but cost is higher. However, there is a sweet point on that trade-off. When the V2X services can share the spectrum with cellular service, it can help to reduce the cost while still keep good QoS.

Proposal-1: NR Sidelink should be able to co-exist with access link on the same spectrum.

In additional, local manager is already similar to relay node. If local manager can help to relay cellular data packet to network, it becomes a mobile relay. That is particularly helpful for both V2X and cellular service because now the same hardware is shared by both services.

Proposal-2: NR Sidelink should be able to provide cellular data services

# Conclusion

In this contribution, we discussed the co-existence of V2X and other services. In particular, we have the following proposals:

Proposal-1: NR Sidelink should be able to co-exist with access link on the same spectrum.

Proposal-2: NR Sidelink should be able to provide cellular data services

# Reference

[1]  	RP-181480   	New SID: Study on NR V2X

[2]	R1-1806986	Autonomous topology management, AT&T