3GPP TSG RAN WG1 Meeting #94                                      R1-1809294

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:	7.2.6.3

Source:			Institute for Information Industry (III)

Title:			Discussion on Configured Grant Enhancements

Document for:	Discussion and Decision

# 1 Introduction

In RAN#80, the study item on physical layer enhancements for NR URLLC has been approved [1]. Objective listed in the SID includes enhancements to configured grant PUSCH:

“Enhanced UL configured grant (grant free) transmissions, with study focusing on improved configured grant operation, example methods such as explicit HARQ-ACK, ensuring K repetitions and mini-slot repetitions within a slot.”

In this contribution, we discuss enhancements that can be done to the configured grant PUSCH in the current specifications.

# 2 Discussion

## 2.1 Repetitions within a slot

In Rel-15 NR, repetitions of a configured grant PUSCH is performed in the same symbols in the repK-1 consecutive slots following the slot where initial transmission takes place. Therefore, to enable repetitions for configured grant PUSCH, periodicity of the configured grant must be at least 2 slots, which is not suitable for use cases requiring low latency.

Observation 1: Repetitions for configured grant PUSCH in Rel-15 NR is not suitable for use cases requiring low latency.

To achieve high reliability without repetitions, more symbols can be allocated for a configured grant. However, in dynamic TDD scenario, time domain resource allocation with large number of symbols for a configured grant can easily cause direction contradiction with the slot format of a slot. If direction contradiction occurs in a slot, then UE will not transmit PUSCH in the slot, which increases latency.

Therefore, to make configured grant PUSCH more suitable for use cases requiring both high reliability and low latency, repetitions within a slot should be supported.

Proposal 1: Repetitions within a slot for configured grant PUSCH is supported.

## 2.2 Explicit HARQ-ACK

In the current specifications, a configured grant PUSCH is considered by the UE to be successfully transmitted to the gNB if the configuredGrantTimer of the HARQ process, which was started when the configured grant PUSCH started, has expired. Before the timer expires, the UE can expect to be scheduled by an UL grant addressed to CS-RNTI for retransmission.

Since for configured grant PUSCH, the HARQ process ID associated with a transmission occasion depends on the time location of the transmission occasion within a system frame, as specified in TS 38.321, and since a transmission occasion cannot be used if the HARQ process associated with it has been used, i.e., the configuredGrantTimer of the associated HARQ process is running, configuredGrantTimer with longer duration will cause latency to be increased if the earliest transmission opportunity after a packet is available is not usable due to configuredGrantTimer is running. Therefore, to enable retransmission for configured grant PUSCH, latency may be increased.

Observation 2: Retransmission for configured grant PUSCH in Rel-15 NR is not suitable for use cases requiring low latency.

To make configured grant PUSCH more suitable for use cases requiring both high reliability and low latency, retransmission should be enabled without increasing the latency. To achieve this goal, configuredGrantTimer should be able to be stopped once gNB correctly received the PUSCH. This can be done by gNB transmitting HARQ-ACK to the UE, and UE stops configuredGrantTimer when it receives ACK in the HARQ-ACK.

The HARQ-ACK should also serve the purpose of early termination, i.e., when UE receives ACK in the HARQ-ACK in the middle of the repetitions of a configured grant PUSCH, it should stop the remaining repetitions. This way can reduce the collision probability of configured grant PUSCH, since the resources of configured grant for different UEs can be overlapped with each other.

Proposal 2: Explicit HARQ-ACK for configured grant PUSCH is supported.

Proposal 3: Remaining repetitions of a configured grant PUSCH is dropped if UE receives ACK in the HARQ-ACK.

For HARQ-ACK transmission, group common DCI should be used to transmit HARQ-ACK of configured grant PUSCH for multiple UEs. In addition, HARQ-ACK of multiple HARQ processes can be transmitted in a group common DCI to lower the overhead of CRC.

Proposal 4: Group common DCI is used for HARQ-ACK transmission for configured grant PUSCH.

## 2.3 Retransmission without dynamic UL grant

When gNB configures a resource for configured grant PUSCH, it may refrain from scheduling PUSCH in the resource. If a resource for configured grant PUSCH for a UE is not overlapped with the resource for configured grant PUSCH for other UE, then the resource is likely to be wasted if not used for retransmission. Although gNB can use dynamic UL grant to schedule UE to retransmit in resource that overlaps with the configured resource, however, a DCI needs to be transmitted.

Retransmission without dynamic UL grant has the advantage of less DCI overhead, but it has the disadvantage that if collision happens in the initial transmission, collision will happen again in the retransmission. Therefore, retransmission should be enabled to perform on the resource for configured grant PUSCH, under the condition that a layer-1 signaling is received by the UE. The layer-1 signaling can be the explicit HARQ-ACK discussed above, so that UE can perform retransmission if NACK is received.

Proposal 5: UE retransmits in the resource for configured grant PUSCH if it receives NACK.

2.4 HARQ process ID

As discussed above, if configuredGrantTimer of a HARQ process is running, transmission occasions associated with the HARQ process cannot be used for configured grant PUSCH. In current specifications, the HARQ process pool is shared between dynamic grant and configured grant. Therefore, if gNB schedules a HARQ process using dynamic grant, then some of the transmission occasions associated with the HARQ process cannot be used for configured grant PUSCH, since the configuredGrantTimer is started when the dynamic UL grant is received. Again, this will cause latency to be increased if the earliest transmission opportunity after a packet is available is not usable due to configuredGrantTimer is running.

Therefore, a method should be used to inform the gNB of the actual HARQ process ID so that UE can transmit in the transmission occasions associated with HARQ process that is not available. For example, carry the HARQ process ID in PUSCH.

Proposal 6: A method should be used to inform the gNB of the actual HARQ process ID so that UE can transmit in the transmission occasions associated with HARQ process that is not available.

# 3 Conclusions

Based on the discussion, we have the following proposals.

Observation 1: Repetitions for configured grant PUSCH in Rel-15 NR is not suitable for use cases requiring low latency.

Observation 2: Retransmission for configured grant PUSCH in Rel-15 NR is not suitable for use cases requiring low latency.

Proposal 1: Repetitions within a slot for configured grant PUSCH is supported.

Proposal 2: Explicit HARQ-ACK for configured grant PUSCH is supported.

Proposal 3: Remaining repetitions of a configured grant PUSCH is dropped if UE receives ACK in the HARQ-ACK.

Proposal 4: Group common DCI is used for HARQ-ACK transmission for configured grant PUSCH.

Proposal 5: UE retransmits in the resource for configured grant PUSCH if it receives NACK.

Proposal 6: A method should be used to inform the gNB of the actual HARQ process ID so that UE can transmit in the transmission occasions associated with HARQ process that is not available.

# 4 Reference

[1] RP-181477, RAN #80, La Jolla, US, June 11th – 14th, 2018.