3GPP TSG-RAN WG1 Meeting #94	Tdoc R1-1809407

Gothenburg, Sweden, August 20st – 24th 2018

Agenda Item:	7.1.3.3

Source:	Ericsson

Title:	Maintenance issues of DL/UL data scheduling and HARQ procedure

Document for:	Discussion and decision

# 1	Introduction

In this contribution, we discuss maintenance issues of DL/UL data scheduling and HARQ procedure for Rel-15.

# 2	UL data transmission procedure

## 2.1	Default configurations to apply when transform precoding is enabled in MSG3 and PUSCH

In 38.211 chapter 6.4.1.1.1.2, when transform precoding is enabled DMRS sequence generation is described as below. The sequence depends on hopping method that is configured from higher layer. Three options exist: apply only group hopping; apply only sequence hopping; apply neither group hopping nor sequence hopping.

In 38.331, IE DMRS-UplinkConfig is used to configure uplink demodulation reference signals for PUSCH, where two parameters are used to indicate which of the 3 alternatives to be used for hopping.

DMRS-UplinkConfig is an optional element in PUSCH-Config. The IE PUSCH-Config is used to configure the UE specific PUSCH parameters applicable to a particular BWP.

In SIB1 message, for MSG3 transmission before dedicated BWP has been assigned, RACH-ConfigCommon is used to configure the transform precoding, PUSCH-ConfigCommon that contains groupHoppingEnabledTransformPrecoding is used to configure the hopping method. The sequence hopping flag is not present in the common part, therefore ambiguity exists when transform precoding is enabled for MSG3. The UE might have been previously enabled or disabled with sequence hopping with dedicated configuration that is unknown to the network at the time of sending MSG3. Related RRC signalling texts from 38.331 are pasted below.

Group hopping or sequence hopping is used for generating DMRS sequences when transform precoding is enabled. The selection of hopping can be indicated with 2 separate flags in an RRC message in dedicated signalling.

Sequence hopping can only be enabled by dedicated signalling (not in SIB). This is done in PUSCH-Config => DMRS-UplinkConfig => sequenceHoppingEnabled. In other words, PUSCH DMRS sequence hopping is not available before the first dedicated RRC (re)configuration. Sequence group hopping can be configured on cell level through system information.

mcs-TableTransformPrecoding is a parameter that is needed to select the transform precoding MCS table which is configured only via dedicated signalling in PUSCH-Config.

mcs-tableTransformPrecoder can only be enabled by dedicated signalling

For MSG3, before dedicated signaling has been received by UE, the transform precoding can be enabled in SIB1. To clarify the expected behaviour so that the network and UE have the same understanding on the applied configurations for transform precoding, we propose an explicit interpretation of the group hopping and MCS table selection at the time transform precoding is enabled in MSG3.

For MSG3 transmission and retransmission, when transform precoding is enabled, UE ignores the previously configured UE-specific parameters by dedicated signalling (for sequence hopping and mcs-tableTransformProcoder) but only applies cell-specific parameters. Sequence hopping should be considered as disabled for DMRS sequence generation, MCS table for transform precoder applies the value 64QAM. Further, groupHoppingEnabledTransformPrecoding in PUSCH-ConfigCommon should be interpreted as follows: (1) absent: both group hopping and sequence hopping disabled; (2) enabled: group hopping enabled, sequence hopping disabled; (3) disabled: group hopping disabled, sequence hopping disabled

From 38.214 6.1.3:

The transform precoding for PUSCH transmission can be configured by msg3-transformPrecodingGroup if dedicated signalling is not configured.

For PUSCH transmission including configured grant, if transform precoding is enabled, it follows the same default configuration as MSG3 transform precoding if dedicated signalling parameters are absent. Sequence hopping should be considered as disabled for DMRS sequence generation, MCS table for transform precoder applies the value 64QAM.

Send LS to RAN2 to clarify the configurations when transform precoding get enabled.

While going through the text in specifications related to transform precoder, the naming and ways to describe the behaviour appears to be inconsistent. One of the reasons is because transform precoder was discussed separately in different sessions: the initial access session, PUSCH transmission, configured grant, MIMO, etc. The information was scattered and difficult to unify. Then in 38.331 RAN2 tried to merge and compile all the requirements sent from RAN1. Next we describe a way to improve the consistency of describing the transform precoder with following TPs.

In RRC, the transform precoder can be configure via these IEs and parameters.

We make the following observations and proposals.

In 38.214, section 6.1.3 and section 6.1.4.1 describe inconsistent UE behaviours on applying transform precoder. The description in Section 6.1.4.1 is aligned with 38.331.

Align transform precoder description in Section 6.1.3 in 38.213 with 38.331. Delete the duplicate text on transform precoder in Section 6.1.4.1 in 38.213.

Adopt TPs in Section 5.1 for transform precoder description.

# 3	DL/UL scheduling and HARQ management

## 3.1	Applicable K1 range

Looking at 38.331 f20, the highlighted parts below show that PDSCH-to-HARQ timing as defined by higher layer parameter dl-DataToUL-ACK can take values from 0 to 15, but the field description restricts the values in the range of 0 to 8 only even though there is no explicit agreement in RAN1 to have this restriction. Since, ASN.1 already supports values 0 to 15, the reason for having such restriction in the field description is not clear.

From 38.331 vf2

dl-DataToUL-ACK							SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)											OPTIONAL,	-- Need M

PUCCH-Config field descriptions

dl-DataToUL-ACK List of timing for given PDSCH to the DL ACK. In this version of the specification only the values [0..8] are applicable. Corresponds to L1 parameter 'Slot-timing-value-K1' (see TS 38.213, section FFS_Section).

As we describe in a companion paper [1], limiting max K1 to 8 creates scheduling restrictions, especially in cases where long processing delay due to A-CSI triggering can correspondingly delay the HARQ feedback of  PDSCH by more than 8 slots, necessitating need for large K1 values.

As shown in the example below (for 30 kHz, reproduced from [1]), triggering A-CSI in slot 6 would require the HARQ-ACK of PDSCH in slots 8/9/10 to be delayed to slot 17, requiring K1 of at least 9. Similarly, other cases exist where the delays can even larger (120 kHz FR2).

Figure 1. Impact of A-CSI request on HARQ-feedback latency (K1).

A-CSI triggering increases HARQ feedback latency, requiring K1 values larger than 8.

Since RRC specification already supports K1 value up to to 15, and given K1 values larger than 8 are necessary in some cases, we propose to simply update the 38.331 field description to say values between 0 to 15 are applicable for K1, and send a corresponding LS to RAN2 to update their specification.

Send LS to RAN2 to make [0..15] applicable for K1.

## 3.2	Buffering requirement for Cross-carrier scheduling with mixed numerology

In last RAN1 meeting, for cross-carrier scheduling with multiple numerologies, definitions on K0/K1/K2 were clarified and agreed. One remaining issue was to address the constraints related to K0 to address the number of symbols that need to be buffered.

R1-1807752

Agreements:

- Cross-carrier scheduling should at least satisfy the causality constraints between scheduling PDCCH and PDSCH as for self-scheduling, also taking into carrier timing difference

  - Note: in the case of mixed numerology, limitations on the number of symbols to buffer need to be taken into account

Agreements:

- For cross-carrier scheduling across different numerology

  - FFS: how to specify additional constraints related to K0 to address the number of symbols which may need to be buffered

From a causality perspective, for the same SCS case, the PDSCH can start at most 3 symbols from the end of PDCCH (which an occur for Type A with PDCCH end in symbol#2, and PDSCH start in symbol #0). Note that with mixed numerology case, the PDSCH processing time is already relaxed since it is derived from the SCS (among PDCCH/PUSCH/UL) that yields the most relaxed processing.

Looking at the different combinations of CCS with different numerologies:

- For low-SCS Ctrl scheduling high SCS PDSCH (Fig 1), overlapped slot (in yellow) is fully utilizable except for the 15kHz->120 kHz case, for which, scheduling from previous 15 kHz control region is feasible.

- For high-SCS Ctrl scheduling low SCS PDSCH (Fig 2), overlapped slot (in yellow) is fully utilizable.

Figure 1. Cross-carrier scheduling with PDCCH on low SCS scheduling PDSCH with high SCS.

Figure 2. Cross-carrier scheduling with PDCCH on high SCS scheduling PDSCH with low SCS.

In case of overlap of PDCCH and Type B PDSCH, the processing time is dependent on the amount of overlap (PDSCH and PDCCH), which can become cumbersome for mixed numerology case. Therefore, for type B PSDCH, it is simple to allow no overlap, while some overlap (such as max 3 symbols of PDSCH numerology) is possible for Type A PDSCH. This satisfies the causality constraint and works well for most cases of interest. Therefore, we propose the following.

For cross-carrier scheduling with different numerology for PDCCH and PDSCH

  - For Type-A, PDSCH start occurs no earlier than three symbols (of PDSCH numerology) from the end of the PDCCH

  - For Type-B, PDSCH starts after PDCCH ends

## 3.3 UE Cap#2 : Confirming WA on Type B values

In last meeting, the processing times for PDSCH mapping Type B with 2 and 4 symbols was captured as a working assumption. These values (a bit tighter than capability#1) allow faster UE processing time that is more suited for URLLC-like and lower latency use cases. We think these values should be confirmed as it is in case they cannot be tightened a bit further.

Additionally, there is an FFS point on how to handle a 3-symol CORESET where first two symbols of CORESET are overlapped with a 2-symbol PDCCH. It was mentioned that this case may require special consideration since the PDSCH ends before the PDCCH end and hence some extra processing time may be provisioned. Our preference is to not create special cases with additional relaxations that the scheduler needs to handle; for this example, there would already be an extra 2-symbol due to the overlapped of PDSCH and PDCCH. Therefore, in our view it is better to support same principle of processing time for all 2-symbol allocations at the UE.

Confirm the WA on capability#2 Type B with 2/4 symbols from RAN1#93 with the following update.

  - (Working assumption) For PDSCH mapping type B with 4 or 2 symbols

    - N1 processing time is increased by ‘d’ symbols relative to the case of PDSCH with 7 symbols, where ‘d’ is the amount of time-domain overlap in symbols between the scheduling PDCCH and the scheduled PDSCH

      - FFS: handling of 3-symbol CORESET where first 2 symbols of CORESET are overlapped with a 2-symbol PDSCH

## 3.4 UE Cap#2 : Proc time for UCI multiplexing

Since, UE capability#2 is an advanced UE, it is expected additional UE processing power is available to support UCI multiplexing with HARQ-ACK without requiring extra processing time. Note the UCI encoding with Polar code can be processed in parallel or in a much more efficient pipelined fashion with the frequency-first mapping of PUSCH with LDPC code. Thus, applying same PUSCH processing time value is preferred whether HARQ-ACK is multiplexed or not. Similarly, same value of N1 is applied for the case when the HARQ-ACK goes on PUSCH vs on PUCCH.

For UE processing capability#2, same PDSCH processing time is applied for case when HARQ-ACK goes on PUCCH or on PUSCH i.e. d1,1 = 0.

For UE processing capability#2, the PUSCH processing time for “PUSCH multiplexing with HARQ-ACK” is same as that for case of no UCI multiplexing on PUSCH.

## 3.5 UE Cap#2 : CA-case

Capability#2 was agreed for the non-CA case while the CA case is FFS, including the possibility of supporting capability#2 on one carrier from the set of carriers in CA case. In our view, reduced latency due to capability#2 is beneficial for the CA case as well and provides feature parity with the LTE sTTI which also supports CA. In case of CA between FR1 and FR2, the capability#2 would be applied only to carriers in FR1.

UE processing capability#2 is supported for CA.

From a UE capability signalling point of view, there are some possible alternatives: either 1) capability#2 supported is separately indicated for the non-CA and CA cases, or 2) jointly indicated for the non-CA and CA cases. Our preference is the latter i.e. if UE indicates support of capability#2, it is applied for both non-CA and CA case.

UE processing capability#2 support is jointly indicated for the non-CA and CA cases.

## 3.6 UE Cap#2 : Processing time for SPS release

The DL SPS release processing for capability#2 UEs should also be shortened to reflect the advanced UE processing capability. Following the same principle as that of capability#1, i.e. using the minimum of PDSCH processing time assuming PDSCH ends in same symbol as PDCCH (and using a 2-symbol Type B PDSCH as reference) and PUSCH processing time, leads to 5/5.5/11 symbols for 15/30/60 kHz cases. Support of capability#2 is separately signaled for uplink and downlink processing. Moreover, if the UE indicates capability#2 support for at least one of the uplink or downlink numerologies, the UE should support the tightened SPS processing time.

SPS release processing times for UE processing capability#2 are as follows:

  - 15kHz: 5 symbols

  - 30kHz: 5.5 symbols

  - 60kHz for FR1: 11 symbols

## 3.7 UE proc time : Editorial corrections

During review, some miscellaneous corrections were identified as shown below for 38.214

Following miscellaneous corrections to section 5.3 of 38.214 are proposed:

  - d1,2should be set to 0 if it is not specified such as following cases

    - for Type A and >= 7-symbol PDSCH

    - for Type B with 7-symbol PDSCH

  - For tables 5.3-1 and 5.3-2, the following change is proposed for 2nd column since the front-loaded value for N1 is used when dmrs-AdditionalPosition =0 for both when Type A and B are configured.

    - dmrs-AdditionalPosition = pos0 in 
DMRS-DownlinkConfig in either both of 
dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB

  - Suggest following change since default may imply a semi-static operation

    - For UE PDSCH processing capability 2 with scheduling limitation when µPDSCH = 1, if the scheduled RB allocation for the PDSCH exceeds 136 RBs, the UE defaults to capability 1 processing timeshall determine Tproc,1 for the PDSCH according to PDSCH processing capability 1.

  - Amend table 5.3-2 to specify PDSCH processing time for UEs with capability#2 with additional DMRS configured to fall back to that of capability#1.

## 3.8 UE proc time : Alignment of 38.213 and 38.214

While reviewing the endorsed versions, the processing time for the case of UCI multiplexing on PUSCH may have been captured with some duplication (as described below) in both 213 and 214.

- 38.213, sec 9.2.5 describes the generic framework considering UCI multiplexing on PUSCH, and this was developed over the last two meetings (Sanya/Busan) including addition of 1 extra symbol.

- 38.214, sec 6.4 has the following sentence added based on comment from MTK in post-Busan review, note a typo here (after “otherwise”, d2,2 should be 0).

  - If the HARQ-ACK is multiplexed on PUSCH, then d2,2 = 1, otherwise d2,2 = 1.

Given the text is 213 is generic enough and covers the case of HARQ-ACK multiplexed on PUSCH as well with the 1 symbol additional proc time, the new sentence in 214 with variable d2,2 is not needed. This also makes aligns the spec further since d2,2 is not referenced in 38.213. We are also open to an alternative way to align the specifications.

Following correction to section 6.4 of 38.214 is proposed for alignment with 38.213:

  - If the HARQ-ACK is multiplexed on PUSCH, then d2,2 = 1, otherwise d2,2 = 1.

## 3.9 UE peak data rate and L1 TBS

In last RAN1 meeting, the following was concluded in soft buffer session wrt peak rates.

Conclusion:

- Performance is not expected to be optimized for the following special cases (while supported in specification) in which retransmissions of a transport block occur at or near peak rate

  - the TB is transmitted with significantly lower code rate (considering initial and retransmission) than R_LBRM(=2/3)

  - the retransmission occurs on a much shorter PDSCH duration than the initial transmission

  - No spec change

There are two potential issues to discuss related to the peak rate

- OH value difference between RAN2 spec peak rate and L1 peak rate?

- Relation between peak rate and L1 TBS.

RAN1 has defined approximate peak rate based on average overheads for use in RAN2 spec (for L2 buffer calculation). If the same formula is used for defining the absolute L1 peak rate, there will be an overall loss because the overheads can be as large as 14% (for FR1, DL). For example, as shown below for 30kHz SCS, the difference between the maximum TBS based on the formula from RAN2 spec (with OH = 14%) and that from TBS calculation (from RAN1 spec) is approximately 10%. One option is to change the overhead from 0.14 to ~ 0.059 or lower.

An example case relation between peak TBS based data rate and formula and OH values are shown below.

| FR | Dmrs Type | scs | nPRB | n_layers | Qm | n_sym | n_dmrs | TBS | dataRate FromTBS | dataRate FromFormula | OH (RAN2) | recOH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | B | 1 | 65 | 2 | 8 | 7 | 4 | 77896 | 0.311584 | 0.278214 | 0.14 | 0.037 |

Considering different case of SCS/RB allocations/DMRS overheads/PDSCH allocation lengths (L), the least OH values are as shown in table below.

|  | FR1 | FR2 |
| --- | --- | --- |
| Only L = 14 duration | 0.059 | 0.062 |
| Only L = 2/4/7/14 durations | 0.037 | 0.04 |
| L = 2 to 14 symbols | 0.012 | 0.019 |

To strike a balance between the various different configurations, we suggest to revise the OH value as follows.

Revise the OH value in maximum data rate formula of 38.306 to ~0.037.

Once a suitable value of OH agreed (e.g. as in above proposal), it is possible to use the formula in 38.306 as an indicator of L1 peak data rate. Consider following illustrative example of three carriers with each carrier with a different numerology and each contributing a 1 Gbps data rate and a SCF = 1 for each carrier, for an aggregate 3 Gbps. The corresponding max TBS is shown in the figure.

Consider now the same case as above, but UE reports SCF = 0.75 for each of the three carriers => aggregate data rate of 3 x 0.75 = 2.25 Gbps.

For this case, we propose that Sum of TB bits (proportionally scaled) over a slot duration does not exceed DataRate*slotDuration. Since the CCs may have different slot durations, condition on the sum TBS can be enumerated as follows (where some SCS-based scaling may be applied for mixed numerology cases):

- For 1 ms reference interval: a condition on may be defined as follows:

  - a0+n=01bn+n=03cn≤DataRate*1ms

- For 0.5 ms reference interval: a condition(s) on may be defined as follows:

  - a02+b0+c0+c1≤DataRate*0.5ms

  - a02+b1+c2+c3≤DataRate*0.5ms

- For 0.25 ms reference interval: a condition(s) on may be defined as follows:

  - for (i,j) in {(0,0),(0,1), (1,2),(1,3)}: a04+bi2+cj≤DataRate*0.25ms

While the above formulation can be generalized to shorter (< slot) durations, we would prefer implementation simplicity provided by slot-level application of the conditions. Applying largest slot duration would be reasonable to allow flexible scheduling framework regarding mixed numerologies, carrier aggregation, and multiple BWPs, though other options might be to define it based on shortest slot duration.  In case when condition is exceeded, UE may skip decoding (or encoding) and report NACK or its performance may not be optimised for processing of those transport blocks.  We prefer the latter case.

Define a sum of TB bits (proportionally scaled to the slot duration) over the largest slot duration across the configured carriers to not exceed DataRate*slotDuration.

  - Note: In case CBG-based transmissions are enabled, sum of TB bits corresponds to sum of bits in the transmitted CBGs of the transport block

# 4	DL/UL resource allocation

## 4.1	New-RNTI missing in the TDRA table

New-RNTI is supposed to be used in same way as C-RNTI as it is used for UE specific data transmission, and it should be included in the TDRA table so UE knows how to interpret the TDRA when received DCI scrambled with new-RNTI.

Add new-RNTI in the same place as C-RNTI in the time domain resource allocation tables: Table 6.1.2.1.1-1 for PUSCH and Table 5.1.2.1.1-1 for PDSCH in 38.214.

## 4.2	Removing the overlapping part of MSG3 timing description in 38.213

To briefly recall the discussions in RAN1 meetings:

RAN1#92Bis

In R1-1805634, the proposal for working assumption:

- For the MSG3 PUSCH time domain resource assignment in RAR, the UE uses the default table or table indicated by RMSI for MSG3 PUSCH for the value of K2

  - Note:  FFS: Numerology for K2

- Additional delay is added to K2 to account for N1+L2. The delay is subcarrier specific and given by the specification

  - Note: If Msg2 and Msg3 have different SCS, value of N1 – in terms of absolute time - are determined by lower SCS between Msg2 and Msg3

- Note: Working assumption can be revisited once default table and/or support for RMSI signaling has been decided

Revist till next meeting

RAN1#93

Agreements:

Adopt the following default tables for ECP:

- For PUSCH time-domain RA table for ECP, j=2 follows the previous agreements on 60kHz, Delta = 4.

Agreements:

- Change Delta from 2 to 3 for 30 kHz for msg 3 processing

There’s overlapping description for MSG3 timing in 38.213 and 38.214 that should be removed to avoid ambiguity. Description in 38.213 is not aligned with the agreements from previous meetings.

8.2	Random access response

…

If higher layer parameter pusch-AllocationList is provided by pusch-ConfigCommon, a value of the Msg3 PUSCH time resource allocation field indicates an entry to a table provided by pusch-AllocationList; otherwise a value of the Msg3 PUSCH time resource allocation field indicates an entry to default time resource allocation table [6, 38.214].

Remove the overlapping description for MSG3 time domain resource allocation in Section 8.2 in TS 38.213.

## 4.3	Missing description for frequency domain resource allocation for MSG3

Frequency domain allocation used in RAR for MSG3 is using 14 bits, which was agreed at RAN#93 meeting. How to interpret these 14 bits is still missing from the specification.

Adopt TP in Section 5.2 to clarify the interpretation of the 14 bits in RAR for frequency domain resource allocation in 38.213 section 8.2 or 38.214 section 6.1.2.

# 5	Text Proposal

## 5.1	TP for clarifying transform precoder procedure

>>> Text Proposal for 38.212 Section 7.3.1.1.2 >>>

##### 7.3.1.1.2	Format 0_1

…

-	Antenna ports – number of bits determined by the following

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled transformPrecoder=enabled, dmrs-Type=1, and maxLength=1;

-	4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled transformPrecoder=enabled, dmrs-Type=1, and maxLength=2;

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled transformPrecoder=disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled transformPrecoder=disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled transformPrecoder=disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled transformPrecoder=disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook.

>>> End Text Proposal >>>

>>> Text Proposal for 38.214 Section 6.1.3 >>>

### 6.1.3	UE procedure for applying transform precoding on PUSCH

For Msg3 PUSCH transmission, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoding.

For PUSCH transmission scheduled with a DCI:

-	If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoding.

-	If the DCI with the scheduling grant was not received with DCI format 0_0

-	If the UE is configured with the higher layer parameter transformPrecoder [transform-precoding-scheduled], the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder [transform-precoding-scheduled], the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoding.

For PUSCH transmission without grant

-	If the UE is configured with the higher layer parameter transformPrecoder [transform-precoding-TWG], the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder [transform-precoding-TWG], the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoding.

>>> End Text Proposal >>>

>>> Text Proposal for 38.214 Section 6.1.4.1 >>>

#### 6.1.4.1	Modulation order and target code rate determination

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, or SP-CSI-RNTI, the transform precoding is enabled if transformPrecoder in PUSCH-Config is set to 'enabled', or if transformPrecoder in PUSCH-Config is not configured and msg3-transformPrecoding in rach-ConfigCommon is set to 'enabled'; otherwise the transform precoding is disabled.

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by CS-RNTI, or the PUSCH with configured grant using CS-RNTI, the transform precoding is enabled if transformPrecoder in ConfiguredGrantConfig is set to 'enabled'; otherwise the transform precoding is disabled.

if transform precoder transformPrecoder is disabled for this PUSCH transmission

>>> End Text Proposal >>>

## 5.2	TP for Msg3 PUSCH frequency domain resource allocation field in RAR grant

>>> Text Proposal for 38.213 Section 8.2 or 38.214 Section 6.1.2>>>

…

The Msg3 PUSCH frequency domain resource allocation field is interpreted as follows:

-	if the frequency hopping flag bit is set to 1

-	NUL, hop MSB bits in the frequency domain resource allocation field are used to indicate the frequency offset according to Subclause 6.3 of [6, TS 38.214 ], where the number of hopping bits NUL, hop is defined in Table 8.3-1.

-	The 14-NUL, hop LSB bits provide the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	else

-	the 14 bits provide the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	end if

>>> End Text Proposal >>>

# Conclusion

In the previous sections we made the following observations:

Observation 1	Group hopping or sequence hopping is used for generating DMRS sequences when transform precoding is enabled. The selection of hopping can be indicated with 2 separate flags in an RRC message in dedicated signalling.

Observation 2	Sequence hopping can only be enabled by dedicated signalling (not in SIB). This is done in PUSCH-Config => DMRS-UplinkConfig => sequenceHoppingEnabled. In other words, PUSCH DMRS sequence hopping is not available before the first dedicated RRC (re)configuration. Sequence group hopping can be configured on cell level through system information.

Observation 3	mcs-tableTransformPrecoder can only be enabled by dedicated signalling

Observation 4	The transform precoding for PUSCH transmission can be configured by msg3-transformPrecodingGroup if dedicated signalling is not configured.

Observation 5	In 38.214, section 6.1.3 and section 6.1.4.1 describe inconsistent UE behaviours on applying transform precoder. The description in Section 6.1.4.1 is aligned with 38.331.

Observation 6	A-CSI triggering increases HARQ feedback latency, requiring K1 values larger than 8.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For MSG3 transmission and retransmission, when transform precoding is enabled, UE ignores the previously configured UE-specific parameters by dedicated signalling (for sequence hopping and mcs-tableTransformProcoder) but only applies cell-specific parameters. Sequence hopping should be considered as disabled for DMRS sequence generation, MCS table for transform precoder applies the value 64QAM. Further, groupHoppingEnabledTransformPrecoding in PUSCH-ConfigCommon should be interpreted as follows: (1) absent: both group hopping and sequence hopping disabled; (2) enabled: group hopping enabled, sequence hopping disabled; (3) disabled: group hopping disabled, sequence hopping disabled

Proposal 2	For PUSCH transmission including configured grant, if transform precoding is enabled, it follows the same default configuration as MSG3 transform precoding if dedicated signalling parameters are absent. Sequence hopping should be considered as disabled for DMRS sequence generation, MCS table for transform precoder applies the value 64QAM.

Proposal 3	Send LS to RAN2 to clarify the configurations when transform precoding get enabled.

Proposal 4	Align transform precoder description in Section 6.1.3 in 38.213 with 38.331. Delete the duplicate text on transform precoder in Section 6.1.4.1 in 38.213.

Proposal 5	Adopt TPs in Section 5.1 for transform precoder description.

Proposal 6	Send LS to RAN2 to make [0..15] applicable for K1.

Proposal 7	For cross-carrier scheduling with different numerology for PDCCH and PDSCH

	For Type-A, PDSCH start occurs no earlier than three symbols (of PDSCH numerology) from the end of the PDCCH

	For Type-B, PDSCH starts after PDCCH ends

Proposal 8	Confirm the WA on capability#2 Type B with 2/4 symbols from RAN1#93 with the following update.

a.	(Working assumption) For PDSCH mapping type B with 4 or 2 symbols

i.	N1 processing time is increased by ‘d’ symbols relative to the case of PDSCH with 7 symbols, where ‘d’ is the amount of time-domain overlap in symbols between the scheduling PDCCH and the scheduled PDSCH

1.	FFS: handling of 3-symbol CORESET where first 2 symbols of CORESET are overlapped with a 2-symbol PDSCH

Proposal 9	For UE processing capability#2, same PDSCH processing time is applied for case when HARQ-ACK goes on PUCCH or on PUSCH i.e. d1,1 = 0.

Proposal 10	For UE processing capability#2, the PUSCH processing time for “PUSCH multiplexing with HARQ-ACK” is same as that for case of no UCI multiplexing on PUSCH.

Proposal 11	UE processing capability#2 is supported for CA.

Proposal 12	UE processing capability#2 support is jointly indicated for the non-CA and CA cases.

Proposal 13	SPS release processing times for UE processing capability#2 are as follows:

	15kHz: 5 symbols

	30kHz: 5.5 symbols

	60kHz for FR1: 11 symbols

- Following miscellaneous corrections to section 5.3 of 38.214 are proposed:

  - d1,2should be set to 0 if it is not specified such as following cases

    - for Type A and >= 7-symbol PDSCH

    - for Type B with 7-symbol PDSCH

  - For tables 5.3-1 and 5.3-2, the following change is proposed for 2nd column since the front-loaded value for N1 is used when dmrs-AdditionalPosition =0 for both when Type A and B are configured.

    - dmrs-AdditionalPosition = pos0 in 
DMRS-DownlinkConfig in either both of 
dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB

  - Suggest following change since default may imply a semi-static operation

    - For UE PDSCH processing capability 2 with scheduling limitation when µPDSCH = 1, if the scheduled RB allocation for the PDSCH exceeds 136 RBs, the UE defaults to capability 1 processing timeshall determine Tproc,1 for the PDSCH according to PDSCH processing capability 1.

  - Amend table 5.3-2 to specify PDSCH processing time for UEs with capability#2 with additional DMRS configured to fall back to that of capability#1.

Proposal 15	Following correction to section 6.4 of 38.214 is proposed for alignment with 38.213:

a.	If the HARQ-ACK is multiplexed on PUSCH, then d2,2 = 1, otherwise d2,2 = 1.

Proposal 16	Revise the OH value in maximum data rate formula of 38.306 to ~0.037.

Proposal 17	Define a sum of TB bits (proportionally scaled to the slot duration) over the largest slot duration across the configured carriers to not exceed DataRate*slotDuration.

a.	Note: In case CBG-based transmissions are enabled, sum of TB bits corresponds to sum of bits in the transmitted CBGs of the transport block

Proposal 18	Add new-RNTI in the same place as C-RNTI in the time domain resource allocation tables: Table 6.1.2.1.1-1 for PUSCH and Table 5.1.2.1.1-1 for PDSCH in 38.214.

Proposal 19	Remove the overlapping description for MSG3 time domain resource allocation in Section 8.2 in TS 38.213.

Proposal 20	Adopt TP in Section 5.2 to clarify the interpretation of the 14 bits in RAR for frequency domain resource allocation in 38.213 section 8.2 or 38.214 section 6.1.2.