3GPP TSG RAN WG1 Meeting 94 			R1-1809627

Gothenburg, Sweden, Aug 20th – Aug 24th, 2018

Source:	Sierra Wireless

Title:	Rel15 Part II - MTC Sub-PRB PUSCH Maintenance Summary

Agenda Item:	6.1.4.4

Document for:	Discussion and Decision

# Introduction

The following tdoc provides a second summary of the remain open issues to the Rel 15 maintenance agenda item for Sub-PRB in RAN1#94.

# Key Issues and Proposals

## RV Cycling supported

The following agreements were made in RAN1#94.

For CE Mode A and B, RV cycling is supported for PUSCH sub-PRB repetitions.

RV changes per repetition which occurs every (RU Size *number of RU) transmission subframes.

For CE Mode A, the first RV is indicated in DCI and subsequent RVs continue following the RV order 0, 2, 3, 1;

For CE Mode B, the first RV is always 0 and the RV order is always 0, 2, 3, 1.

The following Text proposal to TS 36.213 is proposed

-------------------------------------------------------- Text start -----------------------------------------------------------------

8.0	UE procedure for transmitting the physical uplink shared channel

< Unchanged parts are omitted >

A BL/CE UE shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B scheduling PUSCH intended for the UE, perform a corresponding PUSCH transmission in subframe(s) n+ki if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8] with i = 0, 1, …, N-1 according to the MPDCCH, where

-	subframe n is the last subframe in which the MPDCCH is transmitted; and

-	x≤k0<k1<…,kN-1 and the value of  is determined by the repetition number field in the corresponding DCI, where

-	if the UE is configured with higher layer parameter ce-pdsch-puschEnhancement-config with value 'On' are given by {1,2,4,8,12,16,24,32}

-	otherwise, are given in Table 8-2b and Table 8-2c; and

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the corresponding DCI is using uplink resource allocation type 5,  where N ≤ 32 for CE Mode A and N ≤ 2048 for CE Mode B,   is defined in [3] and is determined according to procedure in subclause 8.1.6,  otherwise

-	in case N>1, subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s) starting with subframe n+x, and in case N=1, k0=x;

< Unchanged parts are omitted >

8.6.1	Modulation order and redundancy version determination

< Unchanged parts are omitted >

For a BL/CE UE or for UEs configured with higher layer parameter PUSCH-EnhancementsConfig,

-    if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment is using uplink resource allocation type 5, the redundancy version (rvidx) to use for the i-th BL/CE UL subframe in the physical uplink shared channel is determined according to Table 7.1.7.1-2 using  rv=(iMSlotsUL/2*MRU+rvDCI) mod 4 where i=0, 1, …, N-1, and N is the number of BL/CE UL subframes for the PUSCH transmission as determined in subclause 8.0. For a BL/CE UE configured in CEModeA,  is determined by the 'Redundancy version' field in DCI format 6-0A. For a BL/CE UE configured with CEModeB, .

-    otherwise, the same redundancy version is applied to PUSCH transmitted in a given block of  consecutive subframes. The subframe number of the first subframe in each block of consecutive subframes, denoted as , satisfies . Denote  as the subframe number of the first uplink subframe intended for PUSCH. For BL/CE UEs, the PUSCH transmission spans  consecutive subframes including non-BL/CE subframes where the PUSCH transmission is postponed. For the  block of consecutive subframes within the set of  subframes, the redundancy version (rvidx) is determined according to Table 7.1.7.1-2 using , where , and . The  blocks of subframes are sequential in time, starting with  to which subframe belongs. For a BL/CE UE configured in CEModeA or a UE configured with higher layer parameter PUSCH-EnhancementsConfig,  and  is determined by the 'Redundancy version' field in DCI format 6-0A. For a BL/CE UE configured with CEModeB,  for FDD and  for TDD, and . For UEs configured with higher layer parameter PUSCH-EnhancementsConfig, NabsPUSCH=NrepPUSCH.

< Unchanged parts are omitted >

----------------------------------------------------- Text End -------------------------------------------------------------------

## DCI could indicate longer transmission than 'pusch-maxNumRepetitionCEmodeA'

Issue: Source [3]. The number of transmitted subframes indicated by DCI may be larger than the maximum number of transmitted subframes configured by the higher RRC layer parameter 'pusch-maxNumRepetitionCEmodeA'.

Proposal #1:  UE overrides the number of RU units given in the DCI to ensure the number of transmitted subframes is not greater than 'pusch-maxNumRepetitionCEmodeA'.  number of resource units shall be at most equal to min(' pusch-maxNumRepetitionCEmodeA’/MslotsUL2 ,4), See [3] for TP for 36.213

Supported by: Qualcomm, Sierra Wireless

Proposal #2: UE considers the DCI is valid even if the number of transmitted subframes is greater than 'pusch-maxNumRepetitionCEmodeA'.

Supported by: Qualcomm, Sharp, Sierra Wireless

Feature lead proposal: Make the following agreement:

For Sub-PRB in CE Mode A, the UE considers the DCI valid even if the number of transmitted subframes is greater than 'pusch-maxNumRepetitionCEmodeA'.

## Pi/2 BPSK DMRS Collisions

Issue: Source [3]. The Pi/2 BPSK DMRS resources used by the two cells may collided.

Proposal #1:  Remove the Cell ID dependency on the DMRS formula to avoid this collision. See [3] for TP for 36.211

Supported by: Qualcomm

Feature Lead proposal: This is an optimization and thus can be discussed next meeting.

# Tdocs

- R1-1808625 Remaining issue for Sub-PRB allocation ZTE

- R1-1808987 Maintenance issues for PUSCH sub-PRB allocation for Rel-15 LTE-MTC Ericsson

- R1-1809010 Maintenance for PUSCH subPRB allocation Qualcomm Incorporated

- R1-1809104 Discussion on RV determination for sub-PRB allocation Sharp