3GPP TSG RAN WG1 Meeting #94bis			                                                                   R1-1811245

October 8th – 12th 2018

Chengdu, China

Agenda item:	7.2.1.3

Source: 	Qualcomm Incorporated

Title: 	Procedures Related to NOMA

Document for:	Discussion/Decision

# Introduction

In 3GPP Plenary #80, it was confirmed the scope of NOMA related procedures includes the following [1]:

- UL transmission detection

- HARQ, including transmission scheme, feedback scheme, and combining scheme

- Link adaptation MA signature allocation/selection

- Synchronous and asynchronous operation

- Adaptation between orthogonal and non-orthogonal multiple access

  - FFS the channel structure and procedures for asynchronous.

In this contribution, we start with a discussion on the typical use cases and operation modes of NOMA to identify the performance gain of NOMA. After that, we introduce our considerations for the design of NOMA related procedures, including:

- channel structure for synchronous and asynchronous operation

- MA signature selection/allocation

- contention resolution

- HARQ

- link adaptation and power control

# Use Cases and Operation Modes of NR NOMA

# In NR Rel-15, MU-MIMO has been standardized as an UL solution for concurrent data transmissions. In this SI, NR NOMA has been studied under a similar context as MU-MIMO, wherein the transmitting UEs share the same physical resources in time, frequency and space. Therefore, there are two key questions to be answered:

- How much is the gain of “NR NOMA” relative to MU-MIMO in NR Rel-15?

- Which operation modes and use cases reflect the most significant gain of “NR NOMA”?

In this section, we will provide our views to the questions above.

## NR NOMA vs MU-MIMO

When UE is in RRC_CONNECTED state, both MU-MIMO and NOMA are solutions to non-orthogonal multiple access. When UE is in RRC_INACITIVE state, NOMA can still be used for small data transmission, which facilitates signaling overhead reduction and power saving. Therefore, we have the following observations and proposal:

Observation 1

- When the network operates in grant-based mode, transmission schemes proposed for NOMA can be applied to MU-MIMO as well. The gain of NOMA over MU-MIMO in spectral efficiency is questionable, especially for underloading scenarios.

- When the network operates in grant-free mode and the UL access is contention-free, the gain of NOMA over MU-MIMO in spectral efficiency is questionable.

- The most significant gain of NOMA over MU-MIMO can be achieved in the following scenarios:

  - contention-based, grant-free transmission

  - small data transmission from RRC_INACTIVE state

Proposal 1:  NR NOMA solutions achieving performance gains over NR Rel-15 MU-MIMO should be prioritized in the study and evaluation.

## Use Cases and Operation Modes

To illustrate, Table 1 summarizes the use cases and operation modes of NOMA. In particular, the highlighted features in the third column reflect the major benefits of NOMA, as justified in the SID [1].

Table 1: NR NOMA Use Cases and Features Supported by Different Operation Modes

# Overloading in Grant-based NOMA

A necessary condition for considering the grant-based transmission as NOMA is the overloading of UEs. Specifically, overloading takes place when the number of multiplexed UEs is greater than the available degrees of freedom (DoF). In the context of MU-MIMO and NOMA, we propose the following definition for DoF:

Proposal 2: For grant-based UL transmission, the degree of freedom can be defined as the number of orthogonal physical resource units in time, frequency and space domains, which is measured for a given TTI, a given transmission bandwidth and a given number of transceiver antennas.

To illustrate, assume there are L UEs sharing the same physical resources in time/frequency/space domains, and each UE has one transmit antenna. Besides, assume the base station is equipped with Q receiver antennas, and each UE employs a spreading code of size P in time/frequency domain. In this case, the DoF can be defined as DF≝PQ. A distinction between overloading and underloading can be made based on the comparison of L and DF:

- when L>DF, this is an overloaded transmission.

- when L≤DF, this is an underloaded transmission.

For grant-based and underloaded transmission, the scheduler can consider OMA instead of NOMA to simplify the receiver complexity without degradation in UL capacity and spectral efficiency. NOMA will be useful for grant-based transmission with overloading. Therefore, we have the following proposal:

Proposal 3: For synchronized NOMA transmission, TX solutions incapable of UE overloading and/or unsuitable for scalable configuration should be deprioritized and FFS whether they have significant performance gains over OMA with the same transceiver complexity.

# Two-Step RACH for Asynchronized NOMA

Depending on the arrival time of UL signals, as well as the time alignment methods used by the network, NOMA operation can be classified into synchronized and asynchronized. Take the CP-OFDM waveform as an example and assume the CP length is τCP.  Suppose N NOMA UEs are transmitting within a given TTI, and the arrival time of N UL signals is bounded by an interval τ1 ,  τN, then:

- If |τN-τ1|≤τCP,  the UL transmission is synchronized since the UL waveform of N UEs can be processed by a single FFT/IFFT without ISI.

- If |τN-τ1|>τCP,  the UL transmission is asynchronized since the UL waveform of N UEs cannot be processed by a single FFT/IFFT without ISI.

To enable UL synchronized NOMA, there are two options for DL synchronized UEs:

- Four-step RACH

- Configuration of long CP

However, the system overheads and latency for options a) and b) are significant. Option a) relies on sequential signaling exchange between UE and base station, and the timing advance operation introduces large latency and power consumption. In Option b), the use of “long” CP involves the change of waveform numerology in 3GPP specification, when the SCS is different from 60 kHz or the round-trip delay is greater than the length of ECP. Therefore, we have the following proposal:

Proposal 4:  For normal cell coverage and DL synchronized UE, solutions capable of achieving a better tradeoff than 4-step RACH and CP extension in performance, latency and complexity should be studied for asynchronized NOMA.

According to the agreements on LLS and SLS [1, 3, 4], a typical deployment considered for NR NOMA is ISD 1732 m, carrier frequency 700 MHz, SCS 15 kHz and normal CP (NCP). As a result, an estimate for the round-trip delay can be given by 2×1000300≈6.7 [μs], which is roughly 1.5 times the length of NCP.

The procedures of two-step RACH are shown in Figure 1. The joint use of advanced receiver at gNB and enhanced MA signature mapping at UE will help with MA signature detection, contention resolution and small data detection. The design details of MSG1 and MSG2 can be summarized as follows:

- MSG1 carries both MA signature and small data.

  - MA signatures can be carried by a combination of preamble sequence and DMRS sequence.

  - Preamble/DMRS sequences can be down-selected from the PRACH preamble and DMRS sequences standardized in NR Rel-15.

  - The small data in MSG1 is carried by PUSCH, which can be transmitted without timing advance (TA) and facilitates great savings in latency and transmit power.

  - ML-RSMA can be used as the transmission scheme for UL data. In particular, symbol-level scrambling can be considered to reduce the PAPR and inter-cell interference.

  - After transmitting MSG1, UE monitors the DCI of MSG2 within a pre-configured search space and RAR-Window.

- MSG2 carries a PDCCH (DCI) and a PDSCH (RAR).

  - The DCI specifies the frequency domain resource allocation of RAR. The payload bits of DCI is scrambled with RA-RNTI, whose calculation can be based on the time/frequency resource index (3GPP 38.321-5.1.3) pre-configured for MSG1 transmission as well as the index of MA signature that has been successfully detected by gNB.

  - The inclusion of MA signature index in RA-RNTI calculation helps with contention resolution and latency reduction.

  - After UE successfully decodes the DCI, it can proceed with the decoding of RAR.

  - FFS data structure of MAC PDU for RAR.

Figure 1: Procedures for Two Step RACH

As a result, we have the following proposal for asynchronized NOMA in normal cell coverage:

Proposal 5: The asynchronized NOMA transmission scheme in normal cell coverage, such as two-step RACH, should be studied, wherein MSG1 carries MA signature and small data, and UE does not need to perform timing advance prior to MSG1 transmission.

# DMRS Sequence Design for NOMA

For grant-based or grant-free NOMA, the UL DMRS transmission is synchronized. The DMRS sequences can be orthogonal or non-orthogonal. To obtain reliable channel estimation, orthogonal design for DMRS sequences is preferred. A simple extension of DMRS ports can consider a joint use of OCC, comb-based RE mapping, orthogonal base sequences with cyclic shifting. As a result, at least 24 orthogonal DMRS sequences can be supported by two DMRS symbols within a slot/mini-slot. To accommodate more NOMA UEs, larger overhead of DMRS can be considered.

For asynchronized NOMA, channel estimation can be based on the joint processing of preamble and DMRS. To reduce the ISI, successive detection and data-aided interference cancellation can be used.

Therefore, we propose the following:

Proposal 6: For synchronized NOMA transmission, at least 24 orthogonal DMRS sequences can be supported by two DMRS symbols. Larger overhead and/or non-orthogonal DMRS sequences can be considered for larger overloading factors.

Proposal 7: For asynchronized NOMA transmission, both preamble and DMRS sequences can be transmitted with small data. Channel estimation can be based on the joint processing of preamble and DMRS, as well data-aided interference cancellation.

# HARQ in NOMA UL

Depending on the NOMA operation modes shown in Table 1, the HARQ process can take different forms:

- For grant-free, contention-based transmission with synchronized UL,

  - a scheduling request (SR) can be appended to the first transmission.

  - the re-transmissions can be grant-free as well, but the timeline for re-transmission is staggered and aligned to the symbol/slot boundary within a pre-configured contention window. If the initial transmission fails, the UE will use random access for re-transmission.

- FFS HARQ procedures for asynchronized NOMA.

Considering this, we propose the following:

Proposal 8: For UL NOMA, HARQ needs to be considered for both synchronized and asynchronized transmissions. The re-transmission can be NOMA or OMA, grant-based or grant free.

# Link Adaptation and Power Control

As shown in Table 1, it is not necessary to consider dynamic MCS for initial transmission in grant free NOMA. However, link adaptation and power control can be supported for HARQ re-transmission. Therefore, we have the following proposal:

Proposal 9: Power domain multiplexing and power control can be used in UE grouping and interference management of NOMA. Link adaptation and power control can be supported for grant-based re-transmission of NOMA, which can use dynamic configuration of the spreading factor, code rate and single/multiple branches.

# Conclusions

This contribution has provided our views on the procedures related to NOMA UL transmission. The following proposals have been made:

Observation 1

- When the network operates in grant-based mode, transmission schemes proposed for NOMA can be applied to MU-MIMO as well. The gain of NOMA over MU-MIMO in spectral efficiency is questionable, especially for underloading scenarios.

- When the network operates in grant-free mode and the UL access is contention-free, the gain of NOMA over MU-MIMO in spectral efficiency is questionable.

- The most significant gain of NOMA over MU-MIMO can be achieved in the following scenarios:

  - contention-based, grant-free transmission

  - small data transmission from RRC_INACTIVE state

Proposal 1:  NR NOMA solutions achieving performance gains over NR Rel-15 MU-MIMO should be prioritized in the study and evaluation.

Proposal 2: For grant-based UL transmission, the degree of freedom can be defined as the number of orthogonal physical resource units in time, frequency and space domains, which is measured for a given TTI, a given transmission bandwidth and a given number of transceiver antennas.

Proposal 3: For synchronized NOMA transmission, TX solutions incapable of UE overloading and/or unsuitable for scalable configuration should be deprioritized and FFS whether they have significant performance gains over OMA with the same transceiver complexity.

Proposal 4:  For normal cell coverage and DL synchronized UE, solutions capable of achieving a better tradeoff than 4-step RACH and CP extension in performance, latency and complexity should be studied for asynchronized NOMA.

Proposal 5: The asynchronized NOMA transmission scheme in normal cell coverage, such as two-step RACH, should be studied, wherein MSG1 carries MA signature and small data, and UE does not need to perform timing advance prior to MSG1 transmission.

Proposal 6: For synchronized NOMA transmission, at least 24 orthogonal DMRS sequences can be supported by two DMRS symbols. Larger overhead and/or non-orthogonal DMRS sequences can be considered for larger overloading factors.

Proposal 7: For asynchronized NOMA transmission, both preamble and DMRS sequences can be transmitted with small data. Channel estimation can be based on the joint processing of preamble and DMRS, as well data-aided interference cancellation.

Proposal 8: For UL NOMA, HARQ needs to be considered for both synchronized and asynchronized transmissions. The re-transmission can be NOMA or OMA, grant-based or grant free.

Proposal 9: Power domain multiplexing and power control can be used in UE grouping and interference management of NOMA. Link adaptation and power control can be supported for grant-based re-transmission of NOMA, which can use dynamic configuration of the spreading factor, code rate and single/multiple branches.

# References

- RP-181403, “Revision of Study on 5G Non-orthogonal Multiple Access”, 3GPP TSG RAN #80, ZTE.

- Chairman’s Notes, 3GPP TSG RAN1 #94, Gothenburg, Sweden.

- Chairman’s Notes, 3GPP TSG RAN1 #92, Athens, Greece.

- Chairman’s Notes, 3GPP TSG RAN1 #92b, Sanya, China.

- R1-1811243, “Transmitter Side Signal Processing Schemes for NOMA,” Qualcomm.

- R1-1811246, “Link Level Evaluation for NOMA,” Qualcomm.

A discussion for underloading/overloading can be found in Section 2.4 of this contribution.

A discussion for underloading/overloading can be found in Section 2.4 of this contribution.