3GPP TSG RAN WG1 Meeting #95	R1-1812751

Spokane, USA, 12th – 16th November 2018

Agenda Item:	7.2.10.1

Source: 	Sony

Title:	Remaining issues on NR positioning requirements

Document for:	Discussion / Decision

# Introduction

A new study item “Study on NR positioning support” was approved in RAN plenary meeting #80 [1] . The objective of this study item is to evaluate potential solutions to address NR positioning requirements as defined in TR 38.913, TS 22.261, TR 22.872 and TR 22.804 while considering E911 requirements by analysing positioning accuracy (including latitude, longitude and altitude), availability, reliability, latency, network synchronization requirements and/or UE/gNB complexity to perform positioning, and taking into account a preference to maximize synergy where possible with existing positioning support for E-UTRAN.

With respect to the positioning requirements, RAN1 has made the following agreement in [2] :

Agreement:

- Regulatory requirements are considered as a minimum performance targets for NR Positioning studies

- Additional requirements based on commercial use cases can be used as input performance targets that are subject to further analysis in terms of performance / complexity tradeoffs in different evaluation scenarios

- For regulatory use cases, the following requirements are considered as a minimum performance targets for NR positioning

  - Horizontal positioning error <= 50m for 80% of UEs

  - Vertical positioning error [<5 m] for [80%] of UEs

    - Note: The regulatory requirements refer to floor level vertical accuracy

  - End to end latency and TTFF < 30 seconds

- As a starting point for commercial use cases, the following requirements are considered as performance targets for RAT dependent solutions, which are subject to further analysis in terms of performance / complexity tradeoffs of NR positioning radio-layer solutions

  - Horizontal positioning error < [3]m for [80]% of UEs in indoor deployment scenarios

  - Horizontal positioning error < [10]m for [80]% of UEs in outdoor deployments scenarios

  - Vertical positioning error < [3]m for [80]% of UEs in indoor deployment scenarios

  - Vertical positioning error < [3]m for [80]% of UEs in outdoor deployment scenarios

  - End to end latency < [1]s

- Note: This does not eliminate more or less demanding commercial use cases.

# In this contribution, we discuss our view on the remaining NR positioning requirements, particularly on the commercial use cases requirements.

# Discussion

During the discussion in RAN1 #94bis, we often referred the commercial use cases requirements to the positioning service levels defined by System Architecture (SA) WG1 (SA1). SA1 has included all the Rel-16 positioning RQs and KPIs (e.g. as appears in various technical report TRs 22.872/22.804/22.862) in TS 22.261 [3] as shown below in Table 1.

Table 1: Positioning service levels and performance requirements from SA1[3]

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Availability | Latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99.9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0.3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0.3 m | 2 m | 99.9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0.2 m | 0.2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each others (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |

RAN1 has made progress by adopting the commercial use-cases requirements and defined some requirements (horizontal, vertical, availability and latency) as a starting point. The requirements are not exactly the same as in Table 1 above, but at least they partly cover the above requirements, particularly relating to service level 1. The RAN1 starting point requirement for availability for commercial use-cases has been lowered with respect to Table 1  ([80%] versus 95%). It has been agreed that some values are in “[]” indicating these can be further revised pending simulation results.

We consider the same set of references should be used in order to not mix the requirements and to not convey various messages on positioning requirements. We consider RAN1 should strive to set the commercial requirements for RAN1 studies based on the positioning service levels as defined in TS22.261. Thus, we should align the requirements. For example, RAN1 should strive to fulfil service level 1 during the NR release 16 study item phase for the commercial requirements.

Proposal 1: RAN1 should strive to achieve the commercial requirements based on the positioning service levels as defined in TS22.261 (e.g. service level 1).

During the study item phase, it may be the case that it may be difficult to fulfil the commercial requirements based on the agreed evaluation scenarios. This difficulty may be related to RAN1’s aim is to evaluate the requirements on selected scenarios based on RAT dependent techniques. There are some limitations in the RAT dependent techniques (e.g. signal bandwidth, number of reference signals, cell lay-out, etc) and it may be difficult to achieve the aforementioned requirements. In case such a situation is identified, we consider it to be desirable to provide some analysis / clarifications to be captured in the RAN1 technical report (i.e. TR 38.855). For example, on the need of RAT independent techniques / hybrid positioning or the possibility to achieve the commercial use-cases requirements in a denser network.

Proposal 2: In case some of the requirements are not achieved (i.e. by the RAT dependent technique) some clarifications shall be captured in the RAN1 technical report (i.e. TR 38.855).

# Conclusion

In this contribution, we discuss our view on the remaining issues on NR positioning requirements and obtain the following proposals:

Proposal 1: RAN1 should strive to achieve the commercial requirements based on the positioning service levels as defined in TS22.261 (e.g. service level 1).

Proposal 2: In case some of the requirements are not achieved (i.e. by the RAT dependent technique) some clarifications shall be captured in the technical reports (i.e. TR 38.855).

# References

- RP-181399, “New SID: Study on NR positioning support”

- Chairman’s notes RAN1 Meeting #94bis, Chengdu, October 2018

- TS 22.261 V16.5.0 “Service requirements for next generation new services and markets”