3GPP TSG RAN WG1 Meeting #95			R1-1812947

Spokane, USA, November 12th – 16th, 2018

Agenda item:	6.2.2.2

Source:	Samsung

Title:	Discussion on transmission in preconfigured UL resources for NB-IOT

Document for:	Discussion and Decision

Introduction

In RAN1#94 Meeting, there are following agreements on transmission in preconfigured UL resources for NB-IOT [1]:

Agreement

In idle mode, the UE will at least consider one or more of the following attributes when validating TA (combination of multiple attributes is allowed):

- Serving cell changes (serving cell refers the cell that the UE is camping on)

- Time Alignment Timer for idle mode

- Serving cell NRSRP changes (serving cell refers the cell that the UE is camping on)

- FFS Other attributes:

  - Neighbour cell NRSRP change

  - TDOA of >=2 eNBs

  - TA History

  - Subscription based UE differentiation

  - Others not precluded (for example, attributes that need to be considered for high mobility UEs)

Note that UE power consumption should be taken into account for the FFS attributes

Agreement

For transmission in preconfigured UL resources, an RRC idle UE may use the latest TA that passed the validation criteria

Agreement

Pre-configured UL resources for transmission of data are indicated by RRC signaling. At least UE-specific RRC signaling is supported.

Agreement

The resource configuration includes at least the following

- Time domain resources including periodicity(s)

- Frequency domain resources

- TBS(s)/MCS(s)

Agreement

For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported.

Agreement

Dedicated preconfigured UL resource is defined as an NPUSCH resource used by a single UE

- NPUSCH resource is time-frequency resource

- Dedicated PUR is contention-free

Contention-free shared preconfigured UL resource (CFS PUR) is defined as an NPUSCH resource simultaneously used by more than one UE

- NPUSCH resource is at least time-frequency resource

- CFS PUR is contention-free

Contention-based shared preconfigured UL resource (CBS PUR) is defined as an NPUSCH resource simultaneously used by more than one UE

- NPUSCH resource is at least time-frequency resource

- CBS PUR is contention-based (CBS PUR may require contention resolution)

Agreement

In IDLE mode, HARQ is supported for transmission in dedicated PUR

- A single HARQ process is supported,

  - FFS whether two HARQ processes are supported

- FFS: The design of the corresponding NPDCCH search space

Agreement

In idle mode, dedicated PUR is supported.

- Support for CFS PUR is FFS.

- Support for CBS PUR is FFS.

In this contribution, we discussed some issues to specify support for transmission in preconfigured UL resources for NB-IOT.

Transmission in preconfigured UL resources

RRC-connected mode

Transmission in preconfigured UL resources means grant-free UL transmission, i.e., no dynamic scheduling DCI. There may be multiple possibilities considering diverse signaling for pre-configuration of UL resources, e.g., L1, L2 and/or L3 signaling. In 5G NR system, two types of grant-free UL transmission are specified, wherein, Type 2 grant-free UL transmission is the same as UL SPS in LTE system and Type 1 grant-free UL transmission introduces semi-static resource configuration in RRC without L1 signaling. Comparing with Type 2, Type 1 can improve latency performance, e.g., arrived data can be immediately transmitted in preconfigured UL resources and needn’t wait L1 signaling for resources activation. But, the flexibility of resource allocation in Type 1 is worse than Type 2 since resource activation/reactivation/deactivation is based on L1 signaling in Type 2. And, there may be resource wasting during the ambiguity time of RRC signaling for resource configuration/reconfiguration in Type 1. For a NB-IOT UE, latency performance may not be an important metric. Thus, existing UL SPS with L1 signaling for resource activation/reactivation/deactivation is more practical than other grant-free transmission like Type 1.

For NB-IOT, legacy UL SPS has been supported for periodic BSR in Rel-15, and it can be directly extended to data transmission without much modification. If UL SPS is enabled for data transmission, UL SPS configuration for periodic BSR is unnecessary since BSR can be transmitted with data. For transmission in preconfigured UL resources in RRC-connected mode, the functionality of legacy UL SPS is enough and no new transmission in preconfigured UL resources is needed for Rel-16 NB-IOT.

Proposal #1: For Rel-16 NB-IOT, UL SPS in RRC-connected mode can be extended to data transmission from periodic BSR.

RRC-idle mode

In this section, some issues on transmission in preconfigured UL resources in RRC-idle mode are discussed, e.g., dedicated resources or shared resources, validity period of dedicated PUR, mandatory or optional transmission, activation/deactivation of preconfigured UL resources, TA validation, acquisition and update, HARQ procedure and fallback mechanism.

    - Dedicated resources or shared resources

In the last meeting, dedicated PUR is agreed and shared PUR is FFS. There are two types of shared PUR, i.e., CFS (Contention-free Shared) PUR and CBS (Contention-based Shared) PUR. For CFS PUR, orthogonal multiple access scheme is used for multiple UEs sharing the same time-frequency resource, e.g., MU-MIMO and CDM. For MU-MIMO, it can be transparently supported using dedicated PUR with UE specific DMRS configuration. How many UEs are configured to share the same time-frequency resource using MU-MIMO is up to eNB configuration and transparent for UE. Therefore, CFS PUR using MU-MIMO with UE specific DMRS pattern needn’t be specified. For CDM, OCC may be required and much specification effort will be expected, e.g., how to use OCC with either subframe level or symbol level. In the other hand, in RRC-idle mode, it is difficult to use closed loop power control to achieve approximate receiving power for CDMed UEs, which is a key factor to obtain robust multi-user detection performance for CDM. Therefore, CFS PUR using CDM needn’t be considered due to much specification effort and difficulty of power control.

Proposal #2: CFS PUR needn’t be specified.

For CBS PUR, contention resolution is required to ensure the reliability of PUSCH, and it may cause much more specification effort. Transmission in CBS PUR with contention resolution may be like two-step PRACH procedure. For example, in the first step, UE ID is reported in the contention based PUSCH transmitted in the CBS PUR, and in the second step, contention resolution signaling carrying the reported UE ID will be monitored. From UE’s perspective, additional power consumption may be caused for monitoring of contention resolution signaling. Moreover, assuming only orthogonal multiple-access schemes can be used, the capacity of configurable CBS PUR is very limited comparing with PRACH capacity. Thus, the probability of PUSCH collision will be very large, and successful decoding of collided PUSCH may be be more challenged comparing with PRACH collision. Since CBS PUR may cause much more specification effort, more UE power consumption, and more challenges for eNB receiver, CBS PUR is not supported.

Proposal #3: CBS PUR is not supported.

    - Validity period of dedicated PUR

In the last meeting, it was agreed to support UE-specific RRC signalling for configuration of dedicated PUR. This means dedicated PUR can be configured in RRC-connected mode and used in RRC-idle mode. Similarly, UE-specific RRC signalling can also be used to reconfigure/release dedicated PUR. However, some UE with dedicated PUR may not be able to release the allocation, e.g., goes out of coverage, breaks, or lose power. Therefore, validity period should be introduced for dedicated PUR. If validity period expires, dedicated PUR shouldn’t be used, and eNB can release the allocation and used for other UE. The timer of validity period can be started when receiving dedicated PUR configuration, or when firstly switching into RRC-idle mode after receiving dedicated PUR configuration. Validity period can be configurable, and indicated in UE-specific RRC signaling or system information.

Proposal #4: Validity period should be introduced for dedicated PUR.

On validity of dedicated PUR, another problem is whether dedicated PUR can be used if the camped cell is changed in RRC-idle mode. For a UE in cell edge, cell reselection may often happen, and the issue is worth to be discussed. One option is that dedicated PUR cannot be used in the new camped cell, i.e., dedicated PUR is cell specific. For efficient resource utilization, the UE should release the dedicated PUR, e.g., UE reports the information of dedicated PUR in the new camped cell. Thus, information exchange related to dedicated PUR may be required between neighboring cells. Another option is that dedicated PUR can still be used in the new camped cell. For example, dedicated PUR may be reserved by multiple cells, i.e., dedicated PUR is cell-group specific. Thus, even the camped cell providing dedicated PUR is changed, the dedicated PUR can still be used. The problem needs to be further discussed to clarify UE behavior.

Proposal #5: FFS whether dedicated PUR can be used if camped cell is changed.

    - Mandatory or optional transmission

For transmission in dedicated PUR, one issue is that whether UE can skip transmission in dedicated PUR if no any UL data arrivals at UE side, i.e., mandatory or optional transmission. Since dedicated PUR configuration may not always match to traffic model, UE may have nothing to send in some PUR occasion. If skipping transmission in dedicated PUR is allowed, UE power consumption can be saved. But if a UE is not mandated to transmit on every dedicated PUR, the UE may request more resources than it really needs. For example, a UE may request a dedicated PUR for every 10 seconds but only transmits every 1 hour. This would be very spectrally inefficient. To avoid this, the maximum number of continuous skipping transmission in dedicated PUR should be restricted. If the allowed maximum number of continuous skipping is reached, either dedicated PUR is implicitly released, or a PUSCH carrying zero MAC SDU needs to be transmitted if no any UL data arrivals.

Proposal #6: UE is allowed to skip transmission in dedicated PUR, but the maximum number of continuous skipping should be restricted.

    - Activation/deactivation of preconfigured UL resources

In some case, eNB may want to temporarily turn off transmission in PUR since the cell’s traffic load is relatively high. For example, the cell’s traffic load status may be changed from the time when configuring the PUR. Although a dedicated PUR is within a validity period, eNB may want to take back the PUR for dynamical scheduling, i.e., the PUR is activated. When the cell’s traffic load is mitigated, transmission in PUR can be turned on again, i.e., the PUR is activated. Since the cell’s traffic load is not dependent on a certain UE, taking back of PUR should be used for all of UEs with dedicated PUR provided by the cell. Therefore, eNB can turn on or turn off the functionality of transmission in PUR using a broadcast signaling, i.e., preconfigured UL resources can be activated/deactivated in system information.

Proposal #7: Preconfigured UL resources can be activated/deactivated in system information.

    - TA validation, acquisition and update

To support transmission in PUR in RRC-idle mode, a valid TA is necessary. The initial TA in RRC-idle mode can be acquired in RRC-connected mode. During RRC-idle mode, the initial TA should be validated using some validation criteria. On TA validation, one agreement in the last meeting is that the UE will at least consider one or more of the following attributes when validating TA: serving cell changes, time alignment timer for idle mode, serving cell RSRP changes, and FFS other attributes, e.g., neighbour cell RSRP change, TDOA of >=2 eNBs, TA history and subscription based UE differentiation. Among above attributes, we think RSRP change may not always determine TA change. RSRP change is highly correlated to the change of scatterer environment, e.g., building. However, TA change is highly correlated to the change of distance between UE and eNB. Generally, there may be a large RSRP change but no TA change. Using RSRP change for TA validation may often cause false alarm. Therefore, we propose RSRP change is not considered as one of validation criteria.

Proposal #8: RSRP change is not considered as one of validation criteria.

If TA is determined to be invalid using above validation criteria, how to obtain a new valid TA is a problem. One approach is to fall back to legacy RACH/EDT procedure to send data and obtain a new valid TA. The advantage of this approach is that no new specification changes and just implementation is required. Another approach is to use a non-contention based two-step RACH procedure, wherein a UE-specific PRACH preamble is transmitted and a new valid TA is obtained in an optimized RAR. The advantage of this approach is that the data can still be sent on the PUR. However, one disadvantage is that very much specification work is required to specify the two-step RACH procedure dedicated for TA acquisition. Another disadvantage is that PRACH capacity used for normal RACH procedure may be impacted if the UE-specific PRACH preamble is allocated in legacy RACH group. If the UE-specific PRACH preamble is allocated in a new PRACH group dedicated for TA acquisition, resource overhead is also one problem. Considering much specification effort and limited PRACH capability, non-contention based RACH procedure is not preferred. Fallback to legacy RACH/EDT procedure is recommended.

Proposal #9: If TA is determined to be invalid using validation criteria, a new valid TA can be acquired via legacy RACH/EDT procedure.

During HARQ procedure of transmission in PUR, a small range of TA may be able to be detected using PUSCH/DMRS, wherein the detectable range using PUSCH/DMRS is smaller than that using PRACH. Thus, TA can be updated within a small range for TA tracking. To save signalling overhead, MAC CE for TA update can be excluded, and the updated TA can be indicated in DCI, e.g., 2~4bits. Although a small error of TA may not impact the decoding of PUSCH, accumulation of the small error would cause significant impact and PUSCH may not be decodable. Therefore, TA update is helpful even within a small range. Continuous TA update in each PUR can maintain a valid TA during a long time.

Proposal #10: TA can be updated within a small range in DCI during HARQ procedure.

    - HARQ procedure

For HARQ procedure of transmission in dedicated PUR in RRC-idle mode, a single HARQ process is agreed in the last meeting, and FFS whether more one HARQ processes are supported. The configurable periodicity of dedicated PUR is expected to be large to match the traffic characteristic applicable for preconfigured UL transmission in RRC-idle mode. There is enough time to schedule several retransmissions to ensure successful decoding before next PUR occasion arrivals. Since there is no overlapping for HARQ timelines of transmission in two close PUR occasions, more than two HARQ processes are not required. In addition, at least in this release, multiple activated PUR configurations with separate HARQ process needn’t be considered since the corresponding traffic model is not clear. Therefore, more than one HARQ processes needn’t be supported at least in this release.

Proposal #11: More than one HARQ processes are not supported at least in this release.

To support HARQ procedure of transmission in dedicated PUR in RRC-idle mode, UE should monitor a DCI scrambled with a preconfigured RNTI value in a preconfigured MPDCCH search space within a preconfigured window after transmitting PUSCH. In order to save UE power consumption, ACK feedback can be introduced for early termination of DCI monitoring within the preconfigured window. And, the ACK can be carried by DCI. If ACK DCI is received, UE can immediately terminate DCI monitoring and return sleep. In addition, ACK feedback can be used to explicitly notify the UE whether PUSCH is successfully decoded. If the ACK DCI is not received during the preconfigured window, the UE can fall back to legacy RACH/EDT procedure to send data. The explicit ACK feedback and fallback mechanism can improve the reliability of data transmission. The design of ACK DCI specified for termination of PUSCH in Rel-15 MTC can be used as reference.

Proposal #12: ACK DCI should be introduced.

    - Fallback mechanism

For transmission in preconfigured UL resources, fallback mechanism to RACH/EDT procedure were agreed in the last meeting, but, the details is still FFS. There may be several cases to use fallback mechanism to RACH/EDT procedure. In the first use case, preconfigured UL resources cannot be used to send data. For example, there is no valid TA, the arrival data volume exceeds the allowed maximum TBS of preconfigured UL transmission, or the preconfigured repetition number is not enough due to CE level change, and so on. Thus, UE can fall back to RACH/EDT procedure to send data. In the second use case, transmission in preconfigured UL resources is not successfully decoded, e.g., ACK is not received during a preconfigured window. Thus, UE can fall back to RACH/EDT procedure to resend data. In the third use case, during the HARQ procedure, eNB may trigger UE to fall back to RACH/EDT procedure. For example, the used TA by UE may have a large error and successful decoding cannot be ensured via HARQ combination. Above use cases should be further discussed to conclude which use case can be supported. For the first two use cases, fallback is triggered by UE and no new signalling is required. For the last one use case, fallback is triggered by eNB, and an indication signalling needs to be introduced. If fallback is triggered by eNB, using DCI to indicate fallback to EDT/RACH procedure can be considered.

Proposal #13: Further discuss the use cases of fallback mechanism, and using DCI to indicate fallback to EDT/RACH procedure can be considered for fallback triggered by eNB.

- 3. Conclusion

In this paper, we discussed some issues to specify support for transmission in preconfigured UL resources for NB-IOT. Based on above discussion, we have the following proposals:

For transmission in PUR in RRC-connected mode,

Proposal #1: For Rel-16 NB-IOT, UL SPS in RRC-connected mode can be extended to data transmission from periodic BSR.

For transmission in PUR in RRC-idle mode,

Proposal #2: CFS PUR needn’t be specified.

Proposal #3: CBS PUR is not supported.

Proposal #4: Validity period should be introduced for dedicated PUR.

Proposal #5: FFS whether dedicated PUR can be used if camped cell is changed.

Proposal #6: UE is allowed to skip transmission in dedicated PUR, but the maximum number of continuous skipping should be restricted.

Proposal #7: Preconfigured UL resources can be activated/deactivated in system information.

Proposal #8: RSRP change is not considered as one of validation criteria.

Proposal #9: If TA is determined to be invalid using validation criteria, a new valid TA can be acquired via legacy RACH/EDT procedure.

Proposal #10: TA can be updated within a small range in DCI during HARQ procedure.

Proposal #11: More than one HARQ processes are not supported.

Proposal #12: ACK DCI should be introduced.

Proposal #13: Further discuss the use case of fallback mechanism, and using DCI to indicate fallback to EDT/RACH procedure can be considered for fallback triggered by eNB.

- 4. Reference

- Final Chairman’s notes for RAN1#94 Meeting