3GPP TSG-RAN WG1#95	R1-1813161

Spokane, USA, November 12th – 16th, 2018

Agenda item:		7.2.1.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Complexity Analysis of ESE-based Soft IC Receiver

Document for:		Discussion and Decision

# 1	Introduction

This document is an update of R1-1811132.

In this contribution, we analyze the complexity of the ESE-based soft IC receiver. In RAN1#94 [1], the template for receiver complexity was agreed.

Agreements:

- The following table for computation complexity analysis of the receiver as the starting point, entries can be updated till RAN1#94bis.

Table I	Template of Receiver Computation Complexity breakup

| Receiver component | Detailed component | Computation in parametric number of usages, O(.) analysis, [impact factor] |  |  |
| --- | --- | --- | --- | --- |
|  |  | Receiver type 1 | Receiver type 2 | … |
| Detector | UE detection |  |  |  |
|  | Channel estimation |  |  |  |
|  | Rx combining, if any |  |  |  |
|  | Covariance matrix calculation, if any |  |  |  |
|  | Demodulation weight computation, if any |  |  |  |
|  | UE ordering, if any |  |  |  |
|  | Demodulation, if any |  |  |  |
|  | Soft information generation, if any |  |  |  |
|  | Soft symbol reconstruction, if any |  |  |  |
|  | Message passing, if any |  |  |  |
|  | Others |  |  |  |
| Decoder | LDPC decoding |  |  |  |
| Interference cancellation | Symbol reconstruction(Including FFT operations for DFT-S-OFDM waveform), if any |  |  |  |
|  | LLR to probability conversion, if any |  |  |  |
|  | Interference cancellation |  |  |  |
|  | LDPC encoding, if any |  |  |  |
|  | Others |  |  |  |

- The impact factor is to be estimated based on the analysis of computation, memory size, hardware and software implementation, etc.

  - If/How and which entries are to be combined/compared in order to get the total complexity estimate is FFS.

- Companies may provide the impact factor

  - The impact factor is for each cell

- The rows in the above table are subject to potential re-finement, e.g., adding new row(s), merge some rows, etc.

- Note: the numbers may or may not be a function of UL waveform

- FFS whether or not to add row(s) for memory blocks

In our companion contribution [4], we describe how the impact factor can be determined.

Furthermore, in RAN1#94-bits [2], Tables 8 and 9 of R1-1811938 [6] capturing the computational complexity and assumptions for calculating the computation complexity were agreed.

# 2	Overview of ESE-based Soft IC Receiver

Figure 1 shows the block diagram of an ESE-based soft IC receiver. The elementary signal estimator (ESE) [3] estimates the LLR of each UE by assuming other UEs as AWGN using their prior LLR signal estimates. Extrinsic information can be obtained at the output of the despreader or as the soft output of the LDPC decoder. By excluding the decoder from the feedback loop, latency is improved, and complexity is reduced at the expense of performance.

Figure 1: ESE-based Soft IC Receiver.

Notation of parameters in this contribution (aligned with R1-1811938 [6]):

- NUE	: number of UEs.

- NREDMRS	: Total number of REs for DMRS.

- NREdata	: Total number of REs for data.

- NAPDMRS	: Number of DMRS antenna ports.

- Nrx	: Number of Rx antennas

- NRECE: Number of DMRS taps used for channel estimation filtering across data REs

- NBlkCE: Number of REs in a channel estimation block. There is one channel estimate per channel estimation block.

- NSF: Spreading factor.

- NIterIC: Number of iterations.

# 3	Complexity of ESE-based Soft IC Receiver

In this section, we analyze the computation complexity of an ESE-based soft IC receiver. Figure 2 shows a block diagram of the ESE-based receiver used for IDMA, with the detailed components of each of the three functional units of the receiver.

Figure 2: Block diagram of IDMA receiver.

In the following sections, the following notation is used

NUE is the number of users being received.

The received signal in chip j is: rj=rRej+irIMj

The channel estimate of user k in chip j is: hkj=hkRej+ihkImj

Furthermore, we define the following:

hkRe2=hkRe(j)2, hkIm2=hkIm(j)2

hk2=hkRe2+hkIm2, hk4=(hk2)2

The mean of user k in chip j is: Exkj=ExkRej+i ExkImj

The variance of user k in chip j is: Varxkj=VarxkRej+i VarxkImj

The mean of the received signal in chip j is: Exj=ErRej+iErIMj

The variance of the received signal in chip j is: Varxj=VarrRej+iVarrIMj

## 3.1	Computational Complexity of Detector

The detector is made up of the following components: UE detector, channel estimation, ESE-LLR estimation, user-specific de-interleaver and de-spreader.

### 3.1.1	UE detection

UE detection is performed once per UE per received-slot. UE detection is performed on the DMRS symbols or on the preamble. The order of complexity of UE detection per UE is:

UE Detection Complexity ~ NAPDMRS∙NREDMRS∙Nrx

A simple UE detector can be a correlator with a reference signal, which requires NAPDMRS∙NREDMRS∙Nrx complex multiplications and NAPDMRS∙NREDMRS∙Nrx complex additions. This is done for each antenna port the UE is received on. The square of the amplitude of each antenna’s correlation output is summed, this is followed by a comparison against a threshold.

The number of UE detections per received slot equals the number of potential UEs that can transmit in that slot.

### 3.1.2	Channel estimation

Channel estimation is performed once per UE per received-slot. Channel estimation is performed on the DMRS symbols, and filtered (interpolated) across the data symbols. The order of complexity of channel estimation per UE is:

Channel Estimation Complexity ~ NREDMRS+NRECE∙NREdataNBlkCE∙Nrx

A simple channel estimation can be a multiplication of the received DMRS signal with a reference signal, reused from the UE detection, hence not included in the channel estimation row of Table 1, and then filtering in time domain to get the channel estimate at each data RE. Assuming NRECE DMRS symbols are used for filtering, this requires NRECE∙NREdataNBlkCE∙NRx complex multiplications and additions per UE. The channel estimate is calculated once for each NBlkCE data REs, as described in [5].

The number of channel estimations per received slot equals the number of UEs received in a slot (NUE). This assumes that channel estimation is performed once for each UE and used in all the following iterations.

The channel estimates are buffered to be used in subsequent stages such as ESE-LLR estimation and UE statistics update. The memory for storing the channel estimates is: NREdataNBlkCE∙NRx complex values.

### 3.1.3	ESE-LLR estimation

This block computes the ESE LLRs using the received signal in RE (rj), and the user statistics/receive signal statistics from the previous iteration. This is performed for each antenna, and the ESE-LLRs of the different antennas are combined at the end. These are the equations for calculating the ESE-LLR per antenna per UE, where k is the UE index and j is the chip (RE) index:

E[Re(hk*(j)ζ(j))]=hkRe(j)E[rRe(j)]+hkIm(j)E[rIm(j)]-hk2E[xkRe(j)]

E[Im(hk*(j)ζ(j))]=hkRe(j)E[rIm(j)]-hkIm(j)E[rRe(j)]-hk2E[xkIm(j)]

Var[Re(hk*(j)ζ(j))]=hkRe2Var[rRe(j)]+hkIm2Var[rIm(j)]

+2hkRe(j)hkIm(j)ψ(j)-hk4Var[xkRe(j)]

Var[Im(hk*(j)ζ(j))]=hkRe2Var[rIm(j)]+hkIm2Var[rRe(j)]

-2hkRe(j)hkIm(j)ψ(j)-hk4Var[xkIm(j)]

eESE(xkRe(j))=2hk2Re(hk*(j)r(j))-E[Re(hk*(j)ζ(j))]Var[Re(hk*(j)ζ(j))]

eESE(xkIm(j))=2hk2Im(hk*(j)r(j))-E[Im(hk*(j)ζ(j))]Var[Im(hk*(j)ζ(j))]

ESE-LLR Complexity ~ Nsc∙Nsymbdata∙NRx

Number of operations:

A complex multiply is equivalent to 4 real multiplications. Complex additions are ignored in this analysis. Complex divisions are two real divisions (dividing a complex value by a real value).

Number of complex Multiplications: 5.5∙NREdata∙Nrx

Number of complex divisions: NREdata∙Nrx

The number of ESE LLR estimations per received slot equals the number of UEs that transmit in a slot multiplied by the number of iterations (NUE∙NIterIC).

The ESE-LLR estimates are buffered to be used in subsequent stages such as the user specific interleaver. The ESE-LLR estimates already have the received antennas combined. The memory for storing the ESE-LLR estimates is: NUE∙NREdata Complex values.

### 3.1.4	User Specific De-interleaver

This block is executed after antenna combining, hence the order of complexity is given by:

Deinterleaver Complexity ~ NREdata

This block can be viewed as a memory read followed by a memory write to a different memory location, but can also be implemented transparently, by reading data out of order for the subsequent block.

The number of user specific de-interleavers per received slot equals the number of UEs that transmit in a slot multiplied by the number of iterations (NUE∙NIterIC).

### 3.1.5	Despreader

The despreader adds NSF consecutive LLRs to generate one value that subsequently goes to the demodulator and decoder.

Despreader Complexity ~ NREdata

The number of complex additions in is: NREdataNSF-1NSF≅NREdata

The number of despreaders per received slot equals the number UEs that transmit in a slot multiplied by the number of iterations (NUE∙NIterIC).

The despreader out is buffered to be used in the decoder. The memory for storing the output is: NUE∙NREdataNSF Complex values.

## 3.2	Complexity of Decoder

An LDPC decoder, similar to that of MMSE-based hard IC receiver described in [5], is used for the ESE-based soft IC receiver. The main difference is that soft LLRs are provided by the LDPC decoders for interference cancellation.

The number of LDPC decodes per received slot equals the number UEs that transmit in a slot multiplied by the number of iterations (NUE∙Nitr). We don’t include the computational complexity of LPDC in Table 1, but use the number of usage when comparing the complexity of different receiver structures.

It should be noted that the LDPC decoder can be removed from the iteration loop to reduce the overall latency of the receiver and the computation complexity, albeit at the expense of performance. In this case, the number of LDPC decodes per received slot equals the number UEs that transmit in a slot (NUE).

The LDPC outputs soft symbols. The memory for storing the output of the decoder is: NUE∙NREdataNSF complex values.

## 3.3	Complexity of Interference Cancellation

The interference cancellation block is made up of the following detailed components: spreader and calculation of the extrinsic information, user specific interleaver, UE statistics update and receiver signal statistics update.

### 3.3.1	Spreader and calculation of the extrinsic information

This block performs spreading and generates the extrinsic information:

Spreader and calculation of the extrinsic information Complexity ~ NREdata

The number times this block executes per received slot equals the number of UEs that transmit in a slot multiplied by the number of iterations minus 1 (NUE∙(NIterIC-1)).

The spreader output is buffered to be used in the user specific interleaver, which can be performed on the fly, followed by the UE statistics update. A single user can be process at a time. The memory for storing the output is: NREdata∙Nrx complex values.

### 3.3.2	User specific interleaver

The order of complexity is given by:

Interleaver Complexity ~ NREdata

This block can be viewed as a memory read followed by a memory write to a different memory location, but can also be implemented transparently, by reading data out of order for the subsequent block.

The number of user specific interleavers per received slot equals the number of UEs that transmit in a slot multiplied by the number of iterations minus 1 (NUE∙NIterIC-1).

### 3.3.3	UE statistics update

This block updates the UE statistics using the spreader/decoder extrinsic LLRs. This is performed for each UE for each iteration. These are the equations for updating the UEs statistics using the extrinsic information after interleaving:

E(xkRe(j))=tanhΠExtr(xkRe(j))2

E(xkIm(j))=tanhΠExtr(xkIm(j))2

Var(xkRe(j))=1-E(xkRe(j))2

Var(xkIm(j))=1-E(xkIm(j))2

The tanh function can be implemented with a lookup table.

UE statistics update Complexity ~ NREdata

Number of complex multiplies: NREdata.

The number times this block executes per received slot equals the number of UEs that transmit in a slot multiplied by the number of iterations minus 1 (NUE∙NIterIC-1).

The UE statistics update output is buffered to be used in the receiver signal statics update. Two complex values are stored per user per chip for the mean and variance. Hence, the memory for storing the output is: 2∙NUE∙NREdata complex values.

### 3.3.4	Receiver signal statistics update

This block is executed for each antenna. These are the equation for updating the statistics of the received signal using the updated UE statistics

E[rRe(j)]=k=1NUEhkRe(j)E[xkRe(j)]-hkIm(j)E[xkIm(j)]

E[rIm(j)]=k=1NUEhkRe(j)E[xkIm(j)]+hkIm(j)E[xkRe(j)]

Var[rRe(j)]=k=1NUEhkRe2Var[xkRe(j)]+hkIm2Var[xkIm(j)]+σ2

Var[rIm(j)]=k=1NUEhkIm2Var[xkRe(j)]+hkRe2Var[xkIm(j)]+σ2

ψ(j)=k=1NUEhkRe(j)hkIm(j)(Var[xkRe(j)]-Var[xkIm(j)])

Receiver signal statistics update Compelxity ~ NREdata∙Nrx∙NUE

Number of operations:

A complex multiply is equivalent to 4 real multiplications. Complex additions are ignored in this analysis. Complex divisions are two real divisions (dividing a complex value by a real value).

Number of complex Multiplications:  2.5∙NREdata∙Nrx∙NUE

The number of times this block executes per received slot equals number of iterations minus 1 (NIterIC-1).

The receiver signal statistics update output is buffered to be used in the next iteration for the calculation the ESE LLRs. Two complex values and 1 real value are stored per chip per antenna for the mean, variance and ψ(j). Hence, the memory for storing the output is equivalent to: 2.5∙NREdata∙NRx compelx values.

## 3.4	Memory Aspects

While a detailed memory analysis of the ESE-based soft-IC receiver is beyond the scope of this document. We can identify key buffers that are needed for proper operation of the receiver. In addition to the buffers identified in the previous subsections for the output of channel estimation, ESE-LLR estimates, de-spreader output, spreader output, UE statistics update output, and receive-signal statistics update output, the following memory block is also needed:

- The received signal needs to be buffered to be used later for interference cancellation and subsequent decodes. The amount of data that is buffered depends on the number of resource elements per transmission and number of receive antennas (NREdata∙NRx), and the number of transmissions that need to be buffered NTx. Where NTx depends on the latency and pipelining within the architecture. For this analysis, we assume NTx=2, i.e. double buffering.

## 3.5	Receiver Complexity Table for ESE-based receiver for IDMA

Based on the discussions of the previous sub-sections Table 1 gives the complexity template for ESE-based soft IC receiver.

Table 1: Complexity Template of ESE-based soft IC receiver.

| Receiver Component | Detailed Component | Number of Usage | Computational Complexity | Memory Complexity (Complex Values) |
| --- | --- | --- | --- | --- |
| Detector | UE Detection | NUE,Dect | NUE,Dect∙NAPDMRS∙NREDMRS∙Nrx |  |
|  | Channel Estimation | NUE | NUE∙NRECE∙NREdataNBlkCE∙Nrx | NUE∙NREdataNBlkCE∙Nrx |
|  | ESE LLR | NUE∙NIterIC | Multiply: 5.5∙NIterIC∙NUE∙NREdata∙Nrx Division: NIterIC∙NUE∙NREdata∙Nrx | NUE∙NREdata |
|  | User specific de-interleaver | NUE∙NIterIC |  |  |
|  | De-spreader | NUE∙NIterIC | NIterIC∙NUE∙NREdata | NUE∙NREdataNSF |
| Decoder | LDPC Decoder | NUE∙NIterIC or NUE |  | NUE∙NREdataNSF |
| Interference Cancellation | Spreader and calculation of the extrinsic information | NUE∙NIterIC-1 | NIterIC∙NUE∙NREdata | NREdata |
|  | User specific interleaver | NUE∙NIterIC-1 |  |  |
|  | UE Statistics update | NUE∙NIterIC-1 | NIterIC∙NUE∙NREdata | 2∙NUE∙NREdata |
|  | Rx signal Statistics update | NIterIC-1 | 2.5NIterIC∙NUE∙Nrx∙NREdata | 2.5∙NREdata∙Nrx |
| Memory | Receive Buffer |  |  | NREdata∙Nrx∙NTx NTx is the number of buffering instances, e.g. 2 for double buffering. |

Observation 1: Table 1 gives the complexity template of the ESE-based soft IC receiver.

Numerical example:

Table 2: Computational complexity of ESE-based soft IC receiver.

| Block/ Parameter | Case 1 |  | Case 2 |  | Case 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Compute | Memory | Compute | Memory | Compute | Memory |
| NUE | 12 |  | 12 |  | 12 |  |
| NREDMRS | 144 |  | 144 |  | 144 |  |
| NREdata | 864 |  | 864 |  | 864 |  |
| NAPDMRS | 1 |  | 1 |  | 1 |  |
| Nrx | 2 |  | 4 |  | 2 |  |
| NSF | 4 |  | 4 |  | 4 |  |
| NRECE | 2 |  | 2 |  | 2 |  |
| NBlkCE | 24 |  | 24 |  | 24 |  |
| NIterIC | 5 |  | 5 |  | 5 |  |
| UE Detect | 3,456 |  | 6,912 |  | 3,456 |  |
| Ch Est | 1,728 | 864 | 3,456 | 1,728 | 10,368 | 5,184 |
| ESE LLR | 673,920 | 10,368 | 1,347,840 | 10,368 | 673,920 | 10,368 |
| De-spreader | 51,840 | 2,592 | 51,840 | 2,592 | 51,840 | 2,592 |
| LDPC | 60 Decodes |  | 60 Decodes |  | 60 Decodes |  |
| Spreading/Ext LLR | 41,472 | 864 | 41,472 | 864 | 41,472 | 864 |
| UE Stat Update | 41,472 | 20,736 | 41,472 | 20,736 | 41,472 | 20,736 |
| Rx Signal Stat Update | 207,360 | 4,320 | 207,360 | 4,320 | 207,360 | 4,302 |
| Receive Buffer |  | 3,456 |  | 3,456 |  | 3,456 |

# 4	Conclusion

In this document we presented a detailed analysis of the of the complexity of the ESE-based soft IC receiver. The following observation was made:

Observation 1: Table 1 gives the complexity template of the ESE-based soft IC receiver.

# References

- RAN1 Chairman’s Notes 3GPP TSG RAN WG1 Meeting#94, Gothenburg, Sweden, 3GPP.

- RAN1 Chairmain’s Notes 3GPP TSG RAN WG1 Meeting#94bis, Chengdu, China, 3GPP.

- Li Ping, Lihai Liu, Keying Wu and W. K. Leung, “Interleave-Division Multiple-Access,” in IEEE Transactions on Wireless Communications, Vol. 5, No. 4, pp. 938–947, Apr. 2006.

- R1-1813157, “Receiver considerations for UL NOMA”, Nokia, NSB, Spokane, USA, November 2018.

- R1-1813160, “Complexity Analysis of MMSE-based Hard IC Receiver”, Nokia, NSB, Spokane, USA, November 2018.

- R1-1811938, “Summary of NOMA Receiver Complexity Analysis”, ZTE, Chengdu, China, October 2018.