3GPP TSG RAN WG1 Meeting #94b                                                  R1-1813985

Spokane, USA, November 12th – 16th, 2018

Source:	Qualcomm

Title:	Summary of DMRS Issues

Agenda Item:	7.1.2.4

Document for:	Discussion and Decision

# Working Assumption on DMRS shift for LNC

In RAN1#94bis, the following WA was made:

Working Assumption For PDSCH mapping Type A, when lte-CRS-ToMatchAround is configured and dmrs-AdditionalPosition=’pos1’, and single-symbol DMRS, and if any DMRS symbol and any symbol containing common RS as indicated by lte-CRS-ToMatchAround is the same: - then the DM-RS positions for PDSCH duration (as measured from the beginning of the slot) of 13 and 14 are l0,12 instead of the default l0 , 11. The PDSCH processing time in Table 5.3-1 is increased from 13 to 14 in this case - l0=3 This feature is UE optional

| ATT, Qualcomm, Ericsson, Nokia | Support making the WA an agreement |
| --- | --- |

# Issues to be discussed in other AI

It has already been captured in the Summary on Codebook based UL transmission the issue on “PUSCH and SRS antenna port indexing”.