3GPP TSG-RAN WG1 Meeting #95	R1-1814182

# Spokane, USA, November 12th – 16th, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on power ratio between CSI-RS and PDCCH in 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current specs, the PDCCH EPRE is mistakenly defined as a ratio, taking the value of 0dB, which is incorrect, and the applicable scenario for 0dB power offset assumption between PDCCH and CSI-RS. i.e., for link recovery purpose, has not been captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove erroneous description on PDCCH EPRE. Add applicable scenario for 0dB power offset assumption between PDCCH and CSI-RS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDCCH EPRE is mistakendly fixed to 0 dB, or the 0dB power offset assumption between PDCCH and CSI-RS is unintendedly extended to use cases other than link recovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR impacts UE assumption on PDCCH EPRE and also UE assumption of power ratio of PDCCH to NZP CSI-RS for use cases other than link recovery. |  |  |  |  |  |  |  |  |

4.1	Power allocation for downlink

The gNodeB determines the downlink transmit EPRE.

< Unchanged parts are omitted >

For link recovery, the ratio of the PDCCH EPRE to NZP CSI-RS EPRE is assumed as 0 dB.

< Unchanged parts are omitted >