3GPP TSG RAN WG1 Meeting #96	R1-1901577

Athens, Greece, Feb 25 – March 1, 2019

Agenda Item:	7.2.10.1.4

Source:	Huawei, HiSilicon

Title:	Performance evaluation for NR positioning

Document for:	Discussion and decision

# Introduction

In this contribution, we will present our simulation results for OTDOA, UTDOA, DAOD, UAOA, E-CID, and multi-RTT. The results provided according to the template agreed in Taipei are provided in the Appendix A.

# Evaluated cases

The evaluation assumption and simulation results are included in the TPs provided in the Appendix A, for UMa, UMi, and InH, respectively.

The following cases in Table 1 are evaluated, for FR1 and FR2, respectively. Other parameters can be found in the Appendix A.

Table 1 Evaluated parameter combinations

|  |  | Fc/BW/SCS | {Sync, TOA resolution} |
| --- | --- | --- | --- |
| FR1 | OTDOA/UTDOA | {4GHz, 100MHz, 30kHz} {4GHz, 50MHz, 15kHz} {2GHz, 50MHz, 15kHz} {2GHz, 5MHz, 15kHz} | {perfect, regular resolution} {perfect, super resolution} {50ns Sync error, regular resolution} |
|  | UAOA |  | {N/A, N/A} |
|  | E-CID/multi-RTT |  | {N/A, regular resolution} {N/A, super resolution} |
| FR2 | OTDOA/UTDOA | {30GHz, 400MHz, 120kHz} {30GHz, 100MHz, 120kHz} | {perfect, high resolution} {50ns Sync error, regular resolution} |
|  | DAOD/UAOA |  | {N/A, N/A} |
|  | E-CID/multi-RTT |  | {N/A, super resolution} |

Table 2 shows the evaluated scenario for each technique.

Table 2 Evaluated scenarios

|  |  | InH | UMi | UMa |
| --- | --- | --- | --- | --- |
| FR1 | OTDOA |  |  |  |
|  | UTDOA |  |  |  |
|  | UAOA |  |  |  |
|  | E-CID |  |  |  |
|  | Multi-RTT |  |  |  |
| FR2 | OTDOA |  |  |  |
|  | DAOD |  |  |  |
|  | UTDOA |  |  |  |
|  | UAOA |  |  |  |
|  | E-CID |  |  |  |
|  | Multi-RTT |  |  |  |

# System level simulation results

In this section, we will provide the detailed simulation results of horizontal CDF errors under the agreed percentile points.

## Scenario 1 – Indoor Open Office

The text proposal capturing our results for scenario 1 is provided in appendix.

### Simulation results

Table 3 to Table 8 show the simulation results for indoor open office, including DL, UL, and DL+UL combined with FR1 and FR2. The parameters corresponding each source can be found in the Appendix.

Table 3 CDF error of InH for DL-only positioning in FR1

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | InH | HW1 InH FR1 | 1.1 | 1.5 | 2.4 | 5.7 |
|  |  | HW2 InH FR1 | 2.0 | 2.9 | 4.6 | 8.1 |
|  |  | HW3 InH FR1 | 2.0 | 2.9 | 4.6 | 7.9 |
|  |  | HW4 InH FR1 | 7.5 | 10.1 | 13.0 | 16.0 |
|  |  | HW5 InH FR1 | 0.37 | 0.40 | 0.45 | 0.70 |
|  |  | HW6 InH FR1 | 0.42 | 0.51 | 0.66 | 1.72 |
|  |  | HW7 InH FR1 | 0.43 | 0.52 | 0.70 | 1.72 |
|  |  | HW8 InH FR1 | 4.0 | 5.4 | 7.2 | 9.7 |
|  |  | HW9 InH FR1 | 11.2 | 15.1 | 19.8 | 24.9 |
|  |  | HW10 InH FR1 | 11.2 | 15.1 | 19.7 | 25.3 |
|  |  | HW11 InH FR1 | 11.1 | 14.4 | 18.9 | 23.9 |
|  |  | HW12 InH FR1 | 12.0 | 17.0 | 22.0 | 28.8 |

Table 4 CDF error of InH for DL-only positioning in FR2

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | InH | HW13 InH FR2 | 1.1 | 1.6 | 2.4 | 4.8 |
|  |  | HW14 InH FR2 | 0.2 | 0.4 | 0.7 | 1.4 |
|  |  | HW15 InH FR2 | 15.9 | 21.7 | 29.1 | 38.5 |
|  |  | HW16 InH FR2 | 15.8 | 21.7 | 29.1 | 38.1 |
|  |  | HW17 InH FR2 | 0.6 | 1.0 | 1.8 | 3.8 |
|  |  | HW18 InH FR2 | 0.6 | 1.0 | 1.6 | 3.6 |

Table 5 CDF error of InH for UL-only positioning in FR1

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | InH | HW1 InH FR1 | 1.2 | 2.0 | 3.3 | 5.5 |
|  |  | HW2 InH FR1 | 2.5 | 3.7 | 5.3 | 8.3 |
|  |  | HW3 InH FR1 | 2.5 | 3.7 | 5.5 | 7.8 |
|  |  | HW4 InH FR1 | 13.4 | 17.0 | 20.4 | 28.1 |
|  |  | HW5 InH FR1 | 0.08 | 0.1 | 0.2 | 0.5 |
|  |  | HW6 InH FR1 | 0.2 | 0.3 | 0.5 | 0.9 |
|  |  | HW7 InH FR1 | 0.2 | 0.3 | 0.5 | 1.2 |
|  |  | HW8 InH FR1 | 4.4 | 6.0 | 9.6 | 18.3 |
|  |  | HW9 InH FR1 | 15.6 | 21.3 | 29.0 | 39.9 |
|  |  | HW10 InH FR1 | 15.7 | 21.4 | 28.6 | 37.9 |
|  |  | HW11 InH FR1 | 15.8 | 21.4 | 27.8 | 37.8 |
|  |  | HW12 InH FR1 | 16.6 | 21.9 | 29.4 | 38.7 |
|  |  | HW13 InH FR1 | 0.2 | 0.4 | 1.6 | 3.5 |
|  |  | HW14 InH FR1 | 0.2 | 0.4 | 1.6 | 3.5 |
|  |  | HW15 InH FR1 | 0.2 | 0.5 | 1.6 | 3.6 |
|  |  | HW16 InH FR1 | 0.3 | 0.7 | 2.1 | 4.2 |

Table 6 CDF error of InH for UL-only positioning in FR2

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | InH | HW17 InH FR2 | 1.1 | 1.7 | 2.4 | 4.6 |
|  |  | HW18 InH FR2 | 0.2 | 0.4 | 0.6 | 1.3 |
|  |  | HW19 InH FR2 | 15.8 | 21.7 | 29.1 | 38.0 |
|  |  | HW20 InH FR2 | 15.9 | 21.7 | 28.6 | 38.3 |
|  |  | HW21 InH FR2 | 0.6 | 1.0 | 1.8 | 4.5 |
|  |  | HW22 InH FR2 | 0.6 | 1.0 | 1.6 | 3.8 |

Table 7 CDF error of InH for DL+UL positioning in FR1

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | InH | HW1 InH FR1 | 0.5 | 0.7 | 0.8 | 1.1 |
|  |  | HW2 InH FR1 | 0.8 | 1.2 | 1.7 | 2.1 |
|  |  | HW3 InH FR1 | 0.8 | 1.2 | 1.7 | 2.1 |
|  |  | HW4 InH FR1 | 8.8 | 10.7 | 12.3 | 14.6 |
|  |  | HW5 InH FR1 | 0.1 | 0.1 | 0.2 | 0.3 |
|  |  | HW6 InH FR1 | 0.1 | 0.2 | 0.3 | 0.5 |
|  |  | HW7 InH FR1 | 0.1 | 0.2 | 0.3 | 0.5 |
|  |  | HW8 InH FR1 | 2.2 | 3.3 | 4.4 | 5.8 |
|  |  | HW9 InH FR1 | 0.6 | 0.7 | 0.9 | 1.0 |
|  |  | HW10 InH FR1 | 1.1 | 1.4 | 1.8 | 2.2 |
|  |  | HW11 InH FR1 | 1.1 | 1.4 | 1.8 | 2.1 |
|  |  | HW12 InH FR1 | 5.2 | 7.4 | 9.5 | 11.0 |
|  |  | HW13 InH FR1 | 0.04 | 0.05 | 0.06 | 0.1 |
|  |  | HW14 InH FR1 | 0.08 | 0.1 | 0.2 | 0.3 |
|  |  | HW15 InH FR1 | 0.09 | 0.1 | 0.2 | 0.3 |
|  |  | HW16 InH FR1 | 2.6 | 4.0 | 5.6 | 7.2 |

Table 8 CDF error of InH for DL+UL positioning in FR2

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | InH | HW17 InH FR2 | 0.8 | 1.0 | 1.1 | 1.3 |
|  |  | HW18 InH FR2 | 0.3 | 0.4 | 0.6 | 1.0 |
|  |  | HW19 InH FR2 | 0.8 | 1.0 | 1.1 | 1.3 |
|  |  | HW20 InH FR2 | 0.2 | 0.4 | 0.7 | 1.4 |

### Summary of results for Scenario 1

For indoor open office scenario, all techniques can meet the regulatory requirements. Timing-based positioning techniques can meet the commercial requirement only without network synchronization error and with sufficient bandwidth, e.g., >= 50MHz. Angle-based positioning technique can meet the commercial requirement without synchronization or bandwidth requirement. RTT-based positioning can meet the commercial requirements without synchronization requirement, but with sufficient bandwidth. Super resolution can significantly boost the performance for both timing-based and RTT-based techniques.

## Scenario 2 – Urban micro

The text proposal capturing our results for scenario 2 is provided in appendix.

### Simulation results

Table 9 to Table 14 show the simulation results for urban micro, including DL, UL, and DL+UL combined with FR1 and FR2. The parameters corresponding each source can be found in the Appendix.

Table 9 CDF error of UMi for DL-only positioning in FR1

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMi | HW1 UMi FR1 | 1.2 | 1.6 | 3.5 | 9.1 |
|  |  | HW2 UMi FR1 | 2.0 | 2.8 | 5.5 | 11.0 |
|  |  | HW3 UMi FR1 | 2.0 | 2.8 | 4.5 | 11.0 |
|  |  | HW4 UMi FR1 | 15.8 | 21.7 | 29.8 | 40.3 |
|  |  | HW5 UMi FR1 | 0.6 | 1.1 | 3.0 | 9.9 |
|  |  | HW6 UMi FR1 | 1.1 | 2.1 | 4.1 | 9.9 |
|  |  | HW7 UMi FR1 | 1.1 | 2.1 | 5.0 | 9.8 |
|  |  | HW8 UMi FR1 | 12.9 | 18.5 | 25.8 | 38.5 |
|  |  | HW9 UMi FR1 | 16.6 | 22.4 | 30.4 | 47.6 |
|  |  | HW10 UMi FR1 | 17.2 | 23.5 | 31.1 | 47.6 |
|  |  | HW11 UMi FR1 | 16.6 | 21.8 | 28.8 | 42.6 |
|  |  | HW12 UMi FR1 | 23.1 | 30.5 | 41.2 | 57.8 |

Table 10 CDF error of UMi for DL-only positioning in FR2

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | UMi | HW13 UMi FR2 | 2.4 | 4.4 | 7.9 | 17.5 |
|  |  | HW14 UMi FR2 | 1.3 | 3.0 | 5.7 | 9.7 |
|  |  | HW15 UMi FR2 | 14.8 | 19.3 | 23.8 | 29.5 |
|  |  | HW16 UMi FR2 | 14.2 | 18.5 | 22.2 | 26.9 |
|  |  | HW17 UMi FR2 | 21.9 | 36.3 | 52.3 | 77.6 |
|  |  | HW18 UMi FR2 | 23.2 | 36.7 | 53.6 | 77.5 |

Table 11 CDF error of UMi for UL-only positioning in FR1

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMi | HW1 UMi FR1 | 1.9 | 1.5 | 2.3 | 4.6 |
|  |  | HW2 UMi FR1 | 2.2 | 2.9 | 4.4 | 7.7 |
|  |  | HW3 UMi FR1 | 2.1 | 2.9 | 4.6 | 8.2 |
|  |  | HW4 UMi FR1 | 17.3 | 23.7 | 33.3 | 45.5 |
|  |  | HW5 UMi FR1 | 0.3 | 0.5 | 1.2 | 3.4 |
|  |  | HW6 UMi FR1 | 0.8 | 1.4 | 2.4 | 5.4 |
|  |  | HW7 UMi FR1 | 0.8 | 1.5 | 2.6 | 6.7 |
|  |  | HW8 UMi FR1 | 12.6 | 17.9 | 24.8 | 35.8 |
|  |  | HW9 UMi FR1 | 17.3 | 23.4 | 30.4 | 40.2 |
|  |  | HW10 UMi FR1 | 17.6 | 23.0 | 30.4 | 41.5 |
|  |  | HW11 UMi FR1 | 17.8 | 23.3 | 30.0 | 40.8 |
|  |  | HW12 UMi FR1 | 24.8 | 34.5 | 43.8 | 55.6 |
|  |  | HW13 UMi FR1 | 15.7 | 26.6 | 41.8 | 63.8 |
|  |  | HW14 UMi FR1 | 15.3 | 24.8 | 36.6 | 60.5 |
|  |  | HW15 UMi FR1 | 16.9 | 26.6 | 41.7 | 61.6 |
|  |  | HW16 UMi FR1 | 19.6 | 31.3 | 47.5 | 65.3 |

Table 12 CDF error of UMi for UL-only positioning in FR2

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | UMi | HW17 UMi FR2 | 7.8 | 18.3 | 29.5 | 48.0 |
|  |  | HW18 UMi FR2 | 3.5 | 6.3 | 11.1 | 18.3 |
|  |  | HW19 UMi FR2 | 18.4 | 25.2 | 36.9 | 52.7 |
|  |  | HW20 UMi FR2 | 15.2 | 19.7 | 24.4 | 30.1 |
|  |  | HW21 UMi FR2 | 25.3 | 44.4 | 60.8 | 85.5 |
|  |  | HW22 UMi FR2 | 29.1 | 48.6 | 68.2 | 95.4 |

Table 13 CDF error of UMi for DL+UL positioning in FR1

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMi | HW1 UMi FR1 | 1.0 | 1.5 | 3.0 | 11.3 |
|  |  | HW2 UMi FR1 | 1.6 | 2.2 | 3.3 | 11.2 |
|  |  | HW3 UMi FR1 | 1.6 | 2.2 | 4.6 | 16.7 |
|  |  | HW4 UMi FR1 | 8.9 | 13.6 | 18.2 | 28.3 |
|  |  | HW5 UMi FR1 | 0.7 | 1.4 | 3.1 | 12.1 |
|  |  | HW6 UMi FR1 | 0.7 | 1.4 | 3.2 | 10.1 |
|  |  | HW7 UMi FR1 | 0.7 | 1.6 | 4.3 | 16.2 |
|  |  | HW8 UMi FR1 | 2.2 | 3.8 | 14.7 | 32.7 |
|  |  | HW9 UMi FR1 | 1.2 | 2.0 | 6.4 | 46.7 |
|  |  | HW10 UMi FR1 | 2.2 | 3.7 | 10.3 | 52.0 |
|  |  | HW11 UMi FR1 | 2.2 | 3.2 | 6.5 | 26.6 |
|  |  | HW12 UMi FR1 | 14.1 | 18.2 | 24.0 | 36.2 |
|  |  | HW13 UMi FR1 | 0.4 | 1.9 | 9.6 | 50.7 |
|  |  | HW14 UMi FR1 | 0.8 | 2.7 | 9.4 | 40.6 |
|  |  | HW15 UMi FR1 | 0.9 | 2.4 | 8.5 | 39.1 |
|  |  | HW16 UMi FR1 | 9.5 | 17.3 | 28.0 | 53.5 |

Table 14 CDF error of UMi for DL+UL positioning in FR2

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | UMi | HW17 UMi FR2 | 1.9 | 6.5 | 15.4 | 39.3 |
|  |  | HW18 UMi FR2 | 0.9 | 6.4 | 18.9 | 39.1 |
|  |  | HW19 UMi FR2 | 5.6 | 12.5 | 21.9 | 33.7 |
|  |  | HW20 UMi FR2 | 2.4 | 4.8 | 8.2 | 12.7 |

### Summary of results for Scenario 2

For urban micro scenario, timing-based and RTT-based techniques can meet the regulatory requirements. Timing-based positioning techniques can meet the commercial requirement only without network synchronization error and with sufficient bandwidth, e.g., >= 50MHz. RTT-based positioning can meet the commercial requirement without synchronization requirement, but with sufficient bandwidth. Super resolution can offer the limited performance gain for both timing-based and RTT-based techniques.

## Scenario 3 – Urban macro

The text proposal capturing our results for scenario 3 is provided in appendix.

### Simulation results

Table 15 to Table 17 show the simulation results for indoor open office, including DL, UL, and DL+UL, respectively only in FR1. The parameters corresponding each source can be found in the Appendix.

Table 15 CDF error of UMa for DL-only positioning in FR1

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMa indoor | HW1 UMa FR1 | 1.1 | 1.6 | 6.0 | 27.9 |
|  |  | HW2 UMa FR1 | 1.9 | 2.8 | 8.6 | 40.6 |
|  |  | HW3 UMa FR1 | 2.2 | 3.1 | 10.0 | 46.8 |
|  |  | HW4 UMa FR1 | 18.7 | 26.0 | 38.9 | 81.1 |
|  |  | HW5 UMa FR1 | 0.7 | 1.6 | 13.9 | 23 |
|  |  | HW6 UMa FR1 | 1.2 | 1.8 | 7.4 | 42 |
|  |  | HW7 UMa FR1 | 1.2 | 1.8 | 4.7 | 31 |
|  |  | HW8 UMa FR1 | 13 | 23 | 37 | 87 |
|  |  | HW9 UMa FR1 | 18 | 24 | 36 | 65 |
|  |  | HW10 UMa FR1 | 17 | 26 | 35 | 61 |
|  |  | HW11 UMa FR1 | 17 | 25 | 34 | 73 |
|  |  | HW12 UMa FR1 | 25 | 34 | 46 | 78 |
|  | UMa outdoor | HW1 UMa FR1 | 1.1 | 1.5 | 3.5 | 62.4 |
|  |  | HW2 UMa FR1 | 1.9 | 2.5 | 5.7 | 16.5 |
|  |  | HW3 UMa FR1 | 2.2 | 2.8 | 7.9 | 32.1 |
|  |  | HW4 UMa FR1 | 16.3 | 22.9 | 34.0 | 68.4 |
|  |  | HW5 UMa FR1 | 0.7 | 1.1 | 4.7 | 23 |
|  |  | HW6 UMa FR1 | 1.2 | 1.8 | 4.7 | 17 |
|  |  | HW7 UMa FR1 | 1.2 | 1.8 | 4.7 | 17 |
|  |  | HW8 UMa FR1 | 11 | 18 | 27 | 53 |
|  |  | HW9 UMa FR1 | 15 | 20 | 27 | 42 |
|  |  | HW10 UMa FR1 | 16 | 23 | 28 | 40 |
|  |  | HW11 UMa FR1 | 17 | 23 | 31 | 55 |
|  |  | HW12 UMa FR1 | 23 | 30 | 42 | 62 |

Table 16 CDF error of UMa for UL-only positioning in FR1

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMa indoor | HW1 UMa FR1 | 1.3 | 2.3 | 6.7 | 50.5 |
|  |  | HW2 UMa FR1 | 2.2 | 3.2 | 7.1 | 37.0 |
|  |  | HW3 UMa FR1 | 2.5 | 4.4 | 12.3 | 42.6 |
|  |  | HW4 UMa FR1 | 19.4 | 28.8 | 46.5 | 82.6 |
|  |  | HW5 UMa FR1 | 0.5 | 1.1 | 6.1 | 101.4 |
|  |  | HW6 UMa FR1 | 1.2 | 2.8 | 16.6 | 50.1 |
|  |  | HW7 UMa FR1 | 1.1 | 2.5 | 10.0 | 38.9 |
|  |  | HW8 UMa FR1 | 13.0 | 20.3 | 39.5 | 75.9 |
|  |  | HW9 UMa FR1 | 19.0 | 29.4 | 49.8 | 96.5 |
|  |  | HW10 UMa FR1 | 20.0 | 27.8 | 41.6 | 67.7 |
|  |  | HW11 UMa FR1 | 19.5 | 26.8 | 40.0 | 70.8 |
|  |  | HW12 UMa FR1 | 28.4 | 39.3 | 53.3 | 82.6 |
|  |  | HW13 UMa FR1 | 61.8 | 117.5 | 168.6 | 254.7 |
|  |  | HW14 UMa FR1 | 65.3 | 96.4 | 144.6 | 224.1 |
|  |  | HW15 UMa FR1 | 64.6 | 111.1 | 165.8 | 274.1 |
|  |  | HW16 UMa FR1 | 82.3 | 129.1 | 199.4 | 288.3 |
|  | UMa outdoor | HW1 UMa FR1 | 1.1 | 1.6 | 3.5 | 15.6 |
|  |  | HW2 UMa FR1 | 2.1 | 3.1 | 4.9 | 16.2 |
|  |  | HW3 UMa FR1 | 2.2 | 3.2 | 6.2 | 29.3 |
|  |  | HW4 UMa FR1 | 20.1 | 26.8 | 43.0 | 67.0 |
|  |  | HW5 UMa FR1 | 0.6 | 1.3 | 5.7 | 23.0 |
|  |  | HW6 UMa FR1 | 1.1 | 1.8 | 5.6 | 20.7 |
|  |  | HW7 UMa FR1 | 1.2 | 2.3 | 10.7 | 43.7 |
|  |  | HW8 UMa FR1 | 13.4 | 21.7 | 40.9 | 89.1 |
|  |  | HW9 UMa FR1 | 17.4 | 23.1 | 31.6 | 44.2 |
|  |  | HW10 UMa FR1 | 18.4 | 23.9 | 31.6 | 49.1 |
|  |  | HW11 UMa FR1 | 18.9 | 25.1 | 34.2 | 58.1 |
|  |  | HW12 UMa FR1 | 26.9 | 36.3 | 53.4 | 82.5 |
|  |  | HW13 UMa FR1 | 53.8 | 92.3 | 147.2 | 210.4 |
|  |  | HW14 UMa FR1 | 56.8 | 91.2 | 133.9 | 211.7 |
|  |  | HW15 UMa FR1 | 49.0 | 76.8 | 123.3 | 164.6 |
|  |  | HW16 UMa FR1 | 58.8 | 94.5 | 144.3 | 198.1 |

Table 17 CDF error of UMa for DL+UL positioning in FR1

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMa indoor | HW1 UMa FR1 | 3.7 | 10.8 | 37.5 | 71.3 |
|  |  | HW2 UMa FR1 | 4.2 | 12.6 | 37.2 | 114.4 |
|  |  | HW3 UMa FR1 | 6.0 | 16.0 | 33.7 | 112.6 |
|  |  | HW4 UMa FR1 | 12.9 | 21.1 | 43.3 | 124.9 |
|  |  | HW5 UMa FR1 | 3.7 | 12.4 | 36.6 | 73.4 |
|  |  | HW6 UMa FR1 | 4.6 | 12.4 | 38.5 | 114.4 |
|  |  | HW7 UMa FR1 | 6.1 | 18.0 | 40.5 | 105.4 |
|  |  | HW8 UMa FR1 | 8.0 | 20.7 | 52.8 | 128.7 |
|  | UMa outdoor | HW1 UMa FR1 | 5.7 | 12.2 | 29.8 | 70.3 |
|  |  | HW2 UMa FR1 | 6.5 | 15.2 | 30.8 | 73.2 |
|  |  | HW3 UMa FR1 | 6.3 | 22.3 | 53.9 | 100.8 |
|  |  | HW4 UMa FR1 | 15.6 | 31.3 | 54.7 | 105.9 |
|  |  | HW5 UMa FR1 | 5.7 | 12.4 | 36.6 | 73.4 |
|  |  | HW6 UMa FR1 | 6.4 | 15.2 | 33.0 | 75.6 |
|  |  | HW7 UMa FR1 | 6.5 | 23.0 | 57.3 | 106.2 |
|  |  | HW8 UMa FR1 | 11.9 | 32.8 | 58.0 | 115.6 |

### Summary of results for Scenario 3

For urban macro scenario, OTDOA can meet the regulatory requirement for both indoor and outdoor UEs. UTDOA cases can meet the regulatory requirement for both indoor and outdoor UEs except for 5MHz with synchronization error (HW12 UMa FR1). E-CID cases can meet the regulatory requirements for both indoor and outdoor UEs for half of the cases.

# Conclusion

In this contribution, we provide our simulation results for the techniques agreed in the SI. Based on the simulation, we have the following proposal.

Proposal: Capture in TR 38.855 the TPs in Appendix A.

# References

- 3GPP RAN1, Chairman's Notes RAN1 AdHoc 1901 final, RAN1 AH 1901, Taipei, Jan. 21 – 25, 2019.

# Appendix A: Evaluation results according to agreed template

=========================== Unchanged part omitted ============================

### 8.1.1 System simulations for Indoor office scenario

| Parameter | HW1 InH, FR1 | HW2 InH, FR1 | HW3 InH, FR1 | HW4 InH, FR1 | HW5 InH, FR1 | HW6 InH, FR1 | HW7 InH, FR1 | HW8 InH, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 |
| Reference signal (type of sequence, number of ports, …) | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port |
| Number of sites | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of symbols used per occasion | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm |
| Network synchronization assumptions | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW9 InH, FR1 | HW10 InH, FR1 | HW11 InH, FR1 | HW12 InH, FR1 |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6 | Comb-6 | Comb-6 | Comb-6 |
| Reference signal (type of sequence, number of ports, …) | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port |
| Number of sites | 12 | 12 | 12 | 12 |
| Number of symbols used per occasion | 8 | 8 | 8 | 8 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 |
| Power-boosting level | 7.8dB | 7.8dB | 7.8dB | 7.8dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm |
| Network synchronization assumptions | Sync error 50ns | Sync error 50ns | Sync error 50ns | Sync error 50ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |

| Parameter | HW13 InH, FR2 | HW14 InH FR2 | HW15 InH, FR2 | HW16 InH FR2 | HW17 InH FR2 | HW18 InH FR2 |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 30GHz | 30GHz | 30GHz | 30GHz | 30GHz | 30GHz |
| Subcarrier spacing | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 400MHz | 100MHz | 400MHz | 100MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 |
| Reference signal (type of sequence, number of ports, …) | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port |
| Number of sites | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of symbols used per occasion | 8 | 8 | 8 | 8 | 8 | 8 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | OTDOA, Tylor expansion | OTDOA, Tylor expansion | OTDOA, Tylor expansion | OTDOA, Tylor expansion | ML estimation based on PDP | ML estimation based on PDP |
| Network synchronization assumptions | Sync error 0ns | Sync error 0ns | Sync error 50ns | Sync error 50ns | Sync error 0ns | Sync error 0ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP | 8 TX-beam sweeping, RX beam in the largest RSRP BPL | 8 TX-beam sweeping, RX beam in the largest RSRP BPL |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | DFT codebook | DFT codebook | DFT codebook | DFT codebook | DFT codebook | DFT codebook |
| Additional notes, if any |  |  |  |  |  |  |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | InH | HW1 InH FR1 | 1.1 | 1.5 | 2.4 | 5.7 |
|  |  | HW2 InH FR1 | 2.0 | 2.9 | 4.6 | 8.1 |
|  |  | HW3 InH FR1 | 2.0 | 2.9 | 4.6 | 7.9 |
|  |  | HW4 InH FR1 | 7.5 | 10.1 | 13.0 | 16.0 |
|  |  | HW5 InH FR1 | 0.37 | 0.40 | 0.45 | 0.70 |
|  |  | HW6 InH FR1 | 0.42 | 0.51 | 0.66 | 1.72 |
|  |  | HW7 InH FR1 | 0.43 | 0.52 | 0.70 | 1.72 |
|  |  | HW8 InH FR1 | 4.0 | 5.4 | 7.2 | 9.7 |
|  |  | HW9 InH FR1 | 11.2 | 15.1 | 19.8 | 24.9 |
|  |  | HW10 InH FR1 | 11.2 | 15.1 | 19.7 | 25.3 |
|  |  | HW11 InH FR1 | 11.1 | 14.4 | 18.9 | 23.9 |
|  |  | HW12 InH FR1 | 12.0 | 17.0 | 22.0 | 28.8 |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | InH | HW13 InH FR2 | 1.1 | 1.6 | 2.4 | 4.8 |
|  |  | HW14 InH FR2 | 0.2 | 0.4 | 0.7 | 1.4 |
|  |  | HW15 InH FR2 | 15.9 | 21.7 | 29.1 | 38.5 |
|  |  | HW16 InH FR2 | 15.8 | 21.7 | 29.1 | 38.1 |
|  |  | HW17 InH FR2 | 0.6 | 1.0 | 1.8 | 3.8 |
|  |  | HW18 InH FR2 | 0.6 | 1.0 | 1.6 | 3.6 |

### 8.1.2 System simulations for UMi scenario

| Parameter | HW1 UMi, FR1 | HW2 UMi, FR1 | HW3 UMi, FR1 | HW4 UMi, FR1 | HW5 UMi, FR1 | HW6 UMi, FR1 | HW7 UMi, FR1 | HW8 UMi, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 |
| Reference signal (type of sequence, number of ports, …) | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port |
| Number of sites | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| Number of symbols used per occasion | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm |
| Network synchronization assumptions | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW9 UMi, FR1 | HW10 UMi FR1 | HW11 UMi, FR1 | HW12 UMi, FR1 |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6 | Comb-6 | Comb-6 | Comb-6 |
| Reference signal (type of sequence, number of ports, …) | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port |
| Number of sites | 19 | 19 | 19 | 19 |
| Number of symbols used per occasion | 8 | 8 | 8 | 8 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 |
| Power-boosting level | 7.8dB | 7.8dB | 7.8dB | 7.8dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm |
| Network synchronization assumptions | Sync error 50ns | Sync error 50ns | Sync error 50ns | Sync error 50ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |

| Parameter | HW13 UMi, FR2 | HW14 UMi FR2 | HW15 UMi, FR2 | HW16 UMi, FR2 | HW17 UMi FR2 | HW18 UMi FR2 |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 30GHz | 30GHz | 30GHz | 30GHz | 30GHz | 30GHz |
| Subcarrier spacing | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 400MHz | 100MHz | 400MHz | 100MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 |
| Reference signal (type of sequence, number of ports, …) | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 8 | 8 | 8 | 8 | 8 | 8 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | OTDOA, Tylor expansion | OTDOA, Tylor expansion | OTDOA, Tylor expansion | OTDOA, Tylor expansion | ML estimation based on PDP | ML estimation based on PDP |
| Network synchronization assumptions | Sync error 0ns | Sync error 0ns | Sync error 50ns | Sync error 50ns | Sync error 0ns | Sync error 0ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP | 8 TX-beam sweeping, RX beam in the largest RSRP BPL | 8 TX-beam sweeping, RX beam in the largest RSRP BPL |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | DFT codebook | DFT codebook | DFT codebook | DFT codebook | DFT codebook | DFT codebook |
| Additional notes, if any |  |  |  |  |  |  |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMi | HW1 UMi FR1 | 1.2 | 1.6 | 3.5 | 9.1 |
|  |  | HW2 UMi FR1 | 2.0 | 2.8 | 5.5 | 11.0 |
|  |  | HW3 UMi FR1 | 2.0 | 2.8 | 4.5 | 11.0 |
|  |  | HW4 UMi FR1 | 15.8 | 21.7 | 29.8 | 40.3 |
|  |  | HW5 UMi FR1 | 0.6 | 1.1 | 3.0 | 9.9 |
|  |  | HW6 UMi FR1 | 1.1 | 2.1 | 4.1 | 9.9 |
|  |  | HW7 UMi FR1 | 1.1 | 2.1 | 5.0 | 9.8 |
|  |  | HW8 UMi FR1 | 12.9 | 18.5 | 25.8 | 38.5 |
|  |  | HW9 UMi FR1 | 16.6 | 22.4 | 30.4 | 47.6 |
|  |  | HW10 UMi FR1 | 17.2 | 23.5 | 31.1 | 47.6 |
|  |  | HW11 UMi FR1 | 16.6 | 21.8 | 28.8 | 42.6 |
|  |  | HW12 UMi FR1 | 23.1 | 30.5 | 41.2 | 57.8 |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | UMi | HW13 UMi FR2 | 2.4 | 4.4 | 7.9 | 17.5 |
|  |  | HW14 UMi FR2 | 1.3 | 3.0 | 5.7 | 9.7 |
|  |  | HW15 UMi FR2 | 14.8 | 19.3 | 23.8 | 29.5 |
|  |  | HW16 UMi FR2 | 14.2 | 18.5 | 22.2 | 26.9 |
|  |  | HW17 UMi FR2 | 21.9 | 36.3 | 52.3 | 77.6 |
|  |  | HW18 UMi FR2 | 23.2 | 36.7 | 53.6 | 77.5 |

### 8.1.3 System simulations for UMa scenario

| Parameter | HW1 UMa, FR1 | HW2 UMa, FR1 | HW3 UMa, FR1 | HW4 UMa, FR1 | HW5 UMa, FR1 | HW6 UMa, FR1 | HW7 UMa, FR1 | HW8 UMa, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 | Comb-6 |
| Reference signal (type of sequence, number of ports, …) | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port |
| Number of sites | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| Number of symbols used per occasion | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB | 7.8dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm |
| Network synchronization assumptions | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW9 UMa, FR1 | HW10 UMa, FR1 | HW11 UMa, FR1 | HW12 UMa, FR1 |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6 | Comb-6 | Comb-6 | Comb-6 |
| Reference signal (type of sequence, number of ports, …) | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port | PRS: Gold, 1-port |
| Number of sites | 19 | 19 | 19 | 19 |
| Number of symbols used per occasion | 8 | 8 | 8 | 8 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 |
| Power-boosting level | 7.8dB | 7.8dB | 7.8dB | 7.8dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm | OTDOA, Guass-Newton algorithm |
| Network synchronization assumptions | Sync error 50ns | Sync error 50ns | Sync error 50ns | Sync error 50ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMa indoor | HW1 UMa FR1 | 1.1 | 1.6 | 6.0 | 27.9 |
|  |  | HW2 UMa FR1 | 1.9 | 2.8 | 8.6 | 40.6 |
|  |  | HW3 UMa FR1 | 2.2 | 3.1 | 10.0 | 46.8 |
|  |  | HW4 UMa FR1 | 18.7 | 26.0 | 38.9 | 81.1 |
|  |  | HW5 UMa FR1 | 0.7 | 1.6 | 13.9 | 23 |
|  |  | HW6 UMa FR1 | 1.2 | 1.8 | 7.4 | 42 |
|  |  | HW7 UMa FR1 | 1.2 | 1.8 | 4.7 | 31 |
|  |  | HW8 UMa FR1 | 13 | 23 | 37 | 87 |
|  |  | HW9 UMa FR1 | 18 | 24 | 36 | 65 |
|  |  | HW10 UMa FR1 | 17 | 26 | 35 | 61 |
|  |  | HW11 UMa FR1 | 17 | 25 | 34 | 73 |
|  |  | HW12 UMa FR1 | 25 | 34 | 46 | 78 |
|  | UMa outdoor | HW1 UMa FR1 | 1.1 | 1.5 | 3.5 | 62.4 |
|  |  | HW2 UMa FR1 | 1.9 | 2.5 | 5.7 | 16.5 |
|  |  | HW3 UMa FR1 | 2.2 | 2.8 | 7.9 | 32.1 |
|  |  | HW4 UMa FR1 | 16.3 | 22.9 | 34.0 | 68.4 |
|  |  | HW5 UMa FR1 | 0.7 | 1.1 | 4.7 | 23 |
|  |  | HW6 UMa FR1 | 1.2 | 1.8 | 4.7 | 17 |
|  |  | HW7 UMa FR1 | 1.2 | 1.8 | 4.7 | 17 |
|  |  | HW8 UMa FR1 | 11 | 18 | 27 | 53 |
|  |  | HW9 UMa FR1 | 15 | 20 | 27 | 42 |
|  |  | HW10 UMa FR1 | 16 | 23 | 28 | 40 |
|  |  | HW11 UMa FR1 | 17 | 23 | 31 | 55 |
|  |  | HW12 UMa FR1 | 23 | 30 | 42 | 62 |

### 8.2.1 System simulations for Indoor office scenario

| Parameter | HW1 InH, FR1 | HW2 InH, FR1 | HW3 InH, FR1 | HW4 InH, FR1 | HW5 InH, FR1 | HW6 InH, FR1 | HW7 InH, FR1 | HW8 InH, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 |
| Reference signal (type of sequence, number of ports, …) | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port |
| Number of sites | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Super resolution | Super resolution | Super resolution | Super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm |
| Network synchronization assumptions | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW9 InH, FR1 | HW10 InH, FR1 | HW11 InH, FR1 | HW12 InH, FR1 | HW13 InH, FR1 | HW14 InH, FR1 | HW15 InH, FR1 | HW16 InH, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 |
| Reference signal (type of sequence, number of ports, …) | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port |
| Number of sites | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UAOA, PSO algorithm | UAOA, PSO algorithm | UAOA, PSO algorithm | UAOA, PSO algorithm |
| Network synchronization assumptions | Sync error 50ns | Sync error 50ns | Sync error 50ns | Sync error 50ns | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW17 InH, FR2 | HW18 InH FR2 | HW19 InH, FR2 | HW20 InH FR2 | HW21 InH FR2 | HW22 InH FR2 |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 30GHz | 30GHz | 30GHz | 30GHz | 30GHz | 30GHz |
| Subcarrier spacing | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 400MHz | 100MHz | 400MHz | 100MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 |
| Reference signal (type of sequence, number of ports, …) | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port |
| Number of sites | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UTDOA, Tylor expansion | UTDOA, Tylor expansion | UTDOA, Tylor expansion | UTDOA, Tylor expansion | ML estimation based on PDP | ML estimation based on PDP |
| Network synchronization assumptions | Sync error 0ns | Sync error 0ns | Sync error 50ns | Sync error 50ns | Sync error 0ns | Sync error 0ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP | 8 TX-beam sweeping, RX beam in the largest RSRP BPL | 8 TX-beam sweeping, RX beam in the largest RSRP BPL |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | DFT codebook | DFT codebook | DFT codebook | DFT codebook | DFT codebook | DFT codebook |
| Additional notes, if any |  |  |  |  |  |  |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | InH | HW1 InH FR1 | 1.2 | 2.0 | 3.3 | 5.5 |
|  |  | HW2 InH FR1 | 2.5 | 3.7 | 5.3 | 8.3 |
|  |  | HW3 InH FR1 | 2.5 | 3.7 | 5.5 | 7.8 |
|  |  | HW4 InH FR1 | 13.4 | 17.0 | 20.4 | 28.1 |
|  |  | HW5 InH FR1 | 0.08 | 0.1 | 0.2 | 0.5 |
|  |  | HW6 InH FR1 | 0.2 | 0.3 | 0.5 | 0.9 |
|  |  | HW7 InH FR1 | 0.2 | 0.3 | 0.5 | 1.2 |
|  |  | HW8 InH FR1 | 4.4 | 6.0 | 9.6 | 18.3 |
|  |  | HW9 InH FR1 | 15.6 | 21.3 | 29.0 | 39.9 |
|  |  | HW10 InH FR1 | 15.7 | 21.4 | 28.6 | 37.9 |
|  |  | HW11 InH FR1 | 15.8 | 21.4 | 27.8 | 37.8 |
|  |  | HW12 InH FR1 | 16.6 | 21.9 | 29.4 | 38.7 |
|  |  | HW13 InH FR1 | 0.2 | 0.4 | 1.6 | 3.5 |
|  |  | HW14 InH FR1 | 0.2 | 0.4 | 1.6 | 3.5 |
|  |  | HW15 InH FR1 | 0.2 | 0.5 | 1.6 | 3.6 |
|  |  | HW16 InH FR1 | 0.3 | 0.7 | 2.1 | 4.2 |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | InH | HW17 InH FR2 | 1.1 | 1.7 | 2.4 | 4.6 |
|  |  | HW18 InH FR2 | 0.2 | 0.4 | 0.6 | 1.3 |
|  |  | HW19 InH FR2 | 15.8 | 21.7 | 29.1 | 38.0 |
|  |  | HW20 InH FR2 | 15.9 | 21.7 | 28.6 | 38.3 |
|  |  | HW21 InH FR2 | 0.6 | 1.0 | 1.8 | 4.5 |
|  |  | HW22 InH FR2 | 0.6 | 1.0 | 1.6 | 3.8 |

### 8.2.2 System simulations for UMi scenario

| Parameter | HW1 UMi, FR1 | HW2 UMi, FR1 | HW3 UMi, FR1 | HW4 UMi, FR1 | HW5 UMi, FR1 | HW6 UMi, FR1 | HW7 UMi, FR1 | HW8 UMi, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 |
| Reference signal (type of sequence, number of ports, …) | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port |
| Number of sites | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Super resolution | Super resolution | Super resolution | Super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm |
| Network synchronization assumptions | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW9 UMi, FR1 | HW10 UMi, FR1 | HW11 UMi, FR1 | HW12 UMi, FR1 | HW13 UMi, FR1 | HW14 UMi, FR1 | HW15 UMi, FR1 | HW16 UMi, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 |
| Reference signal (type of sequence, number of ports, …) | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port |
| Number of sites | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UAOA, PSO algorithm | UAOA, PSO algorithm | UAOA, PSO algorithm | UAOA, PSO algorithm |
| Network synchronization assumptions | Sync error 50ns | Sync error 50ns | Sync error 50ns | Sync error 50ns | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW17 UMi, FR2 | HW18 UMi FR2 | HW19 UMi, FR2 | HW20 UMi FR2 | HW21 UMi FR2 | HW22 UMi FR2 |
| --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 30GHz | 30GHz | 30GHz | 30GHz | 30GHz | 30GHz |
| Subcarrier spacing | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 400MHz | 100MHz | 400MHz | 100MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 |
| Reference signal (type of sequence, number of ports, …) | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port |
| Number of sites | 7 | 7 | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UTDOA, Tylor expansion | UTDOA, Tylor expansion | UTDOA, Tylor expansion | UTDOA, Tylor expansion | ML estimation based on PDP | ML estimation based on PDP |
| Network synchronization assumptions | Sync error 0ns | Sync error 0ns | Sync error 50ns | Sync error 50ns | Sync error 0ns | Sync error 0ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP | 8 TX-beam sweeping, RX beam in the largest RSRP BPL | 8 TX-beam sweeping, RX beam in the largest RSRP BPL |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | DFT codebook | DFT codebook | DFT codebook | DFT codebook | DFT codebook | DFT codebook |
| Additional notes, if any |  |  |  |  |  |  |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMi | HW1 UMi FR1 | 1.9 | 1.5 | 2.3 | 4.6 |
|  |  | HW2 UMi FR1 | 2.2 | 2.9 | 4.4 | 7.7 |
|  |  | HW3 UMi FR1 | 2.1 | 2.9 | 4.6 | 8.2 |
|  |  | HW4 UMi FR1 | 17.3 | 23.7 | 33.3 | 45.5 |
|  |  | HW5 UMi FR1 | 0.3 | 0.5 | 1.2 | 3.4 |
|  |  | HW6 UMi FR1 | 0.8 | 1.4 | 2.4 | 5.4 |
|  |  | HW7 UMi FR1 | 0.8 | 1.5 | 2.6 | 6.7 |
|  |  | HW8 UMi FR1 | 12.6 | 17.9 | 24.8 | 35.8 |
|  |  | HW9 UMi FR1 | 17.3 | 23.4 | 30.4 | 40.2 |
|  |  | HW10 UMi FR1 | 17.6 | 23.0 | 30.4 | 41.5 |
|  |  | HW11 UMi FR1 | 17.8 | 23.3 | 30.0 | 40.8 |
|  |  | HW12 UMi FR1 | 24.8 | 34.5 | 43.8 | 55.6 |
|  |  | HW13 UMi FR1 | 15.7 | 26.6 | 41.8 | 63.8 |
|  |  | HW14 UMi FR1 | 15.3 | 24.8 | 36.6 | 60.5 |
|  |  | HW15 UMi FR1 | 16.9 | 26.6 | 41.7 | 61.6 |
|  |  | HW16 UMi FR1 | 19.6 | 31.3 | 47.5 | 65.3 |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | UMi | HW17 UMi FR2 | 7.8 | 18.3 | 29.5 | 48.0 |
|  |  | HW18 UMi FR2 | 3.5 | 6.3 | 11.1 | 18.3 |
|  |  | HW19 UMi FR2 | 18.4 | 25.2 | 36.9 | 52.7 |
|  |  | HW20 UMi FR2 | 15.2 | 19.7 | 24.4 | 30.1 |
|  |  | HW21 UMi FR2 | 25.3 | 44.4 | 60.8 | 85.5 |
|  |  | HW22 UMi FR2 | 29.1 | 48.6 | 68.2 | 95.4 |

### 8.2.3 System simulations for UMa scenario

| Parameter | HW1 UMa, FR1 | HW2 UMa, FR1 | HW3 UMa, FR1 | HW4 UMa, FR1 | HW5 UMa, FR1 | HW6 UMa, FR1 | HW7 UMa, FR1 | HW8 UMa, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 |
| Reference signal (type of sequence, number of ports, …) | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port |
| Number of sites | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Super resolution | Super resolution | Super resolution | Super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm |
| Network synchronization assumptions | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW9 UMa, FR1 | HW10 UMa, FR1 | HW11 UMa, FR1 | HW12 UMa, FR1 | HW13 UMa, FR1 | HW14 UMa, FR1 | HW15 UMa, FR1 | HW16 UMa, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 | Comb-4 |
| Reference signal (type of sequence, number of ports, …) | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port | SRS: ZC, 1-port |
| Number of sites | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| Number of symbols used per occasion | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB | 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UTDOA, Chan’s algorithm | UAOA, PSO algorithm | UAOA, PSO algorithm | UAOA, PSO algorithm | UAOA, PSO algorithm |
| Network synchronization assumptions | Sync error 50ns | Sync error 50ns | Sync error 50ns | Sync error 50ns | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMa indoor | HW1 UMa FR1 | 1.3 | 2.3 | 6.7 | 50.5 |
|  |  | HW2 UMa FR1 | 2.2 | 3.2 | 7.1 | 37.0 |
|  |  | HW3 UMa FR1 | 2.5 | 4.4 | 12.3 | 42.6 |
|  |  | HW4 UMa FR1 | 19.4 | 28.8 | 46.5 | 82.6 |
|  |  | HW5 UMa FR1 | 0.5 | 1.1 | 6.1 | 101.4 |
|  |  | HW6 UMa FR1 | 1.2 | 2.8 | 16.6 | 50.1 |
|  |  | HW7 UMa FR1 | 1.1 | 2.5 | 10.0 | 38.9 |
|  |  | HW8 UMa FR1 | 13.0 | 20.3 | 39.5 | 75.9 |
|  |  | HW9 UMa FR1 | 19.0 | 29.4 | 49.8 | 96.5 |
|  |  | HW10 UMa FR1 | 20.0 | 27.8 | 41.6 | 67.7 |
|  |  | HW11 UMa FR1 | 19.5 | 26.8 | 40.0 | 70.8 |
|  |  | HW12 UMa FR1 | 28.4 | 39.3 | 53.3 | 82.6 |
|  |  | HW13 UMa FR1 | 61.8 | 117.5 | 168.6 | 254.7 |
|  |  | HW14 UMa FR1 | 65.3 | 96.4 | 144.6 | 224.1 |
|  |  | HW15 UMa FR1 | 64.6 | 111.1 | 165.8 | 274.1 |
|  |  | HW16 UMa FR1 | 82.3 | 129.1 | 199.4 | 288.3 |
|  | UMa outdoor | HW1 UMa FR1 | 1.1 | 1.6 | 3.5 | 15.6 |
|  |  | HW2 UMa FR1 | 2.1 | 3.1 | 4.9 | 16.2 |
|  |  | HW3 UMa FR1 | 2.2 | 3.2 | 6.2 | 29.3 |
|  |  | HW4 UMa FR1 | 20.1 | 26.8 | 43.0 | 67.0 |
|  |  | HW5 UMa FR1 | 0.6 | 1.3 | 5.7 | 23.0 |
|  |  | HW6 UMa FR1 | 1.1 | 1.8 | 5.6 | 20.7 |
|  |  | HW7 UMa FR1 | 1.2 | 2.3 | 10.7 | 43.7 |
|  |  | HW8 UMa FR1 | 13.4 | 21.7 | 40.9 | 89.1 |
|  |  | HW9 UMa FR1 | 17.4 | 23.1 | 31.6 | 44.2 |
|  |  | HW10 UMa FR1 | 18.4 | 23.9 | 31.6 | 49.1 |
|  |  | HW11 UMa FR1 | 18.9 | 25.1 | 34.2 | 58.1 |
|  |  | HW12 UMa FR1 | 26.9 | 36.3 | 53.4 | 82.5 |
|  |  | HW13 UMa FR1 | 53.8 | 92.3 | 147.2 | 210.4 |
|  |  | HW14 UMa FR1 | 56.8 | 91.2 | 133.9 | 211.7 |
|  |  | HW15 UMa FR1 | 49.0 | 76.8 | 123.3 | 164.6 |
|  |  | HW16 UMa FR1 | 58.8 | 94.5 | 144.3 | 198.1 |

### 8.3.1 System simulations for Indoor office scenario

| Parameter | HW1 InH, FR1 | HW2 InH, FR1 | HW3 InH, FR1 | HW4 InH, FR1 | HW5 InH, FR1 | HW6 InH, FR1 | HW7 InH, FR1 | HW8 InH, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 |
| Reference signal (type of sequence, number of ports, …) | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port |
| Number of sites | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of symbols used per occasion | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | E-CID | E-CID | E-CID | E-CID | E-CID | E-CID | E-CID | E-CID |
| Network synchronization assumptions | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW9 InH, FR1 | HW10 InH, FR1 | HW11 InH, FR1 | HW12 InH, FR1 | HW13 InH, FR1 | HW14 InH, FR1 | HW15 InH, FR1 | HW16 InH, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 |
| Reference signal (type of sequence, number of ports, …) | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port |
| Number of sites | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of symbols used per occasion | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm |
| Network synchronization assumptions | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW17 InH, FR2 | HW18 InH FR2 | HW19 InH, FR2 | HW20 InH FR2 |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 30GHz | 30GHz | 30GHz | 30GHz |
| Subcarrier spacing | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 400MHz | 100MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 |
| Reference signal (type of sequence, number of ports, …) | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port |
| Number of sites | 12 | 12 | 12 | 12 |
| Number of symbols used per occasion | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 |
| Power-boosting level | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | E-CID | E-CID | Multi-RTT, Tylor expansion | Multi-RTT, Tylor expansion |
| Network synchronization assumptions | Sync error 0ns | Sync error 0ns | Sync error 0ns | Sync error 0ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | DFT codebook | DFT codebook | DFT codebook | DFT codebook |
| Additional notes, if any |  |  |  |  |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | InH | HW1 InH FR1 | 0.5 | 0.7 | 0.8 | 1.1 |
|  |  | HW2 InH FR1 | 0.8 | 1.2 | 1.7 | 2.1 |
|  |  | HW3 InH FR1 | 0.8 | 1.2 | 1.7 | 2.1 |
|  |  | HW4 InH FR1 | 8.8 | 10.7 | 12.3 | 14.6 |
|  |  | HW5 InH FR1 | 0.1 | 0.1 | 0.2 | 0.3 |
|  |  | HW6 InH FR1 | 0.1 | 0.2 | 0.3 | 0.5 |
|  |  | HW7 InH FR1 | 0.1 | 0.2 | 0.3 | 0.5 |
|  |  | HW8 InH FR1 | 2.2 | 3.3 | 4.4 | 5.8 |
|  |  | HW9 InH FR1 | 0.6 | 0.7 | 0.9 | 1.0 |
|  |  | HW10 InH FR1 | 1.1 | 1.4 | 1.8 | 2.2 |
|  |  | HW11 InH FR1 | 1.1 | 1.4 | 1.8 | 2.1 |
|  |  | HW12 InH FR1 | 5.2 | 7.4 | 9.5 | 11.0 |
|  |  | HW13 InH FR1 | 0.04 | 0.05 | 0.06 | 0.1 |
|  |  | HW14 InH FR1 | 0.08 | 0.1 | 0.2 | 0.3 |
|  |  | HW15 InH FR1 | 0.09 | 0.1 | 0.2 | 0.3 |
|  |  | HW16 InH FR1 | 2.6 | 4.0 | 5.6 | 7.2 |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | InH | HW17 InH FR2 | 0.8 | 1.0 | 1.1 | 1.3 |
|  |  | HW18 InH FR2 | 0.3 | 0.4 | 0.6 | 1.0 |
|  |  | HW19 InH FR2 | 0.8 | 1.0 | 1.1 | 1.3 |
|  |  | HW20 InH FR2 | 0.2 | 0.4 | 0.7 | 1.4 |

### 8.3.2 System simulations for UMi scenario

| Parameter | HW1 UMi, FR1 | HW2 UMi, FR1 | HW3 UMi, FR1 | HW4 UMi, FR1 | HW5 UMi, FR1 | HW6 UMi, FR1 | HW7 UMi, FR1 | HW8 UMi, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 |
| Reference signal (type of sequence, number of ports, …) | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port |
| Number of sites | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| Number of symbols used per occasion | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | E-CID | E-CID | E-CID | E-CID | E-CID | E-CID | E-CID | E-CID |
| Network synchronization assumptions | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW9 UMi, FR1 | HW10 UMi, FR1 | HW11 UMi, FR1 | HW12 UMi, FR1 | HW13 UMi, FR1 | HW14 UMi, FR1 | HW15 UMi, FR1 | HW16 UMi, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 |
| Reference signal (type of sequence, number of ports, …) | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port |
| Number of sites | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| Number of symbols used per occasion | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm | Multi-RTT, PSO algorithm |
| Network synchronization assumptions | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

| Parameter | HW17 UMi, FR2 | HW18 UMi FR2 | HW19 UMi, FR2 | HW20 UMi FR2 |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 30GHz | 30GHz | 30GHz | 30GHz |
| Subcarrier spacing | 120kHz | 120kHz | 120kHz | 120kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 400MHz | 100MHz | 400MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 |
| Reference signal (type of sequence, number of ports, …) | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port |
| Number of sites | 7 | 7 | 7 | 7 |
| Number of symbols used per occasion | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 |
| Power-boosting level | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | E-CID | E-CID | Multi-RTT, Tylor expansion | Multi-RTT, Tylor expansion |
| Network synchronization assumptions | Sync error 0ns | Sync error 0ns | Sync error 0ns | Sync error 0ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP | BPL with Largest RSRP |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | DFT codebook | DFT codebook | DFT codebook | DFT codebook |
| Additional notes, if any |  |  |  |  |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMi | HW1 UMi FR1 | 1.0 | 1.5 | 3.0 | 11.3 |
|  |  | HW2 UMi FR1 | 1.6 | 2.2 | 3.3 | 11.2 |
|  |  | HW3 UMi FR1 | 1.6 | 2.2 | 4.6 | 16.7 |
|  |  | HW4 UMi FR1 | 8.9 | 13.6 | 18.2 | 28.3 |
|  |  | HW5 UMi FR1 | 0.7 | 1.4 | 3.1 | 12.1 |
|  |  | HW6 UMi FR1 | 0.7 | 1.4 | 3.2 | 10.1 |
|  |  | HW7 UMi FR1 | 0.7 | 1.6 | 4.3 | 16.2 |
|  |  | HW8 UMi FR1 | 2.2 | 3.8 | 14.7 | 32.7 |
|  |  | HW9 UMi FR1 | 1.2 | 2.0 | 6.4 | 46.7 |
|  |  | HW10 UMi FR1 | 2.2 | 3.7 | 10.3 | 52.0 |
|  |  | HW11 UMi FR1 | 2.2 | 3.2 | 6.5 | 26.6 |
|  |  | HW12 UMi FR1 | 14.1 | 18.2 | 24.0 | 36.2 |
|  |  | HW13 UMi FR1 | 0.4 | 1.9 | 9.6 | 50.7 |
|  |  | HW14 UMi FR1 | 0.8 | 2.7 | 9.4 | 40.6 |
|  |  | HW15 UMi FR1 | 0.9 | 2.4 | 8.5 | 39.1 |
|  |  | HW16 UMi FR1 | 9.5 | 17.3 | 28.0 | 53.5 |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR2 | UMi | HW17 UMi FR2 | 1.9 | 6.5 | 15.4 | 39.3 |
|  |  | HW18 UMi FR2 | 0.9 | 6.4 | 18.9 | 39.1 |
|  |  | HW19 UMi FR2 | 5.6 | 12.5 | 21.9 | 33.7 |
|  |  | HW20 UMi FR2 | 2.4 | 4.8 | 8.2 | 12.7 |

### 8.3.3 System simulations for UMa scenario

| Parameter | HW1 UMa, FR1 | HW2 UMa, FR1 | HW3 UMa, FR1 | HW4 UMa, FR1 | HW5 UMa, FR1 | HW6 UMa, FR1 | HW7 UMa, FR1 | HW8 UMa, FR1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 | 38.901 |
| Carrier frequency | 4GHz | 4GHz | 2GHz | 2GHz | 4GHz | 4GHz | 2GHz | 2GHz |
| Subcarrier spacing | 30kHz | 15kHz | 15kHz | 15kHz | 30kHz | 15kHz | 15kHz | 15kHz |
| Reference Signal Transmission Bandwidth | 100MHz | 50MHz | 50MHz | 5MHz | 100MHz | 50MHz | 50MHz | 5MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 | DL: Comb-6 UL: Comb-4 |
| Reference signal (type of sequence, number of ports, …) | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port | DL: PRS, Gold, 1-port UL: SRS, ZC, 1-port |
| Number of sites | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
| Number of symbols used per occasion | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 | DL: 8 UL: 4 |
| number of occasions used per positioning estimate | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Power-boosting level | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB | DL: 7.8dB UL: 6dB |
| Uplink power control (applied/not applied) | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Regular | Regular | Regular | Regular | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) | Super resolution (MUSIC) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | E-CID | E-CID | E-CID | E-CID | E-CID | E-CID | E-CID | E-CID |
| Network synchronization assumptions | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error | No sync error |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | SVD | SVD | SVD | SVD | SVD | SVD | SVD | SVD |
| Additional notes, if any |  |  |  |  |  |  |  |  |

|  |  | Source | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | UMa indoor | HW1 UMa FR1 | 3.7 | 10.8 | 37.5 | 71.3 |
|  |  | HW2 UMa FR1 | 4.2 | 12.6 | 37.2 | 114.4 |
|  |  | HW3 UMa FR1 | 6.0 | 16.0 | 33.7 | 112.6 |
|  |  | HW4 UMa FR1 | 12.9 | 21.1 | 43.3 | 124.9 |
|  |  | HW5 UMa FR1 | 3.7 | 12.4 | 36.6 | 73.4 |
|  |  | HW6 UMa FR1 | 4.6 | 12.4 | 38.5 | 114.4 |
|  |  | HW7 UMa FR1 | 6.1 | 18.0 | 40.5 | 105.4 |
|  |  | HW8 UMa FR1 | 8.0 | 20.7 | 52.8 | 128.7 |
|  | UMa outdoor | HW1 UMa FR1 | 5.7 | 12.2 | 29.8 | 70.3 |
|  |  | HW2 UMa FR1 | 6.5 | 15.2 | 30.8 | 73.2 |
|  |  | HW3 UMa FR1 | 6.3 | 22.3 | 53.9 | 100.8 |
|  |  | HW4 UMa FR1 | 15.6 | 31.3 | 54.7 | 105.9 |
|  |  | HW5 UMa FR1 | 5.7 | 12.4 | 36.6 | 73.4 |
|  |  | HW6 UMa FR1 | 6.4 | 15.2 | 33.0 | 75.6 |
|  |  | HW7 UMa FR1 | 6.5 | 23.0 | 57.3 | 106.2 |
|  |  | HW8 UMa FR1 | 11.9 | 32.8 | 58.0 | 115.6 |