3GPP TSG RAN WG1 #96	                                                                                     R1-1901699

Athens, Greece, 25th February – 1st March, 2019

Source:	vivo

Title:	DL intra UE Tx prioritization for URLLC

Agenda Item:	7.2.6.4

Document for:	Discussion and Decision

- Introduction

In RAN #81, a new study item regarding I-IOT was approved [1]. The objectives related to URLLC for RAN1 are as follows:

- L2/L3 enhancements: - UL/DL intra-UE prioritization/multiplexing, i.e. prioritization (for example dropping, delaying or puncturing lower priority service) between different categories of traffic in the UE, including both data and control channels and considering (RAN2/RAN1): - different latency and reliability requirements - Different types of resource allocation for example grant-free and grant-based allocations Note: RAN2 to start the work, RAN1 to take action based on RAN2 progress.

In RAN1 #95, a LS from RAN2 was agreed that the targeted intra-UE prioritization/multiplexing scenarios were identified as follows [2]:

Scenario 1: Intra-UE DL Prioritization Scenario 2: Intra-UE UL Prioritization – Resource Conflict between Configured and Dynamic Grant Scenario 3: Intra-UE UL Prioritization – Resource Conflict between Dynamic Grants Scenario 4: Intra-UE UL Prioritization – Resource Conflict between Control Channel and Control Channel Scenario 5: Intra-UE UL Prioritization – Resource Conflict between Control Channel and Data Channel Scenario 6: Intra-UE UL Prioritization – CA-based Concurrent Transmission with Power Limitation Scenario 7: Intra-UE UL Prioritization – Power Control for Traffics with Different Priorities

In this contribution, we provide our views on scenario 1, DL intra-UE prioritization/multiplexing between eMBB and URLLC transmissions.

- Discussion on Scenario 1: Intra-UE DL Prioritization

As mentioned by RAN2, this scenario considers a case UE has sequentially received two DL assignments with overlapping radio resources in time. UE needs to identify the DL assignment with higher priority, which may be used for URLLC transmission. From RAN1’s perspective, it is necessary for a UE to handle the differentiation between the DL assignments with different priorities. Besides, UE behaviour for a UE receiving multiple DL assignments with different priorities need to be discussed, which can be found in our companion contribution [3].In addition, how to deal with multiple DL transmissions overlapping in time with different priorities needs to be further studied.

  - Differentiation of eMBB and URLLC

For a UE supported by both eMBB and URLLC service, it is necessary to differentiate eMBB and URLLC service in DL. Typically, the type of service is differentiated in higher layer. While for URLLC service, it may be not efficient to acquire service type only after delivering the received packet to higher layer, due that larger latency may be caused. Therefore, service type differentiation should be supported in physical layer, such as by DCI format or RNTI. Details of differentiation of eMBB and URLLC can be found in our companion contribution [4].

Proposal 1: NR should support differentiation of DL transmissions with different priorities in PHY layer, such as by DCI format or RNTI.

  - Out-of-order scheduling

In Rel.15, it is not permitted that the HARQ-ACK feedback of a PDSCH is earlier than the HARQ-ACK feedback for another earlier scheduled PDSCH, which is called as out-of-order scheduling. In result, the scheduling cases shown in Figure 2 are not supported. However, such mechanism is not applicable for DL intra-UE prioritization. For a UE with eMBB and URLLC service, URLLC DL traffic may arrive during an eMBB DL transmission (PDCCH/PDSCH). In such case, gNB may have to prioritize the later arrived URLLC DL traffic due to the tight latency and reliability requirement.

Due to the necessity of meeting latency bound, a UE can be scheduled with a URLLC PDSCH after an eMBB PDSCH is scheduled and before the HARQ-ACK for the eMBB PDSCH. Besides, the HARQ-ACK of the URLLC PDSCH also needs to be transmitted before the HARQ-ACK of eMBB PDSCH. Therefore, out-of-order scheduling should be supported for DL intra-UE prioritization, for the tight scheduling timing of URLLC service.

Proposal 2: For a UE supporting both eMBB and URLLC traffic, out-of-order scheduling is supported.

- For any two HARQ process IDs A and B for a given cell, UE can be scheduled that a URLLC PDSCH is before an eMBB PDSCH, where the scheduling DCI for the eMBB PDSCH transmission comes before the scheduling DCI for the URLLC PDSCH transmission.

Figure 1: Example of out-of-order scheduling in case of eMBB DL transmission and URLLC DL transmission

  - eMBB PDSCH and URLLC PDSCH overlapping in time within a slot

For a UE supporting both eMBB and URLLC services, it may have scheduled eMBB and URLLC data transmission simultaneously, i.e. a URLLC PDSCH is scheduled in a slot that is already allocated to an eMBB PDSCH. In such case, the URLLC PDSCH and eMBB PDSCH are scheduled in the overlapped time resources, while overlapped or non-overlapped frequency resources are allocated to eMBB and URLLC PDSCH. Examples are given in Figure 2. As shown in case 1 in Figure 2 (a), a UE is scheduled with an eMBB PDSCH. After that, a URLLC PDSCH is scheduled with non-overlapping frequency resources with eMBB PDSCH. Another example is shown in case 2 in Figure 2. A URLLC PDSCH is scheduled with overlapping frequency resources with eMBB PDSCH. For both cases the URLLC PDSCH has overlapped symbols in time with eMBB PDSCH.

(a)

(b)

Figure 2: Example of overlapping between eMBB PDSCH and URLLC PDSCH for a UE, (a) overlapping in time domain, (b) overlapping in both time and frequency domain

For both cases, handling the reception of PDSCHs from different services overlapping in time is needed. Since latency requirement needs to be satisfied for URLLC service, URLLC PDSCH reception should be prioritized. One solution is that UE decodes both eMBB and URLLC PDSCHs within a slot. However, it could lead to increasing complexity and latency for UE reception. Therefore, such new UE capability needs to be defined. For UE not supporting simultaneously reception of different PDSCHs, the UE decodes URLLC PDSCH and cancels eMBB PDSCH decoding.

Proposal 3: In case of overlapping in time within a slot between eMBB PDSCH and URLLC PDSCH,

- For UE supporting simultaneous eMBB PDSCH and URLLC PDSCH reception, UE decodes both eMBB PDSCH and URLLC PDSCH.

- For UE NOT supporting simultaneous eMBB PDSCH and URLLC PDSCH reception, UE decodes URLLC PDSCH and cancels decoding eMBB PDSCH.

  - HARQ-ACK codebook determination

For a UE with multiple services of different priorities, when UE is configured with semi-static HARQ-ACK codebook, there may be issue while determining the HARQ-ACK codebook size for UE support simultaneous eMBB and URLLC PDSCH reception. It also related to the UL intra-UE multiplexing between control channels. More details of UL multiplexing can refer to our companion contribution [4].

In this section, we focus on the case that UE determines HARQ-ACK codebook size when semi-static HARQ-ACK codebook is applied. According to the specification, UE determines the semi-static HARQ-ACK codebook based on K1 table and TimeDomainResourceAllocation table such that HARQ-ACK codebook size in a slot is equal to the max number of non-overlapping candidate unicast PDSCH occasions per slot. An example in shown in the Figure 3. For this slot, there are three non-overlapped candidate PDSCH occasions according to the SLIV in the TimeDomainResourceAllocation table. In this case, the HARQ-ACK codebook for that slot consists of three fields each of which corresponds to a candidate PDSCH occasion.

Figure 3: Example of semi-static HARQ-ACK codebook determination according to candidate PDSCH occasions

When eMBB PDSCH and URLLC PDSCH are scheduled in the same slot and UE decodes both eMBB PDSCH and URLLC PDSCH, UE has no chance to feedback both eMBB ACK/NACK and URLLC ACK/NACK in the same HARQ-ACK codebook if eMBB PDSCH and URLLC PDSCH belong to the same candidate PDSCH occasion. One simple solution is that gNB does not schedule eMBB PDSCH and URLLC PDSCH in the symbol resources which belong to the same candidate PDSCH occasion. However, it may put limitation on the scheduling flexibility and may lead to increased latency for URLLC scheduling. Therefore, further enhancement for HARQ-ACK codebook determination for UE supporting simultaneous eMBB PDSCH and URLLC PDSCH reception can be considered.

Proposal 4: Further enhancement for HARQ-ACK codebook determination for UE supporting simultaneous eMBB PDSCH and URLLC PDSCH reception can be considered.

- Conclusion

In this contribution, we discussed design and related procedure for DL inter-UE Tx prioritization for eMBB and URLLC transmissions. The observation and proposals are summarized below.

Proposal 1: Study further on how to support differentiation of eMBB DL and URLLC DL in PHY layer.

Proposal 2: For a UE supporting both eMBB and URLLC traffic, out-of-order scheduling is supported.

- For any two HARQ process IDs A and B for a given cell, UE can be scheduled that a URLLC PDSCH is before an eMBB PDSCH, where the scheduling DCI for the eMBB PDSCH transmission comes before the scheduling DCI for the URLLC PDSCH transmission.

Proposal 3: In case of overlapping in time within a slot between eMBB PDSCH and URLLC PDSCH,

- For UE supporting simultaneous eMBB PDSCH and URLLC PDSCH reception, UE decodes both eMBB PDSCH and URLLC PDSCH.

- For UE NOT supporting simultaneous eMBB PDSCH and URLLC PDSCH reception, UE decodes URLLC PDSCH and cancels decoding eMBB PDSCH.

Proposal 4: Further enhancement for HARQ-ACK codebook determination for UE supporting simultaneous eMBB PDSCH and URLLC PDSCH reception can be considered.

- References

- RP-182090, “Study on NR Industrial Internet of Things (IoT)”, Nokia, RAN #81.

- R1-1814342, “LS on Intra-UE Prioritization/Multiplexing”, RAN2 #104.

- R1-1901695, “Enhancement for Scheduling/HARQ/CSI Processing timeline”, vivo, RAN1 #96.

- R1-1901698, “UL intra UE Tx prioritization for URLLC”, vivo, RAN1 #96.