3GPP TSG-RAN1#96	R1-1902672

Athens, Greece, 25 February - 1 March, 2019

Agenda item:       7.2.5.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Details of UE CLI measurements

Document for:     Discussion and Decision

# 1	Introduction

This contribution relates to the ongoing 3GPP NR WI on CLI (cross-link-interference). We address aspects related UE CLI measurements, filtering, and reporting of those. Below is a list of the most relevant 3GPP agreements on this topic from the Ad-hoc RAN1 meeting in January-2019 [1]:

Agreement

The following CLI measurements are supported:

- SRS-RSRP:

  - Linear average of the power contributions of the SRS to be measured over the configured resource elements within the considered measurement frequency bandwidth in the time resources in the configured measurement occasions

- RSSI:

  - The linear average of the total received power observed only in certain OFDM symbols of measurement time resource(s), in the measurement bandwidth, over the configured resource elements for measurement by the UE

For SRS-RSRP:

- The number of SRS to be monitored by the UE should not exceed 8 within a slot

  - Network may configure more than 8 SRSs over different slots

  - The total number of SRSs to be monitored by a UE should not exceed [32]

- The UE is not required to perform time tracking or time adjustment other than a constant offset relative to its own DL timing in order to perform SRS-RSRP measurement

  - FFS: Details on how the constant offset is derived by the UE. The constant offset of 0 is not precluded.

  - FFS: Whether or not to have measurement accuracy relaxation

  - FFS: Timing for the transmission point of view

- UE is not required to measure SRS using different SCS compared to the downlink active BWP SCS of the same carrier

Agreement

For SRS-RSRP measurement report, L3 measurement reporting is applied

Agreement

For CLI-RSSI, L3 measurement reporting is applied

- FFS: Whether SRS-RSRP measurement report and CLI-RSSI measurement report can be configured together for a UE

  - Whether UE should be required to perform SRS-RSRP measurement and CLI-RSSI measurement simultaneously

Agreement

For L3 measurement reporting of RSSI/SRS-RSRP, the specification detail on the reporting and triggering mechanism is discussed in RAN2

- Send an LS to RAN2 (Hyunsoo, LGE)

R1-1901426	[DRAFT] LS on UE-UE CLI measurement and reporting	LG Electronics

The LS is endorsed in R1-1901405

Agreement

From a specification perspective, the measurement/report configuration of CLI-RSSI and SRS-RSRP are independent.

FFS: UE capability discussion

In the context of UE CLI measurement, it is important to recall that the original purpose of having UE CLI measurements available at the NW are:

- Offer the NW information to enable smart scheduling decisions such that scheduling users on “bad resources” that are heavily polluted by CLI can be avoided.

- Offer the NW information to initiate coordinated between neighbor cells, gNBs, and CU and DUs to reduce impact from harmful UE-UE CLI.

By default, scheduling decisions are conducted independently for each cell. Scheduling is part of the MAC-layer. Downlink scheduling decisions by the NW are primary based on buffered/incoming traffic for the different UEs, corresponding QoS constraints, and UE air interface measurements such as CSI measurement reports.

The contribution is organized as follows: In Section 2 we have evaluated the impact of timing offset to UE-to-UE CLI measurement via link level simulation. In Section 3, we present our proposals for UE CLI measurement configuration and reporting framework. and in Section 4 we discuss other CLI-related enhancements. Finally, concluding remarks with a set of proposals appear in Section 5.

# 2 	UE-to-UE CLI measurements

Link-level performance results for UE CLI measurements are presented in this section. In order to understand the impact from the time difference between downlink signals and interference, we have evaluated the BLER of DL signal under CLI and SRS-RSRP/RSSI accuracy with/with various timing offset values.

In this simulation study we assume a victim UE maintaining a DL connection to the serving gNB while simultaneously being interfered by the UL of a nearby UE served by a neighbour gNB. The two gNB use TDD operation and are unsynchronized with each other. Scenario details are in Table 1.

Table 1: UE-to-UE CLI simulation scenario

| UE-to-UE CLI. UE U is the victim UE and UE I is the interfering (aggressor) UE. | - 4 GHz carrier frequency - 30 kHz subcarrier spacing - Variable timing error between interferer and victim: 20, 25, 30 µs - SINR range [-10, 0] dB - INR levels = +/- 5 dB - LDPC coding - SRS: 1/2/4 symbols per slot - Channels models: pedestrian A, typical urban, urban macro. |
| --- | --- |

Timing offset values are chosen with the consideration of TAoffset (20us) and TA from real propagation delay. Due to fixed TAoffset (20us in FR1), we should consider at least 20us timing offset regardless of propagation delay. In addition to this value, the TA of UL in aggressive cells and the propagation delay of DL signal in the serving cell should be added. Assuming the distance between victim UE and aggressive UE is small enough compared to the other delay, the expected timing delay can be calculated as

.

Assume similar cell radius of serving and aggressive cells, 20 us timing error can be used for small cell, while 25us and 30us can be used for the cells having cell radius upto 750ms and 1.5km, respectively.

## BLER performance

To see the performance impact to DL signal reception, we have evaluated the BLER performance.

As shown in the simulation results in Figure 1, BLER performance is mostly influenced by the SINR and invariant to timing errors. Thus the accuracy of the interference measurement is more important than timing alignment.

Observation 1: BLER performance degradation from INR = -5 dB to 5 dB of several orders of magnitude. UE-to-UE CLI level needs to be known at the gNB.

Observation 2: BLER results for the same INR with different timing errors reveal similar UE performance. Timing error is compensated by the link adaptation mechanisms. UE-to-UE CLI can be dealt with by existing interference cancellation mechanisms.

Figure 1: BLER for SINR range [-10, 0] dB

## RSRP and RSSI Accuracy

To analyze the accuracy of the CLI RSRP and RSSI, we use the CDF of the error between the measured and ideal RSRP/RSSI. Same setup as that from Table 1 has been used.

Figure 2 – Figure 6 show the CDF of the RSRP and RSSI errors in different INR levels for pedestrian-a and urban-macro channel models. At low INR, the RSRP error is high for all timing errors (more than 5 dB and upto 20 dB as median error) , regardless of the SRS density. At high INR, the higher the timing error, the higher the RSRP errors. RSSI errors are robust against the timing error in both INR regimes.

Figure 2: CDF of RSRP and RSSI measured without timing error (Black: RS/Slot = 1, Black dashed: RS/Slot = 2, Red: RS/Slot = 4)

Figure 4: CDF of RSRP and RSSI for Pedestrian A channel model at INR = - 5dB

Figure 5: CDF of RSRP and RSSI for Pedestrian A channel model at INR = 5dB

Figure 6: CDF of RSRP and RSSI for urban macro channel model at INR = 5dB

Observations 3: SRS- RSRP is sensitive to the timing error only at high INR, and that the RSSI error is negligible w.r.t. timing error, but high error w.r.t. ideal RSSI.

Observations 4: There are marginal benefits in terms of errors w.r.t. ideal RRM measurements when increasing the density of the SRS from 2 to 4 symbols per slot. It seems that 2 symbols are the better tradeoff.

Observations 5: Without timing alignment, the measured SRS-RSRP cannot provide meaningful measurement.

For SRS-RSRP measurement, with certain relaxation of accuracy of SRS-RSRP, a simplified timing offset can be indicated. We can indicate the timing offset as a unit of fractional symbol, such as (L/2, L/3, 2L/3,....(N-1)L/N…) where L is the symbol length and N is an integer.

There is another option similar to RIM-RS detection. Due to Comb-type RE-mapping pattern of SRS resource, SRS signal can be repeated twice or four times. Figure 6 shows the example of SRS reception with different timing offset.  Regardless of timing offset, UE can receive a complete a half-symbol of SRS signal at the first- or the second-half of DL symbol duration without any change of reception timing. Though a half point FFT is necessary, it can be implemented without big implementation complexity.  Thus, it is also possible to signal which half symbol is used for SRS-RSRP reception.

Figure 6: example of SRS reception with various timing offset

Observation 6: For SRS-RSRP, timing offset can be signaled as function of fractional OFDM symbols.

Proposal 1: For SRS-RSRP measurement, the UE is configured with the expected timing offset between SRS reception and normal DL reception timing as part of the CLI measurement configuration.

# 3	Configuration & triggering of UE CLI measurements

During RAN1 ad-hoc meeting in Taipei, it was agreed to support L3 measurement reporting for SRS-RSRP/CLI-RSSI but the details on measurement configurations are not fully discussed. Though the specification of L3 measurement reporting is up to RAN2, we need to provide the level of details for their work. In this section, we provide our view on the configuration and related operation for CLI measurement and reporting.

We first outline the configuration and reporting of UE CLI measurements with emphasis on UEs in RRC CONNECTED mode. For CLI measurement and report, the existing L3 measurement framework can be reused with some modification.

SRS-RSRP

For SRS-RSRP, SRS measurement resource to be configured. SRS is configured with SRS resource set which includes multiple SRS resource(s) and related parameters. SRS resource set is not applicable to measurement resource for SRS-RSRP. In this case, the multiple SRS resources for measurement may be correspond to different UE group or different cells. Thus, they cannot be configured in a SRS resource set. Also, SRS resource set includes several unnecessary parameters as mandatory field. Thus, SRS-resource IE is better to be included into measurement object for SRS-RSRP measurement. As we have discussed in Section 2, for each SRS resource, additional timing offset parameter should be provided. Summarize that, one or more SRS resource IEs can be included in the measurement object, with the addition of timing offset information for each SRS resource.

CLI-RSSI

For CLI-RSSI, measurement object can be one or more resource specified by the number of symbols and PRBs to be measured. Similar to RateMatchPattern(s) or CSI-IM-Resource Set can be used, and the equivalent resource with RateMatchingPattern is easier to generate an intended resource type. There is also a possibility to re-use existing CSI-IM resource set for measurement object, by combining multiple CSI-IM resources to generate the intended resource type. In addition, different from SRS-RSRP, CLI-RSSI can be configured with additional subband partitioning, the size of subband can be additionally configured with CLI-RSSI measurement resource.

Proposal 2: As part of the CLI measurement object, following resource can be configured

- For SRS-RSRP, one or more SRS resource(s) with additional timing offset information

- For CLI-RSSI, measurement resource can be new resource type equivalent to RateMatchingPattern or CSI-IM resource set

  - Additional Subband information can be configured to facilitate coordinated frequency scheduling.

Configuration of SRS resource set for transmission from the aggressor UE

One remaining issue is how to configure SRS transmission for the aggressor UE. The SRS transmission for CLI measurement may be different from other SRS transmission because it can be configured cell- or UE-group specifically. The existing configuration is designed to support UE-specific configuration, and there are limitations on the number of SRS configurations in a set. In order to de-couple the limitation in each SRS resource configuration, it is beneficial to define a separate SRS resource set intended for CLI-measurement. Other than new usage, the existing SRS configuration can be reused.

Proposal 3: For configuration of SRS transmission for CLI-measurement, reuse the existing RRC-based SRS resource configuration with new usage of “CLI management”.

Dynamic UE CLI measurement reporting:

As complement to the RRC-level solution would be to introduce faster and more agile UE reporting of CLI measurements to the NW that can be utilized for fast scheduling decisions, and potentially also coordination between gNBs at the NW side. This could be enabled by allowing reporting of UE CLI measurement information as follows.

Basic principles are:

The NW configures the UE to measure CLI-RSSI or SRS-RSRP. Those measurements, may be configured to be wideband or frequency selective (i.e. per sub-band). If UE CLI measurement are configured to be measured per subband, the CLI report can be expressed as vector of Boolean values, where each element corresponds to one of the subbands. This CLI report may be reported to the NW as MAC-CE, making it readily available at the NW’s MAC-layer for scheduling decisions.

The UE may also be configured to perform other Rel-15 defined UE interference measurements (e.g. based on CSI-IM resource or NZP CSI-RS resources for interference measurement), and hence could use as trigger for CLI Alert Message if the CLI measurement SRS-RSRP reaches a certain threshold as compared to the interference measurement.

Proposal 4: Support MAC-CE transmission from UE to indicate the subband wise CLI Alert. The indication is simple Boolean for each subband.

The UE may adopt implicit signaling of UE CLI measurements by simply masking the CQI by setting its reporting to “void” or “zero” if too high CLI is measured. This is a simple method that introduces no additional signaling overhead, while still conveying the useful information to the NW when not to schedule UEs that are subject to too high CLI levels (i.e. NW will most likely not schedule UEs when reporting “void” or “zero” CQI values). Notice that this option does not introduce new PHY-layer signaling, and hence is easy to introduce.

Proposal 5: Support the option of implicit UE CLI measurement information signalling to NW by masking the CQI to set it to “void” or “zero” if too high CLI is measured by the UE.

# 4 	Further improvement for CLI measurement

To optimize the signaling overhead between the gNB and the victim UE, and to reduce the computational load of the victim UE, the gNB should first identify which aggressors are near the victim UE. Once the close-by aggressors have been identified, the serving gNB can request from the neighbor the SRS configurations of only those UEs. The serving gNB then triggers the victim UE to measure and report back on a reduced set of SRS signals. The aggressors UEs could be identified through neighbor gNBs cooperation, by using e.g.:

- the location information of the victim UE and signaling such information between the serving gNB and

- the neighbors. time alignment of the victim and potential aggressor UEs to the serving and neighbor gNBs.

- gNB beamformed reception of the SRS of both the served UE and adjacent-cell UEs.

- Side link communication, e.g. V2X or D2D

Furthermore, the aggressor list of each victim UE changes over time, e.g. when either the victim or any of the aggressor UEs changes its position, therefore the list needs to be dynamically updated to avoid unnecessary measurements at the victim UE and reporting overhead.

Proposal 6: The serving gNB should identify for each potential victim UE a list of CLI aggressor UEs and update the list dynamically to reduce the signaling and computational overhead at the victim UE.

It may also be beneficial to consider options where RRC IDLE mode UEs perform early CLI measurements (say CLI-RSSI) upon RRC connection setup, and report those in a compact form during the random access procedure. Having such information early available at the NW would be useful to e.g. avoid scheduling the UE on resources subject to too high CLI, as well as take early actions to alleviate potential UE CLI problems. In order to provide the protection of random access procedure, a “green zone” can be configured for a cell with the coordination with the neighbouring cells.

Proposal 7: NR should support methods to allow the UE to measure CLI during RRC connection setup and convey early UE CLI information to the NW in a compact form during random access.

# 5	Conclusions

Observation 1: BLER performance degradation from INR = -5 dB to 5 dB of several orders of magnitude. UE-to-UE CLI level needs to be known at the gNB.

Observation 2: BLER results for the same INR with different timing errors reveal similar UE performance. Timing error is compensated by the link adaptation mechanisms. UE-to-UE CLI can be dealt with by existing interference cancellation mechanisms.

Observations 3: SRS- RSRP is sensitive to the timing error only at high INR, and that the RSSI error is negligible w.r.t. timing error, but high error w.r.t. ideal RSSI.

Observations 4: There are marginal benefits in terms of errors w.r.t. ideal RRM measurements when increasing the density of the SRS from 2 to 4 symbols per slot. It seems that 2 symbols are the better tradeoff.

Observations 5: Without timing alignment, the measured SRS-RSRP cannot provide meaningful measurement.

Observation 6: For SRS-RSRP, timing offset can be signaled as function of fractional OFDM symbols.

Proposal 1: For SRS-RSRP measurement, the UE is configured with the expected timing offset between SRS reception and normal DL reception timing as part of the CLI measurement configuration.

Proposal 2: As part of the CLI measurement object, following resource can be configured

- For SRS-RSRP, one or more SRS resource(s) with additional timing offset information

- For CLI-RSSI, measurement resource can be new resource type equivalent to RateMatchingPattern or CSI-IM resource set

  - Additional Subband information can be configured to facilitate coordinated frequency scheduling.

Proposal 3: For configuration of SRS transmission for CLI-measurement, reuse the existing RRC-based SRS resource configuration with new usage of “CLI management”.

Proposal 4: Support MAC-CE transmission from UE to indicate the subband wise CLI Alert. The indication is simple Boolean for each subband.

Proposal 5: Support the option of implicit UE CLI measurement information signalling to NW by masking the CQI to set it to “void” or “zero” if too high CLI is measured by the UE.

Proposal 6: The serving gNB should identify for each potential victim UE a list of CLI aggressor UEs and update the list dynamically to reduce the signaling and computational overhead at the victim UE.

Proposal 7: NR should support methods to allow the UE to measure CLI during RRC connection setup and convey early UE CLI information to the NW in a compact form during random access.

# References

- Chairman’s notes, RAN1 Ad-hoc #1901, Taipei, Taiwan