3GPP TSG-RAN1 Meeting #96bis	R1-1905187

Xi’an, China, 8th – 12th April 2019

Source:	Ericsson, Sierra Wireless

Title:	IMT-2020 self-evaluation: mMTC coverage, data rate, latency & battery life

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

# Introduction

The 3GPP Study Item on Self Evaluation towards IMT-2020 Submission [1] is evaluating the performance for the use cases eMBB, mMTC and URLLC. In this contribution we present mMTC results for NB-IoT and LTE Bandwidth reduced Low complexity (BL) UEs operating in Coverage Enhanced (CE) modes A and B, hereafter referred to as LTE-M, in terms of coverage, data rate, latency and battery life. These key performance indicators are part of the set of 3GPP mMTC 5G requirements [4]. The evaluations are performed for the IMT-2020 mMTC simulation configurations [5].

# Coverage

At RAN#81 TR 37.910 [3], which contains the results from the IMT-2020 self-evaluation, was agreed. Part of the results are link budget templates that for NB-IoT and LTE-M presents coverage for the (M/N)PDCCH, (N)PDSCH, (N)PUSCH F1, PUCCH and NPUSCH F2. The details of the mMTC link level evaluations in TR 37.910 can be found in RP-181889 IMT-2020 self-evaluation: mMTC link budget [2]. Here we complete the coverage evaluation for LTE-M and NB-IoT by adding the (N)PSS, (N)SSS, (N)PBCH and (N)PRACH to the set of evaluated physical channels and signals.

The simulation assumptions are presented in Table 1. Note that we here are evaluating the TDL-iii, 2 Hz, channel model and are assuming a device NF of 7 dB as specified in [5] and used in TR 37.910.

The eNB is configured to use 4 receive branches. In the DL TM2 based on 2 transmitters is used in all cases except for the (N)PSS/(N)SSS transmissions where 4 transmitters are used to improve the spatial diversity. The device is configured with single receive and transmit branches.

NB-IoT was evaluated for the guardband mode of operation which is suitable for operating within a NR carrier.

Table 1: Simulation configurations.

| Parameters | Config. A |
| --- | --- |
| Technologies | LTE-M FDD and NB-IoT FDD |
| Physical channels | NB-IoT: NPSS, NSSS, NPBCH, NPDCCH, NPDSCH, NPRACH, NPUSCH F1, NPUSCH F2 LTE-M: PSS, SSS, PBCH, MPDCCH, PDSCH, PRACH, PUSCH, PUCCH |
| Channel models | TDL-iii (NLOS) |
| Doppler | 2 Hz |
| eNB RX/TX | 4/2 and 4/4 |
| eNB NF | 5 dB |
| Device RX/TX | 1/1 |
| Transmission mode | TM2 at eNB; TM1 at UE |
| eNB power | 29 dBm per PRB (corresponding to a 10 MHz system with 46 dBm) 6 dB power boosting for NB-IoT. 3 dB power boosting for LTE-M PSS/SSS/PBCH. |
| UE power | 23 dBm |
| UE NF | 7 dB |
| NB-IoT mode of operation | Guardband |

Table 2 presents the NB-IoT coverage performance in terms of Maximum Coupling Loss (MCL):

MCL=PTX-(SNR+10log10kTB+NF)

The presented data rates are calculated at the physical layer over the (N)PDSCH/(N)PUSCH transmission times.

Table 2: NB-IoT link level results for TDL-iii, 2 Hz.

| Physical channel | NPSS/NSSS | NPBCH | NPDCCH | NPDSCH | NPRACH | NPUSCH F1 | NPUSCH F2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power | 35 | 35 | 35 | 35 | 23 | 23 | 23 |
| Bandwidth [kHz] | 180 | 180 | 180 | 180 | 3.75 | 15 | 15 |
| TBS [Bits] | - | 24 | 23 | 680 | - | 1000 | 1 |
| #RU/#SF/AL | - | - | 2 | 10 | 1 | 8 | 1 |
| #Rep/Aqusition time | 1280 ms | 1280 ms | 512 | 128 | 32 | 32 | 16 |
| #TX/#RX | 4TX/1RX | 2TX/1RX | 2TX/1RX | 2TX/1RX | 1TX/4RX | 1TX/4RX | 1TX/4RX |
| BLER | 10% | 10% | 1% | 10% | 1% | 10% | 1% |
| Phy data rate [bps] | - | - | - | 478 | - | 439 | - |
| SNR [dB] | -14.5 | -14.5 | -16.7 | -14.7 | -8.5 | -13.8 | -13.8 |
| NF | 7 | 7 | 7 | 7 | 5 | 5 | 5 |
| MCL | 164 | 164 | 166.2 | 164.2 | 164.8 | 164 | 164 |

Table 3 presents the LTE-M performance.

Table 3: LTE-M link level results for TDL-iii, 2 Hz.

| Physical channel | PSS/SSS | PBCH | MPDCCH | PDSCH | PRACH | PUSCH | PUCCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power | 39.2 | 39.2 | 36.8 | 36.8 | 23 | 23 | 23 |
| Bandwidth [kHz] | 945 | 945 | 1080 | 1080 | 1048.75 | 30 | 180 |
| TBS [Bits] | - | 34 |  | 328 | - | 712 | 1 |
| #RU/#SF/AL | - | - | 24 | 1 | 1 | 1 | 1 |
| #Rep/Aqusition time | 1500 ms | 800 ms | 256 | 768 | 64 | 1536 | 64 |
| #TX/#RX | 4TX/1RX | 2TX/1RX | 2TX/1RX | 2TX/1RX | 1TX/4RX | 1TX/4RX | 1TX/4RX |
| BLER | 10% | 10% | 1% | 2% | 1% | 2% | 1% |
| Phy data rate [bps] | - | - | - | 419 | - | 454 | - |
| SNR [dB] | -17.5 | -17.5 | -20.8 | -20.5 | -32.9 | -16.8 | -26 |
| NF | 7 | 7 | 7 | 7 | 5 | 5 | 5 |
| MCL | 164 | 164 | 164.2 | 164 | 164.7 | 164 | 165.5 |

Based on the above results it can be concluded that NB-IoT and LTE-M meets the 164 dB MCL target under the assumptions listed in Table 2.

# Data rate

TR 38.913 [4] contains 3GPPs 5G requirements. It adds mMTC requirements on data rate, latency and battery life to the required coverage and capacity KPIs covered in TR 37.919. In case of data rate it is requested that a 5G mMTC system should provide a sustainable data rate of at least 160 bps, calculated over one or more (M/N)PDCCH scheduling cycle(s), at the MCL of 164 dB. The calculations should take overheads (OH) from the radio protocol (PDCP/RLC/MAC) stack into account:

Data rate= 1-BLER∙TBS-OHPDCCH Sched. cycle

Figure 1 and Figure 2 shows NB-IoT DL and UL scheduling cycles at the MCL. Based on the performance presented in Table 2, DL/UL scheduling cycles of 2048 ms and 3072 ms, and an assumed overhead of 5 bytes, a sustainable NB-IoT data rate of 281 bps is achievable for both the downlink and the uplink.

Figure 1: NB-IoT NPDSCH scheduling cycle (G=4) at the MCL giving a DL data rate of 281 bps.

Figure 2: NB-IoT NPUSCH F1 scheduling cycle (G=1.5) at the MCL giving a UL data rate of 281 bps.

Table 4 summarizes the data rates calculated for both NB-IoT and LTE-M. For the LTE-M downlink and uplink data rate calculations scheduling cycles of 3x1.5x256 = 1152 ms and  5x1.5x256=1920 ms are assumed together with a 5 byte PDCP/RLC/MAC overhead.

It is show that in all cases the targeted 160 bps is met.

Table 4: NB-IoT and LTE-M sustainable data rates.

|  | DL data rate | UL data rate |
| --- | --- | --- |
| NB-IoT | 281 bps | 281 bps |
| LTE-M | 245 bps | 343 bps |

# Battery life

TR 38.913 defines a 10-year requirement on a 5-Wh battery for a traffic model defined by a daily delivery of an uplink message of 200 bytes followed by a 20-byte downlink message. The message sizes are assumed to be defined at on top of layer 2 which means that PDCP/RLC/MAC overhead is added to the message sizes in the battery life evaluations.

Based on the RRC resume procedure presented in Figure 3 and Table 5 and the device power consumption profile presented in Table 5 the battery life presented in Table 7 was achieved for NB-IoT and LTE-M.

Figure 3: NB-IoT RRC Resume including uplink and downlink data transmissions.

Table 5: RRC Resume packet sizes.

| Message | LTE-M | NB-IoT |
| --- | --- | --- |
| Msg2: RAR | 7 bytes | 7 bytes |
| Msg3: RRC Connection Resume Request | 7 bytes | 11 bytes |
| Msg4: RRC Connection Resume | 19 bytes | 19 bytes |
| Msg5: RRC Connection Resume Comp + RLC Ack for Msg4 + UL report | 22 + 200 bytes | 22 + 200 bytes |
| RRC Connection Release | 18 bytes | 17 bytes |

Table 6: NB-IoT and LTE-M device power consumption [6].

|  | TX | RX | RRC Connected | RRC Idle |
| --- | --- | --- | --- | --- |
| Power consumption | 500 mW | 80 mW | 3 mW | 0.015 mW |

Table 7: NB-IoT and LTE-M battery life.

|  | RRC Resume |
| --- | --- |
| NB-IoT | 11.8 years |
| LTE-M | 11.9 years |

Table 7 shows that both NB-IoT and LTE-M fulfils the 10-year target.

# Latency

TR 38.913 defines a 10 s requirement for the delivery of a small infrequent packet of size 105 bytes defined at the physical layer, i.e. including both PDCP, MAC and RRC overhead.

The latencies for the RRC procedure as well as the Early Data Procedure (EDT) are here evaluated. It can be noted that EDT is not supported for uplink TBS of size larger than 1000 bits. This explains why the EDT procedure was not used in the battery life evaluations in this contribution.

Figure 4 shows the packet flow for the EDT procedure specified for the User Plane. Table 8 presents the packet sizes for each of the transmissions.

Figure 4: EDT user plane solution procedure illustrated for NB-IoT.

Table 8: EDT packet sizes.

| Message | LTE-M | NB-IoT |
| --- | --- | --- |
| Msg2: RAR | 7 bytes | 7 bytes |
| Msg3: RRC Connection Resume Request + UL report | 11 + 105 bytes | 11 + 105 bytes |
| Msg4: RRC Connection Release | 25 bytes | 24 bytes |

The latency for the delivery of a small infrequent packet of size 105 bytes at the physical layer is presented in Table 7. The 10 s target is met in all cases.

Table 7: NB-IoT and LTE-M latency for small infrequent packet transmission.

|  | RRC Resume | EDT |
| --- | --- | --- |
| NB-IoT | 9.0 s | 5.8 s |
| LTE-M | 7.7 s | 5.0 s |

# Conclusions

In this contribution, we have presented input to the 3GPP self-evaluation study. It is shown that NB-IoT and LTE-M meet the 3GPP requirements on coverage, data rate, latency and battery life under the assumption presented in Table 1. We propose that RAN1 considers including these results in a suitable TR, e.g. TR 37.919.

# References

RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al

RP-181889, “IMT-2020 self-evaluation: mMTC link budget”, RAN#81, Ericsson

TR 37.919 v1.0.0 “Study on Self Evaluation towards IMT-2020 Submission (Rel-15)”

TR 38.913 v15.0.0 “Study on Scenarios and requirements for next generation access technologies”

Report ITU-R M.2411-0 “Requirements, evaluation criteria and submission templated for the development of IMT-2020”, ITU-R

TR 45.820 version 13.1.0 “Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT)”