3GPP TSG RAN WG1#97		R1-197086

Reno, USA, 13th – 17th May 2019

Agenda Item:	7.2.2.2.1

Source:	Lenovo, Motorola Mobility

Title:	Extensions for Channel Access Procedures

Document for:	Discussion

# Introduction

TR 38.889 captures the following aspects for channel access:

For channel access mechanism, LTE-LAA LBT mechanism is adopted as baseline for 5GHz band and adopted as the starting point of the design for 6GHz band. At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz. For CWS adjustment procedure in NR-U, in addition to aspects considered in LTE LAA, NR-U may additionally consider at least the following aspects: CBG based HARQ-ACK operation, NR scheduling and HARQ-feedback delays and processing times, wideband (>20 MHz) operation including BWPs, Configured grant operation. For initiation of a COT by the gNB (operating as an LBE device), the channel access schemes in Table 7.2.1.3.1-1 are used. For initiation of a COT by the UE, the channel access schemes in Table 7.2.1.3.1-4 are used. Table 7.2.1.3.1-4: Channel access schemes for initiating a COT by UE as LBE device Cat 2 LBT; Cat 4 LBT. PUSCH (including at least UL-SCH with user plane data); N/A except for the cases discussed in Note 2 below; Channel access priority class is selected according to the data. SRS-only; N/A; Cat4 with lowest channel access priority class value (as in LTE eLAA). RACH-only; (see Note 2); Cat4 with lowest channel access priority class value. PUCCH-only; (see Note 2); Cat4 with lowest channel access priority class value. Note 1: If the COT includes multiple signals/channels with different channel access categories / priority classes, the highest channel access priority class value and highest channel access category among the channel access priority classes and channel access categories corresponding to the multiple signals/channels applies. Note 2: Applicability of a channel access scheme other than Cat 4 for the following signals / channels have been discussed and details are to be determined when the specifications are developed: - UL control information including UCI only on PUSCH, e.g. HARQ-ACK, Scheduling Request, and Channel State Information - Random Access

In this document, we show our views on channel access for RACH messages in the 4-step RACH procedure.

# Discussion

In LTE LAA, channel access supports the LBT functionality with basically two access types: a contention window based access (type 1) and a one-shot access (type 2). Since LTE supports only SCells for unlicensed carriers, there was no need to consider access types or contention window adaptation for messages exchanges in the random access procedure. However, for all NR-U scenarios where an unlicensed carrier is a PCell, we need not only support RACH transmissions but also consider the appropriate channel access rules. Additionally, NR-U should support the transmission on wideband carriers, where the LBT would still be performed on a subband-level. Therefore we also need to consider the ACK/NACK feedback corresponding to a wideband transmission for subband contention window adjustments.

## Msg 2/3

Msg 2 is basically an ordinary PDSCH messages without HARQ, and Msg 3 is an ordinary PUSCH message. As a consequence, we think the following needs to be supported:

- The RAR Grant conveyed by Msg 2 needs to include an indicator for the channel access type and priority class applicable to Msg 3

Obviously a contention-window based channel access may imply a longer waiting time before Msg 2/3 can be transmitted. To compensate for this drawback, we think that we need to consider extending RACH timers and windows as well as supporting multiple subframes for the transmission of Msg 3. For the timers/windows, it would be suitable to approach RAN2 to inform them that RAN1 sees a benefit for extensions e.g. of the ra-ContentionResolutionTimer. For additional Msg 3 opportunities, the RAR Grant or the DCI assigning Msg 2 resources should additionally include an indicator for the number of Msg 3 transmission opportunities granted to the UE.

Proposal 2: Support the following:

- The RAR Grant conveyed by Msg 2 needs to include

  - an indicator for the channel access type and priority class applicable to Msg 3

- The RAR grant or the DCI assigning Msg 2 resources should include an indicator for the number of Msg 3 transmission opportunities granted to the UE

Proposal 3: Send an LS to RAN2 to suggest increasing the timers/windows involved in the random access procedure (e.g. ra-ContentionResolutionTimer) to compensate for channel access restrictions.

## Msg 4

Msg 4 can be seen as a PDCCH transmission by the gNB carrying generally a regular resource indication. As such, for channel access purposes it should be treated like any other such resource indication (plus data transmission).

## Contention Window Adjustments due to random access procedure transmissions

In LTE LAA, the contention window value is adjusted as a result of the successful/unsuccessful transmission status of TB(s) in a reference subframe, based e.g. on the received NDI in the DCI. At least for Msg 2 and Msg 3 however, there is no NDI included or applicable. Therefore we think that the contention window adjustment procedures need to be extended to include Msg 2 and Msg 3 transmissions.

Proposal 4: Contention Window Adjustment procedures need to be extended to include at least successful/unsuccessful Msg 2 and Msg 3 transmissions of the random access procedure.

## Contention Window Adjustments for wideband transmissions

Considering that LBT may be performed in units of 20 MHz at least on 5GHz, it is preferred to maintain the CWS per subband of the wideband carrier. If CBG-ACK is used for a wideband transmission, it would be easiest for CW adjustments of one subband includes complete code blocks, since then the corresponding CBG-ACKs can be used directly. However, if only HARQ-ACK for a wideband TB is available, some more thinking is required how to reflect TB-level ACK/NACK for subband CW adjustments. One specific issue arises if one or more of the subbands are not available for transmission due to LBT if only TB-ACK is available; in such a case, due to puncturing of the unavailable subbands, the feedback for the TB is likely NACK even though no collision may have occurred on any of the transmitted subbands. This issue needs to be addressed for subband CW adjustments.

Since LTE-LAA already includes a scaling of HARQ-ACK feedback for CW adjustment aspects (80% NACK threshold), we think it is most useful to extend this approach for subband CW adjustments. For example, a TB-level NACK for a subband should be counted with less weight than a CBG-level NACK received for a transmission on the subband, since the TB-level NACK could be the result of collisions or LBT failures on any of the other subbands where the same TB was mapped.

Proposal 5: Contention Window Adjustment procedures for wideband carrier transmissions need to address the cases of CBG-ACK as well as TB-level ACK, especially if a TB-level NACK is received when one or more subbands could not be transmitted due to an LBT failure. Re-using and extending the scaling of NACKs (as done in LTE-LAA) seems to be a proper starting point to establish different weights to different NACKs according to the situation.

## Reference subframe for Contention Window Adjustments

In LTE-LAA, DL reference subframe is defined as the starting subframe(s) of the most recent burst on the carrier made by the eNB, for which at least some HARQ-ACK feedback is expected to be available. A similar mechanism can be reused in NR-U, like that the reference slot for CW adjustment should also be defined as the first slot or the first and second slot of the latest DL burst for NR-U. However, NR-U may support multiple switching points within a COT, which means that there may be more than one DL burst within a channel occupancy. The gNB may use LBT Cat1 or Cat 2 before transmitting the DL bursts except the starting burst, and the HARQ-ACK corresponding to those later bursts do not reflect the channel collision conditions. So the reference resource in time domain should be the starting slot(s) of the most recent channel occupancy.

Proposal 6: In NR-U DL, the reference resource in time domain should be the starting slot(s) of the most recent channel occupancy on the subband where the gNB obtained the channel by Cat4 LBT.

# Conclusion

We propose the following channel access procedure amendments motivated by the random access procedure:

Proposal 1: PRACH should be designed for using channel access type 2 or no LBT.

Proposal 2: Support the following:

- The RAR Grant conveyed by Msg 2 needs to include

  - an indicator for the channel access type and priority class applicable to Msg 3

- The RAR grant or the DCI assigning Msg 2 resources should include an indicator for the number of Msg 3 transmission opportunities granted to the UE

Proposal 3: Send an LS to RAN2 to suggest increasing the timers/windows involved in the random access procedure to compensate for channel access restrictions.

Proposal 4: Contention Window Adjustment procedures need to be extended to include at least successful/unsuccessful Msg 2 and Msg 3 transmissions of the random access procedure.

Proposal 5: Contention Window Adjustment procedures for wideband carrier transmissions need to address the cases of CBG-ACK as well as TB-level ACK, especially if a TB-level NACK is received when one or more subbands could not be transmitted due to an LBT failure. Re-using and extending the scaling of NACKs (as done in LTE-LAA) seems to be a proper starting point to establish different weights to different NACKs according to the situation.

Proposal 6: In NR-U DL, the reference resource in time domain should be the starting slot(s) of the most recent channel occupancy on the subband where the gNB obtained the channel by Cat4 LBT.