3GPP TSG RAN WG1 Meeting #97	R1-1907893

Reno, USA, 13th – 17th, May 2019

Title:	Draft LS on support of Enhancements on multi-TRP/panel transmission

Response to:

Release:	Rel-16

Study Item:	NR_eMIMO

Source:	RAN WG1

To:	RAN WG2

Cc:	-

Contact Person:

Name:	                 	Min Zhang

E-mail Address:	Min.zhang1 at Huawei.com

1. Overall Description:

So far RAN1 has made the following agreements which may have RAN2 specification impact.

For multiple PDCCH based multi-TRP/panel transmission,

- For multi-PDCCH based multi-TRP operation, increase the maximum number of CORESETs per “PDCCH-config” to 5, according to UE capability.

- For separate ACK/NACK feedback for PDSCHs received from different TRPs, the UE should be able to generate separate ACK/NACK codebooks identified by an index, if the index is configured and applied across all CCs.

- Support joint HARQ-Ack feedback for PDSCHs received from different TRPs where multiple DCIs are used.

- When the PUCCH resources are on the different slots, which are indicated by PDSCH-to-HARQ_feedback timing indicator fields of multiple DCIs for different TRPs, both type-1 HARQ-ACK codebook and type-2 HARQ-ACK codebook are supported.

- The index to be used to generate separated ACK/NACK codebook is a higher layer signalling index per CORESET. Note that the index may not be configured for scenarios if there is no ambiguity of codebook generation at the UE, e.g. slot based PUCCH resource allocation per TRP. This does not preclude configuring the index for other purposes.

2. Actions:

RAN1 respectfully asks RAN2 to take into account above RAN1 agreements for future RAN2 work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #98                       26 – 30 August 2019, Prague, Czech Republic

TSG RAN WG1 Meeting #98b                                                14 – 18 October 2019, China