3GPP TSG RAN WG1 Meeting #98					R1-1909932

Prague, Czech Republic, August 26th – 30th, 2019

Source:	Intel Corporation

Title:	List of higher layer parameters for NR Positioning

Agenda item:	7.2.10

Document for:	Discussion and Decision

Introduction

In this contribution we provide initial list of higher layer parameters for NR Positioning framework. This list was endorsed based on RAN1 agreements made till RAN1#98 meeting and is expected to be further revised at RAN1#98bis meeting with a goal to prepare LS to RAN2 WG during RAN1#98bis meeting.

Parameters Related to DL PRS Configuration/Measurements

| Sub-feature group | RAN1 specification | Section | Parent IE in ASN.1 | RAN2 ASN.1 name (suggestions/examples) | New or existing parameter | Parameter name in specification | Description | Value range | Default value | UE specific/Cell specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR DL PRS Configuration |  |  |  | DL-PRS-ResourceSet | New | DL-PRS-ResourceSet | DL PRS Resource Set configuration for NR Positioning (i.e. container of configuration settings for a DL PRS Resource Set) | NA |  | FFS for RAN2 WG | FFS for RAN2 WG | FFS: Max number of sets per TRP |
| NR DL PRS Configuration |  |  |  | DL-PRS-ResourceSetId | New | DL-PRS-ResourceSetId | The ID of this DL PRS Resource Set. Each DL PRS Resource Set has a DL PRS Resource Set ID. | FFS |  | FFS for RAN2 WG | FFS for RAN2 WG |  |
| NR DL PRS Configuration |  |  |  | DL-PRS-Periodicity | New | DL-PRS-Periodicity | Periodicity of DL PRS allocation in slots configured per DL PRS Resource Set (i.e. all DL PRS Resources of a given Set have the same periodicity) ♦ 20480 is not supported for an SCS of 15 kHz | {4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 20480} |  | FFS for RAN2 WG | FFS for RAN2 WG | FFS: Further restrictions on values applicable to different SCS Note that whether DL-PRS-Periodicity & DL-PRS-ResourceSlotOffset can be combined into 1 field is for further discussion. FFS slot offset parameter on the resource set level (additional to a resource parameter for slot offset) |
| NR DL PRS Configuration |  |  |  | DL-PRS-ResourceList | New | DL-PRS-ResourceList | List of DL PRS Resources that belong to a given DL PRS Resource Set |  |  | FFS for RAN2 WG | FFS for RAN2 WG | FFS:A DL PRS resource can be part of multiple DL PRS resource sets FFS whether the DL-PRS-ResourceList contains only the PRS resource IDs or the full PRS resource configurations |
| NR DL PRS Configuration |  |  |  | DL-PRS-Resource | New | DL-PRS-Resource | DL PRS Resource configuration for NR Positioning (i.e. container of configuration settings for a DL PRS Resource) | NA |  | FFS for RAN2 WG | FFS for RAN2 WG |  |
| NR DL PRS Configuration |  |  |  | DL-PRS-ResourceId | New | DL-PRS-ResourceId | Resource Id is assigned to a given DL PRS Resource. A DL PRS Resource ID in a DL PRS Resource Set is associated with a single beam transmitted from a single TRP. | FFS |  | FFS for RAN2 WG | FFS for RAN2 WG | - |
| NR DL PRS Configuration |  |  |  | DL-PRS-SequenceId | New | DL-PRS-SequenceId | Sequence Id used to initialize cinit value used in pseudo random generator [TS38.211, 5.2.1] for generation of DL PRS sequence for transmission on a given DL PRS Resource | At least {0, …, 4095} |  | FFS for RAN2 WG | FFS for RAN2 WG | Note: RAN1 to discuss further whether parameter belong to Resource Set configuration and is applied to all Resources within a Resource Set or not. |
| NR DL PRS Configuration |  |  |  | DL-PRS-CombSizeN | New | DL-PRS-CombSizeN | Resource element (RE) spacing in each symbol of DL PRS Resource. The value N is configured from the set {2, 4, 6}. FFS: Inclusion of other values in the set including values in {1, 8, 12} | N = {2, 4, 6} |  | FFS for RAN2 WG | FFS for RAN2 WG | Note: RAN1 to discuss further whether parameter belong to Resource Set configuration and is applied to all Resources within a Resource Set or not. |
| NR DL PRS Configuration |  |  |  | DL-PRS-ReOffset | New | DL-PRS-ReOffset | Resource element offset in frequency domain for the first symbol in a DL PRS Resource. The relative RE offsets of following symbols are defined relative to the RE Offset in frequency domain of the first symbol in the DL PRS resource | {0,1} for N = 2 {0,1, 2, 3} for N = 4 {0,1, 2, 3, 4, 5} for N = 6 |  | FFS for RAN2 WG | FFS for RAN2 WG | FFS relationship between N and number of symbols per DL PRS Resource Note: RAN1 to discuss further whether parameter belong to Resource Set configuration and is applied to all Resources within a Resource Set or not. |
| NR DL PRS Configuration |  |  |  | DL-PRS-ResourceSlotOffset | New | DL-PRS-ResourceSlotOffset | Points to starting slot of DL PRS Resource |  |  | FFS for RAN2 WG | FFS for RAN2 WG | FFS whether it can be represented by time offset with respect to some reference Note: RAN1 to discuss further whether parameter belong to Resource Set configuration and is applied to all Resources within a Resource Set or not. FFS on DL-PRS-ResourceSetSlotOffset |
| NR DL PRS Configuration |  |  |  | FFS DL-PRS-ResourceOccasionLength | New | FFS DL-PRS- ResourceOccasionLength | FFS Number of consecutive slots containing a DL PRS resource starting from the slot offset provided by DL-PRS-ResourceSlotOffset. |  |  | FFS for RAN2 WG | FFS for RAN2 WG | - DL PRS occasion is one instance of periodically repeated time windows (consecutive slot(s)) where DL PRS is expected to be transmitted FFS: If this definition is introduced for muting or DL PRS resource allocation or both |
| NR DL PRS Configuration |  |  |  | DL-PRS-ResourceSymbolOffset | New | DL-PRS-ResourceSymbolOffset | Points to starting symbol of DL PRS Resource within a slot determined by DL-PRS-ResourceSlotOffset | {0, 1, 2, …, 11} |  | FFS for RAN2 WG | FFS for RAN2 WG | Note: RAN1 to discuss further whether parameter belong to Resource Set configuration and is applied to all Resources within a Resource Set or not. |
| NR DL PRS Configuration |  |  |  | DL-PRS-NumSymbols | New | DL-PRS-NumSymbols | Number of symbols per DL PRS Resource within a slot which is configurable from the following set {2, 4, 6} FFS: Inclusion of other values in the set including values in {1, 3, 8, 12} | {2, 4, 6} |  | FFS for RAN2 WG | FFS for RAN2 WG | Note: RAN1 to discuss further whether parameter belong to Resource Set configuration and is applied to all Resources within a Resource Set or not. |
| NR DL PRS Configuration |  |  |  | DL-PRS-QCL-Info | New | DL-PRS-QCL-Info | QCL indication with other DL reference signals |  |  | FFS for RAN2 WG | FFS for RAN2 WG | The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell. ♦ FFS which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS). ● With regards to QCL relations beyond Type-D of a DL PRS resource, support one or more of the following options: ○ Option 1: QCL-TypeC from an SSB from a TRP. ○ Option 2: QCL-TypeC from a DL PRS resource from a TRP. ○ Option 3: QCL-TypeA from a DL PRS resource from TRP. ○ Option 4: QCL-TypeC from a CSI-RS resource from a TRP. ♦ FFS: Which CSI-RS resource type can be used? (e.g., CSI-RS for CSI, CSI-RS for BM, CSI-RS for TRS, CSI-RS for RLM, CSI-RS for RRM). ○ Option 5: QCL-TypeA from a CSI-RS resource from a TRP. ♦ FFS: Which CSI-RS resource type can be used? (e.g., CSI-RS for CSI, CSI-RS for BM, CSI-RS for TRS, CSI-RS for RLM, CSI-RS for RRM). ● Option 6: No QCL relation beyond Type-D is supported. ♦ NOTE: QCL-TypeA: Doppler shift, Doppler spread, average delay, delay spread ♦ QCL-TypeB: Doppler shift, Doppler spread' ♦ QCL-TypeC: Average delay, Doppler shift ♦ QCL-TypeD: Spatial Rx parameter Note: RAN1 to discuss further whether parameter belong to Resource Set configuration and is applied to all Resources within a Resource Set or not. |
| NR DL PRS Configuration |  |  |  | DL-PRS-SubcarrierSpacing | New | DL-PRS-SubcarrierSpacing | Subcarrier Spacing for DL PRS Resource | 15, 30, 60 kHz for FR1 60, 120 kHz for FR2 |  | FFS for RAN2 WG | FFS for RAN2 WG | FFS if SCS is configured for ○ Alt. 1: A DL PRS resource set ○ Alt. 2: A frequency layer |
| NR DL PRS Configuration |  |  |  | DL-PRS-CyclicPrefix | New | DL-PRS-CyclicPrefix | Cyclic Prefix Type for DL PRS Resource | {Normal, Extended} |  | FFS for RAN2 WG | FFS for RAN2 WG | FFS if CP type is configured for ○ Alt. 1: A DL PRS resource set ○ Alt. 2: A frequency layer |
| NR DL PRS Configuration |  |  |  | DL-PRS-ResourceBandwidth | New | DL-PRS-ResourceBandwidth | Number of PRBs allocated for DL PRS Resource (allocated DL PRS bandwidth) | FFS |  | FFS for RAN2 WG | FFS for RAN2 WG | FFS granularity of bandwidth configuration which is to be down-selected at the RAN1#98bis meeting among the following options: □ Option 1. One PRB □ Option 2. Four PRBs □ Option 3. RBG granularity □ Option 4. One of values configurable from the set 24, 48, 96 192, 264 PRBs □ Option 5: Option 2, 3 or 4 combined with a possibility to blank PRBs at the band edges □ Option 6: Option 2, 3 or 4 combined with the maximum PRBs per carrier bandwidth FFS whether additional constraints such as start PRB and center frequency of the bandwidth for the PRS resources are the same within a frequency layer. Resolve FFS at the next meeting. |
| NR DL PRS Configuration |  |  |  | DL-PRS-StartPRB | New | DL-PRS-StartPRB | Start PRB index defined as offset with respect to reference Point A | FFS: (0..maxNrofPhysicalResourceBlocks-1) |  | FFS for RAN2 WG | FFS for RAN2 WG |  |
| NR DL PRS Configuration |  |  |  | DL-PRS-PointA | New | DL-PRS-PointA | Absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. A single Point A for DL PRS resource allocation is provided per frequency layer. All DL PRS Resources belonging to the same DL PRS Resource Set have common Point A | FFS (ARFCN-ValueNR ?) |  | FFS for RAN2 WG | FFS for RAN2 WG |  |
| FFS NR DL PRS Configuration or NR DL PRS Measurement Configuration |  |  |  | DL-PRS-RstdReferenceInfo | New | DL-PRS-RstdReferenceInfo | ● The network can indicate one or more of the following for the UE to use to determine a reference (reference time based on the DL PRS Resource ID(s)) for DL RSTD measurements. ○ A DL PRS Resource ID ○ A subset of DL PRS Resource IDs from a single DL PRS Resource set ○ A DL PRS Resource set ● The UE may use different DL PRS Resource ID(s) (with the condition that the multiple DL PRS Resource IDs belong to a single DL PRS Resource set) or a different DL PRS Resource set for determining the reference for the RSTD measurement, and if it chooses to do so, it should report the DL PRS Resource ID(s) and/or the information on the DL PRS Resource set used to determine the reference |  |  | FFS for RAN2 WG | FFS for RAN2 WG | ● The network can indicate one or more of the following for the UE to use to determine a reference (reference time based on the DL PRS Resource ID(s)) for DL RSTD measurements. ○ A DL PRS Resource ID ○ A subset of DL PRS Resource IDs from a single DL PRS Resource set ○ A DL PRS Resource set ● The UE may use different DL PRS Resource ID(s) (with the condition that the multiple DL PRS Resource IDs belong to a single DL PRS Resource set) or a different DL PRS Resource set for determining the reference for the RSTD measurement, and if it chooses to do so, it should report the DL PRS Resource ID(s) and/or the information on the DL PRS Resource set used to determine the reference |
| NR DL PRS Measurement Configuration |  |  |  | DL-PRS-RstdMeasurementInfo | New | DL-PRS-RstdMeasurementInfo | Is applied to configure UE to report DL PRS Resource ID(s) or DL PRS Resource Set ID(s) used for determining the timing of each TRP in RSTD measurements |  |  | FFS for RAN2 WG | FFS for RAN2 WG | o Multiple DL PRS resources can be used to determine the received DL subframe timing of the first arrival path of the TRP. o At least the PRS resource ID(s) or PRS resource set ID(s) used for determining the timing of each TRP in RSTD measurements can be configured for reporting in the measurement report |
| NR DL PRS Measurement Configuration |  |  |  | DL-PRS-UE-Rx-Tx-MeasurementInfo | New | DL-PRS-UE-Rx-Tx-MeasurementInfo | Is applied to configure UE to report DL PRS Resource ID(s) or DL PRS Resource Set ID(s) used for determining the timing of each TRP in the UE Rx-Tx time difference measurements |  |  | FFS for RAN2 WG | FFS for RAN2 WG | o Multiple DL PRS resources can be used to determine the received DL subframe timing of the first arrival path of the TRP. o At least the PRS resource ID(s) or PRS resource set ID(s) used for determining the timing of each TRP in the UE Rx-Tx time difference measurements can be configured for reporting in the measurement report. |
| FFS NR DL PRS Configuration or NR DL PRS Measurement Configuration |  |  |  | FFS DL-PRS-expectRSTD and/or DL-PRS-TransmissionTimeDifference and DL-PRS-expectPropagationDelayDifference | New | FFS DL-PRS-expectRSTD and/or DL-PRS-TransmissionTimeDifference and DL-PRS-expectPropagationDelayDifference | For providing an indication of when the DL PRSs are expected to arrive in time at the UE | FFS |  | FFS for RAN2 WG | FFS for RAN2 WG | For providing an indication of when the DL PRSs are expected to arrive in time at the UE, at least one of the following options is supported: • Option 1: Provide an expected RSTD value together with uncertainty (search window) to the UE for the TRPs in the assistance data (analogous to LTE). •Option 2: Provide a TRP transmission time difference (e.g., time offset between SFN#0, subframe offset, slot-, symbol-, sub-symbol-, ns-offset, etc.) and expected propagation delay difference together with uncertainty (search window) to the UE for the TRPs in the assistance data. oA search window may be needed for both, TRP transmission time difference and propagation time difference. oSub-SFN granularity (e.g, subframe offset, slot-, symbol-, sub-symbol-, ns-offset) may be obtained from the DL-PRS configuration information once time offset between SFN#0 is provided to the UE. |
| FFS NR DL PRS Configuration or NR DL PRS Measurement Configuration |  |  |  | FFS DL-PRS-expectRSTD-uncertain and/or DL-PRS-TransmissionTimeDifference-uncertain | New | FFS DL-PRS-expectRSTD-uncertain and/or DL-PRS-TransmissionTimeDifference-uncertain | For providing an indication of when the DL PRSs are expected to arrive in time at the UE with uncertainty (search window). | FFS |  | FFS for RAN2 WG | FFS for RAN2 WG | For providing an indication of when the DL PRSs are expected to arrive in time at the UE, at least one of the following options is supported: •Option 1: Provide an expected RSTD value together with uncertainty (search window) to the UE for the TRPs in the assistance data (analogous to LTE). •Option 2: Provide a TRP transmission time difference (e.g., time offset between SFN#0, subframe offset, slot-, symbol-, sub-symbol-, ns-offset, etc.) and expected propagation delay difference together with uncertainty (search window) to the UE for the TRPs in the assistance data. oA search window may be needed for both, TRP transmission time difference and propagation time difference. oSub-SFN granularity (e.g, subframe offset, slot-, symbol-, sub-symbol-, ns-offset) may be obtained from the DL-PRS configuration information once time offset between SFN#0 is provided to the UE. |
| NR DL PRS Configuration |  |  |  | DL-PRS-MutingPattern | New | DL-PRS-MutingPattern | FFS in RAN1 how it is defined |  |  | FFS for RAN2 WG | FFS for RAN2 WG |  |
| NR DL PRS Configuration |  |  |  | FFS in RAN1 DL-PRS-ResourceSetSpatialFilter | New | FFS in RAN1 DL-PRS-ResourceSetSpatialFilter | FFS in RAN1 whether and how it is defined if introduced. Motivation is to indicate that all DL PRS Resources within DL PRS Resource Set use the same DL spatial transmission filter |  |  | FFS for RAN2 WG | FFS for RAN2 WG |  |
| NR DL PRS Configuration |  |  |  | FFS in RAN1 DL-PRS-ResourceSetRepetition | New | FFS in RAN1 DL-PRS-ResourceSetRepetition | FFS in RAN1 whether and how it is defined if introduced. Motivation is to indicate number of consecutive repetition of DL PRS Resource Set resources |  |  | FFS for RAN2 WG | FFS for RAN2 WG |  |
| NR DL PRS Configuration |  |  |  | FFS in RAN1 DL-PRS-SSB-PowerOffset | New | FFS in RAN1 DL-PRS-SSB-PowerOffset | FFS in RAN1 whether and how it is defined if introduced. Motivation is to indicate offset of DL PRS TX power with respect to SSB TX power |  |  | FFS for RAN2 WG | FFS for RAN2 WG |  |
| NR DL PRS Measurement Configuration |  |  |  | FFS in RAN1 DL-PRS-MeasurementReportConfig | New | FFS in RAN1 DL-PRS-MeasurementReportConfig | FFS in RAN1 whether and how it is defined if introduced. Details TBD. Motivation is to configure UE to conduct measurements and report results of measurements based on DL PRS processing |  |  | FFS for RAN2 WG | FFS for RAN2 WG |  |
| NR DL PRS Configuration |  |  |  | FFS in RAN1 FrequencyLayer | New | FFS in RAN1 FrequencyLayer | A container for all DL PRS Resource Sets from all TRPs classified as in one frequency layer. A single Point A for DL PRS resource allocation is provided per frequency layer. All DL PRS Resources belonging to the same DL PRS Resource Set have common Point A |  |  | FFS for RAN2 WG | FFS for RAN2 WG | FFS: Signaling for the association of a PRS resource to frequency layer. FFS: Max number of frequency layers |
| NR DL PRS Configuration |  |  |  | FFS in RAN1 DL-PRS-ResourceSetList | New | FFS in RAN1 DL-PRS-ResourceSetList | A container for all DL PRS Resource Sets from a TRP. |  |  | FFS for RAN2 WG | FFS for RAN2 WG | - Multiple DL PRS Resource Sets can be configured per TRP FFS how many DL PRS Resources Sets can be configured per TRP |
| NR DL PRS Configuration |  |  |  | FFS in RAN1 DL-PRS-ResourceType | New | FFS in RAN1 DL-PRS-ResourceType | Time domain behavior of DL PRS resource configuration. The network configures DL PRS resources in the same resource set with the same time domain behavior on periodic, aperiodic and semi-persistent DL PRS. | aperiodic, semipersistent, periodic |  | FFS for RAN2 WG | FFS for RAN2 WG |  |

Parameters Related to UL PRS/SRS Configuration/Measurements

| Sub-feature group | RAN1 specification | Section | Parent IE in ASN.1 | RAN2 ASN.1 name (suggestions/examples) | New or existing parameter | Parameter name in specification | Description | Value range | Default value | UE specific/Cell specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR UL PRS/SRS Configuration |  |  | SRS-ResourceSet | SRS-ResourceSet | FFS for RAN2 WG | SRS-ResourceSet | List of SRS Resources with SRS related configuration settings. Each resource set defines a set of SRS-Resources. |  |  |  |  |  |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | SRS-Resource | FFS for RAN2 WG | SRS-Resource | SRS resource configuration settings |  |  |  |  |  |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | SRS-ResourceId | FFS for RAN2 WG | SRS-ResourceId |  | FFS: INTEGER (0..maxNrofSRS-Resources-1) |  | UE specific | FFS for RAN2 WG |  |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | FFS: nrofSRS-Ports | FFS for RAN2 WG | FFS: nrofSRS-Ports |  | FFS: ENUMERATED {port1, ports2, ports4} |  | UE specific | FFS for RAN2 WG | FFS. RAN1 has not discussed/concluded on number of SRS ports for NR Positioning. At least one port can be assumed similar to discussion on DL PRS design. If only one port is agreed this field may not be needed for NR Positioning in R16 Proposal from rapporteur - Single port is used for NR Positioning in R16 (i.e. field is not needed for NR Positioning in R16) |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | FFS: ptrs-PortIndex | FFS for RAN2 WG | FFS: ptrs-PortIndex | FFS: The PTRS port index for this SRS resource for non-codebook based UL MIMO. This is only applicable when the corresponding PTRS-UplinkConfig is set to CP-OFDM. The ptrs-PortIndex configured here must be smaller than the maxNrofPorts configured in the PTRS-UplinkConfig (see TS 38.214 [19], clause 6.2.3.1). | FFS:ENUMERATED {n0, n1} |  | UE specific | FFS for RAN2 WG | FFS. RAN1 has not yet discussed/concluded on applicability of this field (ptrs-PortIndex) for NR Positioning in R16 Proposal from rapporteur - Conclude on applicability of ptrs-PortIndex for NR Positioning at the RAN1#98bis meeting |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | transmissionComb | FFS for RAN2 WG | transmissionComb combOffset transmissionComb cyclicShift | transmissionCombComb value (2 or 4 or 8) and comb offset (0..combValue-1) (see TS 38.214 [19], clause 6.2.1) | n2 combOffset (0,1), cyclicShift (0,…,7) n4 combOffset (0,..,3), cyclicShift (0,…,11) n8 comboffset (0,..,7), cyclicShift (0,…,FFS) |  | UE specific | FFS for RAN2 WG | For positioning, the SRS comb size set is extended from {2,4} to {2,4,8} - The RE pattern of an SRS resource for positioning is configured with a combOffset for the first symbol in an SRS resource. - The relative RE offsets of following symbols are defined relative to the combOffset of the first symbol in the SRS resource. - A relative RE offset of each of the following symbols is derived from the configured number of symbols for an SRS resource, the comb size for the SRS resource and the SRS symbol index within the SRS resource. --FFS: The use of the following for deriving the relative offset in addition to the above: 1) A configurable number of consecutive symbols with the same offset 2) Repetition factor FFS cyclic shift values - RAN1 has not yet discussed/concluded on applicability of cyclic shift values for NR Positioning in R16 Proposal from rapporteur - Conclude on cyclic shift values for NR Positioning in R16 at the RAN1#98bis meeting |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | resourceMapping | FFS for RAN2 WG | resourceMapping startPosition resourceMapping nrofSymbols FFS: resourceMapping repetitionFactor | OFDM symbol location of the SRS resource within a slot including number of OFDM symbols (N = 1, 2 or 4 per SRS resource), startPosition (SRSSymbolStartPosition = 0..5; "0" refers to the last symbol, "1" refers to the second last symbol) and RepetitionFactor (r = 1, 2 or 4) (see TS 38.214 [19], clause 6.2.1 and TS 38.211 [16], clause 6.4.1.4). The configured SRS resource does not exceed the slot boundary. | startPosition INTEGER (0,…,5, 6, …,13) nrofSymbols ENUMERATED {n1,n2,n4, n8, n12} FFS: repetitionFactor ENUMERATED {n1,n2,n4} |  | UE specific | FFS for RAN2 WG | For positioning, the number of consecutive OFDM symbols in an SRS resource is configurable with one of the values in the set {1, 2, 4, 8, 12} For positioning, starting positions in the time domain for the SRS resource can be anywhere in the slot, i.e. an offset loffset range of {0,1,…,13} FFS. RAN1 has not yet discussed/concluded on applicability of repetition factor for NR Positioning in R16 Proposal from rapporteur - Repetition factor field and values are reused for NR Positioning in R16 |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | FFS: freqDomainPosition | FFS for RAN2 WG | FFS: freqDomainPosition | FFS: Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, respectively, and described in Subclause 6.4.1.4 of [4, TS 38.211] | FFS: INTEGER (0,…,67) |  | UE specific | FFS for RAN2 WG | FFS. RAN1 has not yet discussed/concluded on applicability of this parameter for NR Positioning in R16 Proposal from rapporteur - Conclude on frequency hopping at the RAN1#98bis meeting |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | FFS: freqDomainShift | FFS for RAN2 WG | FFS: freqDomainShift | FFS: Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, respectively, and described in Subclause 6.4.1.4 of [4, TS 38.211] | FFS: INTEGER (0,…,268) |  | UE specific | FFS for RAN2 WG | FFS. RAN1 has not yet discussed/concluded on applicability of this parameter for NR Positioning in R16 Proposal from rapporteur - Conclude on frequency hopping at the RAN1#98bis meeting |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | FFS: freqHopping | FFS for RAN2 WG | FFS: freqHopping c-SRS freqHopping b-SRS freqHopping b-hop | FFS: Includes parameters capturing SRS frequency hopping (see TS 38.214 [19], clause 6.2.1). | c-SRS INTEGER (0,…,63) b-SRS INTEGER (0,…,3) b-hop INTEGER (0,…,3) |  | UE specific | FFS for RAN2 WG | FFS. RAN1 has not yet discussed/concluded on applicability of frequency hopping for NR Positioning in R16 Proposal from rapporteur - Conclude on frequency hopping at the RAN1#98bis meeting |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | FFS: groupOrSequenceHopping | FFS for RAN2 WG | FFS: groupOrSequenceHopping | FFS: Parameter(s) for configuring group or sequence hopping (see TS 38.211 [16], clause 6.4.1.4.2). | FFS: ENUMERATED {neither, groupHopping, sequenceHopping } |  | UE specific | FFS for RAN2 WG | FFS. RAN1 has not yet discussed/concluded on applicability of groupHopping, sequenceHopping for NR Positioning in R16 Proposal from rapporteur - The field groupOrSequenceHopping and values are reused for NR Positioning in R16 |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | resourceType | FFS for RAN2 WG | resourceType SRS-PeriodicityAndOffset | resourceType Periodicity and offset for semi-persistent and periodic SRS resource (see TS 38.214 [19], clause 6.2.1) Periodicity and slot offset for this SRS resource. All values in "number of slots". sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots | FFS: sl1 NULL, sl2 INTEGER(0..1), sl4 INTEGER(0..3), sl5 INTEGER(0..4), sl8 INTEGER(0..7), sl10 INTEGER(0..9), sl16 INTEGER(0..15), sl20 INTEGER(0..19), sl32 INTEGER(0..31), sl40 INTEGER(0..39), sl64 INTEGER(0..63), sl80 INTEGER(0..79), sl160 INTEGER(0..159), sl320 INTEGER(0..319), sl640 INTEGER(0..639), sl1280 INTEGER(0..1279), sl2560 INTEGER(0..2559) |  | UE specific | FFS for RAN2 WG | FFS field/values Proposal from rapporteur - FFS PeriodicityAndOffset field and values are reused for NR Positioning in R16 |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | sequenceID | FFS for RAN2 WG | sequenceID | Sequence ID used to initialize pseudo random group and sequence hopping | FFS: INTEGER (0..1023) |  | UE specific | FFS for RAN2 WG | Proposal from rapporteur -SequenceID field and values are reused for NR Positioning in R16 |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | spatialRelationInfo | FFS for RAN2 WG | spatialRelationInfo | spatialRelationInfo Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB | SSB-Index |  | UE specific | FFS for RAN2 WG | For positioning purposes, for UL Beam management/alignment towards serving and neighbouring cells, the following is supported (in addition to UE TX beam sweeping): ● Configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS. ○ Reference DL RS that can be used include at least SSB. - FFS on CSI-RS, DL-PRS Proposal: Conclude the necessary fields that need to be included spatialRelationInfo to indicate the DL reference signal from a neighboring cell to the UE. |
| NR UL PRS/SRS Configuration |  |  | SRS-ResourceSet | srsResourceSetId | FFS for RAN2 WG | srsResourceSetId | The ID of this resource set. FFS It is unique in the context of the BWP in which the parent SRS-Config is defined |  |  | UE specific | FFS for RAN2 WG | Proposal from rapporteur - Reuse field and values for NR Positioning |
| NR UL PRS/SRS Configuration |  |  | SRS-ResourceSet | srsResourceIdList | FFS for RAN2 WG | srsResourceIdList | The IDs of the SRS-Resources used in this SRS-ResourceSet. FFS: If this SRS-ResourceSet is configured with usage set to codebook, the srs-ResourceIdList contains at most 2 entries. If this SRS-ResourceSet is configured with usage set to nonCodebook, the srs-ResourceIdList contains at most 4 entries. |  |  | UE specific | FFS for RAN2 WG | Proposal from rapporteur - Reuse field and values for NR Positioning |
| NR UL PRS/SRS Configuration |  |  | SRS-ResourceSet | resourceType | FFS for RAN2 WG | resourceType | Time domain behavior of SRS resource configuration, see TS 38.214 [19], clause 6.2.1. The network configures SRS resources in the same resource set with the same time domain behavior on periodic, aperiodic and semi-persistent SRS. | aperiodic, semipersistent, periodic |  | UE specific | FFS for RAN2 WG | The following configurations of SRS for positioning are supported ○ Semi-persistent configuration ○ Periodic configuration ○ Aperiodic configuration FFS: - discuss whether reference to accociated RS need to be provided - discuss trigger for SRS transmission in case of aperiodic, semi-persistent |
| NR UL PRS/SRS Configuration |  |  | SRS-ResourceSet | FFS usage | FFS for RAN2 WG | FFS usage | FFS Indicates if the SRS resource set is used for beam management, codebook based or non-codebook based transmission or antenna switching. See TS 38.214 [19], clause 6.2.1. Reconfiguration between codebook based and non-codebook based transmission is not supported | FFS ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching} |  | UE specific | FFS for RAN2 WG | FFS: if usage of "positioning" needs to be added. No consensus so far in RAN1. Alternative option is to define all IEs in LPP Proposal from rapporteur - Conclude on whether usage field is needed for NR Positioning and whether new "positioning" usage need to be added at the RAN1#98bis meeting |
| NR UL PRS/SRS Configuration |  |  | SRS-ResourceSet | alpha | FFS for RAN2 WG | alpha | alpha value for SRS power control (see TS 38.213 [13], clause 7.3). When the field is absent the UE applies the value 1 | FFS: Alpha |  | UE specific | FFS for RAN2 WG | Proposal from rapporteur - alpha field and values are reused for NR Positioning in R16 |
| NR UL PRS/SRS Configuration |  |  | SRS-ResourceSet | P0 | FFS for RAN2 WG | P0 | P0 value for SRS power control. The value is in dBm. Only even values (step size 2) are allowed (see TS 38.213 [13], clause 7.3) | FFS: INTEGER (-202..24) |  | UE specific | FFS for RAN2 WG | Proposal from rapporteur - P0 field and values are reused for NR Positioning in R16 |
| NR UL PRS/SRS Configuration |  |  | SRS-ResourceSet | pathlossReferenceRS | FFS for RAN2 WG | pathlossReferenceRS | A reference signal to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3). | ssb-Index SSB-Index DL-PRS-ResourceId |  | UE specific | FFS for RAN2 WG | For positioning purposes, support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference for the purpose of SRS power control. For the purpose of power control of the SRS for positioning purposes, support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference. One of the following can be configured as the DL reference signal. ○ SSB ○ DL-PRS at least for multi-cell RTT ○ FFS: CSI-RS Proposal Conclude the necessary fields that need to be included in pathlossReferenceRS to indicate the DL reference signal from a neighboring cell to the UE. |
| NR UL PRS/SRS Configuration |  |  | SRS-ResourceSet | FFS: srsPowerControlAdjustmentPhase | FFS for RAN2 WG | FFS: srsPowerControlAdjustmentPhase | FFS: Indicates whether hsrs,c(i) = fc(i,1) or hsrs,c(i) = fc(i,2) (if twoPUSCH-PC-AdjustmentStates are configured) or separate close loop is configured for SRS. This parameter is applicable only for Uls on which UE also transmits PUSCH. If absent or release, the UE applies the value sameAs-Fci1 (see TS 38.213 [13], clause 7.3) | FFS: ENUMERATED { sameAsFci2, separateClosedLoop} |  | UE specific | FFS for RAN2 WG | FFS: Support of closed loop power control requires more discussion in RAN1 Proposal from rapporteur - Conclude on applicability of closed loop power control at the RAN1#98bis meeting |
| NR UL PRS/SRS Measurement Configuration |  |  |  | FFS in RAN1 UL-PRS-MeasurementReportConfig | New | FFS in RAN1 UL-PRS-MeasurementReportConfig | FFS in RAN1 whether and how it is defined if introduced. Details TBD. Motivation is to request gNB to conduct measurements and report results of measurements based on UL PRS processing |  |  | FFS for RAN2 WG | FFS for RAN2 WG |  |
| NR UL PRS/SRS Configuration |  |  | SRS-Resource | FFS in RAN1 symbolBundle | New | FFS in RAN1 symbolBundle | FFS in RAN1 if introduced. Motivation is to configure a number of consecutive SRS symbols with the same offset within the SRS resource. | INTEGER (1...12) |  | UE specific | FFS for RAN2 WG | - The RE pattern of an SRS resource for positioning is configured with a combOffset for the first symbol in an SRS resource. - The relative RE offsets of following symbols are defined relative to the combOffset of the first symbol in the SRS resource. - A relative RE offset of each of the following symbolsolumn is derived from the configured number of symbols for an SRS resource, the comb size for the SRS resource and the SRS symbol index within the SRS resource. --FFS: The use of the following for deriving the relative offset in addition to the above: 1) A configurable number of consecutive symbols with the same offset 2) Repetition factor |
| NR UL PRS/SRS Configuration |  |  | SRS-ResourceSet | FFS in RAN1 clPowerControl | New | FFS in RAN1 clPowerControl | FFS in RAN1 if introduced. Motivation is to enable/disable closed loop power control for SRS | ENUMERATED { disabled } |  | UE specific | FFS for RAN2 WG |  |