3GPP TSG-RAN/WG1 Meeting #98b	R1-1910185

# Chongqing, China, 14th October - 20th October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | <CR#> | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PUCCH power control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In subclause 7.2.1 of 38.213, it is described that a UE refers subcluase 9.2.5 in order to calculate the number of CRC bits. However, the description in subclause 9.2.5 of 38.213 is regarding to UCI multiplexing for HARQ-ACK and/or SR and/or CSI. In other words, the UE cannot get the calculation method for the number of CRC bits if the UE has only HARQ-ACK larger than 11bits. The calculation method for HARQ-ACK only is described in subcluase 9.2.3, in turn the subcluase 9.2.3 refers to subcluase 9.2. Therefore, we suggest that the reference for subcluase 9.2.5 should be replaced with subcluase 9.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the reference sublause 9.2.5 to subcluase 9.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot get the calculation method for the number of CRC bits if the UE has only HARQ-ACK larger than 11bits. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR resolves a right reference for the number of CRC bits. A reasonable UE implementation without this change request already implemented based on the RAN1’s common understanding. Therefore, this CR is backward-compatible. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

7.2.1	UE behaviour

[…]

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits larger than 11, , where

-

-

-	 is a number of HARQ-ACK information bits that the UE determines as described in Subclause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Subclause 9.1.3.1 for Type-2 HARQ-ACK codebook. If the UE is not provided pdsch-HARQ-ACK-Codebook,  if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise,

-	 is a number of SR information bits that the UE determines as described in Subclause 9.2.5.1

-	 is a number of CSI information bits that the UE determines as described in Subclause 9.2.5.2

-	 is a number of CRC bits that the UE determines as described in Subclause 9.2

-	 is a number of resource elements that the UE determines as , where  is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and  is a number of symbols excluding symbols used for DM-RS transmission, as defined in Subclause 9.2.5.2, for PUCCH transmission occasion  on active UL BWP  of carrier  of serving cell .

[…]

< Unchanged parts are omitted >