3GPP TSG RAN WG1 Meeting #98biss	R1-1910397

Chongqing, China, October 14 – 20, 2019

Agenda Item:	7.2.5.5

Source:	Huawei, HiSilicon

Title:	Remaining design aspects for NTN

Document for:	Discussion and Decision

Introduction

A new SI was approved by RAN#80 on studying a set of necessary features/adaptations enabling the operation of NR protocol in non-terrestrial networks (NTN), including physical layer, Layer 2 and above [1]. In this contribution, we discuss the issues of cell measurement, PAPR, and link budget in NTN.

Discussion on Cell measurement

  - Cell measurement for mobility management in NR

One of the fundamental requirements for mobility management is to identify the cell boundary, such that necessary mobility handling operations can be implemented when UE is about to cross cell boundaries. For this purpose, NR conducts cell measurement based on signal strength [2]. Specifically, the signal strength based measurement event A3 triggering handover in NR is listed as [3]:

Observation 1: The cell measurement for NR mobility management (e.g., handover) is executed based on signal strength related parameters.

  - Cell measurement for mobility management in NTN

The signal strength based measurement is reasonable and efficient for NR since a clear near-far effect is expected in terrestrial networks. As depicted in Figure 1(a), a UE can approximately determine whether it is located in the center or edge of the serving cell by measuring the value of RSRP. However, in the context of NTN as shown in Figure 1(b), the near-far effect is not clearly exhibited within a cell.

Figure 1. A sketch of near-far effect in different scenarios: (a) Terrestrial Network; (b) NTN

The unclear near-far effect in NTN is further verified by the path loss value shown in Table 1. For instance, in an NTN cell with a gNB height of 600km and a cell range of 200km, the difference of free space loss between the cell center and cell edge is only around 0.12dB. Considering a 3dB EIRP difference within a cell due to the antenna pattern, the overall differential signal power within a cell is around 3.12dB. Besides, the typical measured intra-frequency/inter-frequency RSRP accuracy requirements for FR1 under normal condition is ±4.5dB [4]. Therefore, it is very difficult to distinguish where a UE is located in an NTN cell by pure received signal power based measurement. As such, cell boundary determination can be restricted to some extent or even ineffective sometimes.

Table 1: Path loss in different network scenarios

| Scenarios |  | LEO based NTN |  | GEO based NTN |
| --- | --- | --- | --- | --- |
| Altitude |  | 600 km | 1200 km | 35786 km |
| Beam footprint size |  | 200 km | 200 km | 500 km |
| Path loss | Cell center | 154.05 dB | 160.05 dB | 206.74 dB |
|  | Cell edge | 154.17 dB | 160.09 dB | 206.75 dB |

Observation 2: The near-far effect is not clearly exhibited in NTN compared with terrestrial network.

Furthermore, even though a UE with high sensitivity can determine its relative position with respect to the center of a serving cell by measuring the received signal power, it is still not sufficient for efficient mobility management in NTN. To be specific, as plotted in Figure 2, a UE residing on the dotted circle with a radius of 50km has a same signal strength. For example, the received RSPP of the three points A, B and C is equal.

Figure 2. An example of different points on a circle inside an NTN cell

However, due to the high mobility of LEO based NTN cell, the residual time required for points A, B and C to cross the cell boundary can be quite different. As seen in Figure 2, the NTN cell diameter is 200km and its movement is indicated by the dashed arrow from the right to the left. Assuming that the speed of the LEO satellite is 7.58km/s. Thus, as shown in Table 2, the residual time for a UE residing in A, B and C to approach the cell boundary is different (i.e., 19.74s, 6.58s and 13.16s, respectively), and so does its mobility management requirement (e.g., preparing time before handover).

Table 2: Residual time for different points on a circle to cross cell boundary

| Reference points | Distance to cross cell boundary | Residual time to cross cell boundary |
| --- | --- | --- |
| A | 150 km | 19.74 s |
| B | 50 km | 6.58 s |
| C | 100km | 13.16 s |

Observation 3: In an LEO based NTN cell, UEs with similar received signal strength can have distinct mobility management requirement.

Combining Observations 2 and 3, RAN1 needs to discuss related physical layer measurements besides traditional signal strength to enhance NTN mobility management in high layers.

Observation 4: Besides signal strength related physical layer measurements, other measurements can be exploited to enhance network mobility management.

Furthermore, three reference scenarios (i.e., scenarios A, C2 and D2) from Table 3 were selected for study in R-16. Different scenarios have distinct characteristics and thus may call for different measurement parameters. In particular, for scenarios A, measuring signal strength may not work at all due to the very small difference in the received power. On the other hand, for scenarios C2 and D2, the UE and satellite position measurement can be crucial considering the high satellite mobility, and signal strength based measurement may be taken as a second criteria to further improve measurement efficiency.

Table 3: Reference scenarios (adopted from [5])

|  | Transparent satellite | Regenerative satellite |
| --- | --- | --- |
| GEO based non-terrestrial access network | Scenario A | Scenario B |
| LEO based non-terrestrial access network: steerable beams | Scenario C1 | Scenario D1 |
| LEO based non-terrestrial access network: the beams move with the satellite | Scenario C2 | Scenario D2 |

Observation 5: For different network scenarios (e.g., scenario A, C2 and D2), suitable combination of parameters (e.g., position and signal strength) should be chosen to facilitate efficient cell measurements.

Discussion on PAPR

In this section, we would like to discuss PAPR effects considering the high power amplifier (HPA) model in NTN. As satellite systems are power-limited, the HPA of the satellite transmitter prefers to work at or near the saturation region for obtaining sufficient transmit power [6]. However, CP-OFDM signals with high PAPR will get into the non-linear region of HPA, which will introduce severe in-band non-linear distortions and out-of-band emission. For avoiding this, the HPA requires a large dynamic linear region and large input back off (IBO). Nevertheless, this kind of HPA for satellites is very expensive and has poor power efficiency. Meanwhile, HPA with low power efficiency may not be able to guarantee the sufficient signal-to-noise ratio (SNR) due to the high propagation path loss in NTN.

Figure 3. Relation between PAPR and SNR

The relation between PAPR and SNR is depicted in Figure 3, and it can be concluded that SNR decreases with increasing PAPR when the satellite uses the same HPA. In power-limited NTN scenarios where the received SNR is low in general, the benefits of reducing the PAPR of CP-OFDM signals before they are fed into the HPA are expected to be obvious.

PAPR effects considering the HPA model

There are mainly two kinds of amplifiers in wireless communication systems, i.e., solid state power amplifier (SSPA) and traveling wave tube amplifier (TWTA). SSPAs are usually used in mobile communication systems and TWTAs are generally applied in satellite communication systems when high transmit power is required [7, 8]. Therefore, for evaluating high PAPR effects on transmitted signals in NTN, the TWTA model is used to simulate the CP-OFDM BER performance.

The TWTA can be modelled as [9, 10]

yt=Ar(t)cosω0t+ψt+ϕ[r(t)]	(1)

where r(t)cos[ω0t+ψt] is the input signal of the HPA. yt is the output signal of the HPA. Amplitude modulation to amplitude modulation (AM/AM) conversion Ar(t) and amplitude modulation to phase modulation (AM/PM) conversion ϕ[r(t)] can be expressed as

Ar(t)=αar(1+βar2)                                                           (2)

ϕr(t)=αϕr(1+βϕr2)                                                                  (3)

where α and β are real-valued parameters that can be used to tune the model to fit a specific amplifier. Here we use the values in [9] as an example, i.e., αa=1.9638, βa=0.9945, αϕ=2.5293, βϕ=2.8168.

| Figure 4. Amplitude and phase transfer functions of original TWTA model | Figure 5. Amplitude transfer function of TWTA model with ideal pre-distorter |
| --- | --- |

Figure 4 shows the nonlinear amplitude and phase transfer functions of the original TWTA model without pre-distorter (PD). It can be seen that output signals of the TWTA experience severe amplitude and phase distortions. Hence, distortion compensation for the amplitude and the phase is needed before signals are fed into the TWTA. In Figure 5, the ideal PD [11] is used to compensate for the distortion, and the linear range for the TWTA can be obtained.

Table 4: Simulation parameters

| Assumption | Value |
| --- | --- |
| Channel | AWGN |
| Modulation | QPSK |
| PD | Ideal PD |
| IBO (input backoff) | 5 dB |
| Subcarrier number | 512 |

Figure 6. BER performance for CP-OFDM signals with the TWTA model

For evaluating the effects of HPA non-linear distortions on demodulation performance at the receiver, some simulations are implemented. Simulation parameters are shown in Table 4. Figure 6 shows that, with QPSK modulation and IBO=5 dB, the BER performance of CP-OFDM signals degrades because of non-linear distortions, compared with the CP-OFDM signals which are not going through the TWTA. Meanwhile, the IBO of input signals reduces the TWTA power efficiency.

Observation 6: High PAPR of CP-OFDM signals would reduce the system BER performance because of non-linear distortions, and the power amplifier efficiency because of the large IBO for power-limited satellites in NTN.

Criteria for selection of PAPR reduction techniques

According to the above analysis, it can be seen that the satellite with the CP-OFDM waveform may need to apply some PAPR reduction techniques to improve its power efficiency and relieve non-linear distortions. Here we list some criteria for selection of PAPR reduction techniques:

- PAPR reduction capability: This is clearly the most important factor for choosing a PAPR reduction technique. The chosen one should be able to reduce the CP-OFDM signals PAPR to an acceptable level.

- BER performance: After applying some PAPR reduction techniques, the original constellation signals may be changed or the signal average transmit power may be increased [12], e.g., amplitude clipping, which makes decoding more difficulty. This is also an important factor, which has to be considered.

- Data rate: Some PAPR reduction techniques need to transmit “redundant signals” to the receiver, e.g., side information of partial transmit sequence (PTS) [13] and selective mapping (SLM) [14], which will reduce the system data rate.

- Implementation complexity: Another important factor, which needs to be considered for PAPR reduction technique selection, is the implementation complexity. Some techniques, such as PTS and SLM, need to search for an optimal or suboptimal solution to reduce PAPR, which need many iterations. It can be generally concluded that higher implementation complexity introduces better PAPR reduction capacity.

Observation 7: Considering the trade-off between PAPR reduction and other performance, some criteria, e.g. PAPR reduction capability, BER performance, data rate and implementation complexity, should be considered before choosing a specific PAPR reduction technique for NTN.

PAPR reduction techniques

In this section, we intend to give some simulation results of existing typical PAPR reduction techniques, which could be a benchmark for further study. Simple distortion techniques and probabilistic technique for reducing PAPR are compared, including clipping and filtering (CAF) [15], companding [16] and PTS [17]

Amplitude clipping removes the peak parts of CP-OFDM signals which are outside the allowed region. However, the clipping would introduce the out-of-band radiation. This issue can be relieved by filtering. Companding technique reduces the PAPR in another non-linear way compared with clipping, i.e., the transform formula is changed. Here, a simple example in [15] is applied. PTS partitions a data block into a number of disjoint sub-blocks. Then they are weighted by phase rotation factors separately after the IDFT. The optimal rotation factor combination is selected for minimizing the PAPR of resulting CP-OFDM signals. Meanwhile, the side information of the optimal rotation factors needs to be transmitted to the receiver for decoding the transmitted signals correctly, which may decrease the system data rate.

Figure 7 shows PAPR performance comparisons of these different PAPR reduction techniques. The parameters of these reduction techniques are selected for obtaining similar PAPR performance, which are listed in Table 2, so that we can compare their BER performance fairly.

Figure 7. PAPR performance of CP-OFDM signals with different PAPR reduction techniques

Table 5: Simulation parameters for PAPR reduction techniques

| Assumption | Value |
| --- | --- |
| Channel | AWGN |
| Modulation | QPSK |
| Subcarrier number | 256 |
| Clipping ratio (CAF) | 3.8 |
| k1 (Companding) | 2 |
| V (PTS) | 4 |
| Rotation factors (PTS) | {±1, ±j} |

Figure 8. BER performance of CP-OFDM signals with different PAPR reduction techniques

Table 6: Comparisons of PAPR reduction techniques

| PAPR reduction technique | BER increase | Data rate loss | Implementation complexity |
| --- | --- | --- | --- |
| CAF | Yes | No | Low |
| Companding | Yes | No | Low |
| PTS | No | Yes | High |

From Figure 8, it can be seen that the distortion techniques (CAF and companding) reduce the CP-OFDM BER performance, and the PTS does not change that. It can be concluded that these reduction techniques can improve CP-OFDM signals PAPR performance and the HPA power efficiency with some other performance loss. In Table 6 we summarize these PAPR reduction techniques according to the criteria of section 3.2.

Observation 8: The HPA power efficiency can be improved with PAPR reduction techniques at the cost of some other performance loss for NTN.

On the whole, the high PAPR problem of CP-OFDM relates with hardware costs, the acceptable power efficiency, nonlinear distortion, the link budget, and so on. For example, high PAPR requires HPA and digital-to-analog converter (DAC) with the large linear range1, which are indeed costly. High PAPR also demands larger solar panels and battery capacity to support the lower power efficiency of hardware, yet they are high-priced.

Therefore, if these extra costs need to be avoided, algorithms for PAPR reduction should be preferred. Considering implementation complexities of PAPR reduction algorithms, devices on the earth (in transparent NTN scenarios) can be assumed powerful and are able to bear the complexities. Regenerative satellites have to bear all the complexity on board and thus low complexity PAPR reduction methods may be preferred. It is noted that PAPR reduction algorithms should be targeted to reach the balance point between hardware costs and signal processing complexities.

Finally, if the PAPR problem is critical in NTN, PAPR reduction algorithms can be utilized. Meanwhile, it is worth pointing out that NR is being adapted for NTN and the one which has smaller changes to NR radio protocols can be considered prior to other PAPR reduction algorithms.

Observation 9: The PAPR reduction algorithm, which has smaller changes to 5G NR radio protocols, can be considered prior to others.

Discussion on link budget

In this section, link budget under different scenarios are evaluated as NTNs are quite different compared with terrestrial network. The link budget or carrier-to-noise-and-interference ratio (CNIR) of transmission link between transmitter and receiver can be derived by carrier-to-noise ratio (CNR) and carrier-to-interference ratio (CIR) as below [18]

|  | CNIR dB=-10log1010-0.1CNR [dB]+10-0.1CIR [dB] | (4) |
| --- | --- | --- |

where CNR calculation is agreed in RAN1#98 [18] as follows

|  | CNR dB=EIRP [dBW]+G/T[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL dB-PLAD [dB]-B [dBHZ] | (5) |
| --- | --- | --- |

where                                          EIRP dBW=PT dBW-LC dB+GT [dBi]                                (6)

GTdB/K=GR dBi-Nf [dB]-10log10T0 [K]+Ta K-T0 [K]10-0.1Nf [dB]     (7)

PLFS(fc,d) =32.45+20log10fc+20log10(d)[2]                          (8)

PLA (α,f)=Azenith(f)/sin⁡(α)                                         (9)

GR dBi=GR,edBi+10log10NR,a-Lp[dB],  &array antenna10log10η∙π2∙D[m]2λ[m]2,  &parabolic antenna                       (10)

the parameters in the above equations are

- EIRP: effective isotropic radiated power

- G/T: figure of merit of the receiver

- k: Boltzmann constant (-228.6 dBW/K/Hz)

- PLFS: free space path loss

- PLA : atmospheric path loss due to gases and rain fades

- PLAD : additional loss

- PLSM : shadowing margin

- PLSL : scintillation loss

- B: channel bandwidth

- PT: antenna transmit power,

- LC: cable loss,

- GT: transmit antenna gain

- GR: receive antenna gain

- Nf: noise figure

- T0: ambient temperature (usually 290K)

- Ta: antenna temperature (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain).

- fc: carrier frequency in GHz

- d: distance between transmitter and receiver

- Azenith(f): zenith attenuation of frequency f

- α: elevation angle

- GT,e: transmit antenna element gain

- NT,a : number of transmit antenna elements

- Lp: polarization loss

- D: equivalent antenna diameter

- λ: wavelength

Parameters for link budget analysis

The system level simulation parameters of satellite and UE agreed by RAN1#97 [19] in Table X.1, Table X.2 and Table X.3 and parameter configuration agreed by RAN1#98 [18] in Table 6.1.3.2-1 can be referred for link budget analysis. Specifically, G/T for DL can be calculated by equation (7) with GR and Nf in Table X.3. EIRP for UL can be evaluated by equation (6), EIPR for DL can be calculated by channel bandwidth (system bandwidth/frequency reuse factor) and the EIRP density in Table X.1 and Table X.2. UE characteristic can be referred in [18]. The distance between satellite and UE is evaluated by the geometry of Figure 5.3.4.4-1 in [2]. Zenith attenuation for calculation of PLA is illustrated in Figure 6 in [20] from 1 to 350 GHz. Note that, at frequencies below 10 GHz, PLA may normally be neglected. The following Table 7 presents the link budget under different communication scenarios. In each scenario, CIR is the average CIR collected for 1000 UEs located in the central beam of 19-beamlayout and interferences from beams that have partial or full coverage outside the earth are neglected according to the agreement in [18]. For some cases (S-band with GEO of set 1, Ka-band and S-band with GEO of set 2), partial coverage of central beams are outside the earth, the results are calculated based on the coverage on the earth.

Table 7: Link budget results

| Case | Satellite orbit | Satellite parameter set | Central beam elevation | Terminal | Frequency Band | Frequency/ Polarization Reuse | UL/DL | CNR [dB] | CIR [dB] | CINR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | Set 1 | 10 deg | VSAT | Ka-band | Option 1 | UL | 3.24 | 10.70 | 2.52 |
|  |  |  |  |  |  |  | DL | 4.84 | 2.35 | 0.41 |
| 2 | GEO | Set 1 | 10 deg | VSAT | Ka-band | Option 2 | UL | 8.01 | 16.90 | 7.48 |
|  |  |  |  |  |  |  | DL | 4.84 | 11.69 | 4.02 |
| 3* | GEO | Set 1 | 10 deg | VSAT | Ka-band | Option 3 | UL | 6.25 | 19.53 | 6.05 |
|  |  |  |  |  |  |  | DL | 4.84 | 5.76 | 2.26 |
| 4* | GEO | Set 1 | 10 deg | Handheld | S-band | Option 1 | UL | -10.80 | 5.76 | -10.89 |
|  |  |  |  |  |  |  | DL | 0.14 | 7.34 | -0.62 |
| 5* | GEO | Set 1 | 10 deg | Handheld | S-band | Option 2 | UL | -10.80 | 12.24 | -10.82 |
|  |  |  |  |  |  |  | DL | 0.14 | 12.09 | -0.13 |
| 6 | LEO-600 | Set 1 | 30 deg | VSAT | Ka-band | Option 1 | UL | 25.15 | 6.60 | 6.54 |
|  |  |  |  |  |  |  | DL | 5.75 | 1.73 | 0.28 |
| 7 | LEO-600 | Set 1 | 30 deg | VSAT | Ka-band | Option 2 | UL | 29.92 | 14.80 | 14.67 |
|  |  |  |  |  |  |  | DL | 5.75 | 10.63 | 4.53 |
| 8* | LEO-600 | Set 1 | 30 deg | VSAT | Ka-band | Option 3 | UL | 28.16 | 17.15 | 16.82 |
|  |  |  |  |  |  |  | DL | 5.75 | 12.27 | 4.87 |
| 9 | LEO-600 | Set 1 | 30 deg | Handheld | S-band | Option 1 | UL | 2.84 | 5.76 | 1.05 |
|  |  |  |  |  |  |  | DL | 6.68 | 1.88 | 0.64 |
| 10 | LEO-600 | Set 1 | 30 deg | Handheld | S-band | Option 2 | UL | 2.84 | 12.24 | 2.37 |
|  |  |  |  |  |  |  | DL | 6.68 | 11.00 | 5.31 |
| 11* | LEO-1200 | Set 1 | 30 deg | VSAT | Ka-band | Option 1 | UL | 19.76 | 6.51 | 6.31 |
|  |  |  |  |  |  |  | DL | 6.36 | 1.38 | 0.18 |
| 12* | LEO-1200 | Set 1 | 30 deg | VSAT | Ka-band | Option 2 | UL | 24.53 | 14.88 | 14.43 |
|  |  |  |  |  |  |  | DL | 6.36 | 10.10 | 4.83 |
| 13* | LEO-1200 | Set 1 | 30 deg | VSAT | Ka-band | Option 3 | UL | 22.77 | 17.35 | 16.25 |
|  |  |  |  |  |  |  | DL | 6.36 | 12.19 | 5.35 |
| 14* | LEO-1200 | Set 1 | 30 deg | Handheld | S-band | Option 1 | UL | -2.55 | 2.73 | -3.68 |
|  |  |  |  |  |  |  | DL | 7.29 | 1.66 | 0.61 |
| 15* | LEO-1200 | Set 1 | 30 deg | Handheld | S-band | Option 2 | UL | -2.55 | 12.93 | -2.67 |
|  |  |  |  |  |  |  | DL | 7.29 | 10.64 | 5.64 |
| 16** | GEO | Set 2 | 10 deg | VSAT | Ka-band | Option 1 | UL | -4.76 | 10.70 | -4.88 |
|  |  |  |  |  |  |  | DL | -3.16 | 7.08 | -3.56 |
| 17** | GEO | Set 2 | 10 deg | VSAT | Ka-band | Option 2 | UL | 0.01 | 16.90 | -0.08 |
|  |  |  |  |  |  |  | DL | -3.16 | 12.08 | -3.29 |
| 18** | GEO | Set 2 | 10 deg | VSAT | Ka-band | Option 3 | UL | -1.75 | 19.53 | -1.78 |
|  |  |  |  |  |  |  | DL | -3.16 | 15.46 | -3.22 |
| 19** | GEO | Set 2 | 10 deg | Handheld | S-band | Option 1 | UL | -15.80 | 5.76 | -15.83 |
|  |  |  |  |  |  |  | DL | -5.36 | 5.76 | -5.68 |
| 20** | GEO | Set 2 | 10 deg | Handheld | S-band | Option 2 | UL | -15.80 | 12.24 | -15.81 |
|  |  |  |  |  |  |  | DL | -5.36 | 12.23 | -5.44 |
| 21** | LEO-600 | Set 2 | 30 deg | VSAT | Ka-band | Option 1 | UL | 17.15 | 6.60 | 6.24 |
|  |  |  |  |  |  |  | DL | -2.25 | 1.93 | -3.66 |
| 22** | LEO-600 | Set 2 | 30 deg | VSAT | Ka-band | Option 2 | UL | 21.92 | 14.80 | 14.03 |
|  |  |  |  |  |  |  | DL | -2.25 | 10.94 | -2.46 |
| 23** | LEO-600 | Set 2 | 30 deg | VSAT | Ka-band | Option 3 | UL | 20.16 | 17.15 | 15.39 |
|  |  |  |  |  |  |  | DL | -2.25 | 13.01 | -2.38 |
| 24** | LEO-600 | Set 2 | 30 deg | Handheld | S-band | Option 1 | UL | -3.16 | 2.73 | -4.16 |
|  |  |  |  |  |  |  | DL | 0.68 | 2.69 | -1.44 |
| 25** | LEO-600 | Set 2 | 30 deg | Handheld | S-band | Option 2 | UL | -3.16 | 12.93 | -3.27 |
|  |  |  |  |  |  |  | DL | 0.68 | 12.97 | 0.43 |
| 26** | LEO-1200 | Set 2 | 30 deg | VSAT | Ka-band | Option 1 | UL | 11.76 | 6.51 | 5.38 |
|  |  |  |  |  |  |  | DL | -1.64 | 1.57 | -3.34 |
| 27** | LEO-1200 | Set 2 | 30 deg | VSAT | Ka-band | Option 2 | UL | 16.53 | 14.88 | 12.62 |
|  |  |  |  |  |  |  | DL | -1.64 | 10.57 | -1.89 |
| 28** | LEO-1200 | Set 2 | 30 deg | VSAT | Ka-band | Option 3 | UL | 16.53 | 17.35 | 13.91 |
|  |  |  |  |  |  |  | DL | -1.64 | 10.57 | -1.89 |
| 29** | LEO-1200 | Set 2 | 30 deg | Handheld | S-band | Option 1 | UL | -8.55 | 5.76 | -8.71 |
|  |  |  |  |  |  |  | DL | 1.29 | 3.17 | -0.88 |
| 30** | LEO-1200 | Set 2 | 30 deg | Handheld | S-band | Option 2 | UL | -8.55 | 12.24 | -8.59 |
|  |  |  |  |  |  |  | DL | 1.29 | 12.81 | 0.99 |
| Note: channel bandwidth B is 360kHz (UL in S-band), system bandwidth/frequency reuse factor/10 (UL in Ka-band), system bandwidth/frequency reuse factor (DL in S and Ka-band) |  |  |  |  |  |  |  |  |  |  |

Observation 10: For GEO (set 1 in S-band, set 2 in S and Ka-band), partial coverage of central beams are outside the earth with 10°elevation.

Observation 11: UL transmission of handheld UE is challenging with high satellite orbit.

Observation 12: VSAT has better link-budgets in general (most are above 0dB and half are above 10dB)

Proposal 1: Capture the link budget analysis results in this contribution into TR.

Conclusion

Based on the discussion in the previous sections, we made the following observations:

Observation 1: The cell measurement for NR mobility management (e.g., handover) is executed based on signal strength related parameters.

Observation 2: The near-far effect is not clearly exhibited in NTN compared with terrestrial network.

Observation 3: In an LEO based NTN cell, UEs with similar received signal strength can have distinct mobility management requirement.

Observation 4: Besides signal strength related physical layer measurements, other measurements can be exploited to enhance network mobility management.

Observation 5: For different network scenarios (e.g., scenario A, C2 and D2), suitable combination of parameters (e.g., position and signal strength) should be chosen to facilitate efficient cell measurements.

Observation 6: High PAPR of CP-OFDM signals would reduce the system BER performance because of non-linear distortions, and the power amplifier efficiency because of the large IBO for power-limited satellites in NTN.

Observation 7: Considering the trade-off between PAPR reduction and other performance, some criteria, e.g. PAPR reduction capability, BER performance, data rate and implementation complexity, should be considered before choosing a specific PAPR reduction technique for NTN.

Observation 8: The HPA power efficiency can be improved with PAPR reduction techniques at the cost of some other performance loss for NTN.

Observation 9: The PAPR reduction algorithm, which has smaller changes to 5G NR radio protocols, can be considered prior to others.

Observation 10: For GEO (set 1 in S-band, set 2 in S and Ka-band), partial coverage of central beams are outside the earth with 10°elevation.

Observation 11: UL transmission of handheld UE is challenging with high satellite orbit.

Observation 12: VSAT has better link-budgets in general (most are above 0dB and half are above 10dB)

Proposal 1: Capture the link budget analysis results in this contribution into TR.

- References

P-181370, “Study on solutions evaluation for NR to support Non-Terrestrial Network”, Thales, RAN#80, La Jolla, USA, June 11-14, 2018

TR 38.811 V15.0.0, “Study on New Radio (NR) to support non terrestrial networks”.

TS 38.215 V15.2.0, “Physical layer measurements”.

TS 38.331 V15.3.0, “Radio Resource Control (RRC) Protocol specification”.

TS 38.133 V15.4.0, “Requirements for support of radio resource management”.

K. Kang, S. Kim, D, Ahn, et al. “Efficient PAPR reduction scheme for satellite MC-CDMA systems”, IEEE Proceedings-Communications, 2005, 152(5): 697-703.

B. M. Lee and R. J. P. de Figueiredo, “Adaptive predistorters forff linearization of high-power amplifiers in OFDM wireless communications”, Circuits, Systems and Signal Processing, 2006, 25 (1): 59-80.

J. M. Weekley and B. J. Mangus, “TWTA versus SSPA: A comparison of on-orbit reliability data”, IEEE Transactions on Electron Devices, 2005, 52(5): 650-652.

A. A. Saleh, “Frequency-independent and frequency-dependent nonlinear models of TWT amplifiers”, IEEE Transactions on communications, 1981, 29(11): 1715-1720.

3GPP TR 38.803 V14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects (Release 14)”, September 2017

A. Brajal and A. Chouly, “Compensation of nonlinear distortions for orthogonal multicarrier schemes using predistortion”, 1994 IEEE GLOBECOM. Communications: The Global Bridge, San Francisco, CA, USA, 1994, pp. 1909-1914

T. Wattanasuwakull and W. Benjapolakul, “PAPR Reduction for OFDM Transmission by using a method of Tone Reservation and Tone Injection”, 2005 5th International Conference on Information Communications & Signal Processing, Bangkok, 2005, pp. 273-277.

S. H. Muller and J. B. Huber, “OFDM with reduced peak-to-average power ratio by optimum combination of partial transmit sequences”, Electronics Letters, 1997, 33(5): 368-369.

R. W. Bauml, R. F. H. Fischer, and J. B. Huber, “Reducing the peak-to-average power ratio of multicarrier by selected mapping”, Electronics Letters, 1996, 32(22): 2056-2057.

X. Li and L. J. Cimini, “Effects of clipping and filtering on the performance of OFDM”, 1997 IEEE 47th Vehicular Technology Conference. Technology in Motion, Phoenix, AZ, USA, 1997, pp. 1634-1638.

R1-1608828, “Overview of PAPR reduction techniques”, Huawei, HiSilicon, RAN1#86bis, Lisbon, Portugal, October 10th – 14th, 2016

S. H. Muller and J. B. Huber, “OFDM with reduced peak-to-average power ratio by optimum combination of partial transmit sequences”, Electronics Letters, 1997, 33(5): 368-369.

“Draft Report of 3GPP TSG RAN WG1 #98 v0.1.0”, Prague, Czech, August 26th – 30th, 2019

“Draft Report of 3GPP TSG RAN WG1 #97 v0.3.0”, Reno, USA, May 13th-17th, 2019

Recommendation ITU-R P.676-11, “Attenuation by atmospheric gases”, Sep. 2016.