3GPP TSG RAN WG1 Meeting #98bis	R1-1910743

Chongqing, China, October 14th – 20th, 2019

Agenda item:	7.2.1.1

Source: 	SONY

Title:	Channel Structure for Two-Step RACH

Document for:	Discussion and Decision

# Introduction

The work item on 2-step RACH for NR aims to specify 2-step RACH as general MAC procedure covering both physical layer and higher layer aspects [1]. In RAN1#98 meeting the following agreements were made [2]:

Agreements:

- The following parameters are further defined per msgA PUSCH configuration

  - Common parameters for both option 1 (separate configuration) and option 2 (relative location)

    - Number of slots (in active UL BWP numerology) containing one or multiple POs, each slot has the same time domain resource allocation

    - Number of time domain POs in each slot

      - POs including guard period are contiguous in time domain within a slot

    - SLIV-based, indicating the start symbol of the first PO in each slot, and the number of occupied symbols of each PO in time domain

      - the number of occupied symbols excludes the guard period

    - PUSCH mapping type A or B

    - Configurable guard period, value range in the unit of symbols FFS

    - Frequency start point with respect to the first PRB of the active UL BWP

      - FFS: configurable PRB-level guard band, up to 1 PRB

- At least support same configuration periodicity for msgA PRACH and PUSCH

    - Single time offset with respect to the start of each PRACH slot, counted as the number of slots (based on the numerology of active UL BWP)

      - Note: The symbol level offset is implied in SLIV-based indication

    - FFS how to handle the overlapping between POs

    - FFS whether and how to support different configuration periodicities

- For the definition of PRU, support both DMRS ports and DMRS sequences at least for CP-OFDM

  - More than 1 DMRS sequence can be configured, FFS the value

  - FFS whether/how to support multiple sequences for DFT-s-OFDM

  - The conditions under which only DM-RS ports are to be specified. FFS details

- Confirm the working assumption that both one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit (PRU) are supported

  - Configurable number of preambles (including one or multiple) mapped to one PRU, explicitly or implicitly

  - FFS 1-to-multiple mapping

- For RRC_INACTIVE/IDLE state, at least support up to two msgA PUSCH configurations for Rel.16

  - Using different preamble groups for the indications of different configurations in case of two configurations

  - Support of more than two configurations is not precluded, and if supported FFS the following mechanisms for the indications of different configurations

    - Alt.1: Using different preamble groups

    - Alt 2: Using different preamble groups and/or RO partitioning

    - Alt.3: Using UCI based indication

    - Alt. 4: Using different DMRS ports/sequences

- At least up to two msgA PUSCH configurations are supported for RRC_CONNECTED state for Rel.16

  - FFS details

- FFS whether the MsgA PUSCH configurations are the same among different RRC states (IDLE, INACTIVE, CONNCETED)

- FFS the rule or BS signaling the criterion for the UE’s selection of msgA PUSCH configuration

In this contribution, we provide our views on the 2-step RACH procedure particularly on the channel structure for the PUSCH in msgA of two-step RACH.

# Discussion

The 2-step RACH operation in general is illustrated in Figure 1. MsgA contains both preamble and information/data. Preamble is transmitted in PRACH and the information/data is transmitted in PUSCH. The information in MsgA can be the legacy Msg.3 of 4-step RACH. The size can be in the range of less than 100 bits (e.g. 72 bits). Furthermore, it may also contain small data transmission from upper layer (e.g. application layer). It may result in larger size, such as around 1000 bits. In response to MsgA transmission, the gNB transmits MsgB which may contain RAR, TA information, and RRC response. This can essentially be similar to the combination of legacy Msg2 and Msg4.

Figure 1: 2 step Random Access Procedure

According to the WID [1], 2-step RACH is applied for RRC_INACTIVE, RRC_CONNECTED and RRC_IDLE state. It implies that the 2-step RACH shall be designed to be operated in all RRC states. The UE must know the 2-step RACH configuration, particularly the MsgA configuration, prior to the 2-step RACH operation. The configuration of msgA may include information regarding data size, resource allocation, supported modulation and coding scheme (MCS), and UEs data multiplexing configuration.  The data size depends on the payload/information that can be carried in MsgA which is mostly defined in RAN2. For 2-step RACH at the initial access stage the msgA configuration can be sent to the UE in system information. This would particularly be beneficial if all UEs in a cell are expected to use the same configuration. In case the UE has been configured from RRC active to RRC inactive mode, the configuration can be sent via dedicated RRC signaling.

  - MsgA PUSCH Configuration

MsgA PUSCH configuration has been discussed in previous meetings and two options related to the PUSCH allocation have been identified:

- Option 1 with separate PUSCH configuration. MsgA PUSCH configuration period may or may not be the same as PRACH configuration period

- Option 2 PUSCH configuration with relative location. MsgA PUSCH configuration period is the PRACH configuration period

We consider 2-step RACH can be beneficial for legacy NR operation in some scenarios and also many specific use-cases, such as NR-unlicensed, Non-Terrestrial Network (NTN), and mMTC. Option 2 (relative location) provides less signaling overhead and also possible low-latency of PUSCH occasion (PO) after the RACH occasion (RO). In addition, as Option 1 is configured separately, the gNB has more freedom and flexibility in allocating PO. We prefer the gNB to have MsgA configuration with Option 2. After the UE transmits the preamble, the UE can immediately transmit PUSCH with a relative timing from the preamble transmission. We expect the UE uses a fix PUSCH transport block size (TBS) for such operation. In addition, some UEs may require to transmit larger transport block sizes or PUSCH transmission with repetition. In such cases, we consider a large PUSCH allocation is required. Furthermore, MsgA PUSCH configuration with Option 1 is applicable in this case and the gNB can provide the allocation regardless of the RACH configuration.

Observation 1: 2-step RACH should aim to support legacy NR operation and also other specific use-cases, such as NR-unlicensed, Non-Terrestrial Network (NTN), and mMTC.

Proposal 1: Support the gNB to have a flexible MsgA PUSCH configuration by supporting both options, such as Option 2 (relative location) is configured as the default mode and Option 1 (separate PUSCH configuration) can be configured to complement Option 2 and/or to be used for specific use-cases.

Proposal 2: Option 2 should be used for the small payload size (e.g. small MCS/TBS)

Proposal 3: Option 1 can be used for the larger payload size (e.g. large MCS/TBS) and PUSCH with repetition.

The WID objectives describe the preamble and MsgA are TDM’ed [1]. We consider there should be a timing relationship (i.e. Option 2 as described above) or a time-gap between preamble and MsgA. The time-gap can be used for example as the guard time period after preamble transmissions, time-alignment to support various SCS, etc. The possibility and benefit of 0 (zero) time-gap between preamble and PUSCH is for further study. The essential part is the gNB can configure the  time-gap between preamble and MsgA PUSCH.

Proposal 4: On option 2: the relative time-offset between preamble and msgA PUSCH can be configured by the gNB (e.g. to consider guard time, time alignment to support various SCS, RACH configuration).

We consider the UE to use MCS for msgA based on the received msgA configuration. In order to simplify the operation, a fixed MCS can be assigned to a UE. It can be a lower MCS to provide robustness and minimize failure of msgA detection. Retransmission of msgA can be relatively costly as the UE has to transmit both preamble and msgA. This type of retransmission was agreed to be supported. However, the occurrence of such retransmission should be minimized. Another option is a UE is configured with a limited set of MCS. The UE can dynamically select the MCS, for example, based on the payload size or pathloss. This may require the gNB to blindly-detect the MCS of the received data. In order to avoid blind-detection mechanism, the MCS could also be associated to the preamble. However, this may cause a limitation for the set of preamble index/resource which the UE can choose, which tends to increase PRACH preamble collision. Another option could be a UE to dynamically signal the MCS on reserved resources within the PUSCH (i.e. signaled with msgA).

Proposal 5: The MCS of MsgA depends on the supported TBS of MsgA and QPSK is at least supported.

A UE can be configured with multiple 2-step RACH configurations. For RRC_INACTIVE/IDLE state, at least support up to two msgA PUSCH configurations for Rel.16. For RRC_CONNECTED state, it can support more than two msA PUSCH configuration. Considering 2-step RACH can be targeted to support various use-cases and scenarios, all four alternatives to indicate the selected 2-step RACH configuration are supported.

Proposal 6: The following mechanisms /alternatives for the indications of different 2-step RACH configurations are supported considering 2-step RACH can be designed to support various use-cases/scenarios.

- Alt.1: Using different preamble groups

- Alt 2: Using different preamble groups and/or RO partitioning

- Alt.3: Using UCI based indication

- Alt. 4: Using different DMRS ports/sequences

  - msgA PUSCH Scrambling

In RAN1#97 meeting [2], msgA PUSCH scrambling was discussed and it is further discussed for the detail of RNTI and the inclusion of DMRS index. For the detail of RNTI, for a UE with RRC_CONNECTED, C-RNTI should be simply used because C-RNTI has been already configured to connected UEs. For a UE with RRC_IDLE/RRC_INACTIVE state, since TC-RNTI/C-RNTI is not known, another scramble ID for PUSCH instead of TC-RNTI/C-RNTI has to be considered.

Since 2-step RACH UE anyway generate RA-RNTI, a simple way is to reuse the RA-RNTI for msgA PUSCH scrambling. For the inclusion of DMRS index, given that multiple PUSCH could occupy one PUSCH occasion, it would be desirable that each of the PUSCHs in any PUSCH occasion have different scramble IDs. If one-to-multiple mapping between preamble and PRU is supported, DMRS index has to be included into the c_init to distinguish multiple PUSCHs in the same PUSCH occasion, since PUSCH could not have different scramble IDs only using RNTI, preamble index, and n_ID.

Proposal 7: For a UE in RRC_IDLE or RRC_INACTIVE state, the RNTI used for msgA PUSCH scrambling should be RA-RNTI. For a UE in RRC_CONNECTED state, the RNTI used for msgA PUSCH scrambling should be C-RNTI.

Proposal 8: DMRS index should be included in the c_init for msgA PUSCH scrambling if one-to-multiple mapping between the preamble and PRUs is supported.

# Conclusion

In this contribution, we discussed the msgA channel structure of 2-step RACH procedure and conclude on the following observation and proposals:

Observation 1: 2-step RACH should aim to support legacy NR operation and also other specific use-cases, such as NR-unlicensed, Non-Terrestrial Network (NTN), and mMTC.

Proposal 1: Support the gNB to have a flexible MsgA PUSCH configuration by supporting both options, such as Option 2 (relative location) is configured as the default mode and Option 1 (separate PUSCH configuration) can be configured to complement Option 2 and/or to be used for specific use-cases.

Proposal 2: Option 2 should be used for the small payload size (e.g. small MCS/TBS)

Proposal 3: Option 1 can be used for the larger payload size (e.g. large MCS/TBS) and PUSCH with repetition.

Proposal 4: On option 2: the relative time-offset between preamble and msgA PUSCH can be configured by the gNB (e.g. to consider guard time, time alignment to support various SCS, RACH configuration).

Proposal 5: The MCS of MsgA depends on the supported TBS of MsgA and QPSK is at least supported.

Proposal 6: The following mechanisms /alternatives for the indications of different 2-step RACH configurations are supported considering 2-step RACH can be designed to support various use-cases/scenarios.

- Alt.1: Using different preamble groups

- Alt 2: Using different preamble groups and/or RO partitioning

- Alt.3: Using UCI based indication

- Alt. 4: Using different DMRS ports/sequences

Proposal 7: For a UE in RRC_IDLE or RRC_INACTIVE state, RNTI used for msgA PUSCH scrambling should be RA-RNTI. For a UE in RRC_CONNECTED state, RNTI used for msgA PUSCH scrambling should be C-RNTI.

Proposal 8: DMRS index should be included into the c_init for msgA PUSCH scrambling if one-to-multiple mapping between the preamble and PRUs is supported.

# References

- RP-190711, “Revised work item: 2-step RACH for NR”, ZTE, Sanechips, Shenzhen, China, March, 2019.

- Chairman notes on RAN1#98, ETSI MCC