3GPP TSG RAN WG1 #98bis		R1-1910990

Chongqing, China, 14th – 20th October 2019

Source:	Panasonic

Title: 	Discussion on UCI enhancement for URLLC

Agenda Item:		7.2.6.2

Document for:	Discussion

Introduction

A work item on physical layer enhancements for NR URLLC was approved [1]. One of objectives of this work item is specification of UCI enhancements, which includes

- More than one PUCCH for HARQ-ACK transmission within a slot

- At least two HARQ-ACK codebooks simultaneously constructed, intended for supporting different service types for a UE

In addition, update of the work item on support of NR Industrial IoT was approved [2]. This work item includes the objective for intra-UE prioritization related to control channel enhancement such as

- Address UL data/control and control/control resource collision by (L1 multiplexing of services of different priority is out of scope):

  - specifying a method to address resource collision between SR associating to high-priority traffic and uplink data of lower-priority traffic for the cases where MAC determines the prioritization [RAN2].

  - specifying prioritization behaviour among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH [RAN1, RAN2].

In this document, we provide our view on UCI enhancement and intra-UE prioritization related to control channel for URLLC. The agreements related to these topics made in previous RAN1 meetings are summarized in Appendix.

This document is update of R1-1908799 [3].

Sub-slot-based HARQ-ACK feedback

In sub-slot-based HARQ-ACK feedback, Rel.15 PUCCH resource overriding procedures is applied in unit of sub-slot is agreed. For two sub-slot configurations (“2-symbol*7” and “7-symbol*2”) which was agreed in RAN1#98, a single configuration for PUCCH resource following Rel.15 applicable for all the sub-slots in a slot is supported. One of remaining issues is whether additionally to support that PUCCH resource configuration can be different for different sub-slots. Whether PUCCH across sub-slot boundary or not is another remaining issue. These two remaining issues should be considered together.

On the issue of PUCCH across sub-slot boundary, assuming the same PUCCH resource overriding procedure with Rel.15 is used, PUCCH transmission is confined within a sub-slot would be simpler. In addition, if PUCCH resource across sub-slot boundary is not allowed, there is no need to additionally support that PUCCH resource configuration can be different for different sub-slots at least for the sub-slot configurations with “2-symbol*7” and “7-symbol*2”. If other sub-slot configurations, such as “3/4-symbol*4” is also supported, different PUCCH resource sets for different sub-slots within a slot could provide more flexibility for unequal size of sub-slot length within a slot. On the other hand, Rel.15 PUCCH resource set framework can still work well. One of concerns of limiting PUCCH transmission confined within a sub-slot is coverage. If coverage is required, network can configure the sub-slot length such that one sub-slot has long PUCCH for better coverage. Alternatively, if coverage is required with keeping sub-slot configuration, multiple sub-slot transmission could be considered by applying Rel.15 multi-slot PUCCH transmission (repetition) procedure.

Although our first preference is that PUCCH transmission across the sub-slot boundary is not allowed, to improve the coverage by multiple sub-slot repetition would require much standardization effort at this stage. Therefore, to support that PUCCH resource across sub-slot boundary is also acceptable to us by considering following points. First, PUCCH transmission should not across slot boundary even if PUCCH resource across sub-slot boundary is allowed. Some consideration is necessary with respect to PUCCH resource set or PUCCH resource allocation at least for the last sub-slot in a slot. Similar to the case to support unequal sub-slot length within a slot, allow to configure additional PUCCH resource set(s) for a specific sub-slot could provide more flexibility. On the other hand, Rel.15 PUCCH resource set framework with network implementation of PUCCH resource configuration/indication can also work with slightly less flexibility. Another point for consideration, if PUCCH across sub-slot boundary is allowed, is how to handle resource overlap when at least one PUCCH resource is across sub-slot boundary. In our view, UCI multiplexing decision in each sub-slot should be independent, i.e., UCI multiplexing decision in the sub-slot n is not required to consider the UCI multiplexing situation in the sub-slot n+1 or sub-slot n-1. One of possibilities is if some PUCCH is continued from sub-slot n-1, the UCI multiplexing decision is “drop the new PUCCH starting from sub-slot n” and “continue the PUCCH from sub-slot n-1”.

It is also not concluded whether Type I (semi-static) HARQ codebook is supported or not needed/deprioritized for Rel.16 URLLC. One of the motivation to introduce semi-static HARQ codebook is CA operation. CA use case might be important for Rel.16 URLLC especially considering intra-UE eMBB/URLLC multiplexing. Multi-carrier intra-UE eMBB/URLLC multiplexing can be much simpler than single-carrier operation. However, for sub-slot-based operation, semi-static codebook could have larger overhead in the uplink. In addition, CA can be still supported by dynamic HARQ codebook. Potential merit of semi-static HARQ codebook would be no need of tight interworking in scheduler of multiple carriers, while even in dynamic HARQ-ACK codebook, the operation similar to semi-static codebook may be possible in case maximum total DAI value is indicated but actually scheduled CC is less than total DCI value. Therefore, our preference is deprioritizing the specification optimized for semi-static codebook.

Proposal 1: For PUCCH transmission in sub-slot-based HARQ-ACK feedback procedure, down select from following options.

- Option 1: PUCCH across sub-slot boundary is not allowed.

  - Support only single configuration for PUCCH resource for all the sub-slots in a slot.

  - Multiple sub-slot transmission is considered for coverage improvement purpose, if necessary.

- Option 2: PUCCH across sub-slot boundary is allowed.

  - Support only single configuration for PUCCH resource for all the sub-slots in a slot. Network implementation handle the issue on slot-boundary.

  - UCI multiplexing in each sub-slot should be independent.

Proposal 2: Semi-static codebook is not defined for Rel.16 URLLC.

Construction of at least two HARQ-ACK codebooks simultaneously intended for supporting different service types for a UE

In Rel.15 NR specification, HARQ-ACK codebook is determined without any consideration of latency and/or reliability requirement and these HARQ-ACK bits are multiplexed in one PUCCH.

In Rel.16 URLLC, in order to ensure latency and/or reliability requirement for URLLC, different handling of HARQ-ACK with different latency and/or reliability should be considered and then, in RAN1 agreed that at least two HARQ-ACK codebooks can be simultaneously constructed, intended for supporting different service types for a UE. Although we see different levels of URLLC and eMBB services simultaneously, we don’t see the need of more than 2 levels of differentiation of HARQ-ACK codebooks handling. Two levels can be used both for URLLC or can be used for URLLC and eMBB. It would not prevent to use two levels of eMBB from the specification. In addition, even in the operation with single HARQ-ACK codebook construction, multiple PUCCH transmissions with HARQ-ACK within a slot can be possible as discussed in Section 2. Therefore, up to 2 HARQ-ACK codebooks, which is always intended for different services types, are sufficient and should be supported. This also can simply the intra-UE Tx prioritization discussion and operation.

Proposal 3: In Rel.16, up to two HARQ-ACK codebooks, which is always intended for different services types are sufficient and should be supported.

In order to support two HARQ-ACK codebooks, how to identify a HARQ-ACK codebook should be specified. In addition, it was agreed in RAN1#98 that the PHY identification of HARQ-ACK codebook is also used to determine the priority of the HARQ-ACK codebook for collision handling. In RAN1#96bis, following four options on PHY identifications for identify HARQ-ACK codebook was agreed.

- Option 1: By DCI format/size

- Option 2: By RNTI

- Option 3: By explicit indication in DCI

- Option 4: By CORESET/search space

As discussed in RAN1 email discussion [98-NR-14], for DCI-based dynamic indication options (Option 1, 2, and 3), some concerns were raised that decoding PDCCH may introduce extra processing delay. For the transmission of HARQ-ACK, PDCCH decoding for PDSCH reception and for determination of PUCCH related information is necessary. The determination of HARQ-ACK codebook and/or priority of HARQ-ACK can also be considered as one of the determination of PUCCH related information. Therefore, for the HARQ-ACK codebook/priority determination perspective, we think that the processing time of PDCCH decoding is not the issue for DCI-based dynamic indication. On the other hand, if this priority information is also used for DL out-of-order or overlapped PDSCHs operation, PDCCH processing time can have the impact on UE complexity. More processing time is available as the order of Option 4 > Option 1 > Option 2 > Option 3. Assuming BD/CCE limitation is not so relaxed compared with Rel.15, we don’t think Option 4 is sufficient and option using a dynamic indication (Option 1, 2, or 3) is necessary. Below we provide our view on dynamic indication options.

Option 1 makes additional DCI size which increase the number of BDs. On Option 2, we already used too many RNTIs. If larger number of industrial IoT, RNTIs may or may not be limiting factor. In addition, in our view, even if DCI format or RNTI is different, additional explicit indication is necessary for priority indication. For example, new DCI for URLLC or MCS-RNTI/new-RNTI is not always the highest priority since new DCI or MCS-RNTI/new-RNTI is not limited for the use case of URLLC only. For Option 3, the concern of increased DCI overhead will be marginal as 1-bit indication (high priority and low priority) is sufficient. On the other hand, adding the few field is only applicable for DCI format 1-1 for Rel.15 DCI format and new DCI format for URLLC in USS as there are flexible/configurable DCI size. DCI format 0-1 in CSS needs to be same size as Rel.15 for backward compatibility reason. Therefore, we think Option 3 is also not always a standalone option and the combination with Option 2 or Option 4 for DCI format 1-0 in CSS is necessary. For example, either of the following should be taken for DCI format 1-0 in CSS.

- Option A: DCI format 1-0 in CSS is always lower priority.

- Option B: For DCI format 1-0 in CSS, priority is determined based on RNTI

Proposal 4: A HARQ-ACK codebook/priority can be identified based on following indication(s).

- Explicit indication in DCI format 1-1 and new DCI for URLLC.

- CORESET/search space or RNTI for DCI format 1-0 in CSS.

Intra-UE UL prioritization

SR priority

In RAN1#97, it was agreed that support that SR priority (e.g., high or low priority) is known at PHY layer. Following three options can be considered on how to know the SR priority at PHY layer.

- Option 1: Derived from the logical channel priority

- Option 2: By an explicit indication in SR configuration

- Option 3: Derived from legacy SR configuration (e.g., periodicity, SR-ID)

Based on the MAC specification [4], each SR configuration corresponds to one or more logical channels. Each logical channel may be mapped to zero or one SR configuration, which is configured by RRC. For Option 2 and 3, the relation between SR priority and specific SR configuration are semi-static. Therefore, if an SR configuration corresponds to multiple priorities of logical channels, UE PHY cannot know which logical channel triggered the SR. One of example is the logical channels which has the same latency requirement but different priority (remote driving related information with 10 ms latency/high priority and voice call with 10 ms latency/low priority). In Option 2 and 3, to assign different SR configurations per logical channel is also possibility but each SR configuration should have different SR resources in order to differentiate SR priority at PHY. To allow overlapped resource but different priority is more efficient than always to have non-overlapped resource for different priorities. In Option 1, logical channel priority is given to UE PHY. Even if an SR configuration corresponds to multiple priorities of the logical channels, UE PHY knows the priority of the logical channel. Note that SR priority at PHY layer does not necessarily have the same granularity level as the logical channel priority. For example, logical channel priority has 16 levels but PHY-level priority can have smaller number of levels such as 2 (high or low). In this case, the association rule between PHY-level priority and logical channel priority should be defined in the specification or configured by RRC.

Proposal 5: UE MAC determines SR’s priority based on the priority of logical channel which triggered the SR. UE MAC instruct the PHY to signal SR and the SR’s priority information to PHY.

Proposal 6: The granularity of PHY-level priority can be different from the number of levels of logical channel priority. The association rule between PHY-level priority and logical channel priority is defined in the specification or configured by RRC.

UL-SCH priority

Similar to the case that UE knows the SR’s priority, since a PUSCH (one TB) can contain the data corresponding to multiple logical channels, the determination of UL-SCH priority can be logical channel priority based approach. UE MAC determines UL-SCH priority based on the highest priority of LCH in the MAC PDU. UE MAC delivers the MAC PDU and UL-SCH priority information to PHY. This approach can unify the UE’s priority determination regardless of dynamic grant and configured grant. Similar to SR case, UL-SCH priority at PHY layer does not necessarily have the same granularity level as the logical channel priority. For example, logical channel priority has 16 levels but PHY-level priority can have smaller number of levels such as 2 (high or low) or more levels. In this case, the association rule between PHY-level priority and logical channel priority should be defined in the specification or configured by RRC.

Although we prefer the same scheme between dynamic grant PUSCH and configured grant PUSCH, for dynamic grant PUSCH, gNB can set the priority based on SR/BSR from the UE with taking into the priority account among different UEs and UE PHY is able to know the priority of logical channel from dynamic priority indication in PHY. Therefore, for dynamic grant PUSCH, the determination of UL-SCH priority based on PHY indication can also be considered.

Proposal 7: UE MAC determines UL-SCH priority based on the highest priority of logical channel in the MAC PDU. UE MAC delivers the MAC PDU and the UL-SCH priority information to PHY.

Solutions for each collision scenario

UE PHY should decide prioritization based on the PHY-level priority of UCI/UL-SCH. The granularity of PHY-level priority should be same among SR, HARQ-ACK, and UL-SCH in order to simply compare the priority. On the priority of CSI, it can have always certain fixed level priority. For example, when PHY-level priority has 2 levels (high/low or URLLC/eMBB), CSI priority is always low/eMBB priority could be one possibility.

Our view on solutions for the collision cases between UCIs or channels are summarized in Table 1. Note that we assume that following table shows the PHY behaviour after determining the priority among UCIs or channels. URLLC means the higher priority and eMBB means the lower priority. The collision cases between URLLC UCIs (such as URLLC SR vs URLLC HARQ-ACK) or URLLC channels (such as URLLC SR/HARQ-ACK vs URLLC PUSCH) is assumed to be the collision scenarios with UCIs or channels with the same priority.

Proposal 8: PHY behaviour in Table 1 should be specified for the collision cases between UCIs or channels.

Table 1 PHY behaviour for the collision cases between UCIs or channels

|  | URLLC SR | URLLC HARQ-ACK | CSI | URLLC PUSCH |
| --- | --- | --- | --- | --- |
| URLLC SR |  |  |  |  |
| URLLC HARQ-ACK | Reuse Rel.15 rule as baseline. |  |  |  |
| CSI | Drop CSI | Drop CSI |  |  |
| URLLC PUSCH | Reuse Rel.15 rule, i.e., drop SR | Reuse Rel.15 rule as baseline | Drop CSI |  |
| eMBB SR | Drop eMBB SR | Drop eMBB SR | Reuse Rel.15 rule | Reuse Rel.15 rule, i.e., drop SR |
| eMBB HARQ-ACK | Drop eMBB HARQ-ACK | Drop eMBB HARQ-ACK | Reuse Rel.15 rule | Drop HARQ-ACK |
| eMBB PUSCH | Drop eMBB PUSCH | Drop eMBB PUSCH | Reuse Rel.15 rule | Drop eMBB PUSCH |

For layer 1, multiplexing aspect has been excluded in RAN#85. Then, we think the solution can be simplified such as “reuse Rel.15” or “drop UCIs or channels with lower priority”.

Conclusion

In this contribution, we discussed enhancement for UCI transmission in Rel.16 URLLC and made following proposals.

Sub-slot-based HARQ-ACK feedback procedure

Proposal 1: For PUCCH transmission in sub-slot-based HARQ-ACK feedback procedure, down select from following options.

- Option 1: PUCCH across sub-slot boundary is not allowed.

  - Support only single configuration for PUCCH resource for all the sub-slots in a slot.

  - Multiple sub-slot transmission is considered for coverage improvement purpose, if necessary.

- Option 2: PUCCH across sub-slot boundary is allowed.

  - Support only single configuration for PUCCH resource for all the sub-slots in a slot. Network implementation handle the issue on slot-boundary.

  - UCI multiplexing in each sub-slot should be independent.

Proposal 2: Semi-static codebook is not defined for Rel.16 URLLC.

Construction of at least two HARQ-ACK codebooks intended for supporting different service types for a UE

Proposal 3: In Rel.16, up to two HARQ-ACK codebooks, which is always intended for different services types are sufficient and should be supported.

Proposal 4: A HARQ-ACK codebook/priority can be identified based on following indication(s).

- Explicit indication in DCI format 1-1 and new DCI for URLLC.

- CORESET/search space or RNTI for DCI format 1-0 in CSS.

Intra-UE prioritization

Proposal 5: UE MAC determines SR’s priority based on the priority of logical channel which triggered the SR. UE MAC instruct the PHY to signal SR and the SR’s priority information to PHY.

Proposal 6: The granularity of PHY-level priority can be different from the number of levels of logical channel priority. The association rule between PHY-level priority and logical channel priority is defined in the specification or configured by RRC.

Proposal 7: UE MAC determines UL-SCH priority based on the highest priority of logical channel in the MAC PDU. UE MAC delivers the MAC PDU and the UL-SCH priority information to PHY.

Proposal 8: PHY behaviour in Table 1 should be specified for the collision cases between UCIs or channels.

Table 1 PHY behaviour for the collision cases between UCIs or channels

|  | URLLC SR | URLLC HARQ-ACK | CSI | URLLC PUSCH |
| --- | --- | --- | --- | --- |
| URLLC SR |  |  |  |  |
| URLLC HARQ-ACK | Reuse Rel.15 rule as baseline. |  |  |  |
| CSI | Drop CSI | Drop CSI |  |  |
| URLLC PUSCH | Reuse Rel.15 rule, i.e., drop SR | Reuse Rel.15 rule as baseline | Drop CSI |  |
| eMBB SR | Drop eMBB SR | Drop eMBB SR | Reuse Rel.15 rule | Reuse Rel.15 rule, i.e., drop SR |
| eMBB HARQ-ACK | Drop eMBB HARQ-ACK | Drop eMBB HARQ-ACK | Reuse Rel.15 rule | Drop HARQ-ACK |
| eMBB PUSCH | Drop eMBB PUSCH | Drop eMBB PUSCH | Reuse Rel.15 rule | Drop eMBB PUSCH |

Reference

[1] RP-190726, “New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC),” Huawei, HiSilicon, RAN#83

[2] RP-192288, “Revised WID: Support of NR Industrial Internet of Things (IoT),” Nokia, Nokia Shanghai Bell, RAN#85

[3] R1-1908799, “Discussion on UCI enhancement for URLLC,” Panasonic, RAN1#98

[4] 3GPP TS38.321 V15.6.0, “NR Medium Access Control (MAC) protocol specification (Release 15),” June 2019.

- Appendix:	Previous agreements

RAN1#94

Agreements:

- Study further how to enable more than one PUCCH for HARQ-ACK transmission within a slot.

Agreements:

- Study further whether/how to enable enhanced reporting procedure/feedback for HARQ-ACK.

  - Enhanced HARQ-ACK multiplexing on PUSCH and PUCCH

  - Finer indication for HARQ feedback timing, e.g. symbol-level, half-slot, etc.

    - Note: this may be related to more than one PUCCH for HARQ-ACK tx within a slot

  - Other enablers are not precluded

RAN1#95

Agreement:

- Multiple PUCCHs for HARQ-ACK within a slot should be supported in R16.

RAN1 AH1901

Agreement:

- For a R16 UE, at least two HARQ-ACK codebooks can be simultaneously constructed, intended for supporting different service types for a UE

  - FFS mode details (including procedures when applicable)

  - FFS: How to identify a HARQ-ACK codebook

  - FFS applicability to semi-static HARQ-ACK codebook, or dynamic HARQ-ACK codebook, or both

  - FFS more than 2

  - FFS whether or not CBG configuration is supported for Rel-16 URLLC

RAN1#96

Agreement:

- Rules for the two HARQ-ACK codebooks for supporting different service types should be specified in R16 if the two HARQ-ACK codebooks are due to transmit in resources overlapping in time

  - FFS details, e.g., multiplexing and/or prioritizing or parallel tx – revisit later this week

Agreement:

- When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, a HARQ-ACK codebook can be identified based on some PHY indications / properties.

  - FFS in potential WI the details of the PHY identification

RAN1#96bis

Agreement:

- For supporting multiple PUCCHs for HARQ-ACK within a slot for constructing HARQ-ACK codebook, support sub-slot-based HARQ-ACK feedback procedure

  - A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot.

    - PDSCH transmission is not subject to sub-slot restrictions (if any)

    - FFS: PDSCH-to-sub-slot association

    - FFS: Allowing PUCCH across sub-slot boundary or not

  - R15 HARQ-codebook construction is applied in unit of sub-slot at least for Type II HARQ-ACK codebook.

    - FFS for Type I HARQ codebook

  - R15 PUCCH resource overriding procedures is applied in unit of sub-slot.

  - Number of length of UL sub-slots in a slot is UE-specifically semi-statically configured.

    - FFS: Limit of number of PUCCH transmissions carrying HARQ-ACKs in a slot.

  - FFS: K1 definition

  - FFS: Details of PUCCH resource configuration and determination.

- FFS: Use “Codebook-less HARQ” as a complementary or not

- FFS: If HARQ-ACK can be omitted in case latency requirement cannot be met.

- FFS: PDSCH groupings and PHY identification for separate HARQ-ACK constructions for different service types.

Agreement:

- For supporting multiple PUCCHs for HARQ-ACK within a slot for constructing HARQ-ACK codebook, K1 is defined following R15 approach but in unit of sub-slot.

Agreement:

- When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, for both Type I (if supported) and Type II HARQ-ACK codebooks (if supported), and dynamically-scheduled PDSCH, down-select from below for the PHY identification for identify a HARQ-ACK codebook:

  - Opt.1: By DCI format

  - Opt.2: By RNTI

  - Opt.3: By explicit indication in DCI (FFS: new field or reuse existing field)

  - Opt.4: By COREST/search space

  - FFS additional option(s) for Type I HARQ-ACK codebook.

- FFS: For SPS PDSCH (including SPS release PDCCH)

RAN1#97

Agreement:

- For sub-slot-based HARQ-ACK feedback procedure, K1 is the number of sub-slots from the sub-slot containing the end of PDSCH to the sub-slot containing the start of PUCCH.

  - Use UL numerology to define the sub-slot grid for PDSCH-to-sub-slot association.

  - FFS: The configuration value range of K1 needs to be extended, and impact to related DCI field bitwidth.

  - Note: It has been agreed that K1 is defined following R15 approach but in unit of sub-slot.

Agreement:

- For sub-slot-based HARQ-ACK feedback procedure, the starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot

  - For a given sub-slot configuration, a UE can be configured with PUCCH resource set(s)

    - FFS same or different PUCCH resource sets can be configured for different sub-slots within a slot

Agreement:

- When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, all Rel-16 parameters in PUCCH configuration related to HARQ-ACK feedback can be separately configured for different HARQ-ACK codebooks except for following.

  - FFS: For PUCCH-SpatialRelationInfo

  - Note: SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList are not related to HARQ-ACK feedback.

- FFS: For other UCI types, e.g., SchedulingRequestResoruceCongfig, multi-CSI-PUCCH-ResourceList.

- FFS: At least one HARQ-ACK codebook follows R15 PUCCH configuration.

Working assumoption:

- Support that SR priority (e.g. high or low priority) is known at PHY layer.

  - FFS how to use the priority information in handling prioritization/multiplexing of UL transmissions.

  - FFS how the SR priority is known

RAN1#97

Agreement:

- At least one sub-slot configuration for PUCCH can be UE-specifically configured to a UE.

  - At least support following two sub-slot configurations for PUCCH: “2-symbol*7” and “7-symbol*2”.

    - FFS other configurable sub-slot configurations, e.g., 4, 14 sub-slots in a slot.

    - For the above two sub-slot configurations (“2-symbol*7” and “7-symbol*2”), support a single configuration for PUCCH resource following R15 applicable for all the sub-slots in a slot.

      - FFS whether or not to additionally support that PUCCH resource configuration can be different for different sub-slots

      - FFS for other sub-slot configurations, if any

  - FFS: If a PUCCH resource across sub-slot boundary is supported.

Agreement:

- When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, following can be separately configured for different HARQ-ACK codebooks:

  - PUCCH-SpatialRelationInfo

  - Sub-slot configuration (only applied for the sub-slot-based HARQ-ACK codebook)

    - FFS whether or not to support the case when there are at least two HARQ-ACK codebooks configured with sub-slots, with the same or different sub-slot configurations

Agreement:

- When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, the PHY identification of HARQ-ACK codebook is also used to determine the priority of the HARQ-ACK codebook for collision handling.

Agreement:

- When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE,

  - In case of SPS PDSCH, the following options for identifying a HARQ-ACK codebook (to down-select, combinations are not precluded)

    - Opt.1: By SPS PDSCH configurations

    - Opt.2: By the DCI activating the SPS PDSCH

    - Opt.3: By the COREST where the activating DCI is received

Agreement:

- Reuse the Rel.15 mechanism for the following scenarios:

  - A URLLC SR collides with a URLLC HARQ-ACK (no other UL signals/channels), except for (to conclude the FFSs by RAN1#98b)

    - FFS if the case in which SR with PF0 vs HARQ-ACK with PF1 needs to be considered.

    - FFS SR with HARQ-ACK in PF 2, 3, 4

  - URLLC HARQ-ACK collides with URLLC PUSCH (no other UL signals/channels) when the corresponding timelines are met

    - To conclude by RAN1#98b for the error cases per Rel.15 (especially for the cases when the timeline is not met)

Agreement:

- In case URLLC (i.e., high priority) HARQ-ACK collides with eMBB (i.e., low priority) SR, down-select from options below (to conclude RAN1#98b):

  - Option 1: Drop eMBB SR

  - Option 2: Multiplex URLLC HARQ-ACK and eMBB SR if the multiplexing rule is met. Otherwise, drop eMBB SR.

    - FFS the details of the rule, e.g.,

      - Timeline

      - Latency

      - Reliability

      - PUCCH formats

- In case eMBB HARQ-ACK (i.e., low priority) collides with URLLC (i.e., high priority) SR, down-select from options below.

  - Option 1: Drop eMBB HARQ-ACK

  - Option 2: Multiplex eMBB HARQ-ACK and URLLC SR if the multiplexing rule is met. Otherwise, drop eMBB HARQ-ACK.

    - FFS the details of the rule, e.g.,

      - Timeline

      - Latency

      - Reliability

      - PUCCH formats, e.g., SR on PF0 collides with HARQ-ACK on PF1/3/4

    - FFS: Resending HARQ-ACK or not after dropping

- In case eMBB HARQ-ACK (i.e., low priority) collides with URLLC (i.e., high priority) HARQ-ACK, down-select from options below.

  - Option 1: Drop eMBB HARQ-ACK

  - Option 2: Multiplex eMBB HARQ-ACK and URLLC HARQ-ACK if the multiplexing rule is met. Otherwise,

  - drop eMBB HARQ-ACK

    - FFS the details of the rule, e.g.,

      - Timeline

      - Latency

      - Reliability

      - Pre-defined rules or configurable rules or dynamically-indicated multiplexing

  - FFS: Resending HARQ-ACK or not after dropping

- FFS details in case of a channel/signal being dropped in handling of collision of UL channels/signals

- High priority vs. low priority HARQ-ACK is made known at the PHY layer (Note: for SR, it’s agreed earlier).