3GPP TSG RAN WG1 Meeting RAN1 #98-Bis                                                       R1-1911063

Chongqing, China, October 14th – 20th, 2019

Agenda item: 7.2.13.4

Source: MediaTek Inc.

Title: On aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

Document for: Discussion and Decision

1	Introduction

In RAN #85, it is agreed to revise the MR DC WID as in RP-1922336 [1] and

- Introduce support for aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH [RAN1]

An RAN1 email discussion is then kicked off for this topic and the following issues are discussed:

  - Issue 1: How should the slot offset for aperiodic CSI-RS relative triggering PDCCH be determined?

  - Issue 2: Should aperiodic CSI-RS triggering when μCSIRS < μPDCCH be supported?

  - Issue 3: What is the additional number of symbols m1 added to the UE reported beamSwitchTiming when μCSIRS > μPDCCH?

  - Issue 4: What is the minimum time m2 between last symbol of triggering PDCCH and first symbol of aperiodic CSI-RS when μCSIRS > μPDCCH?

  - Issue 5: The Rel-15 CSI-RS triggering offset has the value range X∈{0, 1, 2, 3, 4, 16, 24} slots. Are new values needed?

This tdoc focus on the discussions for these 5 issues.

2	On the Offset of Aperiodic CSI-RS and Different SCS

2.1 Slot offset for aperiodic CSI-RS related to triggering PDCCH

The relation of aperiodic CSI-RS related to the triggering PDCCH is shown in Figure 1 as below:

Figure 1: The relation of aperiodic CSI-RS related to the triggering PDCCH (with X=0 and n=0)

According to Figure 1, to avoid unnecessary UE buffering and processing complexity, we think the original text in previous working agreement is quite reasonable, although finally it was not confirmed:

- CSI-RS always in the next slot (with DCI SCS)

- Support only the case where the DCI is contained in the beginning of a slot; we don’t see the need to support other cases for CSI acquisition

Previous working agreement:

When aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 16, 24} slots. The aperiodic CSI-RS is transmitted in slot n+X if the PDCCH SCS is equal to the CSI-RS SCS and n+12μCSIRS2μPDCCH+X if the PDCCH SCS is smaller than the CSI-RS SCS, where n is the slot with the triggering DCI in the numerology of the PDCCH containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset, and μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively. If all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states and the PDCCH SCS is equal to the CSI-RS SCS, the CSI-RS triggering offset X is fixed to zero. If the PDCCH SCS is smaller than the CSI-RS SCS, the UE does not expect that the PDCCH carrying the triggering DCI is contained in the last 10 symbols of the slot.

To ensure that the A-CSI-RS always appears in the next slot (with DCI SCS), using offset as (n+1)2μCSIRS2μPDCCH+X is reasonable. In additional to assure enough UE processing time, it should be noted that in another RAN1 working item (UE power saving), it is emphasize that cross-slot A-CSI-RS triggering is beneficial to UE power saving (RAN1 #97 7.2.9.2 Procedure of cross-slot scheduling power saving techniques).

Moreover, UE should have enough processing time to decode the PDCCH and then receive the A-CSI-RS. Therefore, if the PDCCH SCS is smaller than the CSI-RS SCS, the UE does not expect that the PDCCH carrying the triggering DCI is contained in the last 10 symbols of the slot.

Proposal 1: The aperiodic CSI-RS triggering offset is set to (n+1)2μCSIRS2μPDCCH+X, where

- n is the slot with the triggering DCI in the numerology of the PDCCH containing the triggering DCI,

- X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset

- μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively

Proposal 2: If the PDCCH SCS is smaller than the CSI-RS SCS, the UE does not expect that the PDCCH carrying the triggering DCI is contained in the last 10 symbols of the slot.

2.2 Support of aperiodic CSI-RS triggering when μCSIRS < μPDCCH

We think there is no clear justification yet to support μCSIRS < μPDCCH. However, since high-to-low scheduling for CCS with different numerology is supported in Rel-16, for spec completeness, having independent UE capability for low-to-high and high-to-low A-CSI-RS triggering can be discussed.

Proposal 3: Do not support aperiodic CSI-RS triggering when μCSIRS < μPDCCH, or introduce it as a capability.

2.3 The additional number of symbols m1 added to the UE reported beamSwitchTiming when μCSIRS > μPDCCH

Our understanding on introducing m1 is to provide sufficient UE processing time for DCI decoding. The parameter ‘beamSwitchTiming’ somehow takes PDCCH processing time into account, but it is derived under the assumption of the same numerology between DCI and CSI-RS so we need additional offset m1 for different numerology case. A conservative value like m1=14 is more proper. How to understand that m1 is equal to the span of the PDCCH monitoring region?

Proposal 4: Set the additional number of symbols added to the UE reported beamSwitchTiming when μCSIRS > μPDCCH, which means m1, to be 14.

2.4 The minimum time m2 between last symbol of triggering PDCCH and first symbol of aperiodic CSI-RS when μCSIRS > μPDCCH

In our opinion, a conservative value like m2=14 is more proper. With m2 large enough, UE buffering of data for slots in higher numerology may not be an issue.

Proposal 5: Set the minimum time between last symbol of triggering PDCCH and first symbol of aperiodic CSI-RS when μCSIRS > μPDCCH, which means m2, to be 14.

2.5 The range of Rel-15 CSI-RS triggering offset

The Rel-15 CSI-RS triggering offset has the value range X∈{0, 1, 2, 3, 4, 16, 24} slots. According to the text proposal to 38.214 regarding aperiodic CSI Reporting/aperiodic CSI-RS provided in the email discussion, the UE behavior would be: (DCI to A-CSI-RS delay = X)

- If  X > Y + d

  - Apply the TCI state in DCI to the received A-CSI-RS

- If  Y+d > X > m22μCSIRS2μPDCCH

  - If there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS

- If  X < m22μCSIRS2μPDCCH

  - Do not process the A-CSI-RS (drop it)

Note:

- Y is the UE reported threshold beamSwitchTiming (one of the values {14,28,48} in  FR2 symbols)

- d=0 if the PDCCH SCS is larger than or equal the CSI-RS SCS and d=m12μCSIRS2μPDCCH otherwise, where μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively,

Take m1=14, 30kHz PDCCH scheduling a 120kHz A-CSI-RS, and beamSwitchTiming =14 as an example:

- Y+d = 14 + 14*4 = 70

Since the maximum value of X is 24, the range of X should be enlarged.

Proposal 6: Enlarge the range of aperiodicTriggeringOffset (X) in current spec (0~24 for now).

3	Summary

In this contribution, we focus on the discussions for the issues discussed in RAN1 offline discussion after RAN #85 and have the following proposals:

Proposal 1: The aperiodic CSI-RS triggering offset is set to (n+1)2μCSIRS2μPDCCH+X, where

- n is the slot with the triggering DCI in the numerology of the PDCCH containing the triggering DCI,

- X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset

- μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively

Proposal 2: If the PDCCH SCS is smaller than the CSI-RS SCS, the UE does not expect that the PDCCH carrying the triggering DCI is contained in the last 10 symbols of the slot.

Proposal 3: Do not support aperiodic CSI-RS triggering when μCSIRS < μPDCCH, or introduce it as a capability.

Proposal 4: Set the additional number of symbols added to the UE reported beamSwitchTiming when μCSIRS > μPDCCH, which means m1, to be 14.

Proposal 5: Set the minimum time between last symbol of triggering PDCCH and first symbol of aperiodic CSI-RS when μCSIRS > μPDCCH, which means m2, to be 14.

Proposal 6: Enlarge the range of aperiodicTriggeringOffset (X) in current spec (0~24 for now).

Reference

RP-1922336, Revised WID: Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements, RAN #85