3GPP TSG RAN WG1#98bis                                      R1-1911468

Chongqing, China, October 14th – 20th, 2019

Source:	vivo

Title:	Summary#2 of offline discussion on Full TX Power UL transmission

Agenda Item:	7.2.8.4

Document for: Discussion and Decision

Offline agreement:

- For 2Tx in mode 1,

  - For rank=1, TPMI=2, TPMI=0, TPMI=1 are included in new codebook subset for non-coherent UEs with power scaling defined as in [38.213] Rel-15

  - For rank=2, TPMI=0 is included in the new codebook subset

Offline agreement:

- For Mode2,

  - power scaling factor is equal to 1 for the reported TPMI precoders that supports full power Tx

  - FFS: power scaling for other remaining TPMI precoders

Details are as following options (no new options will be considered):

| OPPO | - power scaling factor is equal to 1 for the reported TPMI precoders that supports full power Tx - for the other TPMI precoders, the power scaling factor is determined by #non-zero PUSCH port/#SRS ports in the SRS resource indicated by SRI |
| --- | --- |
| CATT | - For TPMIs that can support full power transmission reported by UE, the power scaling factor is 1; - For other TPMIs, - if full power transmission is supported by the combination of any two ports, the power scaling factor is min{1,2 #non-zero-PUSCH-port divided by #SRS-ports in the SRS resource indicated by SRI }; - otherwise, the power scaling factor is determined by #non-zero-PUSCH-port divided by #SRS-ports in the SRS resource indicated by SRI. |
| Intel | For Mode 2 operation, if the TPMI supports full power transmission, then the power scaling factor should be 1; otherwise the power scaling factor is determined by the number of non-zero power PUSCH ports divided by the actual number of SRS antenna ports in the SRS resource indicated by SRI. |
| Ericsson | - power scaling factor for 4 Tx UEs with full power capability is - 1 for ranks 3 & 4, for rank 2 with 2 port SRS, and for 1 port SRS, otherwise - 1 for TPMIs that provide full power, otherwise - 1/2 [or 1/3] for rank one with 4 ports if supported by UE capability, otherwise - determined by #non-zero PUSCH port/#SRS ports - where the number of SRS ports is the number of ports in the SRS resource associated with the PUSCH transmission |