3GPP TSG RAN WG1 Meeting #98bis	R1-1911494

Chongqing, P.R. China, October 14th – 20th 2019

Agenda item:		7.2.13.4

Source:	Nokia, Nokia Shanghai Bell

Title:	FL summary#2 on A-CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

Document for:		Discussion

# Introduction

This document summarizes the contributions Numerologies submitted under agenda-item 7.2.13.4 “7.2.13	Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) – aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH” [1-8]. The objective of this document is to summarise the proposals related to the feature.

FL summary #2 captures the Tuesday offline discussions

# Proposals

## Scenarios

| Vivo | Proposal 1: Reuse the scenario and definition of scheduling timing for cross-carrier PDSCH scheduling. - For the case of higher SCS PDCCH triggering lower SCS aperiodic CSI-RS, the motivation and usecase need to be clarified. |
| --- | --- |
| ZTE | Proposal 1: For cross carrier CSI-RS triggering with mixed numerology, both μPDCCH<μCSI-RS and μPDCCH>μCSI-RS are supported. |
| Sam | Proposal 2: Support of this feature is restricted to μPDCCH≤μCSI-RS. |
| Intel | Proposal 1: Support aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH for μCSIRS < μPDCCH and μCSIRS > μPDCCH |
| Eri | Proposal 3: Both low-to-high CSI-RS triggering as well as high-to-low CSI-RS triggering is supported, but correspond to separate UE capabilities |
| MTek | Proposal 3: Do not support aperiodic CSI-RS triggering when μCSIRS < μPDCCH, or introduce it as a capability. |

Offline discussion status:

- Support cross-carrier A-CSI-RS triggering when µPDCCH < µCSI-RS

- Support cross-carrier A-CSI RS triggering when µPDCCH > µCSI-RS

- The two are understood to be subject to separate UE capabilities

  - FFS if these are linked to the capabilities related to the cross-carrier spacing with different numerologies

## Triggering offset definition

| Vivo | Proposal 4: Considering new values added to the CSI-RS triggering offset in case of mixed numerology CSI-RS triggering. |
| --- | --- |
| ZTE | Proposal 2: Given the PDCCH slot n1 in carrier A and the configured X value for triggering offset, the CSI-RS slot in carrier B is n12μCSI-RS2μPDCCH+X. Proposal 3: For a cross carrier CSI-RS triggering with μPDCCH<μCSI-RS, the number of symbols between the first symbol of CSI-RS and the last symbol of PDCCH should be at least m2, which is defined per PDCCH SCS - m2=4, 4 and 8 for 15kHz, 30kHz and 60kHz, respectively. Proposal 4: Support adding more values (e.g. X can be from 0 to 24) in the candidate set for X>4 for all the aperiodic CSI-RS triggering cases, i.e., not restricted to cross-carrier triggering with mixed numerology. Proposal 5: If the slot offset is not configured for the triggered aperiodic CSI-RS set, transmission of the triggered aperiodic CSI-RS is expected to be in the first valid slot which satisfies the timing requirement. |
| Sam | Proposal 1: The reference slot for the slot offset for A-CSI-RS transmission is n'=n+12μCSI-RS2μPDCCH if μPDCCH<μCSI-RS. Proposal 5: Additional values for the CSI-RS triggering offset X is not supported. |
| Intel | Proposal 2: For aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH, the slot index for CSI-RS transmission in the numerology of CSI-RS is derived based on the following equation (n+1)2μCSIRS2μPDCCH+X - n is the slot index corresponding to the triggering DCI in the numerology of the PDCCH containing the triggering DCI - X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset |
| Eri | Proposal 1: The slot offset for CSI-RS is defined asn2μCSIRS2μPDCCH+X, where n is the slot with the triggering DCI in the numerology of the PDCCH, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset, μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively Proposal 2: Increase the value range of aperiodicTriggeringOffset to X=0,1,…,31 |
| MTek | Proposal 1: The aperiodic CSI-RS triggering offset is set to (n+1)2μCSIRS2μPDCCH+X, where - n is the slot with the triggering DCI in the numerology of the PDCCH containing the triggering DCI, - X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively Proposal 6: Enlarge the range of aperiodicTriggeringOffset (X) in current spec (0~24 for now). |
| Hua | Observation 1: The value range of time offset, minimum time gap etc. for aperiodic-CSI-RS triggering by DCI with different SCS is depending on the adoption of timing determination approach and the output of UE capability discussion for support of multi-TB scheduling. Observation 2: Asynchronous-CA might have impact on the value range of timing related parameters for aperiodic-CSI-RS triggering. Proposal 1: The aperiodic CSI-RS triggering offset is given by (n+1)2μCSIRS2μPDCCH-1+X, where - n is the slot with the triggering DCI in the numerology of the PDCCH containing the triggering DCI, - X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively. |

FL proposal:

Definition of the slot offset

- n12μCSI-RS2μPDCCH+X, where the n1 is the PDCCH carrier slot with the DCI

- Note: The case when the frame boundaries of the two carriers are not aligned may require additional compensation when determining the actual slot number

- This with the assumption that there is always a non-zero delay between the PDCCH and A-CSI RS

New slot offset values

- Extend the X values for cross-carrier A-CSI RS triggering when the PDCCH and A-CSI RS have different SCS

Reference slot for for the slot offset

- PDCCH carrier slot with the DCI

- slot n'=n+12μCSI-RS2μPDCCH on the A-CSI-RS carrier

Definition of the slot offset

- n12μCSI-RS2μPDCCH+X, where the n1 is the PDCCH carrier slot with the DCI

- (n+1)2μCSIRS2μPDCCH+X, where the n is the PDCCH carrier slot with the DCI

- (n+1)2μCSIRS2μPDCCH-1+X, where the n is the PDCCH carrier slot with the DCI

- n’ + X, where n'=n+12μCSI-RS2μPDCCH and n is the PDCCH carrier slot with the DCI

## Minimum triggering offset

| Vivo | Proposal 3: Reuse the delta for cross-carrier scheduling PDSCH as the minimum time between last symbol of triggering PDCCH and first symbol of aperiodic CSI-RS. |
| --- | --- |
| Sam | Proposal 4: Support a fixed minimum gap between PDCCH and A-CSI-RS for UE processing relaxation when the CSI-RS triggering offset X=0; reuse the value for cross-carrier scheduling as the fixed minimum gap. |
| Eri | Proposal 4: When the PDCCH SCS is smaller than the CSI-RS SCS, the UE is not required to process aperiodic CSI-RS if the offset between the last symbol of the PDCCH and the first symbol of the CSI-RS is smaller than m22μCSIRS2μPDCCH in the CSI-RS numerology, where m2 is equal to 4 for 15 and 30 kHz SCS and 8 for 60 kHz SCS. |
| MTek | Proposal 5: Set the minimum time between last symbol of triggering PDCCH and first symbol of aperiodic CSI-RS when μCSIRS > μPDCCH, which means m2, to be 14. |
| QCOM | Proposal 1: The minimum offset between the end of the triggering PDCCH and the start of the aperiodic CSI-RS for the UE to receive the CSI-RS is Δ2μCSIRS2μPDCCH, where Δ is the minimum PDCCH-to-PDSCH delay between the received PDCCH and the received PDSCH at the UE defined for cross-carrier scheduling without different numerologies. μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively. Δ takes values of [4, 4, 8, 12] for PDCCH SCS = [15, 30, 60, 120] kHz, respectively. |

FL proposal:

Minimum A-CSI RS triggering offset at least when not switching the beam for cross-carrier triggering of A-CSI RS when the PDCCH SCS and the A-CSI RS SCS are not the same

- Minimum delay from he end of the triggering PDCCH and the start of the CSI-RS in the CSI-RS carrier’s slots is defined as m

  - m = 4, 4, 8, [12] symbols for PDCCH SCS = 15, 30, 60, 120 kHz, respectively as defined for cross-carrier scheduling of PDSCH with different PDCCH and PDSCH SCS.

  - When µPDCCH < µCSI-RS the minimum delay is quantized to the beginning of the next A-CSI RS carrier slot

## Triggering DCI location

| MTek | Proposal 2: If the PDCCH SCS is smaller than the CSI-RS SCS, the UE does not expect that the PDCCH carrying the triggering DCI is contained in the last 10 symbols of the slot. |
| --- | --- |

Discuss the proposal

## Beam switching

| Vivo | Proposal 2: Clarify the motivation for introducing the additional number of symbols added to the UE reported beamSwitchTiming. |
| --- | --- |
| ZTE | Proposal 6: If extra value has to be added for the beam switch time when μPDCCH<μCSI-RS, d=14 OFDM symbols defined w.r.t. PDCCH SCS can be a start point. |
| Sam | Proposal 3: Support additional d=2μCSI-RS2μPDCCH-1Z, where Z is fixed, symbols added to the UE reported beamSwitchTiming when μPDCCH≤μCSI-RS. |
| Eri | Proposal 5: When the PDCCH SCS is smaller than the CSI-RS SCS, an additional m1 symbols are added to the UE’s reported capability beamSwitchTiming, where m1 is equal to m2 |
| MTek | Proposal 4: Set the additional number of symbols added to the UE reported beamSwitchTiming when μCSIRS > μPDCCH, which means m1, to be 14. |
| QCOM | Observation 1: For cross-carrier aperiodic CSI-RS triggering, early proposals have defined the beam switch timing threshold as summation of self-scheduling threshold and an adjustment term. The self-scheduling threshold is based on the scheduled cell’s SCS and the adjustment term is based on the scheduling cell’s SCS. Proposal 2: For cross-carrier aperiodic CSI-RS triggering with different numerologies between the triggering PDCCH and the aperiodic CSI-RS, beam switch timing threshold is equal to a number of symbols based on the CSI-RS’s SCS and the symbol number is X+Δ2μCSIRS2μPDCCH where X is the beam switch timing threshold defined for same cell scheduling (i.e., beamSwitchTiming) and Δ is the minimum PDCCH-to-PDSCH delay between the received PDCCH and the received PDSCH at the UE defined for cross-carrier scheduling without different numerologies. μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively. Δ takes values of [4, 4, 8, 12] for PDCCH SCS = [15, 30, 60, 120] kHz, respectively. |

Additional symbols to the UE reported beamSwitchTiming

- Needed

- Not needed

- If answer to above is: “needed”, the additional symbols to the UE reported beamSwitchTiming is

- d = 14 symbols

- d=2μCSI-RS2μPDCCH-1Z, where Z = [12 or 14]

- d is the same as the minimum triggering offet

- d = X+Δ2μCSIRS2μPDCCH, where X is the same as the triggering offset and Δ takes values of [4, 4, 8, 12] for PDCCH SCS = [15, 30, 60, 120] kHz, respectively.

## Default QCL assumption

| Vivo | Proposal 5: For cross-carrier triggering of aperiodic CSI-RS on a SCell without any monitored CORESET, if the offset between the last symbol of the PDCCH and the first symbol of the aperiodic CSI-RS resources is smaller than the UE reported threshold beamSwitchTiming, and if there is not any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the serving cell where the aperiodic CSI-RS resource is configured. |
| --- | --- |
| ZTE | Proposal 7: When CSI-RS time offset is smaller than BeamSwitchTiming, UE apply QCL Type A/B/C parameters derived from TCI indication in the trigger state associated with the DCI codepoint, and QCL-Type D derived from the default beam, where the QCL-Type D of the default beam is defined as - QCL Type D from the lowest CORESET ID of the CSI-RS CC, if there is PDCCH QCL Type D configuration for the CSI-RS CC; - QCL Type D from the lowest CORESET ID of the lowest CC with PDCCH QCL Type D configuration in the CSI-RS band, otherwise. |

Discuss the following

If the offset between the triggering PDCH and the CSI-RS is less than the applicable beam switching timing

- Apply QCL type A/B/C derived from the TCI indication

- Apply QCL type D derived from the default beam, where

  - TCI state with the lowest ID applicable to PDSCH in the active BWP of the serving cell where the aperiodic CSI-RS resource is configured.

  - One of

    - QCL Type D from the lowest CORESET ID of the CSI-RS CC, if there is PDCCH QCL Type D configuration for the CSI-RS CC;

    - QCL Type D from the lowest CORESET ID of the lowest CC with PDCCH QCL Type D configuration in the CSI-RS band, otherwise.

# References

- R1-1910243	Discussion on aperiodic CSI-RS triggering, vivo

- R1-1910291	Cross-carrier CSI-RS triggering with mixed numerology, ZTE

- R1-1910509	A-CSI-RS triggering by PDCCH with different numerology, Samsung

- R1-1910683	Discussions on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH, Intel Corporation

- R1-1910898	On support of aperiodic CSI-RS triggering with mixed numerology	Ericsson

- R1-1911063	Support of aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH, MediaTek Inc.

- R1-1911140	A-CSI-RS with different numerology between CSI-RS and triggering PDCCH, Qualcomm Incorporated

- R1-1911265	Discussion on aperiodic CSI-RS triggering with different numerologies between CSI-RS and triggering PDCCH, Huawei, HiSilicon

# Annex – Proposals and observations

[1]	R1-1910243	Discussion on aperiodic CSI-RS triggering, vivo

Proposal 1:

- Reuse the scenario and definition of scheduling timing for cross-carrier PDSCH scheduling.

  - For the case of higher SCS PDCCH triggering lower SCS aperiodic CSI-RS, the motivation and usecase need to be clarified.

Proposal 2:

- Clarify the motivation for introducing the additional number of symbols added to the UE reported beamSwitchTiming.

Proposal 3:

- Reuse the delta for cross-carrier scheduling PDSCH as the minimum time between last symbol of triggering PDCCH and first symbol of aperiodic CSI-RS.

Proposal 4:

- Considering new values added to the CSI-RS triggering offset in case of mixed numerology CSI-RS triggering.

Proposal 5:

- For cross-carrier triggering of aperiodic CSI-RS on a SCell without any monitored CORESET, if the offset between the last symbol of the PDCCH and the first symbol of the aperiodic CSI-RS resources is smaller than the UE reported threshold beamSwitchTiming, and if there is not any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the serving cell where the aperiodic CSI-RS resource is configured.

[2]	R1-1910291	Cross-carrier CSI-RS triggering with mixed numerology, ZTE

Proposal 1: For cross carrier CSI-RS triggering with mixed numerology, both μPDCCH<μCSI-RS and μPDCCH>μCSI-RS are supported.

Proposal 2: Given the PDCCH slot n1 in carrier A and the configured X value for triggering offset, the CSI-RS slot in carrier B is n12μCSI-RS2μPDCCH+X.

Proposal 3: For a cross carrier CSI-RS triggering with μPDCCH<μCSI-RS, the number of symbols between the first symbol of CSI-RS and the last symbol of PDCCH should be at least m2, which is defined per PDCCH SCS

- m2=4, 4 and 8 for 15kHz, 30kHz and 60kHz, respectively.

Proposal 4: Support adding more values (e.g. X can be from 0 to 24) in the candidate set for X>4 for all the aperiodic CSI-RS triggering cases, i.e., not restricted to cross-carrier triggering with mixed numerology.

Proposal 5: If the slot offset is not configured for the triggered aperiodic CSI-RS set, transmission of the triggered aperiodic CSI-RS is expected to be in the first valid slot which satisfies the timing requirement.

Proposal 6: If extra value has to be added for the beam switch time when μPDCCH<μCSI-RS, d=14 OFDM symbols defined w.r.t. PDCCH SCS can be a start point.

Proposal 7: When CSI-RS time offset is smaller than BeamSwitchTiming, UE apply QCL Type A/B/C parameters derived from TCI indication in the trigger state associated with the DCI codepoint, and QCL-Type D derived from the default beam, where the QCL-Type D of the default beam is defined as

- QCL Type D from the lowest CORESET ID of the CSI-RS CC, if there is PDCCH QCL Type D configuration for the CSI-RS CC;

- QCL Type D from the lowest CORESET ID of the lowest CC with PDCCH QCL Type D configuration in the CSI-RS band, otherwise.

[3]	R1-1910509	A-CSI-RS triggering by PDCCH with different numerology, Samsung

Proposal 1: The reference slot for the slot offset for A-CSI-RS transmission is n'=n+12μCSI-RS2μPDCCH if μPDCCH<μCSI-RS.

Proposal 2: Support of this feature is restricted to μPDCCH≤μCSI-RS.

Proposal 3: Support additional d=2μCSI-RS2μPDCCH-1Z, where Z is fixed, symbols added to the UE reported beamSwitchTiming when μPDCCH≤μCSI-RS.

Proposal 4: Support a fixed minimum gap between PDCCH and A-CSI-RS for UE processing relaxation when the CSI-RS triggering offset X=0; reuse the  value for cross-carrier scheduling as the fixed minimum gap.

Proposal 5: Additional values for the CSI-RS triggering offset X is not supported.

[4]	R1-1910683	Discussions on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH, Intel Corporation

Proposal 1:

- Support aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH for μCSIRS < μPDCCH and μCSIRS > μPDCCH

  - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH respectively

Proposal 2:

- For aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH, the slot index for CSI-RS transmission in the numerology of CSI-RS is derived based on the following equation (n+1)2μCSIRS2μPDCCH+X

  - n is the slot index corresponding to the triggering DCI in the numerology of the PDCCH containing the triggering DCI

  - X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset

  - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively

[5]	R1-1910898	On support of aperiodic CSI-RS triggering with mixed numerology	Ericsson

Proposal 1	The slot offset for CSI-RS is defined as n2μCSIRS2μPDCCH+X, where n is the slot with the triggering DCI in the numerology of the PDCCH, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset, μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively

Proposal 2	Increase the value range of aperiodicTriggeringOffset to X=0,1,…,31

Proposal 3	Both low-to-high CSI-RS triggering as well as high-to-low CSI-RS triggering is supported, but correspond to separate UE capabilities

Proposal 4	When the PDCCH SCS is smaller than the CSI-RS SCS, the UE is not required to process aperiodic CSI-RS if the offset between the last symbol of the PDCCH and the first symbol of the CSI-RS is smaller than  m22μCSIRS2μPDCCH in the CSI-RS numerology, where m2 is equal to 4 for 15 and 30 kHz SCS and 8 for 60 kHz SCS.

Proposal 5	When the PDCCH SCS is smaller than the CSI-RS SCS, an additional m1 symbols are added to the UE’s reported capability beamSwitchTiming, where m1 is equal to m2

[6]	R1-1911063	Support of aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH, MediaTek Inc.

Proposal 1: The aperiodic CSI-RS triggering offset is set to (n+1)2μCSIRS2μPDCCH+X, where

- n is the slot with the triggering DCI in the numerology of the PDCCH containing the triggering DCI,

- X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset

- μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively

Proposal 2: If the PDCCH SCS is smaller than the CSI-RS SCS, the UE does not expect that the PDCCH carrying the triggering DCI is contained in the last 10 symbols of the slot.

Proposal 3: Do not support aperiodic CSI-RS triggering when μCSIRS < μPDCCH, or introduce it as a capability.

Proposal 4: Set the additional number of symbols added to the UE reported beamSwitchTiming when μCSIRS > μPDCCH, which means m1, to be 14.

Proposal 5: Set the minimum time between last symbol of triggering PDCCH and first symbol of aperiodic CSI-RS when μCSIRS > μPDCCH, which means m2, to be 14.

Proposal 6: Enlarge the range of aperiodicTriggeringOffset (X) in current spec (0~24 for now).

[7]	R1-1911140	A-CSI-RS with different numerology between CSI-RS and triggering PDCCH, Qualcomm Incorporated

Observation 1: For cross-carrier aperiodic CSI-RS triggering, early proposals have defined the beam switch timing threshold as summation of self-scheduling threshold and an adjustment term. The self-scheduling threshold is based on the scheduled cell’s SCS and the adjustment term is based on the scheduling cell’s SCS.

Proposal 1: The minimum offset between the end of the triggering PDCCH and the start of the aperiodic CSI-RS for the UE to receive the CSI-RS is

Δ2μCSIRS2μPDCCH

where Δ is the minimum PDCCH-to-PDSCH delay between the received PDCCH and the received PDSCH at the UE defined for cross-carrier scheduling without different numerologies. μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively. Δ takes values of [4, 4, 8, 12] for PDCCH SCS = [15, 30, 60, 120] kHz, respectively.

Proposal 2: For cross-carrier aperiodic CSI-RS triggering with different numerologies between the triggering PDCCH and the aperiodic CSI-RS, beam switch timing threshold is equal to a number of symbols based on the CSI-RS’s SCS and the symbol number is

X+Δ2μCSIRS2μPDCCH

where X is the beam switch timing threshold defined for same cell scheduling (i.e., beamSwitchTiming) and Δ is the minimum PDCCH-to-PDSCH delay between the received PDCCH and the received PDSCH at the UE defined for cross-carrier scheduling without different numerologies. μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively.  Δ takes values of [4, 4, 8, 12] for PDCCH SCS = [15, 30, 60, 120] kHz, respectively.

[8]	R1-1911265	Discussion on aperiodic CSI-RS triggering with different numerologies between CSI-RS and triggering PDCCH, Huawei, HiSilicon

Observation 1: The value range of time offset, minimum time gap etc. for aperiodic-CSI-RS triggering by DCI with different SCS is depending on the adoption of timing determination approach and the output of UE capability discussion for support of multi-TB scheduling.

Observation 2: Asynchronous-CA might have impact on the value range of timing related parameters for aperiodic-CSI-RS triggering.

Proposal 1:

The aperiodic CSI-RS triggering offset is given by (n+1)2μCSIRS2μPDCCH-1+X, where

- n is the slot with the triggering DCI in the numerology of the PDCCH containing the triggering DCI,

- X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset

- μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively.