3GPP TSG-RAN WG1 Meeting #99	R1-1912282

Reno, Nevada, U.S.A, Nov 18th – 22nd, 2019

Source:	Nokia, Nokia Shanghai Bell

Title: 		Half-duplex operation in CA based on [95-NR-06]

Agenda item:	7.2.14

Document 		Discussion and Decision

# Introduction

The outcome of the email discussion [95-NR-06] has been the following set of agreements:

Phase 1:

Agreements: For a UE configured with multiple serving cells in a band or band combination, where the UE is not capable of simultaneous transmission and reception, support a behavior in the following table:

- FFS: other combinations of Semi SFI, RRC and Dynamic

- FFS: behaviour for SSB in RRM measurements

- FFS: down-select further between Alt1 and Alt2. Down-selection may be performed independently for case 3 and 6.

- For the agreed behaviour introduce a new feature as “mandatory with capability”

  - Note: Sep./Dec. UEs are still able to operate HD TDD CA if network ensures same transmission direction across all the serving cells.

- Reference (Ref) cell is the cell with the lowest cell ID among cells: (i) within the band or band combination and (ii) with conflicting directions, and “Other cell” is any cell within the band or band combination other than the Ref cell.

| No | Ref cell | Other cell | UE behavior | Note |
| --- | --- | --- | --- | --- |
| 1 | Semi SFI D | Semi SFI U | Allowed to drop U for inter-band Error case in intra-band | Dropping U on other cell Error case in intra-band |
| 2 | Semi SFI D | RRC U | Allowed to drop U | Dropping on other cell |
| 3 | Semi SFI D | Dynamic U | Alt 1: Allowed to drop D for inter-band Error case in intra-band | Overriding semi SFI D to F on reference cell for the UE |
|  |  |  | Alt 2: Error | Error |
| 4 | Semi SFI U | Semi SFI D | Allowed to drop D for inter-band Error case in intra-band | Dropping D on other cell Error case in intra-band |
| 5 | Semi SFI U | RRC D | Allowed to drop D | Dropping on other cell |
| 6 | Semi SFI U | Dynamic D | Alt 1: Allowed to drop U for inter-band Error case in intra-band | Overriding semi SFI U to F on reference cell for the UE |
|  |  |  | Alt 2: Error | Error |
| 7 | RRC D | RRC U | Allowed to drop U | Dropping on other cell |
| 8 | RRC U | RRC D | Allowed to drop D | Dropping on other cell |
| 9 | Dynamic D | Dynamic U | Error | Error |
| 10 | Dynamic U | Dynamic D | Error | Error |

Note, in above tables, the terminology is as follows:

- Semi SFI D and U:  D and U symbols configured by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated

- Semi SFI F: flexible symbols configured by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated, when provided to a UE, or when TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated are not provided to the UE

- RRC D: symbols corresponding to a higher-layer configured PDCCH, or a PDSCH, or a CSI-RS on semi SFI F of the same cell

- RRC U: symbols corresponding to a higher-layer configured SRS, or PUCCH, or PUSCH, or PRACH on semi SFI F of the same cell

- Dynamic D and U: symbols scheduled as D and U by DCI formats other than DCI format 2_0 on semi SFI F of the same cell

- Note: there is no intention to change the existing behavior w.r.t. how to update the reference cell

Phase 2

Agreement:

For case 3, down-select Alt 1

For case 6, down-select Alt 2

Note: above down-selection applies to NR CA only

For the remaining cases, adopt the following behavior:

| No | Ref cell | Other cell | UE behavior | Note |
| --- | --- | --- | --- | --- |
| 11 | RRC U | Semi SFI D | Allowed to drop D | Dropping on other cell |
| 12 | Dynamic U | Semi SFI D | Allowed to drop D | Dropping on other cell |
| 13 | RRC D | Semi SFI U | Allowed to drop U | Dropping on other cell |
| 14 | Dynamic D | Semi SFI U | Allowed to drop U | Dropping on other cell |
| 15 | RRC U | Dynamic D | Alt 2: Error |  |
| 16 | RRC D | Dynamic U | Alt 1: Allowed to drop D for inter-band Error case in intra-band |  |
| 17 | Dynamic U | RRC D | Allowed to drop D | Dropping on other cell |
| 18 | Dynamic D | RRC U | Allowed to drop U | Dropping on other cell |

Agreement:

In symbols indicated in ssb-PositionsInBurst on reference and other cell(s), half-duplex CA UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot.

Agreement:

In R15, behavior for mobility CSI-RS related to HD CA is not specified.

Agreement:

- Half-duplex CA UE determines reference cell per symbol as a cell with the lowest cell ID among multiple serving cells in a band or band combination having direction determined by RRC D/U or semi SFI D/U

- Note: this overrides earlier agreement ”Reference (Ref) cell is the cell with the lowest cell ID among cells: (i) within the band or band combination and (ii) with conflicting directions, and “Other cell” is any cell within the band or band combination other than the Ref cell. “

- Note: Agreed cases 12, 14, 17 and 18 are not needed anymore

- Note: Agreed cases 9 and 10 should apply to collisions between two cells irrespective of a cell being reference or other

Agreement:

Inform RAN4 about the option below and ask RAN4 to discuss/confirm RRM behavior.

- When the scheduling restrictions due to RRM measurements are applicable, half-duplex CA UE does not transmit in UL in one cell, if the UL transmission would collide with any SSB symbols other than indicated in ssb-PositionsInBurst inside SMTC window of another cell. RRM measurements in SMTC window  are always protected.

- When the scheduling restrictions due to RRM measurements are NOT applicable, if the half-duplex CA UE is scheduled by DCI (Dynamic U) to transmit in UL in one cell, and if the UL transmission collides with any SSB symbols other than indicated in ssb-PositionsInBurst inside SMTC window of another cell, UE is not required to perform RRM measurements on the colliding SSB symbols.

- RRM measurements during the symbols indicated in ssb-PositionInBurst are always protected.

In RAN1#96 it has been concluded:

Agreement: (RAN1#96)

Agreements made during email discussion [95-NR-06] on directional collision handling for half duplex CA operation are postponed to Re1-16.

In this contribution we provide text proposal for TS 38.213 to capture agreements from [95-NR-06]

# HD UE behaviour

The final behavior is summarized in Table 1, where we separated behavior into: (i) applicable to intra-band only (ii) applicable to inter-band only (iii) applicable to both intra and inter-band.

Table 1 Combined agreed behavior in [95-NR-06]

| No | Ref cell | Other cell | UE behavior | Note |
| --- | --- | --- | --- | --- |
| 1 | Semi SFI D | Semi SFI U | Allowed to drop U for inter-band Error case in intra-band | Dropping U on other cell Error case in intra-band |
| 2 | Semi SFI D | RRC U | Allowed to drop U | Dropping on other cell |
| 3 | Semi SFI D | Dynamic U | Allowed to drop D for inter-band Error case in intra-band | Overriding semi SFI D to F on reference cell for the UE |
| 4 | Semi SFI U | Semi SFI D | Allowed to drop D for inter-band Error case in intra-band | Dropping D on other cell Error case in intra-band |
| 5 | Semi SFI U | RRC D | Allowed to drop D | Dropping on other cell |
| 6 | Semi SFI U | Dynamic D | Error | Error |
| 7 | RRC D | RRC U | Allowed to drop U | Dropping on other cell |
| 8 | RRC U | RRC D | Allowed to drop D | Dropping on other cell |
| 9 | Dynamic D | Dynamic U | Error | Error |
| 10 | Dynamic U | Dynamic D | Error | Error |
| 11 | RRC U | Semi SFI D | Allowed to drop D | Dropping on other cell |
| 13 | RRC D | Semi SFI U | Allowed to drop U | Dropping on other cell |
| 15 | RRC U | Dynamic D | Error | Error |
| 16 | RRC D | Dynamic U | Allowed to drop D for inter-band Error case in intra-band | Dropping RRC D on reference cell |

where the reference cell is determined as:

“Half-duplex CA UE determines reference cell per symbol as a cell with the lowest cell ID among multiple serving cells in a band or band combination having direction determined by RRC D/U or semi SFI D/U”

And the definition of colliding symbols is the following:

- Semi SFI D and U:  D and U symbols configured by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated

- Semi SFI F: flexible symbols configured by TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated, when provided to a UE, or when TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated are not provided to the UE

- RRC D: symbols corresponding to a higher-layer configured PDCCH, or a PDSCH, or a CSI-RS on semi SFI F of the same cell

- RRC U: symbols corresponding to a higher-layer configured SRS, or PUCCH, or PUSCH, or PRACH on semi SFI F of the same cell

- Dynamic D and U: symbols scheduled as D and U by DCI formats other than DCI format 2_0 on semi SFI F of the same cell

Based on above, we formulated behavior for sub-clause 11.1 TS38.213:

If a UE is configured with multiple serving cells operating in a band or different bands, the UE is not capable of simultaneous transmission and reception, UE indicates support of capability [FG 6-25] and the UE is not configured to monitor PDCCH for DCI format 2-0, the UE determines reference cell per symbol as a cell with the lowest configured cell index among multiple serving cells in a band or band combination having direction determined by any of - TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated - higher-layer configured SRS, PUCCH, PUSCH or PRACH on flexible symbols - higher-layer configured PDCCH, PDSCH or CSI-RS on flexible symbols. If a UE is configured with multiple serving cells operating in a band, and the UE is not capable of simultaneous transmission and reception, UE indicates support of capability [FG 6-x] and the UE is not configured to monitor PDCCH for DCI format 2-0, the UE is not expected to receive - [1,4] indications on reference and another cell by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated indicating opposite directions in a symbol, - [3] indication by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated indicating a symbol to be uplink in reference cell and DCI scheduling the symbol as downlink on another cell, - [16] higher-layer configured PDCCH, PDSCH or CSI-RS on a flexible symbol in reference cell and DCI scheduling the symbol as uplink on another cell. If a UE is configured with multiple serving cells, the reference cell and an another cell operate in different bands, UE is not capable of simultaneous transmission and reception, UE indicates support of capability [FG 6-x] and the UE is not configured to monitor PDCCH for DCI format 2-0, the UE - [1,4] assumes a symbol configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated in another cell as flexible, if TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated in reference cell indicates opposite direction for the symbol, - [3] transmits uplink transmission scheduled by DCI on a symbol of another cell despite the symbol being indicated as downlink by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated on reference cell, - [16] does not expect to receive higher-layer configured PDCCH, PDSCH or CSI-RS on flexible symbols in reference cell in a set of symbols, if receives DCI scheduling one of the symbols as uplink in another cell. If a UE is configured with multiple serving cells operating in band or different bands, UE is not capable of simultaneous transmission and reception, UE indicates support of capability [FG 6-x] and the UE is not configured to monitor PDCCH for DCI format 2-0, the UE - [6] does not expect to receive an indication by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated indicating a symbol to be downlink in reference cell and DCI scheduling the symbol as uplink on another cell - [15] does not expect to receive a higher-layer configured SRS, PUCCH, PUSCH or PRACH on a flexible symbol in reference cell and DCI scheduling the symbol as downlink on another cell - [2,5,7,8] does not transmit higher-layer configured SRS, PUCCH, PUSCH or PRACH on a set of symbol or receives higher-layer configured PDCCH, PDSCH or CSI-RS on set of symbols in another cells, if at least one symbol from the set of symbols is indicated with opposite direction by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated or by higher-layer configured SRS, PUCCH, PUSCH, PRACH, PDCCH, PDSCH or CSI-RS on reference cell - [11,13] assumes a symbol configured by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated in another cell as flexible, if symbol is indicated with opposite direction by higher-layer configured SRS, PUCCH, PUSCH, PRACH, PDCCH, PDSCH or CSI-RS on reference cell - [9,10] is not expected to receive DCIs indicating contradicting directions on a symbol in different cells.

# RRM behaviour

Agreement:

In symbols indicated in ssb-PositionsInBurst on reference and other cell(s), half-duplex CA UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot.

To capture the above agreement, we have prepared following TP for sub-clause 11.1 of TS38.213:

If a UE is configured with multiple serving cells operating in a band or different bands, the UE is not capable of simultaneous transmission and reception, UE indicates support of capability [FG 6-x] and the UE is not configured to monitor PDCCH for DCI format 2-0, for a set of symbols of a slot that are indicated to the UE by ssb-PositionsInBurst in SystemInformationBlockType1 or ssb-PositionsInBurst in ServingCellConfigCommon in any of multiple serving cells , when provided to the UE, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot in any of multiple serving cells.

# Capability

The capability below should serve as starting point for the discussion in R16 Features:

|  | 6-x | Half-duplex UE behavior in TDD CA | Support for directional collision handling between reference and other cell(s) |  | Yes |  | Type 3 | No | N.A. |  | Note: UEs that do not indicate this capability should still be able to operate half-duplex TDD CA if network ensures same transmission direction across all the serving cells | RAN1 | Mandatory with capability signaling for intra-band CA band and for inter-band CA in band combination without RAN4 FG 2-5 capability | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |