3GPP TSG RAN WG1 Meeting #99	R1-1912332

Reno, USA, November 18th – 22nd, 2019

Agenda Item:	6.2.1.1

Source: 	Sony

Title:	Remaining issues on UE group wake up signal for MTC

Document for:	Discussion and Decision

# Introduction

One of the objectives in eMTC release 16 is to support a UE-group wake-up signal (WUS) mechanism in order to improve DL transmission efficiency and/or UE power consumption [1]. In RAN1 meeting #98bis [2], the following related agreements on UE-group wake up signal for MTC were made:

Agreement

- The configured WUS resources associated with same gap are consecutive in time/frequency domain.

- The WUS resources in the same time resource are consecutive to each other in the frequency domain.

- When the WUS resources are configured in two time resources, the WUS resources in first time resource are consecutive to a WUS resource in the second time resource.

- Choose supported patterns for WUS resource configuration considering the location of legacy WUS resource if Rel-15 WUS is configured.

- No more than 3 bits are used for configuration of the pattern for WUS resource

  - FFS: Considering the location of legacy WUS resource if Rel-15 WUS is configured

- For group WUS resources sharing WUS gap, per default, the number of UE groups per resource is configured for all group WUS resources. Optionally, the number of UE groups for each WUS resource can be configured individually.

- The number of UE groups per WUS resource is 1, 2, 4, or 8

- The optional eDRX configurability and configuration values regarding the number of consecutive POs a WUS is associated same as in Rel-15 legacy WU

- Design pre-defined method to allow alternating UE group to monitor different WUS resources at different POs implicitly.

- Consider both cell-specific DRX cycle and UE-specific DRX cycle.

- At least the following parameters are used in the pre-defined method

  - H_SFN of current PO

  - DRX cycle

- The group WUS resource that may coincide with legacy WUS is assigned  and the other group WUS resource are using .

- are used to differentiate different scrambling sequences for WUS sequence.

- Regarding group WUS configuration for different gap durations (DRX, short eDRX and long eDRX), the following applies:

- If eDRX long gap is configured but no group WUS configuration for that gap is provided, the eDRX short gap configuration shall be used if configured, otherwise the DRX configuration shall be used.

- If eDRX short gap is configured but no group WUS configuration for that gap is provided, the DRX gap configuration shall be used.

In this contribution, we discuss some remaining aspects on supporting UE-group wake-up signal for MTC (MWUS). A companion document submitted to RAN2 considers RAN2 aspects of UE-group WUS [3].

# Discussion

The grouping mechanism of UEs within an MWUS will allow only a group or all groups of UEs to be woken up by the MWUS for a paging occasion (PO). The UE grouping operation will reduce the number of UEs having to wake up unnecessarily to detect MPDCCH and decode PDSCH. Thus, it reduces UE power consumption for such UEs.

# WUS resource(s) configuration

Up to 4 orthogonal WUS resources including legacy WUS resource are supported. It was agreed that all of those 4 orthogonal WUS resources are to be placed adjacent to each other in both frequency and time domain. WUS resources allocations are illustrated in Figure 1 and numbered as . An eNB can configure the number of WUS resources. The UE needs to know how many and which resources are used by an eNB. In order to facilitate the signaling of WUS resources pattern from an eNB to UE(s), WUS resource reference numbering must be defined. For example, the WUS resource numbering is shown Figure 1. In this example, the WUS reference #0 is basically the WUS resource location as indicated in legacy rel-15 WUS freqLocation (RRC parameters in rel.15). Three possible legacy rel-15 WUS freqLocation are shown in Figure 1.

F (RB)

0,1, 2,3 ,4,5

#3

#0

#0

#3

#2

#1

#1

#0

#3

#2

#1

#2

(C)

(B)

(A)

Time

Figure 1: Illustration of 4 WUS resources allocation.

Proposal 1: Define the WUS resource reference numbering to facilitate the signalling of WUS resource pattern to the UE.

Proposal 2: The legacy WUS freqLocation is used as the WUS resource reference (index #0)

In RAN1 #98bis, it was agreed that no more than 3 bits are used for configuration of the pattern for WUS resource (i.e. the parameter name is gWUS-resource-config-DRX). We consider all 3 bits are used that explicitly tell the UE the number of WUS resources and the selected WUS resources with respect to the WUS reference resource numbering (e.g as illustrated in Figure 1). The usage of 3 bits to represent WUS resource pattern indicating the number of WUS resource(s) and used/active WUS resource(s) is shown in Table 1.

Table 1: WUS resource pattern represented in 3 bits signalling.

| Bit pattern | Num WUS resource | Used Resource |
| --- | --- | --- |
| 000 | 1 | #0 |
| 001 |  | #1 |
| 010 | 2 | #0, #1 |
| 011 |  | #0, #3 |
| 100 | 3 | #0,#1,#2 |
| 101 |  | #1,#2,#3 |
| 110 |  | #0,#1,#3 |
| 111 | 4 | #0,#1,#2,#3 |

Proposal 3: 3 bits WUS pattern is used to represent the number of WUS resources and the used WUS resource(s) by an eNB.

# Dynamic mapping of UE group to a WUS resource

Considering many UE groups can be supported in up to 4 WUS resources, the WUS grouping assignment to a WUS resource should be based on certain criteria or objectives. One of the objectives is to support fairness in paging performance such that blocking is avoided/minimized. Other objectives include reduced false wake-up and low UE power consumption. For example, a WUS resource is shared between legacy WUS UE (Rel-15) and group WUS UE (Rel-16). There could be the case, especially in the first deployment, the number of legacy WUS UEs is relatively much larger than the group WUS UEs. Consequently, the eNB may needs to frequently page legacy UEs due to the large number of legacy UEs. In this case, group WUS UE that shares the same WUS resource as that used by the legacy WUS UE has a small opportunity to be paged, i.e. causing blocking for that group WUS UE. In another example related to fairness, assume there are two TDM-ed WUS resources each with 2 UE groups, as illustrated in Figure 2. In this example, if being paged, UEs in group 3,4 will always “wake-up” earlier than the UEs in group 1,2, leading to increasing power consumption.

g3,g4

g1,g2

Paging

Tgap

#1

#0

Figure 2: Illustration of 2 TDM-ed WUS resources

Observation1: The WUS grouping assignment to WUS resources should target to support fairness in paging performance such that blocking is avoided, reduced false wake-up, and low UE power consumption.

We consider the dynamic mapping to change WUS resource of a UE group can be designed to provide fairness among UE groups in multiple WUS resources. If the groups can switch from one WUS resource to another then the fairness and equality of the power consumption issue as shown in Figure 2 can be resolved. The dynamic mapping of UE groups and WUS resources is illustrated in Figure 3. In this example, in one occasion UE groups (g3,g4) are assigned in WUS resource #1 and in another occasion UE groups (g3,g4) are assigned in WUS resource #0. Hence, there is a fairness on power consumption among UE groups.

g1,g2

g3,g4

Paging

Tgap

g3,g4

g1,g2

Paging

Tgap

#1

#0

#0

#1

Time

Figure 3: Illustration of UE groups switching across WUS resources

Observation 2: Dynamic mapping to change WUS resource of a UE group from time-to-time can result in achieving fairness of power consumption.

The aforementioned dynamic mapping operation can be beneficial (e.g. provide fairness). However, the dynamic mapping should also consider supporting mobility in WUS operation. During idle mode, a UE can move from one cell to another. Once the UE moves to a new cell, the UE must be able to identify which WUS resource is to be monitored in relation to its WUS group ID.

Observation 3: Dynamic mapping to change WUS resource of a UE group from time-to-time supports mobility in WUS operation, particularly when the UE moves from one cell to another.

An eNB must be configured with the number of supported groups and the number of WUS resources allocation as described earlier. In practice, it could be the case that all eNBs within an area have the same number of supported groups but with different numbers of WUS resources depending on the cell load or cell planning. In terms of UE operation, firstly the UE needs to know which group it belongs to, particularly if the grouping is based on paging probability. Secondly, the UE needs to know which WUS resource the UE needs to monitor and which WUS sequence is to be used. It has been agreed that a UE only needs to monitor one WUS resource. Hence, the UE needs to monitor one out of up to 4 WUS resources at a time. In order to support UEs with group WUS operation, both implicit and explicit parameters are required, particularly to support dynamic mapping to enable UE group to be changed among WUS resources as agreed in RAN1 #98. Implicit parameters shall be defined to minimize the number of bits required for explicit parameters between eNB and UE. Explicit parameters related to the dynamic mapping can be gWUS resource-config, gWUS numGroups, that have been agreed in RAN1#98bis.

There are up to 4 possible WUS resources in eMTC and dynamic mapping means the groups can be changed among WUS resources. To support the dynamic mapping, a WUS resource reference and WUS resource numbering must be defined. This can be used as reference/initial WUS resource index. The WUS resource (R0) as shown in Figure 1 can be considered as WUS reference. In practice, this can also be the case where WUS resource for a legacy UE is located.

An example of the configurability is shown in Figure 4. Here, there are 8 UE groups distributed equally and 4 shared WUS resources. UE groups are rotated across different WUS resources for each WUS/PO occasion and/or as a function of SFN number. In the first occasion UE WUS group1 (g1) monitors WUS resource index #0, in the subsequent occasion UE WUS group1 (g1) monitors WUS resource index #1, and so on.

g7,g8

g5,g6

g3,g4L

g1,g2

#0

#3

#1

#2

g1,g2

g7,g8

g5,g6,L

g3,g4

#0

#3

#1

#2

g3,g4

g1,g2

g7,g8,L

g5,g6

#0

#3

#1

#2

g5,g6

g3,g4

g1, g2, L

g7,g8

#0

#3

#1

#2

Time

Figure 4: Rotation of UE groups between WUS resources at different POs

Proposal 4: The dynamic mapping of a WUS resource for a WUS group should be based on the initial WUS resource index (reference) and time reference number (e.g. SFN, DRX cycle).

# 3	Conclusions

In this contribution, the following observations and proposals are made:

Observation1: The WUS grouping assignment to WUS resources should target to support fairness in paging performance such that blocking is avoided, reduced false wake-up, and low UE power consumption.

Observation 2: Dynamic mapping to change WUS resource of a UE group from time-to-time can result in achieving fairness of power consumption.

Observation 3: Dynamic mapping to change WUS resource of a UE group from time-to-time should supports mobility in WUS operation, particularly when the UE moves from one cell to another.

Proposal 1: 4 Define the WUS resource reference numbering to facilitate the signalling of WUS resource pattern to the UE.

Proposal 2: The legacy WUS freqLocation is used as the WUS resource reference (index #0)

Proposal 3: 3 bits WUS pattern is used to represent the number of WUS resources and the used WUS resource(s) by an eNB.

Proposal 4: The dynamic mapping of a WUS resource for a WUS group should be based on the initial WUS resource index (reference) and time reference number (e.g. SFN, DRX cycle).

# References

- RP-190770, Revised WID: Additional MTC enhancements for LTE, March 2019

- RAN1 #98bis Chairman notes, Chongqing, China, October 2019

- R2-1915235, On supporting UE-group wake-up signal for MTC/NB-IoT, Sony, RAN2#108, Reno, USA, November 2019.