3GPP TSG RAN WG1 #99		R1-1912339

Reno, USA, November 18th– 22nd, 2019

Agenda Item:	7.2.2.1.2

Source:	Sony

Title:	DL Signals and Channels for NR-U

Document for:	Discussion / Decision

Introduction

The following agreements were made so far.

Agreement:

When GC-PDCCH is configured, explicit indication via GC-PDCCH is supported as a mechanism to inform the UE that one or more carriers and/or LBT bandwidths are not available or available for DL reception, at least for slot(s) that are not at the beginning of DL transmission burst.

- FFS: Signalling details of the indication, including e.g., the time domain validity of the indication

- FFS: Whether and how to support the mechanism at the beginning of DL transmission burst

- FFS: Whether and how to handle the case when GC-PDCCH is not configured or not received by the UE

Agreement:

Support bit field corresponding to available LBT bandwidths in GC-PDCCH (add a bitmap in the GC-PDCCH DCI)

Agreement:

Add a COT duration bit-field per serving cell in GC-PDCCH, i.e., DCI format 2_0

- The following are configurable by RRC:

  - Presence of this bit-field

  - Location of this bit-field in the DCI

  - Length of this bit-field in the DCI

    - FFS: Whether a single value will suffice in which case, the length is not configurable

  - Encoding of the bit field value, i.e., what COT duration corresponds to which bit-field value

- If a UE receives this bit-field, it applies the knowledge about end-of-COT at least for the purpose of UL transmission LBT category switching in a gNB-acquired COT

- If this field is not present, UE should use SFI indication to determine end-of-COT (if SFI is available)

  - FFS: details for this SFI-based mechanism

- FFS: Whether the duration is encoded as e.g., total length or remaining length.

- FFS: Granularity the signalled duration

At RAN#84, NR-U prioritization listed the following issues as essential or optimizations:

Essential

- Channel occupancy time and frequency domain structure indication for LBE

- UE COT detection for FBE

- gNB control for the Cat 4 UL transmission (including CG) switching to cat 2 when fall in gNB COT

- Behavior when P/SP-CSI-RS fail to transmit due to LBT

Optimizations

- CSI-RS enhancement outside of DRS

  - More opportunities for CSI-RS to compensate the possible LBT failure

  - A-TRS directly QCL with SSB

In this contribution, we discuss the design of DL signals and channels for unlicensed NR.

Discussion

COT structure indication

Channel occupancy time and frequency domain structure indication for LBE

In the RAN1#98bis meeting, it was agreed that a bit-field indicates the COT duration, if this field is configured, and SFI indicates the COT duration if this field is not configured. It was FFS on the details for the SFI-based mechanism. In this case, to indicate end-of-COT, we consider whether a new state (e.g. “out-of-COT” state) should be introduced or an existing state (e.g. “flexible”) could be reused.

The benefit of introducing an “out-of-COT” state is that the UE is obviously aware of the end-of-COT, so that the UE can determine the LBT type for configured UL transmission as well as switch to PDCCH monitoring. However, the pattern of slot format (e.g. “F” after “U”) could indicate the end-of-COT. Therefore, no new state such as “out-of-COT” is necessary.

Proposal 1: No new state such as “out-of-COT” is necessary.

- If the field of COT duration is not present in GC-PDCCH, “flexible” should be used for the indication of “end-of-COT”.

Although no new state such as “out-of-COT” is necessary, additional slot formats, such as “F” after “U”, would be beneficial if “flexible” is used for the indication of the end-of-COT. In the current specification, there is no definition of slot format pattern such as “F” after “U” in the middle of a slot (as shown in the Appendix). If additional patterns of slot format, such as “F” after “U” is defined, partial UL transmission on the last slot of COT can be performed.

Proposal 2: Additional patterns of slot format should be defined by using reserved formats (56-254) for the SFI-based mechanism.

Another FFS point is “Whether the duration is encoded as e.g., total length or remaining length.”. In the case that the indicated COT duration is total length, the UE must always be aware of the start of COT, otherwise the end of COT cannot be estimated. We have concerns about mis-matching of the end of COT between gNB and UE in the case that UE fails to detect the start of COT. Although it could be resolved that CG-PDCCH carries additional information relating to the start of COT, this brings extra signaling overhead. On the other hand, by indicating only the remaining COT length, such mis-matching between gNB and UE would not occurr. Therefore, remaining COT length should be indicated by GC-PDCCH.

Proposal 3: The field of COT duration should indicate the remaining COT length from the slot where GC-PDCCH is received.

Conclusions

In this contribution, based on the above discussion we have the following proposals:

Proposal 1: No new state such as “out-of-COT” is necessary.

- If the field of COT duration is not present in GC-PDCCH, “flexible” should be used for the indication of “end-of-COT”.

Proposal 2: Additional patterns of slot format should be defined by using reserved formats (56-254) for the SFI-based mechanism.

Proposal 3: The field of COT duration should indicate the remaining COT length from the slot where GC-PDCCH is received.

References

- TS 38.213, v15.7.0

Appendix: Slot Format

Table 11.1.1-1: Slot formats for normal cyclic prefix [1]

| Format | Symbol number in a slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| 0 | D | D | D | D | D | D | D | D | D | D | D | D | D | D |
| 1 | U | U | U | U | U | U | U | U | U | U | U | U | U | U |
| 2 | F | F | F | F | F | F | F | F | F | F | F | F | F | F |
| 3 | D | D | D | D | D | D | D | D | D | D | D | D | D | F |
| 4 | D | D | D | D | D | D | D | D | D | D | D | D | F | F |
| 5 | D | D | D | D | D | D | D | D | D | D | D | F | F | F |
| 6 | D | D | D | D | D | D | D | D | D | D | F | F | F | F |
| 7 | D | D | D | D | D | D | D | D | D | F | F | F | F | F |
| 8 | F | F | F | F | F | F | F | F | F | F | F | F | F | U |
| 9 | F | F | F | F | F | F | F | F | F | F | F | F | U | U |
| 10 | F | U | U | U | U | U | U | U | U | U | U | U | U | U |
| 11 | F | F | U | U | U | U | U | U | U | U | U | U | U | U |
| 12 | F | F | F | U | U | U | U | U | U | U | U | U | U | U |
| 13 | F | F | F | F | U | U | U | U | U | U | U | U | U | U |
| 14 | F | F | F | F | F | U | U | U | U | U | U | U | U | U |
| 15 | F | F | F | F | F | F | U | U | U | U | U | U | U | U |
| 16 | D | F | F | F | F | F | F | F | F | F | F | F | F | F |
| 17 | D | D | F | F | F | F | F | F | F | F | F | F | F | F |
| 18 | D | D | D | F | F | F | F | F | F | F | F | F | F | F |
| 19 | D | F | F | F | F | F | F | F | F | F | F | F | F | U |
| 20 | D | D | F | F | F | F | F | F | F | F | F | F | F | U |
| 21 | D | D | D | F | F | F | F | F | F | F | F | F | F | U |
| 22 | D | F | F | F | F | F | F | F | F | F | F | F | U | U |
| 23 | D | D | F | F | F | F | F | F | F | F | F | F | U | U |
| 24 | D | D | D | F | F | F | F | F | F | F | F | F | U | U |
| 25 | D | F | F | F | F | F | F | F | F | F | F | U | U | U |
| 26 | D | D | F | F | F | F | F | F | F | F | F | U | U | U |
| 27 | D | D | D | F | F | F | F | F | F | F | F | U | U | U |
| 28 | D | D | D | D | D | D | D | D | D | D | D | D | F | U |
| 29 | D | D | D | D | D | D | D | D | D | D | D | F | F | U |
| 30 | D | D | D | D | D | D | D | D | D | D | F | F | F | U |
| 31 | D | D | D | D | D | D | D | D | D | D | D | F | U | U |
| 32 | D | D | D | D | D | D | D | D | D | D | F | F | U | U |
| 33 | D | D | D | D | D | D | D | D | D | F | F | F | U | U |
| 34 | D | F | U | U | U | U | U | U | U | U | U | U | U | U |
| 35 | D | D | F | U | U | U | U | U | U | U | U | U | U | U |
| 36 | D | D | D | F | U | U | U | U | U | U | U | U | U | U |
| 37 | D | F | F | U | U | U | U | U | U | U | U | U | U | U |
| 38 | D | D | F | F | U | U | U | U | U | U | U | U | U | U |
| 39 | D | D | D | F | F | U | U | U | U | U | U | U | U | U |
| 40 | D | F | F | F | U | U | U | U | U | U | U | U | U | U |
| 41 | D | D | F | F | F | U | U | U | U | U | U | U | U | U |
| 42 | D | D | D | F | F | F | U | U | U | U | U | U | U | U |
| 43 | D | D | D | D | D | D | D | D | D | F | F | F | F | U |
| 44 | D | D | D | D | D | D | F | F | F | F | F | F | U | U |
| 45 | D | D | D | D | D | D | F | F | U | U | U | U | U | U |
| 46 | D | D | D | D | D | F | U | D | D | D | D | D | F | U |
| 47 | D | D | F | U | U | U | U | D | D | F | U | U | U | U |
| 48 | D | F | U | U | U | U | U | D | F | U | U | U | U | U |
| 49 | D | D | D | D | F | F | U | D | D | D | D | F | F | U |
| 50 | D | D | F | F | U | U | U | D | D | F | F | U | U | U |
| 51 | D | F | F | U | U | U | U | D | F | F | U | U | U | U |
| 52 | D | F | F | F | F | F | U | D | F | F | F | F | F | U |
| 53 | D | D | F | F | F | F | U | D | D | F | F | F | F | U |
| 54 | F | F | F | F | F | F | F | D | D | D | D | D | D | D |
| 55 | D | D | F | F | F | U | U | U | D | D | D | D | D | D |
| 56 – 254 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 255 | UE determines the slot format for the slot based on tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, if any, on detected DCI formats |  |  |  |  |  |  |  |  |  |  |  |  |  |