3GPP TSG RAN WG1 Meeting #99	R1-191yyyy

Reno, USA, November 18th – 22nd, 2019

Agenda Item:	7.2.10.3

Source: 	Sony

Title:	Remaining issues on UE measurements for NR Positioning

Document for:	Discussion / Decision

# Introduction

A new work item “NR positioning support” was approved In RAN plenary #83 [1] . During study item phase, NR positioning techniques (e.g. DL-TDOA, DL-AoD) were presented and evaluated. The preliminary results can be found in [2] .

In RAN1 meeting #98bis, the following selected agreements were made [3]:

Agreement:

- UE can be configured to measure DL PRS RSTD, UE Tx-Rx time difference, and DL PRS RSRP in up to X5 total number of DL PRS resources within

- Up to X1 positioning frequency layers, FFS: X1

- Up to X2 TRPs per positioning frequency layer, FFS: X2

- Up to X3 DL PRS resource sets per TRP, FFS: X3

- Up to X4 DL PRS resources per PRS resource set, FFS: X4

- FFS: X5

- UE can be configured to transmit up to Y1 SRS resources for positioning per SRS resource Set

- UE can be configured to transmit up to Y2 SRS resources for positioning across all SRS resource Sets

- UE can be configured to transmit up to total Y3 SRS Resource Sets for positioning

- FFS: Y1, Y2, Y3

- FFS: whether Y1 and/or Y2 and/or Y3 are per BWP or all BWPs

- A limit on the maximum number of DL PRS resources configured to the UE for all TRPs within a measurement window is defined.

- This limit can be signalled as a UE capability.

- FFS: the relationship of maximum numbers of DL PRS resources with the following factors

    - the type of the measurement window, e.g., sliding window

    - the maximum number of DL PRS OFDM symbols

    - the maximum measurement bandwidths

- UE can be configured to measure and report up to [M] DL PRS RSTD measurements with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and the M measurements being performed on the same pair of TRPs subject to UE capability

- All the RSTD measurements in a single report should have a single reference timing

- Note: Each RSTD measurement is between DL PRS Resources corresponding to different TRP IDs.

- M=[3]

- UE can be configured to measure and report up to N (> 1) DL PRS RSRP measurements on different DL PRS resources from the same TRP

- N=[3]

- A UE measurement can be associated with a time stamp.

- For DL RSTD and RSRP measurements, the time stamp can include at least the SFN

    - FFS: SFN corresponds to the reference DL PRS resource or DL PRS resource sets (can be determined by the UE)

- For Rx-Tx measurements, the time stamp can include the SFN, as well as the slot number for a subcarrier spacing

    - FFS: these values correspond to the reference DL PRS resource or DL PRS resource set (can be determined by the UE)

# In this contribution, we discuss our view on UE positioning measurements for downlink-based positioning solutions in NR.

# Discussions

In RAN1#98bis, we discussed the number of DL PRS resources to be measured by UE (i.e. X1, X2, …, X5) and also the configured SRS resources to be transmitted by the UE (i.e. Y1, Y2, Y3). We consider those parameters (X, Y) depends on the UE capability and the target positioning accuracy/latency. A UE with limited capability in term of operating bandwidth may not measure some of the frequency layers. On the other hand, a UE with tight positioning requirement may require more DL PRS resources.

Proposal 1: The number of DL PRS resources to be measured by UE (i.e. X1, X2, …, X5) and the configured SRS resources to be transmitted by the UE (i.e. Y1, Y2, Y3) shall consider UE capability and the target positioning requirements (i.e. horizontal/vertical accuracy & latency).

In order to ensure the UE can perform positioning measurement efficiently, the configuration on PRS multiplexing should be introduced within preconfigured PRS resources, which can accommodate the PRS transmitted from multiple beams and from multiple gNBs. Furthermore, a set of PRS resources (e.g. multiple resource elements carrying PRS) can represent the transmit beam of PRS from a gNB and basically the angle of departure (AoD) or. This is illustrated in Figure 1. Here, a gNB is illustrated with possible 4 different beam directions and it translates to 4 different PRS resources.

Figure 1: Illustration of PRS multiplexing supporting multiple beams

During the reception of a configured set of PRS resources, the UE performs measurement for each gNB. Furthermore, the measurement for each gNB can results in multiple measurements in case the gNB has been configured with multiple set of PRS resources where each PRS resources represent certain beam direction (AoD). The UE can perform timing measurements (RSTD) and/or received power measurements (RSRP) for each set of PRS resources. It has been agreed that a UE can be indicated to report the DL PRS Resource ID(s) or the DL PRS Resource set ID(s) associated with the DL PRS Resource (s) or the DL PRS Resource set(s). Hence the reported RSTD / RSRP measurements can be accompanied with the corresponding DL PRS Resource ID(s) or the DL PRS Resource set ID(s).	The DL PRS resource ID(s) or the DL PRS Resource set ID(s) is essentially DL-AoD technique to be calculated at the LS.

Proposal 2: DL-AoD technique is essentially based on the PRS resource ID that can be obtained from DL PRS RSRP measurements and/or DL PRS RSTD measurements.

Referring to the configured PRS multiplexing and beam operation as shown in Figure 1, the UE can perform these set of measurements as shown in Figure 2. The measurement can be an RSTD measurement and/or RSRP measurement. In this example, if the UE is located between gNB1 - beam 1 and gNB 2 – beam 3 then the UE is expected to obtain the best measurements based on those beams. Assuming the UE has known knowledge of the PRS configuration, the UE can identify the beam index in the form of resource ID of that measurement.

Meas (gNB1,b4)

Meas (gNB1,b3)

Meas (gNB1,b2)

Meas (gNB1,b1)

Meas (gNB2,b4)

Meas (gNB2,b3)

Meas (gNB2,b2)

Meas (gNB2,b1)

Figure 2: UE measurements data and the selected report

Proposal 3: UE can report one or the combination of RSTD and RSRP measurements together with the corresponding PRS resource ID / PRS resource set(s) ID.

As one of the forms of the indication to the UE, we consider it can be based on the selected gNBs in order to limit/control the measurement report size. For example, a UE can report the measurement report of selected gNBs. The selection can be based on the measurement quality, for example best N measurement reports from a set of gNBs. This operation would be beneficial in order to improve the efficiency of positioning measurement report.

Proposal 4: Support to limit the positioning measurement report (i.e. based on the selected gNBs/TRPs).

In the actual deployment, it is common that the PRS with line-of-sight (LOS) may not always be available, particularly in the urban/indoor scenarios due to the reflection. The received DL-PRS could be mostly dominated by NLOS components. The UE RSTD report based on NLOS components could lead to high positioning estimation error. By providing RSTD reporting with multipath measurements would enable the LS to have a better judgement on proper RSTD measurement result and result in improving UE positioning estimation accuracy. Furthermore, LTE positioning supports RSTD reporting with multipath measurements. Hence, NR positioning without multipath measurements can compromise NR positioning accuracy.

Proposal 5: Support RSTD reporting with multipath measurements

# Conclusion

In this contribution, we discussed our view on downlink-based positioning particularly on the UE measurement aspects. We have made the following proposals:

Proposal 1: The number of DL PRS resources to be measured by UE (i.e. X1, X2, …, X5) and the configured SRS resources to be transmitted by the UE (i.e. Y1, Y2, Y3) shall consider UE capability and the target positioning requirements (i.e. horizontal/vertical accuracy & latency).

Proposal 2: DL-AoD technique is essentially based on the PRS resource ID that can be obtained from DL PRS RSRP measurements and/or DL PRS RSTD measurements.

Proposal 3: UE can report one or the combination of RSTD and RSRP measurements together with the corresponding PRS resource ID / PRS resource set(s) ID.

Proposal 4: Support to limit the positioning measurement report (i.e. based on the selected gNBs).

Proposal 5: Support RSTD reporting with multipath measurements

# References

- RP-190752, “New WID: NR Positioning Support”

- TR38.855,” Study on NR positioning support”

- Chairman notes RAN1 Meeting #98bis

- TS36.214