3GPP TSG RAN WG1 #99	R1-1912583

Reno, USA, November 18th – 22nd, 2019

Agenda item:	7.2.4.3

Source:			ITRI

Title:			Remaining Issues on NR Sidelink Synchronization for 5G V2X

Document for:	Discussion and Decision

- Introduction

NR V2X SI was completed in February 2019, and the new NR V2X work item [1] was approved in RAN#83. Sidelink synchronization mechanism is one of the objectives for the new work item of NR V2X, and the sidelink synchronization mechanism related objectives can be shown as follows:

4.1 Objective of SI or Core part WI or Testing part WI

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage.

- Support of sidelink signals, channels, bandwidth part, and resource pools [RAN1, RAN2]

- Resource allocation [RAN1, RAN2]

…….

- Sidelink synchronization mechanism as per the study outcome [RAN1, RAN2]

- Procedures selecting synchronization reference

- S-SSB and procedures to transmit and receive it, including when GNSS and gNB/eNB are unavailable

- Use of RS for sidelink synchronization if specification impact is identified

- Solution for ‘not co-channel’ in-device coexistence between LTE and NR sidelinks

……….

- Sidelink physical layer procedures as per the study outcome

……………..

For the NR V2X SI, the sidelink synchronization mechanism related agreements at RAN1#94 [2] are shown as follows,

Agreements:

- NR V2X Sidelink Synchronization includes at least the following

  - Sidelink synchronization signal(s)

  - PSBCH

  - Sidelink synchronization sources and procedure(s)

    - Study potential synchronization sources –GNSS, gNB, eNB, UE, LTE UE

      - Note: this doesn’t mean all of them are to be supported

Besides, the sidelink synchronization mechanism related agreements at RAN1#94bis [3] are shown as follows,

Agreements:

At least GNSS, gNB, NR UE, and eNB are supported as the synchronization source for NR V2X.

eNB as a synchronization source for NR V2X UEs supporting LTE Uu/PC5 or Uu only (no change to the eNB behaviour)

Whether a source is supported is for further NR V2X UE capability consideration

Agreements:

- The design of NR V2X sidelink synchronization signals and PSBCH uses NR SSB structure as the starting point with the following properties,

  - NR V2X synchronization signals include sidelink PSS (S-PSS) and sidelink SSS (S-SSS) and are structured with PSBCH in a block format (S-SSB)

Moreover, the sidelink synchronization mechanism related agreements at RAN1#95 [4] are shown as follows,

Agreements:

- The aspects of synchronization sequence for NR V2X to be considered for the evaluation include,

- The length of S-PSS and S-SSS sequences

- If and how to distinguish from NR Uu PSS and SSS sequences

Agreements:

- Using the below table as a starting point for evaluation assumptions for sidelink synchronization LLS.

  - Detection probability of S-PSS/S-SSS

  - Decoding BLER of PSBCH

  - Check further offline regarding UE speeds (absolute vs. relative, including current channel model assumptions in the TR)  on Friday, confirmed to be relative speed and thus, the speeds in the table below need to be doubled

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 6 GHz | 30 GHz |
| Channel Model | CDL channel models |  |
| Subcarrier Spacing(s) | 15, 30, 60 kHz | 60, 120 kHz |
| SNR Range | > -6 dB | > -6 dB |
| UE Speed | 3 km/h, 120 km/h (mandatory) 30km/h, 250 km/h (optional) | 3 km/hr, 120 km/h (mandatory) |
| Interference model | Scenario 1: no interference Scenario 2: effect of interference includes in the model | Scenario 1: no interference |
| Initial Frequency Offset | TX: Uniform distribution within [-5, 5] ppm of nominal carrier frequency RX: Uniform distribution within [-5, 5] ppm of nominal carrier frequency |  |

Furthermore, the sidelink synchronization mechanism related agreements at RAN1 Ad-Hoc 1901 [5] are shown as follows,

Agreements:

- For NR SLSS, as the baseline:

  - The sequence type for S-PSS is the same type as the M-sequence used for NR-PSS

  - The sequence type for S-SSS is the same type as the Gold sequence for NR-SSS

Agreements:

- At least for single-carrier operation:

- For the SL synchronization procedure, each type of synchronization reference has a respective sync priority

  - FFS the priority between eNB and gNB (if necessary)

- For the SL synchronization procedure, among the available references, a UE selects the synchronization reference with the highest priority as the reference to derive its transmission timing

  - FFS other potential usage

  - FFS how to handle the case when there are two or more references of a same priority to be selected as the highest priority

Then, the sidelink synchronization mechanism related agreements at RAN1#96 [6] are shown as follows,

Agreements:

- NR V2X supports using a sidelink RS for synchronization purpose

  - Applicable only on unlicensed (ITS) carrier with no network deployment on this carrier

  - This RS is not a standalone RS and not part of SLSS.

  - This RS will not appear in the synchronization procedure for the selection of sync sources.

  - RS used for the synchronization purpose would not impact any sidelink RS design

For the NR V2X WI, the sidelink synchronization mechanism related agreements at RAN1#96bis [7] are shown as follows,

Agreements:

- RAN1 will consider the design of NR SLSS where

  - S-PSS of LTE and NR candidates have similar PAPR.

  - S-SSS of LTE and NR candidates have similar PAPR.

  - SLSS design should take into consideration PAPR difference between S-PSS and S-SSS.

Agreements:

- In NR V2X, S-SSB bandwidth is 11RBs.

  - PSBCH spans 11RBs.

- The S-SSB is designed following combination 1.

  - Length-127 M-sequences for S-PSS and length-127 Gold sequences for S-SSS

  - Two symbols are used for each of S-PSS and S-SSS, respectively.

Agreements:

- For the evaluation of PSBCH performance, the payload size of NR V2X PSBCH is 56 bits including 24 bits of CRC.

For the NR V2X WI, the sidelink synchronization mechanism related agreements at RAN1#97 [8] are shown as follows,

Working assumption:

- For the NR SLSS,

  - Same sequence is used for both symbols of S-PSS

  - Same sequence is used for both symbols of S-SSS

Agreements:

The impact on detection probability performance of having or not having a transient period between S-PSS and S-SSS symbols is used to evaluate the following:

- Alt 1: S-PSS symbols and S-SSS symbols are adjacent.

- Alt 2: S-PSS symbols and S-SSS symbols are not adjacent.

FFS (aim to conclude this week – see below)

- The power difference between S-PSS and S-SSS symbols.

- The transient duration.

Agreements:

The following parameters are assumed for evaluation:

- Power Difference for S-PSS and S-SSS symbols:

  - Opt.1) MPR values: S-PSS = 0 dB, S-SSS = 3 dB;

  - Opt.2) MPR values: S-PSS = 3 dB, S-SSS = 3 dB

  - Opt.3) companies to report the assumed MPR values

- Transient period is

  - 10us for FR1; 5us for FR2

  - Waveform puncturing during the transient period

- S-PSS detection search window: 80ms and 160ms

For the NR V2X WI, the sidelink synchronization mechanism related agreements at RAN1#98 [9] are shown as follows,

Agreements:

- For S-SSB pattern design, the first symbol is PSBCH

  - Note: no specific symbol(s) reserved for AGC tuning

Agreements:

- The number of NR V2X SSID is 672 with the combination of {2 S-PSS candidates * 336 S-SSS candidates}.

Agreements:

- NR S-SSB structure for NCP is as follows:

  - For the case of ECP, the structure is the same as the above except that the number of PSBCH symbols after S-SSS is only 6

Agreements:

- 160ms is supported as the S-SSB periodicity for all SCS.

- The number of S-SSB transmissions within one S-SSB period is (pre)configurable. The following values with change marks are further agreed:

Note: the values in bracket are subject to further discussion regarding potential removal all-together

For FR1:

  - For 15kHz SCS, {1, [2]}

  - For 30kHz SCS, {1, 2, [4]}

  - For 60kHz SCS, {1, 2, 4, [8]}

For FR2:

  - For 60kHz SCS, {1, 2, 4, 8, 16, 32}

  - For 120kHz SCS, {1, 2, 4, 8, 16, 32, 64}

FFS details for the multiple S-SSB transmissions within one S-SSB period (the set of slots, repetition?, etc.)

Agreements:

- RS based synchronization can be supported by UE implementation without RAN1 specification impact.

For the NR V2X WI, the sidelink synchronization mechanism related agreements at RAN1#98bis [10] are shown as follows,

Agreements:

- When two or more UE synchronization sources have same priority, S-SSB RSRP is used for the selection of the synchronization source.

Agreements:

- The following fields with “green” background are agreed

| PSBCH contents | Number of bits | Notes |
| --- | --- | --- |
| DFN |  |  |
| Indication of TDD configuration |  | System-wide information, e.g. TDD-UL-DL common configuration and/or potential SL slots |
| SL-BWP information |  |  |
| In-coverage indicator |  | FFS how to indicate it. |
| Type of sync source |  |  |
| Slot index within a subframe |  |  |
| ??? |  |  |
| CRC | 24 |  |
| Total bits |  |  |

Agreements:

- The procedure for signalling, identifying priority for one or more synchronization references and selecting the synchronization reference from the LTE is re-used (as a working assumption) for NR SL

  - FFS SSIDs used for each priority

  - FFS other potential impacts due to P3/P4/P5

- FFS whether there is an issue with prioritization among references of the same priority

Agreements:

672 SL-SSIDs are divided into 2 sets to indicate different synchronization priorities following a similar approach as in LTE-V2X:

- Set id_net {0, 1, …, 335}

- Set id_oon{336, 337, 338, …, 671}

- The usage of 0 is the same as 0 as in LTE

- The usage of 336 is the same as 168 as in LTE

- The usage of 337 is the same as 169 as in LTE

Agreement:

- Do not support 2/4/8 as the number of S-SSB transmissions within one S-SSB period for 15/30/60 KHz SCS for FR1, respectively.

This paper is the revision of R1-1910860. In this paper, we will focus on sidelink synchronization issues and mechanisms discussion for NR V2X. At first, we will introduce the issues and mechanisms on synchronization procedures including synchronization reference selection, eNB/gNB timing misalignment, and synchronization signal forwarding. Then, we will discuss the issues and mechanisms on the design of sidelink synchronization signal block (S-SSB) for NR V2X, especially for the considerations of sidelink PSS (S-PSS) design, the working assumption in NR V2X WI at RAN1#97 meeting, and the high speed case agreed in RAN1#95 meeting. Finally, we will investigate the sidelink synchronization issues due to different subcarrier spacing (SCS).

- Discussion

In this section, we will discuss the issues and mechanisms on the synchronization procedures, including synchronization reference selection, eNB/gNB timing misalignment, and synchronization signal forwarding. Then, we will introduce the design of S-SSB with the considerations of S-PSS design, the working assumption in NR V2X WI at RAN1#97 meeting, and the high speed case agreed in RAN1#95 meeting. Finally, sidelink synchronization issues due to different SCS is discussed.

2.1 Issues and mechanisms on synchronization procedures

In NR V2X synchronization discussion, RAN1 reached the following agreements by email discussion [98-NR-09], and RAN1 will send an LS to RAN2/4 to inform this agreement:

- For confirmation of the working assumption of synchronization priority rules, eNB/gNB should be included into the priority order of GNSS-based synchronization. GNSS-based synchronization; gNB/eNB-based synchronization. • P0: GNSS • P1: UE directly synchronized to GNSS • P2: UE indirectly synchronized to GNSS • P3: gNB/eNB • P4: UE directly synchronized to gNB/eNB • P5: UE indirectly synchronized to gNB/eNB • P6: the remaining UEs have the lowest priority.; • P0’: gNB/eNB • P1’: UE directly synchronized to gNB/eNB • P2’: UE indirectly synchronized to gNB/eNB • P3’: GNSS • P4’: UE directly synchronized to GNSS • P5’: UE indirectly synchronized to GNSS • P6’: the remaining UEs have the lowest priority.. - When GNSS-based synchronization is (pre-)configured, it is also supported to disable the use of P3, P4, P5 by (pre-)configuration (i.e., a UE skips P3, P4, P5 in synchronization reference selection procedure). Details are up to RAN2.

Besides, the use of GNSS-based synchronization and gNB/eNB-based synchronization can be based on that in Rel-14 LTE V2X, where the related synchronization source priority agreements in LTE-based V2X Services WI in [11] are shown as follows,

Agreements:

- If the UE detects no eNB in a carrier which is (pre-)configured as the carrier which potentially includes eNBs used as sync reference, the following priority rules should be applied:

  - P1: GNSS

  - P2: the following UE has the same priority:

    - UE directly synchronized to GNSS

    - UE directly synchronized to eNB

  - P3: the following UE has the same priority:

    - UE indirectly synchronized to GNSS (if RAN1 decides to differentiate between direct and indirect synchronization to GNSS)

    - UE indirectly synchronized to eNB

  - P4: the remaining UEs have the lowest priority.

- The priority order between P1 and P2 can be (pre-)configured.

  - Note: It is RAN1 understanding that P1 will be prioritized over P2 if the eNB prioritizes GNSS over eNB timing.

Agreements:

- If the UE detects an eNB in a carrier which is (pre-)configured as the carrier which potentially includes eNBs used as sync reference,

  - If GNSS is prioritized over eNB by eNB configuration, UE directly or indirectly synchronized to GNSS has a higher priority than eNB.

From last RAN1 meeting, it is agreed that the procedure for signalling, identifying priority for one or more synchronization references and selecting the synchronization reference from the LTE is re-used (as a working assumption) for NR SL. Hence, we propose that GNSS–based synchronization in the synchronization source priority table should be adopted if the UE detects no gNB/eNB in a carrier which is (pre-)configured as the carrier which potentially includes gNBs/eNBs used as sync reference. Similarly, we also propose that gNB/eNB-based synchronization or GNSS–based synchronization in the synchronization source priority table should be adopted by gNB/eNB pre-configuration if the UE detects an gNB/eNB in a carrier which is (pre-)configured as the carrier which potentially includes gNBs/eNBs used as sync reference.

Proposal 1: The GNSS–based synchronization in the synchronization source priority table should be adopted if the UE detects no gNB/eNB in a carrier.

Proposal 2: The gNB/eNB-based synchronization or the GNSS-based synchronization in the synchronization source priority table should be adopted by gNB/eNB pre-configuration if the UE detects a gNB/eNB in a carrier.

Moreover, there will be a timing misalignment issue with prioritization among references of the same priority. For example, when there are two synchronization references with same priority, two NR V2X UEs located between the two synchronization references may synchronize to the two different synchronization references, and then timing misalignment issue will occur when the two NR V2X UEs want to communicate to each other. SSIDs used for each priority can mitigate the timing misalignment issue, because the synchronization references will not have the same priority. The mapping between SSIDs and priorities should be pre-configured, where gNB should assign an SSID to a mode 1 NR V2X UE, and a mode 2 NR V2X UE can randomly select an SSID for itself.

Proposal 3: There will be a timing misalignment issue with prioritization among references of the same priority, and SSIDs used for each priority can mitigate the timing misalignment issue. The mapping between SSIDs and priorities should be pre-configured, where gNB should assign an SSID to a mode 1 NR V2X UE, and a mode 2 NR V2X UE should randomly select an SSID for itself.

If the same synchronization source priority is pre-configured for eNB and gNB, the NR V2X UE can choose eNB or gNB as synchronization reference by itself when the NR V2X UE receives synchronization signals from both eNB and gNB. As mentioned above, the NR V2X UE can select the synchronization reference that the received synchronization signal with largest signal strength is transmitted by it. However, for this case, the NR V2X UEs may select different synchronization sources as synchronization references when the NR V2X UEs are located around the middle region between the eNB coverage and gNB coverage. Then, the NR V2X UEs may follow different synchronization reference timings and the timing misalignment issue may occur for the NR V2X communication among these NR V2X UEs, when eNB and gNB have different reference timings. Hence, some mechanisms are needed for the timing misalignment issue between eNB and gNB. For example, some coordination mechanisms between eNB and gNB are needed to coordinate the reference timings between eNB and gNB. Therefore, we propose that some coordination mechanisms between eNB and gNB are needed to coordinate the reference timings between eNB and gNB if the same synchronization source priority is pre-configured for eNB and gNB.

Proposal 4: In order to overcome timing misalignment issue between eNB and gNB, some coordination mechanisms between eNB and gNB are needed to coordinate the reference timings between eNB and gNB if the eNB and gNB have different reference timings but have a same synchronization source priority.

After a NR V2X UE selects a synchronization source and follows its reference timing, the NR V2X UE needs to decide to become a new synchronization source and forward the synchronization signal or not. Sidelink synchronization signal (SLSS) forwarding/transmission can extend the coverage of the network synchronization. However, when the number of misaligned reference timings distributed by SLSS transmission increases, interference among misaligned timings will also become severe. Hence, to keep the number of synchronization sources to a minimum can eliminate such interference and maintain the unity of a synchronous network. Therefore, we propose that sidelink synchronization signal forwarding should be considered to extend coverage for NR V2X, the details are FFS.

Proposal 5: Sidelink synchronization signal forwarding should be considered to extend coverage for NR V2X, where the details are FFS.

2.2 Issues and mechanisms on the design of Sidelink SS Block

From the sidelink synchronization mechanism related agreements shown in section 1, the design for NR V2X synchronization signals, including sidelink PSS (S-PSS)/ sidelink SSS (S-SSS) and PSBCH, can refer to that agreed in NR SI/WI.

The agreements and working assumption related to sequence design for NR-PSS and NR-SSS in Rel-15 NR WI in [12-13] are shown as follows,

Agreements:

- Confirm following working assumptions on NR-PSS as agreements

  - Number of PSS sequences: 3

    - PSS sequence details:

      - Frequency domain-based pure BPSK M sequence (fixing the time/freq. offset ambiguity)

        - 1 polynomial: Decimal 145 (i.e. g(x) = x7 + x4 + 1)

        - In freq. domain 3 cyclic shifts (0, 43, 86) to get the 3 PSS signals

        - Initial poly shift register value: 1110110

Working assumption:

- NR-SSS sequence design is 1 polynomial with 127 cyclic shifts, and 1 another polynomial with 9 cyclic shifts

    - Two generator polynomials will be defined for m-sequences, and cyclic shift according to NR-cell ID is applied to each m-sequence

    - Two polynomials are generated by g0(x) = x7 + x4 + 1 and g1(x) = x7 + x + 1

      - Initial state is [0000001]

    - The cyclic shift values m0 and m1 are jointly determined by the cell IDs carried by NR PSS (i.e., NID(2)=0, 1, 2) and NR SSS (i.e., NID(1)=0, 1, …, 335)), where the cell ID is given by NIDcell=3NID(1)+NID(2)

      - m0=3NID(1)112+NID(2)

      - m1=NID1   mod 112+m0+1

Agreement:

Confirm Working Assumption for NR-SSS design from RAN1#89.

In RAN1 Ad-Hoc 1901 meeting, it is agreed that the sequence type for S-PSS is the same type as the M-sequence used for NR-PSS and the sequence type for S-SSS is the same type as the Gold sequence used for NR-SSS. Besides, in RAN1#96bis meeting, it is agreed that length-127 M-sequences are adopted for S-PSS and length-127 Gold sequences are adopted for S-SSS. Moreover, it is agreed in last RAN1 meeting that there are 2 S-PSS candidates and 336 S-SSS candidates. However, some parameters in NR-PSS and NR-SSS, including the values of cyclic shift, should be revised in order to meet the purposes and requirements for NR V2X. In the following, we will show the simulation results of the peak-to-average power ration (PAPR), cubic metric (CM), and autocorrelation for LTE-PSS and NR-PSS. Besides, based on the simulation results, we will provide our viewpoint for the design of S-PSS.

Large PAPR/ CM will introduce non-linear distortion when the transmitted signals pass through the power amplifier. Fig. 1(a) and 1(b) show the simulated PAPR & raw CM values in dB for LTE-PSS with all root indices and NR-PSS with all cyclic shift indices, respectively. The LTE-PSS are the Zadoff-Chu sequences with length 63, and the NR-PSS is the mentioned above M-sequences with length 127. From the simulation results, we can observe that several conjugated root pairs {(1,62),(8,55),(16,47),(25,38),(29.34), (31,32)} for LTE-PSS are with smaller values of both PAPR and raw CM. However, several conjugated root pairs {(9,54),(18,45),(27,36)} for LTE-PSS are with larger values of both PAPR and raw CM. Compare to LTE-PSS, the values of both PAPR and raw CM for NR-PSS with all cyclic shifts are smaller.

Observation 1: LTE-PSS only with some specific conjugated root pairs have good PAPR and raw CM properties.

Observation 2: NR-PSS with all cyclic shifts have good PAPR and raw CM properties.

(a)                                                 (b)

Figure 1. PAPR & raw CM values for (a) LTE-PSS with all root indices and (b) NR-PSS with all cyclic shift indices

Besides, we use the peak to side lobe ratio (PSR) to evaluate the autocorrelation of LTE-PSS and NR-PSS, where the PSR is defined as the ratio of the first tallest autocorrelation peak value to the second tallest autocorrelation peak value. The autocorrelation is better when the corresponding PSR is larger, and the sequences with good autocorrelation property will be robust to interference caused by carrier frequency offset values (CFO) and can be detected easily in the receiver. Fig. 2(a) and 2(b) show the simulated PSR values in dB at different CFOs for LTE-PSS with all root indices and NR-PSS with all cyclic shift indices, respectively. From the simulation results, we can observe that the PSR values of LTE-PSS with all roots are sensitive to CFOs. Fortunately, the PSR values of NR-PSS with all cyclic shifts are robust to CFOs when CFOs are less than 0.5 subcarrier spacing (SCS).

Observation 3: The autocorrelation property of LTE-PSS with all roots are sensitive to CFOs, while the autocorrelation property of NR-PSS with all cyclic shifts are robust to CFOs when the CFOs are less than 0.5 SCS.

(a)                                                  (b)

Figure 2. PSR of Autocorrelation at different CFOs for (a) LTE-PSS with all root indices and (b) NR-PSS with all cyclic shift indices

Proposal 6: The sequence types and length for NR-PSS and NR-SSS are agreed to be reused for S-PSS and S-SSS in NR V2X. However, some parameters, including the values of cyclic shift, should be revised for S-PSS and S-SSS in order to meet the purposes and requirements for NR V2X, where the details are FFS.

In 5G NR, the PBCH payload consists of MIB and the additional timing information. The NR PBCH content can be classified into the following information,

- System timing: SFN, half radio frame;

- TDD configuration

- Priority information

- Initial access: subCarrierSpacingCommon, ssb-SubcarrierOffset, dmrs-TypeA-Position, pdcch-ConfigSIB1, MSB of the subcarrier offset between SSB and the common resource block grid (FR1) / SSB index(FR2);

- Cell related: cellBarred, intraFreqReselection;

In RAN1#96bis meeting, it is agreed that the payload size of NR V2X PSBCH is 56 bits including 24 bits of CRC for the evaluation of PSBCH performance. Besides, at least DFN, indication of TDD configuration and CRC should be included in NR V2X PSBCH, as shown in the agreement of NR V2X WI in last RAN1 meeting. In order to meet the purposes and requirements for NR V2X, the content of PSBCH for NR V2X can reuse that in PBCH for 5G NR, and the details are FFS.

Proposal 7: The content of PSBCH for NR V2X can reuse that in PBCH for 5G NR, and the details are FFS.

Besides, the working assumption in NR V2X WI at RAN1#97 meeting is that same sequence is used for both symbols of S-PSS and same sequence is used for both symbols of S-SSS for the NR SLSS. From our viewpoint, two different sequences used in two different symbols for S-PSS and also that for S-SSS do not have obvious advantages for the performance of S-PSS and S-SSS detection in the receiver. And same sequence is used for both symbols of S-PSS and same sequence is used for both symbols of S-SSS are simple and with lower computation complexity in the receiver. Hence, we confirm the working assumption that same sequence is used for both symbols of S-PSS and same sequence is used for both symbols of S-SSS for the NR SLSS.

Proposal 8: We confirm the working assumption that same sequence is used for both symbols of S-PSS and same sequence is used for both symbols of S-SSS for the NR SLSS.

Moreover, for the power difference between S-PSS and S-SSS symbols, we think that different power should be per-configured for S-PSS and S-SSS symbols when the PAPR values of S-PSS symbol and S-SSS symbol are different. In order to maintain the average signal to noise power ratio (SNR) for a symbol, larger power should be configured for the symbol when the symbol has larger PAPR value and larger power back off is used for the symbol to alleviate non-linear distortion due to large PAPR, and the details are FFS.

Proposal 9: Different power should be per-configured for S-PSS and S-SSS symbols when the PAPR values of S-PSS symbol and S-SSS symbol are different, and the details are FFS.

Another important issue for the design of S-SSB is the adoption of subcarrier spacing in the S-SSB, especially in the cases of high UE speed & high carrier frequency that may introduce severe inter-carrier interference and hence significantly degrade system performance.

It is agreed that S-SSB has the same numerology, which includes SCS and CP length, as that of control and data channels for a given carrier in RAN1#95 meeting. Besides, in RAN1#95 meeting, several values of carrier frequency, subcarrier spacing, and UE speed are also agreed in the evaluation assumptions for sidelink synchronization LLS including subcarrier spacing of 15, 30, 60 kHz and UE speed of up to 120km/h (mandatory) and 250km/h (optional) are used for carrier frequency of 6GHz. Also, it is agreed that subcarrier spacing of 60, 120 kHz and UE speed of up to 120km/h (mandatory) are used for carrier frequency of 30GHz.

In the cases of high UE speed & high carrier frequency for NR V2X, for example carrier frequency is 6GHz and the UE speed is 120km/h (the maximum absolute vehicle speed may be 240km/h), the normalized Doppler frequency defined as fd*TU where fd is the maximum Doppler frequency and TU is the useful symbol duration (TU=66.67 sec when subcarrier spacing is 15 kHz) will be up to 8.8% when subcarrier spacing is 15 kHz. And the normalized Doppler frequency which is up to about 0.1 will significantly degrade system performance, as shown in section 2.2 in [14].

From above-mentioned analysis, subcarrier spacing of 15kHz is not appropriate to be adopted for the case that carrier frequency is 6GHz and the UE speed is 120km/h because  is 8.8%. Similarly, subcarrier spacing of 15kHz and 30kHz are not appropriate to be adopted for the case that carrier frequency is 6GHz and the UE speed is 250km/h because is 18.4% and 9.2%, respectively. Besides, subcarrier spacing of 60kHz is also not appropriate to be adopted for the case that carrier frequency is 30GHz and the UE speed is 120km/h because  is 11%. Therefore, adoption of larger subcarrier spacing should be considered in the cases of high UE speed & high carrier frequency to alleviate inter-carrier interference due to Doppler effect for NR V2X.

Proposal 10: In the design of S-SSB, adoption of larger subcarrier spacing should be considered in the cases of high UE speed & high carrier frequency to alleviate inter-carrier interference due to Doppler effect for NR V2X.

2.3 Considerations on the Sidelink Synchronization Issues due to Different SCS

Compare to LTE, support of multiple numerologies is one of the key features for NR. In order to meet the low latency requirements for NR V2X, multiple SCS will be supported for NR V2X. And the SCS related agreements on the physical channel for NR V2X are shown in [2] as follows,

Agreements:

- RAN1 to continue study on the physical channel considering at least the following aspects:

- Subcarrier spacing

  - Candidates for further study are:

    - FR1: 15 kHz, 30 kHz, 60 kHz, 120 kHz

    - FR2: 30 kHz, 60 kHz, 120 kHz, 240 kHz

  - Companies are encouraged to consider the potential issues and benefit of introducing new subcarrier spacing.

Besides, in RAN1#95 meeting, subcarrier spacing of 15, 30, 60 kHz for carrier frequency of 6GHz in FR1 and subcarrier spacing of 60, 120 kHz for carrier frequency of 30GHz in FR2 are agreed in the evaluation assumptions for sidelink synchronization LLS.

From above agreements, multiple SCS may be supported for NR V2X SLSS/PSBCH transmission. Hence, there will be a case that NR V2X UEs located in different cells transmit SLSS/PSBCH with different subcarrier spacing, or in-coverage NR V2X UEs and out-of-coverage NR V2X UEs transmit SLSS/PSBCH with different subcarrier spacing. In this case, a new synchronization mechanism should be considered for NR V2X.

Proposal 11: For a case that NR V2X UEs located in different cells transmit SLSS/PSBCH with different subcarrier spacing, or in-coverage NR V2X UEs and out-of-coverage NR V2X UEs transmit SLSS/PSBCH with different subcarrier spacing, a new synchronization mechanism should be considered for NR V2X. The details are FFS.

- Conclusion

In this contribution, we discuss sidelink synchronization for NR V2X. Based on the analysis, we have the following observations and proposals:

Proposal 1: The GNSS–based synchronization in the synchronization source priority table should be adopted if the UE detects no gNB/eNB in a carrier.

Proposal 2: The gNB/eNB-based synchronization or the GNSS-based synchronization in the synchronization source priority table should be adopted by gNB/eNB pre-configuration if the UE detects a gNB/eNB in a carrier.

Proposal 3: There will be a timing misalignment issue with prioritization among references of the same priority, and SSIDs used for each priority can mitigate the timing misalignment issue. The mapping between SSIDs and priorities should be pre-configured, where gNB should assign an SSID to a mode 1 NR V2X UE, and a mode 2 NR V2X UE should randomly select an SSID for itself.

Proposal 4: In order to overcome timing misalignment issue between eNB and gNB, some coordination mechanisms between eNB and gNB are needed to coordinate the reference timings between eNB and gNB if the eNB and gNB have different reference timings but have a same synchronization source priority.

Proposal 5: Sidelink synchronization signal forwarding should be considered to extend coverage for NR V2X, where the details are FFS.

Observation 1: LTE-PSS only with some specific conjugated root pairs have good PAPR and raw CM properties.

Observation 2: NR-PSS with all cyclic shifts have good PAPR and raw CM properties.

Observation 3: The autocorrelation property of LTE-PSS with all roots are sensitive to CFOs, while the autocorrelation property of NR-PSS with all cyclic shifts are robust to CFOs when the CFOs are less than 0.5 SCS.

Proposal 6: The sequence types and length for NR-PSS and NR-SSS are agreed to be reused for S-PSS and S-SSS in NR V2X. However, some parameters, including the values of cyclic shift, should be revised for S-PSS and S-SSS in order to meet the purposes and requirements for NR V2X, where the details are FFS.

Proposal 7: The content of PSBCH for NR V2X can reuse that in PBCH for 5G NR, and the details are FFS.

Proposal 8: We confirm the working assumption that same sequence is used for both symbols of S-PSS and same sequence is used for both symbols of S-SSS for the NR SLSS.

Proposal 9: Different power should be per-configured for S-PSS and S-SSS symbols when the PAPR values of S-PSS symbol and S-SSS symbol are different, and the details are FFS.

Proposal 10: In the design of S-SSB, adoption of larger subcarrier spacing should be considered in the cases of high UE speed & high carrier frequency to alleviate inter-carrier interference due to Doppler effect for NR V2X.

Proposal 11: For a case that NR V2X UEs located in different cells transmit SLSS/PSBCH with different subcarrier spacing, or in-coverage NR V2X UEs and out-of-coverage NR V2X UEs transmit SLSS/PSBCH with different subcarrier spacing, a new synchronization mechanism should be considered for NR V2X. The details are FFS.

4.	Reference

- RP-190766, “New WID on 5G V2X with NR sidelink”, 3GPP TSG RAN Meeting #83, Shenzhen, P. R. China, March 18 -21, 2019.

- Chairman notes, 3GPP RAN1 #94, Aug. 20 - 24, 2018

- Chairman notes, 3GPP RAN1 #94bis, Oct. 8 - 12, 2018

- Chairman notes, 3GPP RAN1 #95, Nov. 12 - 16, 2018

- Chairman notes, 3GPP RAN1 NR Ad-Hoc 1901, Jan. 21 - 25, 2019

- Chairman notes, 3GPP RAN1 #96, Feb. 25 - Mar. 1, 2019

- Chairman notes, 3GPP RAN1 #96bis, Apr. 8 - 12, 2019

- Chairman notes, 3GPP RAN1 #97, May 13 - 17, 2019

- Chairman notes, 3GPP RAN1 #98, Aug. 26 - 30, 2019

- Chairman notes, 3GPP RAN1 #98bis, Oct. 14 - 20, 2019

- Chairman notes, 3GPP RAN1 #86bis, Oct. 10 - 14, 2016

- Chairman notes, 3GPP RAN1 #89, May 15 - 19, 2017

- Chairman notes, 3GPP RAN1 NR Ad-Hoc#2, June 27 - 30, 2017

- R1-1905078, “On Sidelink Synchronization Issues and Mechanisms for NR V2X,” proposed by ITRI, Apr. 2019.