3GPP TR 36.276 V0.1.0 (2019-11)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

LTE-based 5G terrestrial broadcast;

Overall Description;

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Introduction	6

4.1	General	6

4.2	Use cases and requirements	7

4.3	Enhancements targeting LTE terrestrial broadcast	7

5	Architecture	8

6	Protocol aspects	8

6.1	Frame structure and numerologies	8

6.2	MBMS Transmission	9

6.3	MAC Layer	9

6.4	RLC layer	9

6.5	RRC layer	9

7	ROM aspects	10

7.1	General	10

7.2	Physical layer procedures	10

7.3	Idle mode procedures	10

7.4	RRM requirements	10

Appendix	Upper layer aspects	11

Annex <X>: Change history	12

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document provides an overview and overall description of the LTE-based 5G terrestrial broadcast comprising:

-	a service delivering Free To Air content [2];

-	a radio network comprising only MBMS-dedicated cells [3] as transmitters; and

-	Receive Only Mode (ROM) devices [4] as receivers.

Details of the radio interface protocols and procedures are specified in companion specifications of the 36 series.

This document is a 'living' document, i.e. it is permanently updated and presented to TSG-RAN meetings.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.101: "Service aspects; Service principles".

[3]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[4]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[5]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[6]	3GPP TS 36.331: "Radio Resource Control (RRC) Protocol".

[7]	3GPP TS 24.116: "Stage 3 aspects of system architecture enhancements for TV services".

[8]	3GPP TS 36.211: "E-UTRA; Physical Channels and Modulation".

[9]	3GPP TR 36.776: " Study on LTE-based 5G terrestrial broadcast".

[10]	3GPP TR 38.913: "Study on scenarios and requirements for next generation access technologies".

[11]	3GPP TR 36.440: " General aspects and principles for interfaces supporting Multimedia Broadcast Multicast Service (MBMS) within E-UTRAN".

[12]	3GPP TS 24.117: "TV service configuration Management Object (MO)".

[13]	3GPP TS 36.213: "E-UTRA; Physical layer procedures".

[14]	3GPP TS 36.304: "E-UTRA; Procedures in idle mode".

[15]	3GPP TS 36.133: "E-UTRA; Requirements for support of radio resource management".

[16]	3GPP TS 36.321: "E-UTRA; MAC protocol specification".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DRX	Discontinuous reception

eMBMS	Evolved MBMS

HPHT	High power high tower

ISD	Inter-site distance

LPLT	Low power low tower

MPMT	Medium power medium tower

NAS	Non-access startum

MBMS	Multimedia Broadcast/Multicast System

MBSFN	Multicast/Broadcast Single Frequency Network

PBCH	Physical Broadcast Channel

PDSCH	Physical Downlink Shared Channel

PSS	Primary synchronization signal

ROM	Receive only mode

RRC	Radio Resource Control

RRM	Radio resource management

SSS	Secondary synchronization signal

TV	Television

# 4	Introduction

## 4.1	General

The main aspects of the LTE-based 5G terrestrial broadcast described in this specification are:

-	radio network comprising only MBMS-dedicated cells [3] as transmitters; and

-	ROM devices [4] as receivers.

NOTE: ROM devices support only ROM service [5]. ROM service uses one of the reserved TMGI values [7].

MBMS-dedicated cells support only MBMS transmission and do not support unicast transmission. MBSFN subframes of a MBMS-dedicated cell may not have control region and can therefore be 100% allocated to MBMS. Non-MBSFN subframes, also called Cell Acquisition Subframes (CAS), which have the control region, are used for transmission of the system acquisition signals (PSS/SSS) and system information on PBCH and PDSCH. CAS are transmitted with periodicity of 40ms and use subframes with f = 15 kHz. PBCH of a MBMS-dedicated cell uses a different scrambling sequence initialization than PBCH of a non-MBMS-dedicated cell, which prevents UEs not supporting MBMS-dedicated cell from camping on it. For more information about MBMS-dedicated cell see 3GPP TS 36.300 [3].

ROM devices support MBMS transmission but do not support unicast transmission. ROM devices may not have USIM. As such, ROM devices do not support two-way signalling procedures with the network, including connection establishment procedures and security procedures. ROM devices only support the idle mode. Not all idle mode procedures are supported, as described in subclause 7.3. For more details on ROM devices see clause 7, 3GPP TS 36.300 [3] subclause 15.11, 3GPP TS 36.246 [4] Annex D and 3GPP TS 24.116 [7] clause 4.

NOTE 1:	As a matter of implementation, a cellular device can host a ROM device and a traditional UE capable of unicast. Such device is further described in 3GPP TS 36.246 [4] Annex E and called ROM device with independent unicast. The co-hosted UE is connected to a different cell from the MBMS-dedicated cell serving the co-hosted ROM device  If the co-hosted UE and ROM device share baseband resources, the co-hosted UE can use MBMSInterestIndication signalling procedure, specified in TS 36.331 [6], to inform the serving RAN about the baseband resources occupied by the co-hosted ROM device and therefore not available for unicast.

NOTE 2: There may be awareness at the application layer of the ROM device with independent unicast. How this awareness is created is outside of the scope of specifications.

## 4.2	Use cases and requirements

In Release 14, the use cases and scenarios for eMBMS services based on LTE have been expanded to include terrestrial broadcasting (the feature also referred to as "EnTV"). This included new requirements:

-	network dedicated to TV broadcast via eMBMS;

-	deployments with ISD significantly larger than a typical ISD associated with legacy cellular deployments;

-	support for ROM device.

NOTE: At the upper layers, the requirements included the support for Free to Air service [2] and for eMBMS network sharing [4].

In Release 16, gap analysis documented in TR 36.776 [9] compared the Release 14 LTE terrestrial broadcasting capabilities with the requirements for 5G dedicated broadcast networks in TR 38.913 [10]. As a result of this analysis, the following two requirements were deemed unfulfilled by Release 14 LTE eMBMS:

1.	Support for service over large geographic area, comprising networks with ISD > 100km;

2.	Support for mobility scenarios including speeds of up to 250 km/h.

In relation to the first requirement, the following three ISD are defined:

-	15 km, referred to as LPLT network with sectorized cells;

-	50 km, referred to as MPMT network with omni-directional transmitters; and

-	125 km, referred to as HPHT network with omni-directional transmitters.

The first requirement is associated with receivers with high-gain rooftop directional antennas, low mobility and a line-of-sight channel.

The second requirement was associated with receivers mounted on the cars, with omni-directional antennas.

In addition to the above two requirements, a third requirement was added related to improving the CAS reception for both large ISD and high mobility scenarios.

## 4.3	Enhancements targeting LTE terrestrial broadcast

In Release 14, the following key RAN enhancements were made to the specifications to enable LTE terrestrial broadcast:

-	MBMS-dedicated cell [3];

-	Subframes using f = 1.25 kHz [8];

-	New information blocks on PBCH and PDSCH of CAS [3], [6]:

-	MIB-MBMS with a 40ms periodicity, containing resource allocation for SIB1-MBMS on PDSCH; and

-	SIB1-MBMS, with an 80ms periodicity, containing information relevant for receiving MBMS service and, optionally, the scheduling of other system information blocks;

-	MBMSInterestIndication RRC signalling procedure (see subclause 4.1).

NOTE: 	For upper layer enhancements, see 3GPP TS 23.246 [4] Annex D and E, 3GPP TS 24.116 [7], 3GPP TS 24.117 [12] and 3GPP TS 26.346 [5] (ROM service aspects).

In Release 16, the following RAN enhancements were made to address the use cases described in subclause 4.2:

-	Transmission using f = 0.37 kHz, the cyclic prefix duration of 300us and the symbol duration of 3ms, for support of large ISD;

-	Subframes using f = 2.5 kHz for support of high mobility;

-	PDCCH enhancements:

-	CFI indication in MIB [6] to avoid the need to decode PCFICH;

-	New aggregation level 16; and

-	Repetition of PBCH within CAS to increase PBCH robustness.

Editor’s note: Further Rel.16 enhancements may be added to the list.

Editor’s note: Whether a new PDCCH aggregation level 16 or a repetition of AL8 is specified is FFS.

# 5	Architecture

The network architecture for LTE-based 5G terrestrial broadcast is described in 3GPP TS 36.300 [3] subclause 15.1.1, with the exception that only ROM reception via MBMS-dedicated cell is supported.

RAN interfaces for LTE-based 5G terrestrial broadcast are described in 3GPP TS 36.300 [3] subclause 15.1.1 and in 3GPP TS 36.440 [11], except that the counting procedure is not supported by the eNB.

User plane and control plane protocol stack for LTE-based 5G terrestrial broadcast is described in 3GPP TS 36.300 [3] subclause 15.1.2 and subclause 15.1.3, respectively.

NOTE: For upper layer architecture, see 3GPP TS 23.246 [4].

# 6	Protocol aspects

## 6.1	Frame structure and numerologies

Only frame structure type 1 is supported. All numerologies specified in 3GPP TS 36.211 [8] are supported. For subframes using f other than 0.370 kHz, the frame structure is according to Figure 6.1-1. For transmissions using f = 0.370 kHz, the frame structure is shown in Figure 6.1-2.

Figure 6.1-1: Frame structure type 1 for subframes not using f = 0.370 kHz

CAS

#1

#2

#3

…………

#13

One 1m subframe

One 3ms slot, one symbol per slot

Four radio frames = 40ms

#1

Figure 6.1-2: Frame structure type 1 for transmissions using f = 0.370 kHz

Editor’s note: FFS if additional non-MBSFN subframes are configurable for transmissions using f = 0.370 kHz.

## 6.2	MBMS Transmission

MBMS transmission is performed according to 3GPP TS 36.300 [3] subclause 15.3.3.

MCCH configuration and scheduling is performed according to 3GPP TS 36.300 [3] subclause 15.3.5 and 3GPP TS 36.331 [6] subclause 5.8.1, except that the MBMS counting configuration is not supported.

MCCH information acquisition is performed according to 3GPP TS 36.300 [3] subclause 15.3.5 and 3GPP TS 36.331 [6] subclause 5.8.2, except that only RRC_IDLE is supported.

## 6.3	MAC Layer

MAC layer supports only:

-	BCH reception for BCCH;

-	DL-SCH reception for BCCH; and

-	MCH reception for MCCH/MTCH.

BCH reception and DL-SCH reception in the MAC layer use transparent MAC [16], i.e. single MAC PDU per TTI with no headers. HARQ entity uses the dedicated broadcast HARQ process, defined in [16].

MCH reception in the MAC layer is specified in 3GPP TS 36.321 [16] subclause 5.12 and in 3GPP TS 36.300 [3] subclause 15.3.3.

## 6.4	RLC layer

BCCH uses the RLC-TM mode.

MTCH and MCCH use the RLC-UM mode. RLC operation for MTCH and MCCH is described in 3GPP TS 36.300 [3] subclause 15.3.3.

## 6.5	RRC layer

RRC layer supports only:

-	System information reception (see 3GPP TS 36.331 [6] subclause 5.2) in MBMS-dedicated cell; and

-	MBMS reception (see 3GPP TS 36.331 [6] subclause 5.8) in MBMS-dedicated cell.

For system information reception, the following applies:

-	only BCCH-BCH-Message-MBMS and BCCH-DL-SCH-Message-MBMS message class is supported;

-	acquisition of system information messages is performed according to 3GPP TS 36.331 [6] subclause 5.2.3b.

For MBMS reception, the following applies:

-	MBMS counting procedure and MBMS interest indication procedure are not supported.

# 7	ROM aspects

## 7.1	General

Since a ROM device does not support uplink transmission or two-signalling procedures, and does not comprise USIM, it cannot support all the physical layer procedures of the traditional UE. By the same token, only a subset of idle mode procedures and RRM requirements applicable to a traditional UE will be supported. The following subsections provide an overview of the physical layer and idle mode procedures and the RRM requirements applicable to a ROM device.

## 7.2	Physical layer procedures

ROM device only supports the following physical layer procedures specified in 3GPP TS 36.213 [13]:

-	Cell search;

-	Timing synchronization;

-	PDSCH procedures;

-	PDCCH assignment procedure;

-	PMCH procedures; and

-	Assumptions independent on physical channels (clause 12) related to MBMS-dedicated cell.

## 7.3	Idle mode procedures

ROM device only supports the following idle mode procedures specified in 3GPP TS 36.304 [14]:

-	Cell selection; and

-	Cell reselection.

PLMN prioritization for cell reselection is specified in 3GPP TS 36.304 [14] subclause 5.2.4.1.

NOTE: 	NAS layer PLMN selection does not apply to ROM device. PLMN selection for ROM device is specified in 3GPP TS 24.116 [7].

Editor’s note: FFS is further clarifications are needed, e.g. not all the idle mode states are supported.

ROM device does not support DRX.

## 7.4	RRM requirements

ROM device only supports the following requirements specified in 3GPP TS 36.133 [15]:

-	Cell selection; and

-	Cell reselection, except for:

-	IRAT reselection;

-	paging-related requirements; and

-	CSG cell-related requirements.

# Appendix	Upper layer aspects

Pointers only

<Transparent mode>

<Service and session configuration>

<Network and service selection>

Annex <X>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-08 | RAN1#97 | R1- 1908844 |  |  |  | Skeleton TR | 0.0.1 |