3GPP TSG RAN WG1 Meeting #99	R1-1912908

Reno, USA, November 18-22, 2019

Agenda Item:	7.2.2.3

Source:	Huawei, HiSilicon

Title:	SLS evaluation of LBT for short control signaling

Document for:	Discussion and Decision

Introduction

In RAN#96b, following agreement was approved [1]

- The maximum DRS transmission window duration is 5 ms.

  - The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS.

    - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions.

  - FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values.

- Agreement from the SI is updated as shown:

Table 7.2.1.3.1-1: Channel access schemes for gNB as LBE device

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) | When the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA) | When DRS duty cycle is > 1/20, or total duration > 1 ms Cat4 with any channel access priority class value can be used |
| DRS multiplexed with unicast data | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |
| PDCCH and PDSCH | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |

In this contribution, we mainly discuss various types of LBT for NR-U DRS transmission and its impact on Wi-Fi system throughput and beacon transmission delay. This tdoc is a resubmission of R1-1910395

LBT type of DRS transmission

In RAN1#97, three options of channel access mechanism for DRS was discussed offline.

Scheme 1: Cat2 LBT is allowed before each candidate DRS starting point if the previous Cat2 LBT fails in the DRS windows. For example, , at most 20 Cat2 LBTs are allowed, if the DRS windows is 5ms with 30 kHz SCS and DRS is allowed at every half slot within the DRS transmission.

Scheme 2: Cat4 LBT with highest priority class is allowed before each candidate DRS used for DRS transmission. It is same as Wi-Fi that EDCA with access category of AC_VO is used for beacon frame according to IEEE802.11 specification. In the evaluation, post back off is adopted. According to EN 301 893, if the back off counter decrease to negative, when the channel becomes busy, the channel access engine should initiate a new back off procedure with least CW value after an idle period of 25us.

Scheme 3: A combination LBT method of Cat2 and Cat4 can be used. At most 5 Cat2 LBT is allowed in the initial part of the DRS windows. If all CAT2 LBT attempts fail, Cat4 LBT with highest priority class is used for the rest of the DRS window.

Besides, we also evaluate the following scheme.

Scheme 4:  gNB is allowed to additionally use one shot LBT within the DRS window before each candidate DRS starting points, after it finish the CAT4 LBT before DRS transmission window. An example is illustrated as

Figure 1 channel access of DRS: scheme 4

Simulation results

Impact on Wi-Fi User Perceived Throughput

Assuming DRS transmission periodicity is 20 ms, DRS transmission duration is 1 ms with 30 kHz SCS. The traffic arrival rates of 0.2/0.4/0.6 file/sec are selected such that the mean buffer occupancies (BO) of Wi-Fi coexist with Wi-Fi operators is about 20%/50%/80% corresponding to low/medium/high traffic load. If gNB acquires DL COT for unicast data with CAT4 LBT before the DRS transmission window, DRS burst is multiplexed with unicast data in the same COT. The impact of missing DRS at UE is not considered in the SLS. The system-level simulation of UPT results in 5%, 50% and 95% are shown in the figure below. The detail of simulation assumption can be found in the appendix. It can be observed that there is not obvious difference of system performance for Wi-Fi network under DRS transmission with Cat2 LBT (scheme 1), Cat4 LBT (scheme 2),  a mixture of Cat2 and Cat4 LBT with a limitation to the number of Cat2 LBT attempts within a DRS window (scheme 3) or a hybrid LBT method which allows post back off before the DRS window and Cat2 LBT within the DRS window before each SSB position (scheme 4).

Observation 1: There is not an obvious difference of system performance for Wi-Fi network in coexistence scenario under DRS transmission with Cat2 LBT, Cat4 LBT, or a mixture of Cat2 and Cat4 LBT with a limitation to the number of Cat2 LBT attempts within a DRS window or a hybrid LBT with Cat4 LBT before a DRS window and Cat2 LBT within the window.

- Low traffic load

- Medium traffic load

- High traffic load

Figure 2.  5%, 50%, 95% throughput of Wi-Fi network in coexistence when NR-U using different LBT types for DRS transmission in low/medium/high traffic load

Impact on Wi-Fi beacon delay

The Wi-Fi beacon delay is also evaluated when coexists with NRU operator under low/medium/high traffic load. The period of Wi-Fi beacon is 100ms, the average of beacon delay is shown in the table below. It can be observed that not obvious differences of Wi-Fi delay under DRS with Cat2 LBT or Cat4 LBT.

Table 1. Mean Wi-Fi delay coexist with NRU operator under low/medium/high traffic load

| Average beacon delay(ms) | Low load | Medium load | High load |
| --- | --- | --- | --- |
| Scheme 1(DRS with Cat2) | 8.6 | 8.9 | 9.4 |
| Scheme 2(DRS with Cat4) | 8.6 | 8.7 | 9.0 |

To evaluate the impact on Wi-Fi beacon delay in the dense deployment, we change the topology from 3 nodes per operator to 4 nodes per operator with a smaller room size of 40m*50m , LOS probability of open office is used to make sure all nodes can hear each other. Full buffer traffic is loaded in the Wi-Fi operator. The other SLS configurations are same as usual. The Wi-Fi beacon delay are compared under scheme 1 and scheme 2. From the simulation results, we observe that the latency of Wi-Fi beacon when NRU DRS uses scheme 2 increase only by 0.5% to 10.7% compared with scheme1.

Table 2. CDF of % increase in delay of Wi-Fi beacon

| CDF of % increase in delay | 5% | 25% | 50% | 75% | 95% |
| --- | --- | --- | --- | --- | --- |
| Percent of value | 10.7% | 4.5% | 2.8% | 0.7% | 0.5% |

Observation 2: Cat2 LBT for NRU DRS transmission will neither lead to Wi-Fi system throughput loss nor beacon delay increasing even in dense deployment.

Conclusions

In this contribution, we mainly discussed different LBT types for NR-U DRS transmission and its impact on Wi-Fi system throughput. Based on the discussion and evaluations, we have made the following observations:

Observation 1: There is not an obvious difference of system performance for Wi-Fi network in coexistence scenario under DRS transmission with Cat2 LBT, Cat4 LBT, or a mixture of Cat2 and Cat4 LBT with a limitation to the number of Cat2 LBT attempts within a DRS window or a hybrid LBT with Cat4 LBT before a DRS window and Cat2 LBT within the window.

Observation 2: Cat2 LBT for NRU DRS transmission will neither lead to Wi-Fi system throughput loss nor beacon delay increasing even in dense deployment.

References

Draft Report of 3GPP TSG RAN WG1 #96b v0.1.0

TR 38.889 V16.0.0

- [3] IEEE 802.11-2016

- Appendix I: system level simulation assumptions

Table A: Summary of simulation assumptions for NR-U

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Layout for nodes | Layout dimensions: 120mx80m a=20 meters, b=40 meters, c=20 meters, and d=40 meters |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline |
| Number of carriers | 1 |
| Number of users per operator | Exactly 5 per gNB per 20MHz |
| SCS | 60KHz 30KHz(DRS) |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| CCA-ED | -72dBm for NRU |
| UE receiver | MMSE-IRC |
| TXOP | 8ms for NRU and WiFi |
| MCS | NR MCS with 256QAM (LDPC) |
| MIMO | TM9 with one layer |
| UE Processing Time Capability | #1 |
| Link adaptation | CQI feedback + OLLA |
| Scheduling | Proportional Fairness |
| DMTC duration | 5ms, with 0.25ms shift for DRS |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | DL and UL |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

Table B: Summary of simulation assumptions for 802.11ac

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| MCS |  |  | 802.11ac MCS with 256QAM |
| Carrier Frequency |  |  | 5GHz |
| Carrier Channel Bandwidth |  |  | 20MHz |
| Number of carriers |  |  | 1 |
| Number of users per operator |  |  | Exactly 5 per gNB per 20MHz |
| MCOT |  |  | 8 ms |
| Frame aggregation |  |  | A-MPDU |
| MPDU size |  |  | 1500B MSDU + 14 B header |
| Guard interval |  | 11ac | 0.8us |
| Channel Model |  |  | NR UMi street canyon |
| AP Tx Power |  |  | 23dBm |
| STA Tx Power |  |  | 18dBm |
| Beacon Period |  |  | 100ms |
| MAC | Coordination |  | DCF |
|  | SIFS/DIFS |  | 16us/43us |
|  | Detection |  | Energy detection & preamble detection |
|  | RTS/CTS |  | No |
|  | Contention |  | EDCA |
| CCA-PD |  |  | -82dbm for 11ac |
| CCA-ED |  |  | -62dbm for 11ac |
| ACK Modeled |  |  | Immediate ACK |
| OFDM symbol length |  |  | 4 us |

- Appendix II: system level simulation results

Table 1: Throughput of Wi-Fi system in coexist with NR-U with different LBT types for NR-U DRS transmission in low traffic load

| Reported parameters |  | DRS windows = 5ms |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in coex (Case 1) | Wi-Fi in coex (Case 2) | Wi-Fi in coex (Case 3) | Wi-Fi in coex (Case 4) |
| DL UPT CDF [Mbps] | 5% | 26.04 | 26.19 | 26.05 | 25.23 |
|  | 50% | 67.56 | 66.78 | 66.92 | 67.16 |
|  | 95% | 76.94 | 76.91 | 76.87 | 76.81 |
|  | Mean | 59.21 | 59.07 | 58.77 | 59.09 |
| DL Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 | 0.05 |
|  | 50% | 0.06 | 0.06 | 0.06 | 0.06 |
|  | 95% | 0.15 | 0.15 | 0.15 | 0.16 |
|  | Mean | 0.08 | 0.08 | 0.08 | 0.08 |
| UL UPT CDF [Mbps] | 5% | 22.69 | 24.18 | 23.65 | 23.44 |
|  | 50% | 60.36 | 60.12 | 59.30 | 59.97 |
|  | 95% | 76.43 | 76.56 | 76.43 | 76.43 |
|  | Mean | 55.37 | 55.48 | 55.38 | 55.35 |
| UL Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 | 0.05 |
|  | 50% | 0.06 | 0.07 | 0.07 | 0.07 |
|  | 95% | 0.16 | 0.16 | 0.15 | 0.16 |
|  | Mean | 0.08 | 0.08 | 0.08 | 0.08 |
| ρDL |  | 99.47% | 99.44% | 99.44% | 99.44% |
| ρUL |  | 99.10% | 99.14% | 99.12% | 99.13% |
| Mean BO |  | 13.72% | 13.70% | 13.77% | 13.69% |
| λ |  | 0.2 |  |  |  |

Table 2: Throughput of Wi-Fi system in coexist with NR-U with different LBT types for NR-U DRS transmission in medium traffic load

| Reported parameters |  | DRS windows = 5ms |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in coex (Case 1) | Wi-Fi in coex (Case 2) | Wi-Fi in coex (Case 3) | Wi-Fi in coex (Case 4) |
| DL UPT CDF [Mbps] | 5% | 13.15 | 12.90 | 12.77 | 12.77 |
|  | 50% | 44.65 | 44.55 | 44.32 | 44.91 |
|  | 95% | 78.45 | 78.29 | 78.63 | 78.26 |
|  | Mean | 46.05 | 45.82 | 45.98 | 46.12 |
| DL Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 | 0.05 |
|  | 50% | 0.09 | 0.09 | 0.09 | 0.09 |
|  | 95% | 0.30 | 0.29 | 0.30 | 0.30 |
|  | Mean | 0.12 | 0.12 | 0.12 | 0.12 |
| UL UPT CDF [Mbps] | 5% | 8.96 | 9.12 | 9.55 | 8.46 |
|  | 50% | 38.15 | 38.29 | 39.21 | 37.97 |
|  | 95% | 78.14 | 78.17 | 78.18 | 78.12 |
|  | Mean | 41.10 | 41.17 | 41.71 | 40.71 |
| UL Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 | 0.05 |
|  | 50% | 0.10 | 0.10 | 0.10 | 0.10 |
|  | 95% | 0.42 | 0.42 | 0.41 | 0.45 |
|  | Mean | 0.15 | 0.16 | 0.15 | 0.16 |
| ρDL |  | 98.90% | 98.79% | 98.83% | 98.96% |
| ρUL |  | 99.06% | 99.13% | 99.12% | 99.06% |
| Mean BO |  | 36.3% | 36.7% | 36.0% | 36.5% |
| λ |  | 0.4 |  |  |  |

Table 3: Throughput of Wi-Fi system in coexist with NR-U with different LBT types for NR-U DRS transmission in high traffic load

| Reported parameters |  | DRS windows = 5ms |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in coex (Case 1) | Wi-Fi in coex (Case 2) | Wi-Fi in coex (Case 3) | Wi-Fi in coex (Case 4) |
| DL UPT CDF [Mbps] | 5% | 6.63 | 7.19 | 7.05 | 6.81 |
|  | 50% | 30.50 | 31.73 | 32.28 | 30.42 |
|  | 95% | 73.71 | 73.70 | 74.13 | 73.81 |
|  | Mean | 34.36 | 35.30 | 35.73 | 34.92 |
| DL Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 | 0.05 |
|  | 50% | 0.13 | 0.12 | 0.12 | 0.13 |
|  | 95% | 0.58 | 0.53 | 0.52 | 0.55 |
|  | Mean | 0.19 | 0.18 | 0.18 | 0.19 |
| UL UPT CDF [Mbps] | 5% | 4.64 | 4.83 | 5.21 | 4.81 |
|  | 50% | 24.61 | 25.69 | 25.55 | 25.68 |
|  | 95% | 69.78 | 71.21 | 71.72 | 71.47 |
|  | Mean | 28.80 | 29.80 | 29.74 | 29.67 |
| UL Delay CDF [s] | 5% | 0.06 | 0.06 | 0.06 | 0.05 |
|  | 50% | 0.16 | 0.15 | 0.15 | 0.15 |
|  | 95% | 0.75 | 0.76 | 0.66 | 0.70 |
|  | Mean | 0.25 | 0.24 | 0.23 | 0.24 |
| ρDL |  | 98.63% | 98.74% | 98.50% | 98.31% |
| ρUL |  | 97.77% | 97.84% | 97.89% | 98.16% |
| Mean BO |  | 60.47% | 60.48% | 59.48% | 60.46% |
| λ |  | 0.6 |  |  |  |