3GPP TSG RAN WG1 #99		R1-1912914

Reno, USA, November 18–22, 2019

Title:	[DRAFT] Draft Reply LS on SFN LSB indication in msg2/msgB

Response to:	R1-1911584 (R2-1914064)

Release:	Rel-16

Work Item:	NR_unlic-Core, NR_2step_RACH-Core

Source:	Huawei, HiSilicon [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name:	Yi Wang

#### E-mail Address:	wangyi6@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN1 would like to thank RAN2 for the LS regarding agreements related to SFN LSB indication in msg2/msgB on the unlicensed carrier. Regarding the inclusion of 2 LSBs of SFN in DCI used for scheduling of msg2 in 4-step RACH and msgB in 2-step RACH on the unlicensed carrier, RAN1 has the following considerations:

- If the PDCCH scheduling msg2/msgB is addressed to C-RNTI, there is no need to indicate SFN in DCI.

- If the PDCCH scheduling msg2 is addressed to RA-RNTI, SFN indication in DCI can be feasible as there are 16 reserved bits in DCI; If the PDCCH scheduling msgB is addressed to msgB-RNTI, it is not yet clear whether SFN indication in DCI can be feasible, as msgB PDCCH needs to be used for scheduling successRAR and/or fallbackRAR, and may be used for the indication of PUCCH resource for HARQ-ACK feedback of msgB, for which case the DCI content is under discussion.

- From RAN1 point of view, if the RAR windows have overlap, the SFN LSBs can be indicated in msg2/msgB payload, and RA responses of more than one SFNs can be multiplexed in the same message. As there will be less PDCCH/PDSCH transmissions, it can be more beneficial than SFN indication in DCI, in terms of power consumption and signaling overhead.

2. Actions to RAN2:

ACTIONS:

RAN1 respectfully asks RAN2 to take the above into account for the work on SFN LSB indication in msg2/msgB on the unlicensed carrier.

3. Dates of Upcoming TSG RAN WG1 Meetings:

TSG-RAN WG1 Meeting #100						24 – 28 February 2020		GR

TSG-RAN WG1 Meeting #100						20 – 24 April 2020			KR