3GPP TSG RAN WG1 #99		R1-1913154

Reno, USA, November 18–22, 2019

Title:	[DRAFT] Draft Reply LS on CSI-RS measurement outside DRX active time

Response to:	R1-1911808(R4-1912770)

Release:	Rel-15

Work Item:	NR_newRAT

Source:	Huawei, HiSilicon [RAN1]

To:	RAN4

Cc:

Contact Person:

#### Name:	Xiaolei TIE

#### E-mail Address:	tiexiaolei@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN1 would like to thank RAN4 for the LS on CSI-RS measurement outside DRX active time.

Regarding Question#1 from RAN4:

- Is UE expected to monitor CSI-RS resources for Radio Link Monitoring (while T310 timer is not running), Beam Failure Detection, Candidate beam detection [the set q1] (before beam failure detected and after beam failure detected) and L1-RSRP computation outside DRX active time, if configured?

RAN1 considers that UE is not expected to monitor CSI-RS for Radio Link Monitoring, Beam Failure Detection, Candidate beam detection and L1-RSRP computation outside DRX active time.

Regarding the Question#2 from RAN4:

- Will the CSI-RS resources for Radio Link Monitoring, Beam Failure Detection, Candidate Beam Detection [the set q1] and L1-RSRP Computation be always available outside DRX active time, if configured?

RAN1 considers that UE may assume that the configured CSI-RS is transmitted outside DRX active time.

2. Actions to RAN4:

ACTIONS:

RAN1 respectfully asks RAN4 to take the above replies into account.

3. Dates of Upcoming TSG RAN WG1 Meetings:

TSG-RAN WG1 Meeting #100						24 – 28 February 2020		GR

TSG-RAN WG1 Meeting #100bis						20 – 24 April 2020			KR