3GPP TSG-RAN WG1 Meeting #99	R1-1913362

# Reno, Nevada, U.S.A Nov 18th  – 22nd , 2019

| CR-Form-v11.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | V15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On TRS muting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | On a carrier smaller than 52 RB, TRS need to span across whole BWP. In addition, R15 UEs support only BWP sizes corresponding to nominal channel BW (CBW), e.g. 5MHz and 10MHz. An operator may not deploy R15 NR with reduced BW between 5MHz and 10MHz due to necessesity to transmit TRS within whole 10MHz BWP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce TRS muting when a subset of TRS RBs is overlapping with RBs indicated in RateMatchPattern and certain conditions are met. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An operator may not deploy R15 NR with BW between 5MHz and 10MHz due to necessesity to transmit TRS within whole 10MHz BWP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 5.1.6.1.1	CSI-RS for tracking

A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.

For a NZP-CSI-RS-ResourceSet configured with the higher layer parameter trs-Info, the UE shall assume the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same. For frequency range 1, the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. For frequency range 2 the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic CSI-RS resources in one slot or with a NZP-CSI-RS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot.

A UE configured with NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-Info may have the CSI-RS resources configured as:

-	Periodic, with the CSI-RS resources in the NZP-CSI-RS-ResourceSet configured with same periodicity, bandwidth and subcarrier location

-	Periodic CSI-RS resource in one set and aperiodic CSI-RS resources in a second set, with the aperiodic CSI-RS and periodic CSI-RS resource having the same bandwidth (with same RB location) and the aperiodic CSI-RS being 'QCL-Type-A' and 'QCL-TypeD', where applicable, with the periodic CSI-RS resources. For frequency range 2, the UE does not expect that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is smaller than the UE reported ThresholdSched-Offset. The UE shall expect that the periodic CSI-RS resource set and aperiodic CSI-RS resource set are configured with the same number of CSI-RS resources and with the same number of CSI-RS resources in a slot. For the aperiodic CSI-RS resource set if triggered, and if the associated periodic CSI-RS resource set is configured with four periodic CSI-RS resources with two consecutive slots with two periodic CSI-RS resources in each slot, the higher layer parameter aperiodicTriggeringOffset indicates the triggering offset for the first slot for the first two CSI-RS resources in the set.

A UE does not expect to be configured with a CSI-ReportConfig that is linked to a CSI-ResourceConfig containing an NZP-CSI-RS-ResourceSet configured with trs-Info and with the CSI-ReportConfig configured with the higher layer parameter timeRestrictionForChannelMeasurements set to 'configured'.

A UE does not expect to be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to other than 'none' for aperiodic NZP CSI-RS resource set configured with trs-Info.

A UE does not expect to be configured with a CSI-ReportConfig for periodic NZP CSI-RS resource set configured with trs-Info.

A UE does not expect to be configured with a NZP-CSI-RS-ResourceSet configured both with trs-Info and repetition.

Each CSI-RS resource, defined in Subclause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter NZP-CSI-RS-Resource with the following restrictions:

-	the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), as defined by higher layer parameter CSI-RS-resourceMapping, is given by one of

-	, , or for frequency range 1 and frequency range 2,

-	, , , , ,  or  for frequency range 2.

-	a single port CSI-RS resource with density  given by Table 7.4.1.5.3-1 from [4, TS 38.211] and higher layer parameter density configured by CSI-RS-ResourceMapping.

-	the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks

-	the UE is not expected to be configured with the periodicity of  slots if the bandwidth of CSI-RS resource is larger than 52 resource blocks.

-	the periodicity and slot offset for periodic NZP CSI-RS resources, as given by the higher layer parameter periodicityAndOffset configured by NZP-CSI-RS-Resource, is one of slots where 10, 20, 40, or 80 and where µ is defined in Subclause 4.3 of [4, TS 38.211].

-	same powerControlOffset and powerControlOffsetSS given by NZP-CSI-RS-Resource value across all resources.

For a NZP-CSI-RS-ResourceSet configured with the higher layer parameter trs-Info on a carrier Ngridsize,μ≤52,

- when a contiguous subset of the resource blocks of NZP CSI-RS resources overlaps with resoure blocks indicated in resourceBlocks given by RateMatchPattern in ServingCellConfigCommon, bitmap configured in symbolsInResourceBlock of the RateMatchPattern indicates all the symbols as not available for PDSCH, periodicityAndPattern is not provided in RateMatchPattern, and

- when the index of the overlapping RB and symbol level resource set is not contained in rateMatchPatternGroup1 or rateMatchPatternGroup2, and

- when the subset of non-overlapping resource blocks of the NZP CSI-RS resources is contiguous, and

- when the number of non-overlapping resource blocks is at least 34,

the UE does not expect NZP CSI-RS to be present in these overlapping resource blocks.