3GPP TSG-RAN WG Meeting #99	R1-1913409

# Reno, USA, November 18th – 22nd, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1283 | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 64-QAM TBS determination for eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-15, support of 64-QAM was introduced by reusing the legacy 5-bit MCS table. Current specification, however, does not describe the TBS determination procedure for the “reserved” MCS entries for retransmissions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that, as per legacy procedure, MCS entries 29-31 indicate that the previous TBS is reused |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCS entries 29-31 have no defined procedure on how to calculate the TBS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.1.7.2	Transport block size determination

<Unchanged parts are omitted>

else if the UE supports ce-pdsch-pusch-maxBandwidth with value ≥5MHz, or if the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config-r15 and the MPDCCH DCI format 6-1A is mapped onto the UE specific search space and the repetition number field in the DCI indicates PDSCH repetition level 1,

-	for, the TBS is assumed to be as determined from DCI transported in the latest MPDCCH for the same transport block using . If there is no MPDCCH for the same transport block using, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment MPDCCH.

-	forthe UE shall first determine the TBS index () usingand Table 7.1.7.1-1

-	if the UE is configured with higher layer parameter ce-pdsch-maxBandwidth-config with value 5MHz or if the UE is configured with higher layer parameter pdsch-MaxBandwidth-SC-MTCH with value 24 PRBs

-	For CEModeA,

-	if the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config-r15 and the MPDCCH DCI format 6-1A is mapped onto the UE specific search space and the repetition number field in the DCI indicates PDSCH repetition level 1,

-	set  to the TBS determined by the procedure in Subclause 7.1.7.2.1,

-

-	otherwise, TBS is determined by the procedure in Subclause 7.1.7.2.8 for

-	For CEModeB, TBS is determined by the procedure in Subclause 7.1.7.2.8 for

-	if the UE is configured with higher layer parameter ce-pdsch-maxBandwidth-config with value > 5MHz

-	For CEModeA,

-	if the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config-r15 and the MPDCCH DCI format 6-1A is mapped onto the UE specific search space and the repetition number field in the DCI indicates PDSCH repetition level 1,

-	set  to the TBS determined by the procedure in Subclause 7.1.7.2.1,

-

-	otherwise, TBS is determined by the procedure in Subclause 7.1.7.2.1 for

-	For CEModeB, TBS is determined by the procedure in Subclause 7.1.7.2.1 for

-	otherwise,

-	if the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config-r15 and the MPDCCH DCI format 6-1A is mapped onto the UE specific search space and the repetition number field in the DCI indicates PDSCH repetition level 1,

-	set  to the TBS determined by the procedure in Subclause 7.1.7.2.1,

-	TBS=minTBS',1000

-	otherwise

-	TBS is determined by the procedure in Subclause 7.1.7.2.1

<Unchanged parts are omitted>